| DSSAD Guidance | ADS text | Comments |
| --- | --- | --- |
| (New requirement) | 4.2.8. The Regulation includes requirements for Data Storage System for Automated Driving (DSSAD), which refers to the data storage capability of a vehicle to monitor the safety performance of ADS. This system contributes to the evaluation of ADS performance and supports the identification of safety-relevant behaviour during vehicle operation. | The general requirements in the ADS Regulation have been significantly simplified. Accordingly, please confirm with the DSSAD members whether this text can be deleted. |
| (New requirement) | 5.3.3. Data Storage System for Automated Driving 5.3.3.1. The ADS vehicle shall be equipped with a DSSAD capable of monitoring the safety performance of the ADS in accordance with the provisions of this Regulation. |  |
| 6.3.1.12.[The manufacturer shall describe the following aspects of the data storage system:] (a) [Storage location and crash survivability,] (b) [Data recorded during vehicle operation and occurrences,] (c) [Data security and protection against unauthorized access or use, and] (d) [Means and tools to carry out authorized access to data.] | 6.3.1.12. Data Storage System for Automated Driving 6.3.1.12.1. In accordance with Annex 7, the manufacturer shall describe the DSSAD installed on the ADS vehicle, including: (a) Capability to record time-stamped data, (b) Capability to record time-series data, (c) List of recordable data elements, (d) Means for enabling access to stored data, and (e) Means for protecting data against unauthorized access and manipulation. 6.3.1.12.2. The manufacturer shall justify the use of data elements provided by an alternative format listed in Annex 7. |  |
| (New requirement) | 6.3.2.10. Data Storage System for Automated Driving 6.3.2.10.1The manufacturer shall provide evidence demonstrating the following: (a) Recording of the data elements listed under 6.3.1.12.1.(c), (b) Storage of recorded data in accordance with Annex 7. |  |
| (New requirement) | 7.3.2. Assessment of the DSSAD 7.3.2.1 The documentation provided under paragraph 6.3.1.12. shall be verified for consistency with the provisions of Annex 7. |  |

| DSSAD Guidance | ADS text | Comments |
| --- | --- | --- |
| 1. Introduction | 1. Purpose |  |
| DSSAD refers to a capability of a vehicle to monitor the performance of its Automated Driving System (ADS). This document provides recommendations to enable evaluations of ADS performance. | 1.1 This annex provides DSSAD specifications in accordance with paragraphs 6.3.1.1., 7.3.1.13., 8.3.1.6., and 8.3.3.1. The manufacturer shall address these specifications in its description of the DSSAD installed on the ADS in accordance with paragraph 7.3.1.13. |  |
| This document has been prepared to support WP.29 deliberations under the 1958, 1997, and 1998 Agreements. | (Deleted) |  |
| 2. Terms and Definitions | (Deleted. Already described in other chapters.) 3. Definitions |  |
| This section defines terms used in this document. Use of these terms and their definitions is recommended in the development of legal requirements related to ADS and ADS vehicles. | (Deleted) |  |
| Source: WP.29 GRVA Guidelines and recommendations for Automated Driving System safety requirements, assessments, and test methods to inform regulatory development. | (Deleted) |  |
| 2.1 Automated Driving System (ADS) means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis.1 | (Deleted. Already described in other chapters.) 3.1. “Automated Driving System (ADS)” means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis. |  |
| 2.2 (ADS) Function means an ADS hardware and software capability designed to perform a specific portion of the DDT. | (Deleted. Already described in other chapters.) 3.5. “(ADS) function” means an ADS hardware and software capability designed to perform a specific portion of the DDT. |  |
| 2.3 ADS feature means an ADS designed specifically for use within an Operational Design Domain (ODD). | (Deleted. Already described in other chapters.) 3.6. “(ADS) feature” means an application of an ADS designed specifically for use within an Operational Design Domain (ODD). |  |
| 2.4 ADS vehicle means a vehicle equipped with an ADS. | (Deleted. Already described in other chapters.) 3.2. “ADS vehicle” means a vehicle equipped with an ADS. |  |
| 2.5 Driver means a human user who performs in real time part or all of the DDT and/or DDT fallback for a particular vehicle. | (Deleted. Already described in other chapters.) 3.9.2. “Driver” means a user who performs in real time part or all of the DDT and/or DDT fallback for a particular vehicle. |  |
| 2.6 Dynamic Driving Task (DDT) means the real-time operational and tactical functions required to operate the vehicle. | (Deleted. Already described in other chapters.) 3.3. “Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle. |  |
| 2.6.1. When the ADS is in operation, the DDT is always performed in its entirety by the ADS which means the whole of the tactical and operational functions necessary to operate the vehicle (i.e., the ADS performs “the entire DDT” as stated in the definition of an “Automated Driving System” under paragraph 2.1.). These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control. | When the ADS is in operation, the DDT is always performed in its entirety by the ADS which means the whole of the tactical and operational functions necessary to operate the vehicle (i.e., the ADS performs “the entire DDT” as stated in the definition of an “Automated Driving System” under para. 3.2.). These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control. | This paragraph is under ADS IWG discussion. |
| 2.6.1.1. Sensing and perception include: | (Deleted. Already described in other chapters.) 3.3.1. Sensing and perception include: |  |
| - Monitoring the driving environment via object and event detection, recognition, and classification. | (a) Monitoring the driving environment via object and event detection, recognition, and classification. |  |
| - Perceiving other vehicles and road users, the roadway and its fixtures, objects in the vehicle’s driving environment and relevant environmental conditions. | (b) Perceiving other vehicles and road users, the roadway and its fixtures, objects in the vehicle’s driving environment and relevant environmental conditions. |  |
| - Sensing the ODD boundaries, if any, of the ADS feature. | (c) Sensing the ODD boundaries, if any, of the ADS feature. |  |
| - Positional awareness. | (d) Positional awareness. |  |
| 2.6.1.2 Planning and decision include: | (Deleted. Already described in other chapters.) 3.3.2. Planning and decision include: |  |
| - Predicting actions of other road users. | (a) Predicting actions of other road users. |  |
| - Response preparation. | (b) Response preparation. |  |
| - Maneuver planning. | (c) Manoeuvre planning. |  |
| 2.6.1.3 Control includes: · | (Deleted. Already described in other chapters.) 3.3.3. Control includes: |  |
| - Object and event response execution | (a) Object and event response execution. |  |
| - Lateral vehicle motion control | (b) Lateral vehicle motion control. |  |
| - Longitudinal vehicle motion control. | (c) Longitudinal vehicle motion control. |  |
| - Enhancing conspicuity via lighting and signaling. | (d) Enhancing conspicuity via lighting and signalling. |  |
| 2.7 ADS fallback response means a system-initiated deactivation of the ADS or an ADS-controlled procedure to place the vehicle in a minimal risk condition. | (Deleted. Already described in other chapters.) 3.15.1. ADS fallback response” means a system-initiated deactivation of an ADS feature or an ADS-controlled procedure to place the vehicle in a mitigated risk condition (MRC). | This paragraph is under ADS IWG discussion. |
| 2.8 Fallback user means a user designated to perform the DDT pursuant to an ADS fallback response. | (Deleted. Already described in other chapters.) 3.14.3. “Fallback user” means a user designated to perform the DDT pursuant to an ADS fallback response. |  |
| 2.9 Minimal Risk Condition (MRC) means a stable and stopped state of the vehicle that reduces the risk of a crash. | (Deleted. Already described in other chapters.) 3.17. “Mitigated Risk Condition (MRC)” means a stable and stopped state of the vehicle that reduces the risk of a crash. |  |
| 2.10 Operational Design Domain (ODD) means the operating conditions under which an ADS feature is specifically designed to function. | (Deleted. Already described in other chapters.) 3.12. “Operational Design Domain (ODD)” means the operating conditions under which an ADS feature is specifically designed to function. |  |
| 2.11 System-initiated deactivation of the ADS means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle user. | (Deleted. Already described in other chapters.) 3.15.2. “System-initiated deactivation of the ADS” means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle fallback user. |  |
| 2.12 User initiated deactivation of the ADS means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user. | (Deleted. Already described in other chapters.) 3.15.3. “User-initiated deactivation of the ADS” means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user. |  |
| 2.13. (ADS) User means a human user of an ADS vehicle. | (Deleted. Already described in other chapters.) 3.14. “ADS user” means a human user of an ADS vehicle. |  |
| 2.14 Failure means the termination of an intended behaviour of an element (system, component, software) or an item (system or combination of systems that implement a function of a vehicle) | (Deleted. Already described in other chapters.) 3.20. "Failure" means the termination of an intended behaviour of an element or an item. |  |
| 2.15 (DSSAD) Triggering Event means a time stamped data element which triggers the recording and storing of time series data elements |  | Add to the definitions chapter in paragraph 3. |
| 2.16. “Emergency manoeuvre” is a manoeuvre performed by the system in case of an event in which the vehicle is at imminent collision risk and has the purpose of avoiding or mitigating a collision. |  | Add to the definitions chapter in paragraph 3. |
| 2.17. “Imminent collision risk” describes a situation or an event which leads to a collision of the vehicle with another road user or an obstacle which cannot be avoided by a braking demand with lower than 5 m/s2. |  | Add to the definitions chapter in paragraph 3. |
| 2.18. “Data Storage System for Automated Driving (DSSAD)” means the capability to record and store data concerning the safety performance of a vehicle’s ADS. |  | Add to the definitions chapter |
| 3. Data Storage and Security | 2. (The same text) |  |
| 3.1. The data storage system should be sufficient to record and store the DSSAD time stamped data elements and time series data elements listed in Paragraph 6. | 2.1. The DSSAD shall be capable of recording and storing time-stamped and time-series data elements as defined in paragraph 5 of this annex. |  |
| 3.2. The DSSAD shall be protected against both unauthorized access and manipulation. | 2.2. (The same text) |  |
| 3.3. Data elements under paragraph 6 that may be stored off-board the vehicle should remain stored on the vehicle until the data has been successfully uploaded to an off-board storage facility. | 2.3 Data elements under paragraph 5 of this annex that may be stored off-board the vehicle shall remain stored on the vehicle until the data has been successfully uploaded to an off-board storage facility. |  |
| 4. Data Format: | 3. (The same text) |  |
| 4.1. Each data element listed in Paragraph 6 shall be available in accordance with Paragraph 5. The output shall be provided in an open standard format (e.g. JSON, CSV, XML), with the exception of ‘sensor data’, and the data shall be in a readable form, aside from ‘sensor data’ and ‘visual images’. | 3.1. Each data element listed in paragraph 5 of this annex shall be available in accordance with paragraph 4 of this annex. The output shall be provided in an open standard format (e.g. JSON, CSV, XML), with the exception of ‘sensor data’, and the data shall be in a readable2 form, aside from ‘sensor data’ and ‘visual images’. |  |
| 4.2. Information required to interpret the output to correlate it with respect to the data elements required in Paragraph 6 shall be provided by the manufacturer to an authorized entity on request and subject to applicable national law(s). | 3.2. Information required to interpret the output to correlate it with respect to the data elements required in paragraph 5 of this annex shall be provided by the manufacturer to an authorized entity on request and subject to applicable national law(s). |  |
| 4.3. Time stamp data format | 3.3. (The same text) |  |
| 4.3.1. Time stamp data should be recorded in a clearly identifiable way with following data: | 3.3.1. Time stamp data shall be recorded in a clearly identifiable way with following data: |  |
| 4.3.1.1. The time stamped data element, as listed in paragraph 6.2.1. | 3.3.1.1. The time stamped data element, as listed under paragraph 5.2.1. of this annex. |  |
| 4.3.1.2. The additional information noted in 6.2 for each time stamped data element as appropriate. | 3.3.1.2. The additional information noted in the table under paragraph 5.2 for each time stamped data element as appropriate. |  |
| 4.3.1.3. Date (Resolution: yyyy/mm/dd); | 3.3.1.3. (The same text) |  |
| 4.3.1.4. Timestamp | 3.3.1.4. (The same text) |  |
| 4.3.1.4.1 Resolution: hh/mm/ss timezone e.g. 12:59:59 UTC; | 3.3.1.4.1. (The same text) |  |
| 4.3.1.4.2 Accuracy: +/- 1.0 s. | 3.3.1.4.2. (The same text) |  |
| 4.3.1.5. Location : global longitude + latitude; shall be recorded in decimal degrees and to at least five (5) decimal places, but shall be unrounded. | 3.3.1.5. (The same text) |  |
| 4.3.2. A single timestamp may be allowed for multiple elements recorded simultaneously within the time resolution of the specific data elements. If more than one element is recorded with the same timestamp, the information from the individual elements shall indicate the chronological order. | 3.3.2. (The same text) |  |
| 5. Data Accessibility | 4. (The same text) |  |
| 5.1 The stored data defined in paragraph 6 shall be readily accessible to the approval authority and authorised entities for DSSAD data collected by the vehicles in their territory of deployment. The authorised entities and the conditions of access shall be defined by the applicable laws of the territory of deployment. | 4.1 The stored data defined in paragraph 5 of this annex shall be readily accessible to the approval authority and authorised entities for DSSAD data collected by the vehicles in their territory of deployment. The authorised entities and the conditions of access shall be defined by the applicable laws of the territory of deployment. |  |
| 5.2. The DSSAD data (whether stored on or off-board the vehicle) shall be readily available and retrievable through an electronic communication interface that complies with a publicly available interface standard. It is recommended to use an internationally recognized standard. | 4.2. (The same text) |  |
| 5.3. Instructions for retrieving the DSSAD data via the electronic communication interface shall be maintained by the manufacturer.3 | 4.3. (The same text) |  |
| 5.4. The stored DSSAD data shall be retrievable even when the main onboard vehicle power supply is not available. | 4.4. The stored DSSAD data shall be retrievable even when the main onboard vehicle power supply is not available. |  |
| 5.5. The DSSAD data shall be retrievable even after an impact to the vehicle of a severity level set by relevant regulations. | 4.5. The DSSAD data shall be retrievable even after an impact to the vehicle of a severity level set by relevant regulations. |  |
| 5.6. If the DSSAD data is intended to be stored on-board the vehicle, then the following applies. | 4.6. (The same text) |  |
| 5.6.1. The data elements concerning the activation and deactivation of the feature in paragraph 6.2.1. shall be available via the vehicle’s information display/user interface where controls related to manual performance of the DDT are provided. | 4.5.1. The data elements concerning the activation and deactivation of the feature in paragraph 5.2.1. of this annex shall be available via the vehicle’s information display/user interface where controls related to manual performance of the DDT are provided. |  |
| 5.5.2. Upon request of an authorized entity, the manufacturer shall make available to them the manufacturer-specific tools, software, webservice interfaces, and/or support to retrieve the DSSAD data. | 4.6.2. Upon request of an authorized entity, the manufacturer shall make available to them the manufacturer-specific tools, software, webservice interfaces, and/or support to retrieve the DSSAD data. |  |
| 5.7. If the DSSAD data is intended to be stored off-board the vehicle, then the following applies. | 4.7. (The same text) |  |
| 5.7.1. An authorized entity shall not have to install any manufacturer specific systems or software to retrieve the DSSAD data. | 4.7.1. An authorized entity shall not have to install any manufacturer specific systems or software to retrieve the DSSAD data. |  |
| 6. Data Elements: | 5. (The same text) |  |
| 6.1. The DSSAD shall record and store the data elements listed under Paragraph 6.2. and 6.3. This requirement shall be without prejudice to applicable laws governing access to data, availability, privacy and data protection. | 5.1. The DSSAD shall record and store the data elements listed under paragraph 5.2. and 5.3. of this annex. This requirement shall be without prejudice to applicable laws governing access to data, availability, privacy and data protection. |  |
| 6.2. Data elements of time-stamp data | 5.2. Time-stamp data elements |  |
| 6.2.1. The following table details the data elements of time-stamp data to be recorded, along with any additional information and recording condition. These data elements are recorded only whilst the feature is active. | 5.2.1. (The same text) |  |

| Event | Additional Information |
| --- | --- |
| Activation of the feature | ADS feature is activated by the: - system, or - user |
| The following data elements shall be recorded if they occur while an ADS feature is active. |  |
| Deactivation of the feature | ADS feature is deactivated by the - Initiated by the system, or - Initiated by a user. |
| ODD exit |  |
| Start of ADS fallback to user, if applicable | ADS fallback to user initiated due to: |
|  | (i) Foreseen condition |
|  | (ii) Unforeseen condition |
|  | (iii) Failure |
|  | (iv) Input to the driving controls, or |
|  | (v) ODD exit. |
| Start of ADS fallback to an MRC | Fallback to an MRC initiated due to: |
|  | (i) ODD exit |
|  | (ii) ADS failure |
|  | (iii) collision detected |
|  | (iv) Detection that fallback user is not available when they have no longer met the conditions of paragraph 6.2.2.1.6/4.2.2.1.6 of this regulation (if applicable), or |
|  | (v) failure of the fallback user to take control following a system-initiated deactivation of the ADS. |
| User input to the driving controls, if applicable | Application of: |
|  | (i) brake control, |
|  | (ii) acceleration control, |
|  | (iii) steering control, or |
|  | (iv) direction indicator. |
| Application of the passenger stop request as designated in paragraph 4.2.3.1./6.2.3.1. of this regulation |  |
| Prevention of user takeover, if applicable | Prevention of user takeover (if applicable) due to: |
|  | (i) Unintentional user input, |
|  | (ii) The current situation being unsuitable, |
|  | (iii) The current situation being unsafe Or |
|  | (iv) The user not being suitably engaged. |
| Detection that fallback user is not available when they have no longer met the conditions of paragraph 6.2.2.1.6/4.2.2.1.6 of this regulation, if applicable |  |
| Start of Emergency Manoeuvre |  |
| End of Emergency Manoeuvre |  |
| Event Data Recorder (EDR) trigger input |  |
| Detected collision |  |
| MRC achieved | Indication of end states per paragraph 5.3.1.14/7.3.1.14 of this regulation |
| Detected failure that compromises the ADS capability to perform the DDT | Nature of failure in accordance with para. 7.3.1.15. |
| Remote intervention in a tactical function, if applicable. |  |

| 6.3. Time series data elements | 5.3. (The same text) |  |
| --- | --- | --- |
| 6.3.1. The data elements shall be recorded in compliance with paragraph 6.3.1 if the following thresholds are reached or conditions occur: | 5.3.1. The data elements shall be recorded in compliance with paragraph 5.3.X if the following thresholds are reached or conditions occur: |  |
| a) Detected collision | (The same text) |  |
| b) EDR trigger input (excluding last stop trigger) | (The same text) |  |
| 6.3.1.1 If there is no system or sensor designed to provide the data element to be recorded and stored under section 6.3, alternative data may be utilized if the data provides equivalent information to the specified data element. | 5.3.1.1 If there is no system or sensor designed to provide the data element to be recorded and stored under paragraph 5.3, alternative data may be utilized if the data provides equivalent information to the specified data element. |  |
| 6.3.2. The following table details the data elements of time-series data to be recorded during a triggering event. | 5.3.2. (The same text) |  |

| Data element | Condition for requirement | Recording interval/time (relative to time stamp) |
| --- | --- | --- |
| Visual images | Mandatory |  |
| Detected object distance, longitudinal | Mandatory if available |  |
| Detected object distance, lateral | Mandatory if available |  |
| Detected object relative velocity, longitudinal | Mandatory if available |  |
| Detected object relative velocity, lateral | Mandatory if available |  |
| Detected object classification | Mandatory if available |  |
| Sensor data | Mandatory if ‘Detected object elements’ are not available |  |
| ADS-requested accel demand | Mandatory |  |
| ADS-requested service braking demand | Mandatory |  |
| ADS-requested parking brake demand | Mandatory |  |
| ADS-requested steering demand | Mandatory |  |
| Vehicle acceleration, longitudinal | Mandatory |  |
| Vehicle acceleration, lateral | Mandatory |  |
| ADS-determined vehicle speed | Mandatory |  |

This definition is based on SAE J3016 and ISO/PAS 22736 (Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles). These standards define levels of driving automation based on the functionality of the driving automation system feature as determined by an allocation of roles in DDT and DDT fallback performance between that feature and the (human) user (if any). The term “Automated Driving System” is used specifically to describe a Level 3, 4, or 5 driving automation system.

Readable means that the data is of numerical values and natural language which can be understood to represent a specific data point with a value associated with it (e.g. <<speed>>35<<speed/>>, not hexadecimal or binary).

Contracting parties may further define technical specifications for data accessibility and/or availability of instructions under national law.

Excluding any last stop trigger

This data element is generally represented by a camera image; however, this image may be a construct of other sensor data if camera images are unavailable.

e.g. camera, radar, LiDAR, used by the ADS for decision making. This shall be documented in the information package provided to the Authorised Entity. This shall include a “Visual Representation“ submitted to the Authorised Entity at the time of providing the DSSAD Data and shall comply with the requirements of 4.1 and 5.4.