Proposal for supplement 6 to the 01 series of amendments to UN Regulation No. 21 (Interior fittings)

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS [feature type 2]. The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

0.1.	For supplement 6 to the 01 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles with an ADS where those vehicles are also equipped with a manual driving mode operating at a speed of more than 6 km/h it is expected that in the manual driving mode the technical requirements can be applied as they would for a conventional vehicle. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled.”

Paragraph 1., amend to read:

1. 	Scope: This Regulation applies to the interior fittings of vehicles of category M11 with regard to:

(a)1.1.	the interior parts of the passenger compartment other than the rear-view mirror or mirrors;

(b)1.2.	the arrangement of the controls;

(c)1.3.	the roof or opening roof, and

(d)1.4.	the seat-back and the rear parts of seats.

(e)1.5.	power-operation of windows, roof panels and partition systems.

Vehicles of category Y1 are not in the scope of this regulation.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 2.3., amend to read:

“2.3.	"reference zone" is the head impact zone as defined in Annex 1 to this Regulation, or at the choice of the manufacturer, according to Annex 8, excluding the following areas, if applicable:  (see Annex 10, explanatory notes, paragraphs 2.3. and 2.3.1.).

2.3.1.	the area bounded by the forward horizontal projection of a circle circumscribing the outer limits of the steering control, increased by a peripheral band 127 mm in width; this area is bounded below by the horizontal plane tangential to the lower edge of the steering control when the latter is in the position for driving straight ahead. (see annex 10, explanatory notes, paragraphs 2.3. and 2.3.1.)

2.3.2.	the part of the surface of the instrument panel comprised between the edge of the area specified in paragraph 2.3.1. above and the nearest inner sidewall of the vehicle; this part of the surface is bounded below by the horizontal plane tangential to the lower edge of the steering control and; (see annex 10, explanatory notes, paragraphs 2.3. and 2.3.1.)”

Insert new paragraphs 2.19. and 2.20., to read:

“2.19. 	“ADS feature of type 1 (ADSF-1)” means an ADS feature which includes an ADS fallback response requiring a fallback user.

2.20.	“ADS feature of type 2 (ADSF-2)” means an ADS feature which does not include an ADS fallback response requiring a fallback user.”

Paragraph 5.8.2.9., amend to read:

“5.8.2.9.	One-touch closing shall be permitted, only for the power-operated window of the driver’s door and the roof panel, and only during the time when the ignition key is in the engine running position. It is also allowed when the engine has been switched off or the ignition key/power key has been removed, or an equivalent condition has happened in case of a non mechanical device, as long as neither of the two front doors has been opened sufficiently to permit egress of occupants.

In case an ADSF-2 is active, one-touch closing shall not be permitted on the driver’s door.”

Paragraph 5.8.4.2., amend to read:

“5.8.4.2.	All rear-window, roof-panel and partition switches intended for use by occupants in the rear of the vehicle shall be capable of being switched off by a driver- controlled switch which is located forward of a vertical transverse plane passing through the R points of the front seats. The driver controlled switch is not required if a rear window, roof panel or partition is equipped with an auto-reversing device. If, however, the driver-controlled switch is present, it shall not be able to override the auto-reversing device or prevent lowering of the partition system.

The driver-controlled switch shall be located so as to minimise any accidental manipulating. It shall be identified by the symbol shown in Figure 2 of annex 9 to this Regulation or an equivalent symbol, for example according to ISO 2575:1998 reproduced in figure 3 of annex 9 to this Regulation.

If an ADSF-2 is able to operate power operated windows, roof panels or partition systems, they shall be equipped with auto-reversing devices. It shall not be possible for the ADS to override the auto-reversing device.”

Paragraph 5.8.6.1. to 5.8.6.3., amend to read:

“5.8.6.1. 	The owner’suser’s manual of the vehicle shall contain clear instructions relating to the power-operated window/roof panel/partition, including:

5.8.6.2. 	use of the driver-controlled switch (if applicable),

5.8.6.3.	a “WARNING” message indicating the dangers, particularly to children in the case of improper use/activation of the power-operated windows/roof-panel systems/partition systems. This information should indicate the responsibilities of the driver and/or ADS, including instructions for other occupants and the recommendation to leave the vehicle only if the ignition key/power key has been removed, or an equivalent condition has happened in case of a non mechanical device,”

II.	Justification

See paragraph 0. in the proposal.