|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 7. September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-seventh session

Geneva, 1–4 December 2025

Item Y of the provisional agenda

UN Regulation No. 114 (Replacement Airbag Modules)

Proposal for supplement 2 to the original version of UN Regulation No. 114 (Replacement Airbag Modules) *

Submitted by Germany on behalf of the GRSP Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Insert a new paragraph 0, to read:

"0.	Introduction

  - For supplement 2 to the original version of the Regulation:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles with an Automated Driving System (ADS)1 where those vehicles are also equipped with a manual driving mode operating at a speed of more than 6 km/h it is expected that in the manual driving mode the technical requirements can be applied as they would for a conventional vehicle. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled.”

Paragraph 1., amend to read:

1. 	SCOPE

This Regulation applies to the following aftermarket equipment:

1.1. 	to airbag modules for replacement airbag systems intended to be installed on motor vehicles to reduce the severity of injuries of the occupants;

1.2. 	to replacement steering wheels for vehicles of categories M11 and N11, equipped with an airbag module of an approved type and intended to be installed as an additional restraint system beside safety-belts and other restraint systems in power-driven vehicles, i.e. a system which, in the event of a severe impact, automatically deploys a flexible structure intended to reduce the severity of injuries of the occupants;

1.3. 	to replacement airbag systems other than those installed in steering wheels and equipped with airbag modules of an approved type as an additional restraint system beside safety-belts and other restraint systems in power-driven vehicles of categories M1 and N1, i.e. a system which, in the event of a severe impact, automatically deploys a flexible structure intended to reduce the severity of injuries of the occupants.

Paragraph 1.1, insert  new footnote 1,  to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraphs 2.6.1. and 2.6.2., amend to read:

“2.6.1. 	Category A: Device intended to protect the driver occupants of a vehicle against the steering mechanism (if fitted) in the event of a frontal collision.

2.6.2. 	Category B: Device intended to protect front seat passenger(s) other than the driver in the event of a frontal collision.”

Paragraph 5.3.4., amend to read:

" 5.3.4. 	The replacement steering wheel shall allow the drivers (if applicable) a direct view to all important instruments and indicators such as

(a) the speedometer and

(b) the tell tale signs for:

direction indicator

main beam

rear fog lamp

hazard warning signal

automatic antilock system

brake system malfunctions

airbag function indicator

The tests shall be carried out according to the prescription laid down in paragraph 6.2.2.;”

II.		Justification

See paragraph 0.

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.