| United Nations | ECE/TRANS/WP.29/XXXX |
| --- | --- |
| Economic and Social Council | Distr.: General XX XXXX 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Consolidated Resolution No. 8 (R.E.8) concerning Automotive Life Cycle Assessment (A-LCA)

The text reproduced below was prepared by the Informal Working Group on Automotive - Life Cycle Assessment (A-LCA) and was adopted on XX XXXX 202X by the World Forum for Harmonization of Vehicle Regulations (WP.29). It is based on document ECE/TRANS/WP.29/202X/XXX.

Consolidated Resolution No. 8 (R.E.8) concerning Automotive Life Cycle Assessment (A-LCA)

Table of contents

PageConsolidated Resolution No. 8 (R.E.8) concerning Automotive Life Cycle Assessment (A-LCA)	1

Consolidated Resolution No. 8 (R.E.8) concerning Automotive Life Cycle Assessment (A-LCA)	2

1	Preamble	4

2	Statement of technical rationale and justification [SG7]	5

2.1	Introduction [SG1]	5

2.2	Methodology background [SG1]	5

2.3	Existing regulations and standards [SG7/SG1]	6

2.4	Technical rationale and justification [SG1/SG7]	6

3	Text of the Mutual Resolution	8

3.1	Introduction [SG1]	8

3.2	General Methodology	8

3.2.1	Level Concept [SG1]	8

3.2.2	Product Category [SG1]	8

3.2.3	Functional Unit and Reference flow [SG4]	8

3.2.4	System boundaries [SG1]	8

3.2.5	Environmental impact category and indicator [SG1]	10

3.2.6	Data collection requirements and data types [SG1/SG3]	10

3.2.7	Cut off rule [SG1]	10

3.2.8	Energy modelling [SG6]	10

3.2.9	Allocations [SG3]	10

3.2.10	Waste treatment, disposal & Recycling modelling [SG5/SG3]	10

3.2.11	Chain of Custody [SG3]	10

3.2.12	Logistics [SG1]	10

4	Annexe: Specific methodology per life cycle stages	11

4.1	Material production stage [SG2]	11

4.1.1	Declared unit	11

4.1.2	Automotive material classification	11

4.1.3	System boundaries	11

4.1.4	Data collection and data type	11

4.1.5	Energy modelling [SG6]	11

4.2	Parts production and vehicle assembly stage [SG3]	11

4.2.1	Levels in parts production and vehicle assembly	11

4.2.2	Declared unit	11

4.2.3	System boundaries	11

4.2.4	Data collection and data types	11

4.2.5	Energy modelling [SG6]	12

4.3	Use stage [SG4]	12

4.3.1	Service life and functional unit	12

4.3.2	System boundaries	14

4.3.3	Use phase consumption	17

4.3.4	Data Collection and data type	25

4.3.5	Application of the energy modelling schemes [SG6]	27

4.3.6	Maintenance	27

4.4	End of Life [SG5]	32

4.4.1	Levels in disposal and recycling stage	32

4.4.2	System boundaries	32

4.4.3	Data collection and data types	32

4.4.4	Scenario	32

4.4.5	Energy modelling [SG6]	33

4.4.6	GHG calculation for each process	33

4.5	Interpretation [?]	33

4.6	Sensitivity Analysis [?]	33

4.7	Reporting [?]	33

4.8	Verification process [?]	33

# Preamble

1.1.	The Administrative Committee of the 1958 Agreement (AC.1) and the Executive Committee of the 1998 Agreement (AC.3),

1.2.	DESIRING to harmonise technical requirements while ensuring high levels of safety, environmental protection, energy efficiency and anti-theft performance of wheeled vehicles, equipment and parts which can be fitted and/or be used on wheeled vehicles,

1.3.	BEARING IN MIND that the 1958 Agreement established on 20 March 1958 provides for the adoption of uniform technical prescriptions for wheeled vehicles, equipment and parts which can be fitted and/or be used on wheeled vehicles and the conditions for reciprocal recognition by Contracting Parties of approvals granted on the basis of these prescriptions,

1.4.	BEARING IN MIND that the 1998 Agreement provides for the establishment of global technical regulations for wheeled vehicles, equipment and parts which can be fitted and/or be used on wheeled vehicles was opened for signature in Geneva on 25 June 1998,

1.5.	BEARING IN MIND that both Agreements facilitate the trade of wheeled vehicles, equipment and parts with harmonized performance requirements among the respective Contracting Parties,

1.6.	BEARING IN MIND that this Resolution does not hold regulatory status within Contracting Parties.

1.7.	RECOMMENDS that Contracting Parties and manufacturers refer to this Mutual Resolution when establishing studies used for the assessment of life cycle CO2 equivalent emissions in the framework of the 1958 or 1998 Agreements respectively.

# Statement of technical rationale and justification [SG7]

## Introduction [SG1]

## Methodology background [SG1]

## Existing regulations and standards [SG7/SG1]

## Technical rationale and justification [SG1/SG7]

# Text of the Mutual Resolution

## Introduction [SG1]

## General Methodology

### Level Concept [SG1]

### Product Category [SG1]

### Functional Unit and Reference flow [SG4]

The primary function of a passenger car [light duty vehicle] is to transport people from one location to another. For the present application of this Resolution, the functional unit (FU) for a passenger car [light duty vehicles] is defined as the distance travelled per passenger over vehicle lifetime.

[However, the definition of a functional unit for light-duty vehicles beyond passenger cars presents notable complexity. For vans, which are designed primarily for goods transport but often serve mixed functions such as carrying tools, equipment, or passengers, the choice of an appropriate functional unit is challenging.]

Given these considerations, considering only passenger cars [light duty vehicles] and the fact that their real-life occupancy (number of passengers per vehicle) is difficult to estimate, a more conservative approach should be considered and single passenger per vehicle should be assumed, with the presumption that vehicle technologies and energy sources will not change the passenger car [light duty vehicle] occupancy rate. This will also ensure comparability between different vehicles.

Considering the above assumption, the functional unit (FU) for a passenger car [light duty vehicles] is defined as the transportation of 1 passenger per km of distance travelled over the vehicle lifetime.

The reference flow for a vehicle is defined as the measurable quantity of inputs and outputs necessary to meet the defined functional unit of the product over its lifecycle. The reference flow in automotive LCAs translates this functional unit into specific, quantifiable measures of resources consumed and emissions produced, covering aspects such as:

- Vehicle components and materials (e.g., steel, aluminum, plastics required to manufacture the car),

- Fuel and/or energy consumption over the vehicle’s lifetime,

- Maintenance and replacement parts (such as tires, fluids, SLI batteries, etc.),

- End-of-life treatment for disposal or recycling.

The standardized reference flow enables consistent and comparable assessments across different automotive products, focusing on the impacts associated with achieving the defined service.

### System boundaries [SG1]

Life cycle stages

### Environmental impact category and indicator [SG1]

### Data collection requirements and data types [SG1/SG3]

#### Primary data quality [SG1]

#### Primary data share [SG3]

#### Secondary data quality [SG1]

### Cut off rule [SG1]

### Energy modelling [SG6]

### Allocations [SG3]

#### Waste vs Co-product

#### Multi-output allocation

### Waste treatment, disposal & Recycling modelling [SG5/SG3]

#### Material and parts recycling modeling [SG5]

#### Incineration with energy recovery [SG5]

#### Disposal [SG5]

### Chain of Custody [SG3]

### Logistics [SG1]

# Annexe: Specific methodology per life cycle stages

## Material production stage [SG2]

### Declared unit

### Automotive material classification

### System boundaries

### Data collection and data type

### Energy modelling [SG6]

## Parts production and vehicle assembly stage [SG3]

### Levels in parts production and vehicle assembly

#### Level 1

#### Level 2

#### Level 3

#### Level 4

### Declared unit

### System boundaries

### Data collection and data types

### Energy modelling [SG6]

## Use stage [SG4]

### Service life and functional unit

The vehicle lifetime (or service life) is defined as the full period of time during which a vehicle is operated and thereby contributes to GHG emissions. This lifetime is expressed in terms of kilometres (km) driven and is chosen to represent typical usage patterns. Alongside the distance travelled, the years of operation for a vehicle is defined as the assumed total number of years a vehicle remains in active use between point of sale and point of recycling. Both are relevant for the purpose of the methodology.

Vehicle lifetime mileage is required to translate vehicle production emissions (in tonnes of CO2e) into the functional unit. Vehicle lifetime in years is needed to calculate the lifetime mileage, but also to determine the period of time over which the changes in the emission intensity of the fuel and electricity will be accounted for. Lifetime duration is also needed to calculate the number of replacements/refills for certain maintenance and consumable items. As for the annual driven distance, a constant average annual mileage is assumed, based on gathered data on total vehicle activity (in kilometres) and vehicle lifetime.

#### Service life: methodology / guideline

Precisely assessing these values for each region may present further challenges, underscoring the need for flexibility and regional adaptation in their definition and application.

The actual service life and lifetime distance reference flow of a vehicle can vary significantly depending on the region and its specific applications. This variability makes it challenging to accurately estimate these values universally. Therefore, this guideline proposes to define service life for each region or country of usage.

De-registration in a country (i.e., as indicated by “survival curves”) can be significantly earlier than reaching end of life. In some countries, a significant share of vehicles is exported before reaching the end of service life. Hence, the average age when being de-registered is lower than the average age when being recycled. In general, these guidelines recommend to consider statistical evidence on the average vehicle age at the point of recycling. For the reasons discussed above, statistics on the average age at the point of de-registration in a country is not recommended, as these values exclude the potential export and continued operation of a proportion of the vehicle in other countries. Therefore, for all vehicles, the lifetime used to calculate life cycle emissions should align with the average age of vehicles at the time of recycling, considering only those registered and recycled within the same country.

Some frameworks adopt different lifetime values by vehicle segment. There are both advantages and disadvantages to offering differentiation by segments. (1) Differentiation offers a more accurate representation of the typical usage expected, which also partly highlights the greater potential utility that larger vehicles can provide. (2) However, near the boundaries between segments, it may unintentionally influence consumer decisions (for instance, a specific user is unlikely to choose a vehicle solely because it belongs to a different segment – it's typically bought for a particular use). Until further in-depth analysis is conducted, this guideline does not recommend differentiating service life by segment.

Although statistics indicate that service life varies considerably between powertrains, this may be influenced by user behaviour, as well as current economic/taxation policies, which are subject to change. Since one of the objectives of LCA is to compare different powertrains, based on the same functionality, i.e., the same use case, this guideline does not recommend assigning different service lives to different powertrains.

Moreover, vehicle annual mileage generally declines over time. Consequently, higher mileage in the early years should be assigned greater weight to account for the typically higher emissions intensity of electricity mixes during that period. Nevertheless, the guidelines do not prescribe a specific function to model the evolution of annual mileage over vehicle age; instead, a constant value may be assumed, as previously noted. Alternatively, practitioners may adopt a different approach, provided that the chosen method is clearly documented and transparently reported.

Some vehicles, such as special purpose or sports vehicles, have very low usage, leading to a significantly reduced service life. The guidelines suggest that the practitioners can indicate a shorter service life supported by primary data, which will lead to a more stringent functional unit. However, the guidelines currently do not advise declaring a longer service life than the regional default value, as this could result in a more favourable functional unit.

#### Service life values

Service life (in both km and years) will be defined by each contracting parties according to official available data on vehicle service life until point of recycling.

In the absence of the above values, the following values as defined in official databases or peer-reviewed reports (see Annex for reference) can be used as an indication of typical service life as fall-back option.

| Region / Country | Year | Duration (Years) | Source |
| --- | --- | --- | --- |
| EU-27 | 2024 | 20 | Multiple sources as in the table below |
| United Kingdom | 2022 | 18 | LSE, 2024 |
| Japan | 2022 | 16.5 | Ministry of Economy, Trade and Industry, 2023 - Status of the Enforcement of the Automobile Recycling Law |
| USA | 2003 | 18 | US DoT |
| Brazil | 2020 | 22 | Ministry of Science, Technology and Innovations of Brazil, 2020 |

### System boundaries

When defining the system boundaries, it is pivotal to highlight that the scope is to provide a comprehensive methodology for calculating realistic GHG emissions and energy consumption over vehicle use stage at various levels of detail and considering the availability of different information and datasets. Therefore, in an intermediate step this requires the calculation of energy consumption as activity data for electrified vehicles such as HEV, PHEV, FCHV and PEV. As outlined in the figure below, the use stage encompasses the operation of the vehicle itself and direct impacts from this, as well as impacts from the production and distribution of fuel/electricity, and for impacts relating to vehicle maintenance and replacement parts.

Impacts directly from the energy used in operation of the vehicle include in the CO2 equivalent calculation the tank-to-wheel (TTW) contribution are hereby addressed, hence:

- Electric energy: from vehicle charging port to the wheels, if already included in the certification protocol;

- Fuel: from tank to wheel.

Conversely, impacts resulting from the production and distribution of fuel and electricity for the operation of the vehicle, i.e. well-to-tank (WTT), shall follow the methodology outlined in Section xx.

These latter WTT GHG emissions shall be covered in the conversion factor of each energy type according to Section xx.

Use stage boundaries are depicted in the figure below:

Material production

Material

Material transport

Maintenance parts

Disposal/

Recycle

Maintenance parts & Consumables transport

Fuel production/ energy generation

Fuel/ electricity

Driving

AC Refrigerant

Fluorocarbon emissions

Material parts production

Waste transport

ELV

Maintenance parts & Consumables

Finished vehicle transport

Leakages

Leakage emissions

Processes

Input/Output

Fuel

Combustion CO2 emissions

Combustion non-CO2 GHG emissions

GHG from fuel system

Figure 4.3.2-1: Use stage Life-cycle flow schematic

As a matter of fact, the use stage covers mainly two aspects of the whole life cycle: the “In-use energy consumption and GHG emissions” (covering WTT and TTW impacts), and the “Maintenance and consumables” impacts. It is relevant to address also leakages such as emissions of methane and other hydrocarbons, together with emissions of climate active species from the vehicles (e.g. methane leakage from CNG cars) and the formation of non-CO2 GHG gasses (e.g. N2O) during the fuel combustion. More in detail, the figure below shows the system boundaries related to vehicle operation.

Driving Fuel and Energy Consumption

Finished vehicle transport

EoL vehicle transport

Maintenance & Regular Consumables

Leakages and Fluorocarbons

Figure 4.3.2-2: Vehicle operation system boundaries

The handover point from the “Parts Production and Vehicle Assembly” stage is defined at the point of delivery to the final customer, once the finished vehicle has been handed over. On the disposal and recycling side, GHG emissions after the vehicle service life fall within the EoL (End-of-Life) domain. Therefore, it is acknowledged that the transportation of the vehicle to EoL treatment facilities is included within the EoL domain.

Considering the proposed system boundaries, the CO2 equivalent emissions for the use stage comprise emissions from in-use consumption, emissions due to maintenance parts and consumables, and the contribution of leakages and fluorocarbon emissions. The following equation highlights these components:

C Use Stage=C Fuel+C Leakages+C Maintenance+C Fluorocarbon

Where,

C Use Stage 	carbon emissions for the whole use stage (kgCO2e);

C Fuel		carbon emissions due to the fuel combustion, including both CO2 and non-CO2 exhaust emissions (kgCO2e);

C Maintenance 	carbon emissions due to maintenance and consumables (kgCO2e);

C Leakages 	carbon emissions due to leakages, including unburnt GHG species emissions (kgCO2e);

C Fluorocarbon 	carbon emissions due to fluorocarbons emissions (kgCO2e);

In the following paragraphs, the methodology for assessing each term of the equation is detailed.

### Use phase consumption

Contrary to upstream activities where carbon footprint is unique for a vehicle, downstream activities are different for different region for the same vehicle. The LCA vehicle's GHG emissions are declared at the point of sale, but it is difficult to measure energy consumption after sale. As a matter of fact, for Level 1 and Level 2 analysis, projections or estimations can be made based on available information such as certification values. These should be considered as minimum requirements and do not prevent the use of more realistic or detailed data to better characterize vehicle behaviour, where such data is available. For Level 3 and Level 4, where stricter requirements are in place, please refer to guidelines in Table 4.3.3-2.

Usage of passenger cars [light duty vehicles] depends on regional customer behaviour. To provide a standard approach that ensures repeatability, comparability and verifiability by authorities, each region has defined homologation driving cycles and test conditions to be followed. In some regions, these certification values are repeatedly found to underestimate the average fuel consumption in real-life operation. In this regard, Table 4.3.3 1 provides an overview of the primary certification protocols used globally. This table serves as an illustrative example of the methodologies employed in key regions to determine fuel and energy consumption for passenger cars [light duty vehicles], depending on the powertrain type. The selection and verification of the appropriate homologation procedure remain the responsibility of each contracting party, to be determined on a case-by-case basis. As a result, using regional certification values is considered to be a good starting point to later reach an accurate representation of in-use energy consumption, compared to using globally standardized values.

The scientific community widely acknowledges that there remains a substantial disparity between certification values and actual energy or fuel consumption in real-world conditions across all types of powertrains in some regions. The gap may vary per powertrain, region, driving behaviour and other variables that have to be taken into account. However, in some regions such as Europe and Japan, real-world fuel consumption can be monitored through on-board systems. This data is compiled annually and shows the gap between certified values and actual usage for each powertrain. To address this, the guidelines propose using a "Discrepancy factor" if data are available for the region in question. If the factor is not available for a specific region, then the factor should be calculated based on best available data and expert qualified assumptions with a minimum value of ‘1’.  For vehicles that have more than one mode of operation (e.g. PHEVs working in Charge Depleting and Sustaining mode), distinct discrepancy factors should be applied to accurately characterize each operational mode.

For example, considering a ‘Level 4’ calculation (see Level Concept [SG1]), OEM-specific average data may be used to determine the “Discrepancy factor” based on analysis of data from their vehicles operating in the real-world for similar powertrains (e.g. for ICEVs, or ZEV/electric powertrains such as PEVs, FCHVs, etc.), matched to the defined region of operation (i.e. sale). [However, OEM-specific average data cannot substitute publicly available official data, such as EU OBFCM data]. Practitioners must disclose details about the fleet sample used to derive the data, including, for example, the sample size and period of collection. “Discrepancy factors” can only be quantified at the first time of vehicle registration (0 km). These factors should be defined and updated on a regular basis by the authorities.

Vehicle emissions may increase over time due to wear, component deterioration and other factors.  If emissions "Deterioration factors" for the specific modules are available for a particular region, they should be used to account for this issue. Emissions “Deterioration factors” based upon measurement and used for vehicle emissions certification and/or type approval shall be considered primary data. If a “Deterioration factor” is not available for a region, then a “Deterioration factor” should be calculated based on best available data and expert qualified assumptions with a minimum value of ‘1’. Since the deterioration effect covers vehicle aging, it is important to note that this factor may also be reflected in on-board fuel consumption monitoring data used to quantify the “Discrepancy factor”, and hence double counting shall be avoided.

As far as the mere Fuel and/or Energy consumption is considered, the in-use consumption, ECin-use shall be calculated as follows for vehicles with powertrains with only one mode of operation:

ECin-use=ECcertification×fdiscrepancy×fdeterioration

Where,

ECcertification 	energy consumption or fuel consumption (MJ/km or Wh/km);

fdiscrepancy 	discrepancy factor (if not available for a region, then 1 should be used)

fdeterioration 	deterioration factor (if not available for a region, then 1 should be used)

[ For vehicles with powertrains with two modes of operation (for example OVC-HEVs, including plug-in hybrid vehicles – PHEVs, or range-extended electric vehicles – REEVs), the in-use energy consumption shall be defined separately for each mode of operation, so that overall in-use energy consumption may be defined.  An additional formula shall be used to account also for changes to the share of operation (i.e. in mode 1 – e.g. charge depleting electric - CD, or mode 2 –charge sustaining - CS) that may result from accounting for the discrepancy and deterioration factors for each fuel mode:

ECin-use=ECin-use Mode 1×UFin-use Mode 1+ ECin-use [Mode 2]×(1-UF in-use Mode 1)

And,

| Mode 1 (e.g. charge depleting) | ECin-use M1= ECcertification [M1]×fdiscrepancy [M1]×fdeterioration [M1] |
| --- | --- |
| Mode 2 (e.g. charge sustaining) | ECin-use M2= ECcertification [M2]×fdiscrepancy [M2]×fdeterioration [M2] |

Where,

Mode 1/M1 = The first mode of operation, e.g. Charge Depleting (CD) where the on-board battery is being depleted. In some OVC-HEV architectures (e.g. pure range extended electric vehicles), this may be 100% operation on electricity. However, in other PHEV architectures, this may include still some mixed operation (i.e. use of electricity from the battery as well as a smaller degree of the use of the combustion engine) – and therefore energy consumption of both electricity and fuel may need to be accounted for.

Mode 2/M2 = The second mode of operation, e.g. Charge Sustaining (CS) where the on-board battery is not being depleted/the vehicle is essentially operating as a non-plug-in hybrid vehicle mode. In this case there is only fuel consumption (and no net consumption of electricity).

UF in-use [Mode 1] is the in-use Utility Factor (= share of operation) in mode 1 (=charge depleting for PHEVs and REEVs), calculated according to the regional vehicle certification procedures.  For example, under European WLTP this is defined by the electric range in charge depleting mode.

For regions where the Utility Factor (or equivalent) is defined according to the range operating in Mode 1 / charge depleting, the in-use electric range is needed, which shall be calculated according to the following equation:

Rangein-use [electric]=Rangecertification [electric]×ECcertification [CD]ECin-use [CD]

Where,

Rangecertification [electric] is the certified electric range (i.e. in charge depleting mode of operation) in km or miles, as defined in the regional type-approval requirements. In some regions (and for some specific powertrain types – e.g. range extended electric vehicles), this may be defined as the full/equivalent all electric range. In other regions, such as the EU WLTP, the charge depleting range may include also some use of the combustion engine in mixed operation, which is included in the certified charge-depleting cycle range (RCDC).  In WLTP, the equivalent all electric range (EAER) is the part of the actual RCDA range that can be attributed to the use of the electric battery (and is the value included in public disclosures).

Rangein-use [electric] is the in-use electric range (i.e. in charge depleting mode of operation) in km or miles, i.e. including accounting for the discrepancy and deterioration factors.

ECcertification [CD] is the certified electric energy consumption (in MJ/km or Wh/km) in charge depleting mode (i.e. draining the on-board battery). This is defined in WLTP as the distance driven in charge depleting on a single full charge. For some vehicle types, this may include mixed operation (i.e. use of electricity from the battery as well as a smaller degree of the use of the combustion engine), which should be accounted for in the calculations.

ECin-use [CD] is the in-use electric energy consumption (in MJ/km or Wh/km) in 100% charge depleting mode (which may include small amounts of fuel consumption, as well as electricity in some cases).]

[Table 4.3.3-1 : Level Concept for Use phase Energy Consumption certification values]

| Level | Representativeness | Energy Consumption Certification |
| --- | --- | --- |
| Level 1 | LCA family reference vehicle | Regional typical consumption factors or other local representative realistic data |
| Level 2 |  |  |
| Level 3 | Vehicle High in selected IP family/Inertia class | OEM segment-powertrain certification values |
| Level 4 | Specific vehicle configuration | OEM vehicle specific certification values |

Note: in defining regional discrepancy and deterioration factors for different modes of operation based on real-world energy consumption monitoring, care should be taken to avoid double-counting of the effects of the Utility Factor.

[Table 4.3.3-2 : Energy consumption certification protocols for main regions ]

| Region | Powertrain | Protocol |
| --- | --- | --- |
| Europe | ICE, HEV, PHEV, FCHV, PEV | WLTP (WLTC 4 phases) |
| Japan | ICE, HEV, PHEV, FCHV, PEV | WLTP (WLTC 3 phases) |
| China | ICE, HEV, PHEV | WLTP (WLTC 4 phases) |
|  | FCHV, PEV (Passenger car) | CLTC-P |
|  | FCHV, PEV (Vans/LCV) | CLTC-C |
| Korea | ICE, HEV, PHEV, FCHV, PEV | Combined (FTP75+HWFET) |
| US | ICE, HEV, PHEV, PEV, FCHV | Combined (FTP-75+HWFET) |
| Canada | ICE, HEV, PHEV, PEV, FCHV | Combined (FTP-75+HWFET) |

#### Calculation of ‘Discrepancy Factor’

[A significant gap remains between certification values and real-world energy or fuel consumption across all powertrains in some regions. To address this, guidelines suggest applying a "discrepancy factor" where regional data exists. If unavailable, the factor should be estimated using the best available data and expert assumptions, with a minimum value of 1. For vehicles with multiple modes, such as PHEVs, discrepancy factors should reflect each mode of operation separately, as outlined in the previous section.]

Table 4.3.3.2-1: Prioritisation for discrepancy factor calculation

| Level | Discrepancy Factor |
| --- | --- |
| Level 1 | If OEM/ CPs / supplier-specific data is not available, assume official monitoring info, inventories like EMEP/EEA Guidebook, COPERT, MOVES, APEI, etc… |
| Level 2 |  |
| Level 3 | Default values provided by CPs based on methodology development/impact assessment/internal studies on real-world gap (OBFCM or equivalent standardised) |
| Level 4 | Default values provided by CPs or OEM-specific average data based on analysis of data from their vehicles operating in the real-world for similar powertrains (i.e. for electric powertrains = PEVs, FCHVs, etc.), matched to the region of operation. |

#### Calculation of ‘Deterioration Factor’

[ Given the loss in (charge/discharge) efficiency of batteries over the lifetime of the vehicle is reportedly relatively low, and no approaches have been identified to quantify this objectively, it is not proposed to include this aspect for PEVs.  However, the situation for fuel cells is different, where efficiency degradation is expected to be significant, particularly for HDVs. Therefore, the following overall methodological approach is proposed for determining fuel cell efficiency degradation over lifetime of the vehicle.]

[Fuel Cell Hybrid Electric Vehicles]

[ For fuel cells, efficiency losses occur over the operational life of the vehicle. It is proposed to calculate the average loss of efficiency (used to calculate an amended lifetime average energy consumption in MJ/km) based on the fuel cell durability assumptions and operational lifetime km, as outlined below. Fuel cell durability is defined as the number operational hours to reach 10% degradation of the original fuel cell rated power (in kW). The following general methodological approach is therefore proposed to determine the average loss in efficiency over the service life of a vehicle using fuel cell based powertrain (i.e. an FCHV or FC-REEV powertrain).

Potential for further development of knowledge in this area is expected. To adjust to this perspective, the recommended practices allow OEM or suppliers to propose an alternative owned methodology to define operational fuel cell efficiency loss, as long as it is validated by an independent third party expert on fuel cells.

For operation on hydrogen for FCHV – amended to account for fuel cell degradation, the efficiency loss should be to a maximum of 5% degradation over the life of the vehicle, i.e. in the case where FC [lifetime energy] >FC [max energy], where a fuel cell replacement will be required in any case. The following formula is proposed to calculate the average efficiency reduction in the intermediate cases up to this point:

EnCon AvLife=  EnCon [Start]1- 10% * FCEV[lifetime energy]FC[max energy]2

Where:

EnCon [AvLife] = average input hydrogen energy consumption in MJ/km over the entire lifetime of the vehicle.

EnCon [Start] = input hydrogen energy consumption in MJ/km at the start of the vehicle life (i.e. before any FC degradation), as defined in vehicle certification (i.e. before any real-world adjustments being applied).

FCHV [lifetime energy] = Lifetime vehicle operational electrical energy requirement (i.e. fuel cell output, kWh) based on the input hydrogen energy consumption (in kWh/km), the lifetime activity (in km) and the average fuel cell efficiency (%).

FC [max energy] = maximum energy delivered by the fuel cell (in kWh) over the defined service life (in hours) at the average fuel cell running power (in kW). As per the formula given below.

NB: Fuel cell durability/service life is defined as based on the number of operational hours to 90% of original peak power rating, hence an efficiency loss of 10% over the life of the fuel cell, equal to an average reduction in overall efficiency of 10% divided by 2.

The (i) maximum lifetime energy that can be delivered by the fuel cell before reaching 10% degradation (FC [max energy]), and (ii) fuel cell electric vehicle lifetime energy requirements (FCHV [lifetime energy]) is proposed be calculated as follows:

(i) Fuel cell lifetime maximum electrical energy output (FC [max energy]):

FCmax energykWh=Fuel cell durability hrs* Fuel cell stack max running power (kW)

Where:

(ii) Fuel cell electric vehicle lifetime electrical energy requirement (i.e. energy output from fuel cell)

(= FCHV [lifetime energy]):

FClifetime energykWh=EnConStartMJkm*Fuel cell stack efficiency[Start]*EnConConversionkWhMJ* Lifetime activity (km)

Where:

EnCon [Start] = input hydrogen energy consumption in MJ/km at the start of the vehicle life (i.e. before any FC degradation), as defined in vehicle certification (i.e. before any real-world adjustments being applied).

EnConConversion = conversion factor for converting MJ to kWh = 3,6 MJ/kWh

Prioritisation for fuel cell durability assumptions:

The following recommended prioritisation is proposed for the underlying assumptions of fuel cell life and average operational efficiency, with the choice of which option is most appropriate or feasible left to the practitioner (i.e. depending on the availability of data and objective of the study).

The different options are listed in order of accuracy and preference. The third option is proposed as a mandatory minimum default approach, where sufficient information is not available for the other options.

Table 4.3.3.2-1: Prioritisation for fuel cell durability assumptions

| Level | Fuel Cell Deterioration Factor |
| --- | --- |
| Level 1 | If OEM/ supplier-specific data is not available, assume an operational life of 6000/24000 hours (for LDVs/HDVs)(a), an efficiency of 55%/52% (at the start of the fuel cell life for LDVs/HDVs)(b), with efficiency loss of 10% over the life of the fuel cell, and running at an average of 25%(c)/25%(d) (for LDVs/HDVs) of the peak power rating |
| Level 2 |  |
| Level 3 | Optional, depending on availability OEM / supplier specific data on fuel cell life (to 10% loss in power) and average operational power level (as % of the peak power of the fuel cell, according to regulatory testing cycles) |
| Level 4 | Optional, depending on availability OEM / supplier specific methodological approach to define operational fuel cell efficiency loss, if validated by an independent third-party expert on fuel cells. |

Notes: (a) based on 2025 targets from FCH2JU KPIs FCH 2 JU - MAWP Key Performance Indicators (KPIs) - European Commission (europa.eu); (b) based on Ricardo review of typical fuel cell efficiency for LDV and HDV applications; (c) based on Fuel Cell Electric Vehicle Durability and Fuel Cell Performance (nrel.gov), (d) average approximation based on Ricardo analysis of VECTO simulation results for different HDVs and cycles.]

[Off-Vehicle Charging Hybrid Electric Vehicles (OVC-HEV)]

[For OVC-HEVs (i.e. including plug-in hybrid electric vehicles – PHEVs - and range-extended electric vehicles – REEVs), the proposed methodology should account for the effect of the change in the battery energy capacity (i.e. State of Health / Certified Energy as defined in GTR 22) over the lifetime of the vehicle and its impact on electric range and via the Utility Factor UF on the share of operation in charge sustaining and charge depleting modes, which ultimately affects the combined fuel consumption and CO2 emissions]

[The average lifetime in-use electric range for vehicles operating in dual-mode shall be calculated according to the following formula, accounting for the average loss in energy capacity (i.e. as defined by the battery SOH – state-of-health):

Rangein-use [electric]=Rangecertification [electric] ×Battery Capacityin-use averageBattery CapacityStart

Where,

Rangecertification [electric] is the certified electric range (i.e. in charge depleting mode of operation) in km or miles.

Rangein-use [electric] is the in-use electric range (i.e. in charge depleting mode of operation) in km or miles, i.e. including accounting for the discrepancy and deterioration factors.

Battery CapacityStart is the net usable battery energy capacity (in kWh) available at the start of the vehicle lifetime, defined as UBEcertified in GTR22.

Battery Capacityin-use average is the average net usable battery capacity (in kWh) available over the lifetime of the vehicle, which may be defined as the average of the capacity available at the start of the vehicle lifetime, and the capacity left at the end-of-life of the vehicle (i.e. according to the state-of-health – SOH), i.e. according to the following equation:

Battery Capacityvehicle EoL = SOHvehicle EoL×Battery CapacityStart

And where,

Battery Capacityvehicle EoL is the average net usable battery energy capacity (in kWh) available at the end of the vehicle lifetime.

SOHvehicle EoL is the average battery state-of-health (SOH) in % of the original capacity at the end-of-life of the vehicle (or battery, whichever comes first).

(* UN GTR22 defines the “State Of Certified Energy” - SOCE to provide a precise definition of battery state of health based on energy, and should be used as the primary definition/basis where available).

The average available battery capacity at end-of-life of the vehicle (or SOHvehicle EoL) may be defined according to alternative methodologies, as outlined in [Table 16] below.  Where OEM / supplier specific calculated value for end-of-life available battery capacity or SOHvehicle EoL is not available, the following formula shall be used to estimate the SOHvehicle EoL, which assumes as an approximation a linear relationship for battery degradation based on the number of full charge/discharge cycles:

SOHvehicle EoL=1-(20% × Batterylifetime cyclesBatterycycle life )

Where,

Batterycycle life is the operational cycle lifetime (i.e. number of full charge/discharge cycles) of the battery to reach 80% SOH. (Note: depending on the definition of, for example minimum battery durability requirements, in different regions for different vehicle types, the equation may be adjusted to account for the defined minimum SOH for end-of-life use in a vehicle).

Batterylifetime cycles is the average number of full battery charge/discharge cycles over the use-stage vehicle lifetime, which may be estimated according to the following formula:

Batterylifetime cycles =ECin-use CD × Service Life×UFcertificationBattery CapacityStart × (1+Charging Losses)

Where,

Service Life = number of km driven over the use stage lifetime.

Charging Losses = charging losses included within the certified or in-use vehicle electricity consumption in charge-depleting mode of operation.

Other parameters are defined as earlier.

Table xxxxx: Prioritisation for average battery SOH reduction assumptions

| Level | Traction Battery Deterioration Factor |
| --- | --- |
| Level 1 | If OEM / supplier-specific data is not available, assume an operational cycle life of 2000/3000 charge/discharge cycles hours (for LDVs/HDVs) to calculated the average SOH according to the defined formula. |
| Level 2 |  |
| Level 3 |  |
| Level 4 | Optional, depending on availability OEM / supplier specific data on battery cycle life (i.e. to 80% loss in energy capacity*), according to regulatory durability testing requirements. |

Notes: * Depending on the definition of battery end-of-life in a vehicle in different regions for example via minimum battery durability requirements for different vehicle types, the SOH/SOCE cycle life definition may be adjusted.]

#### Quantification of Leakages

This section deals with GHG emissions arising from the fuel system, such as evaporation, hydrogen and methane leakages.

[Evaporative emissions

Evaporative emissions occur in vehicles due to the nature of the fuel system and the volatility of petrol fuel. As part of the homologation process, vehicles are subjected to emissions testing to ensure compliance with regulations, including limits on evaporative emissions.

Limits of evaporative emission (2g to 0.5g of HC) and duration of measurement (24h to 48h) varies between different regions of the world (worst case limit is 2g / 24h).

Given the very low impact on the entire use stage GhG emissions, this recommended practices neglect the emissions arising from fuel evaporation.

Hydrogen leakage

Hydrogen emissions primarily occur during production and distribution, mainly due to fugitive leakage. To a lesser extent, emissions can also arise directly from hydrogen-fuelled vehicles, although standardized test methods to quantify these emissions are currently lacking. Hydrogen can also escape from combustion engines and storage systems, particularly in the case of liquefied hydrogen.

Recent research suggests that hydrogen leakage rates across the supply chain may be comparable to methane losses in the natural gas supply chain. Estimates indicate that green hydrogen supply chains could experience net leakage rates between 2.6% and 6.9% (Cooper, Dubey, Bakkaloglu, & Hawkes, 2022).

Hydrogen emissions are not commonly included in life cycle inventory (LCI) datasets, and a characterization factor for hydrogen is currently absent from established impact assessment methods (e.g., the Environmental Footprint (EF) method), due to its omission from the explicit list of greenhouse gases in the latest IPCC Assessment Report. As such, it is recommended that the inclusion of hydrogen as a greenhouse gas in default LCA calculations be deferred until formal consensus is reached on its global warming potential (GWP) and/or its integration into the EF method.

In the interim, practitioners are required to account for hydrogen emissions as a mandatory flow indicator until a standardized GWP value is formally adopted. The following hierarchy shall be applied by practitioners to account for typical fugitive hydrogen emissions from both the supply chain and vehicle use:

- Where available, practitioners should use official governmental estimates or supplier-specific data on typical fugitive hydrogen emissions. These should differentiate between various hydrogen production pathways, local versus imported hydrogen, and distinct hydrogen vehicle types.]

[ Methane leakage

To account for methane emissions, practitioners should use official governmental estimates or supplier-specific data on typical fugitive CH4 emissions, where available. In the absence of such official or supplier-specific data, practitioners shall apply peer-reviewed data from scientific literature, such as the ones proposed hereafter as an example.

Based on studies on Euro 6 cars in Europe (Prussi et al., 2020); Valverde & Giechaskiel, 2020; Vojtíšek-Lom et al., 2018; Hagos & Ahlgren, 2018), the methane emissions from gasoline, diesel, and CNG cars are considered with 5 mg CH 4 /km, 9 mg CH 4 /km, and 60 mg CH 4 /km. This corresponds to about additional 0.15g CO2 eq. /km, 0.12 g CO2 eq. /km, and 1.81 g CO2 eq. /km for the 100-year GWP.

For CNG cars in China, several studies report a high methane slip of 2% (Pan et al., 2020), which corresponds to about 1 g CH 4 /km and thus additional 30 g CO2 eq. /km for the 100-year GWP.]

#### Quantification of fluorocarbons emissions

[ Refrigerant leakage has a direct environmental impact, primarily due to the use of hydrofluorocarbons (HFCs), a class of synthetic gases that replaced hydrochlorofluorocarbons (HCFCs). HCFCs were phased out due to their destructive effects on the ozone layer, attributed to their chlorine content. While HFCs do not pose a significant threat to the ozone layer, they are potent greenhouse gases with high global warming potential (GWP).

Although HFCs are considered short-lived climate pollutants with an atmospheric lifespan of approximately 15 years, many exhibit strong global warming effects. In automotive applications, HFC leakage is nearly unavoidable, leading to continuous emissions throughout a vehicle's lifespan.

Refrigerant emissions shall be included in the life cycle inventory as elementary flows. For mobile air conditioning (MAC) systems in passenger vehicles, such emissions should be reported as non-exhaust emissions if the refrigerant used has a GWP₁₀₀ value equal to or greater than 150 kg CO₂-eq/kg. Refrigerants with a GWP₁₀₀ below this threshold may also be reported as non-exhaust emissions, at the discretion of the practitioner.

For Level 1 and Level 2, these recommended practices do not prescribe a specific methodology for quantifying refrigerant leakage to the environment. Instead, the approach for estimation is left to the practitioner, provided that the method is clearly documented and transparently reported.

However, for Level 3 and Level 4, the calculation of non-exhaust emissions arising from processes concerning MAC systems use should be performed with the following data:

- Air conditioning refrigerant leakage volume [kg/year]

- Fluorocarbon GWP100

- Fluorocarbon production intensity [kg-CO2e/kg-fluorocarbon]

- Years of use [years]

The refrigerants leakage may be calculated as follows:

C Fluorocarbon=Leak MassHFC × Years ×GWPHFC+[Replenishment VHFC × Years ×EFHFC]

]

#### Quantification of vehicle emissions exported out of region of sales

Ideally, the GHG emissions related to operation of vehicles exported from the country where they are sold/used shall be evaluated by using energy consumption performance (and fuel mix) and vehicle lifetime (and km activity) of the country where they are exported, used and eventually disposed/recycled. However, to avoid unnecessary complexity, if data in the region to which they are exported cannot be tracked down or it is difficult to grasp the needed values of the country where they are exported, used and eventually disposed/recycled, a simplified approach can be adopted. Hence, the full lifetime period (and km activity) shall be considered but the energy consumption performance (and fuel mix) and vehicle life (activity) of the primary region of sale/use are adopted for the calculations. For those countries and regions that export used vehicles, the actual average vehicle lifetime is higher than the average period during which the vehicles are used domestically.

Therefore, for all vehicles, the lifetime used to calculate life cycle emissions shall align with the average age of vehicles at the time of recycling, considering only those registered and recycled within the same country.

### Data Collection and data type

Data shall be collected for the item outlined below. Where applicable, activity data shall be collected as primary data. In case of the difficulty, secondary data may be applied to activity data. GHG emission intensity data shall be collected as secondary data. Secondary data source shall be reported. The following data shall be collected:

Table 4.3.4-1: Processes and Data to be included in Use stage

| Processes | Activity data | GHG emissions Intensity Data |
| --- | --- | --- |
| Driving | Certified Fuel consumption [l/100km] and/or electricity consumption [kWh/100km] |  |
|  |  | Discrepancy factor RW data |
|  |  | Degradation factor |
|  | Separate Certified fuel/electricity consumption for the charge depleting (CD) and charge sustaining (CS) modes for OVC-HEVs |  |
|  | Utility Factor definition for dual-mode powertrains (e.g. OVC-HEVs) |  |
|  | Certified charge depleting (CD) range [km] for dual-mode powertrains (e.g. OVC-HEVs) |  |
|  |  | Average lifetime Battery SOH loss for dual-mode powertrains (e.g. OVC-HEVs) |
|  |  | Average xEV charging/discharging efficiency if not included in certification values |
|  | Vehicle lifetime (years), and activity (km) | Vehicle recycling statistics/ Nationally authorized statistics/OEM's average vehicle life |
|  |  | GHG emission factor for burning fuel |
|  |  | Vehicle occupancy rates for potential scenario analysis |
| Leakages | Evaporative emissions, hydrogen, CNG |  |
| Emitting Fluorocarbons | Fluorocarbon emissions | GHG emission factor for emitting fluorocarbons |
| Maintenance and Consumables - Production | List of maintenance parts and consumables | Fixed percentage of upstream emission attributed to maintenance processes depending upon powertrain, service life, vehicle segment and region of use |
|  | Frequency of replacement/service intervals |  |
|  |  | GHG emission factor for producing maintenance parts, in line with Parts production and vehicle assembly stage [SG3] |
| Maintenance and Consumables - Transportation | List of maintenance parts and consumables |  |
|  | Frequency of replacement/service intervals |  |
|  | Transport Weight [kg] Transport Distance [km] | GHG emission factor for transporting maintenance parts |
| Maintenance and Consumables - End of Life | To be evaluated in line with End of Life [SG5] | To be evaluated in line with End of Life [SG5] |
| Vehicle activity out of region of sales | Out of scope | Out of scope |
| Second life of components | Out of scope | Out of scope |

### Application of the energy modelling schemes [SG6]

### Maintenance

Vehicle maintenance refers to the recommended regular inspection defined by the vehicle manufacturer in order to ensure its optimal performance, longevity, and safety by preventing issues from arising and preserving its overall condition. Maintenance in the use stage of a vehicle's life cycle can have a none-negligible impact on its carbon footprint. Recent life cycle assessment studies estimate that the greenhouse gas emissions produced by vehicle maintenance during the use stage can range from [2%- 5%] of total vehicle carbon footprint, depending on the powertrain type and vehicle segment.

#### Maintenance scope

Two types of maintenance are included in the guideline:

- Consumables: These are materials, fluids, and components that are regularly used, replaced, or replenished during the operation and maintenance of a vehicle. These items typically have a limited lifespan and require periodic replacement to ensure the vehicle's proper functioning, safety, and performance. This includes items such as engine oil, transmission fluid, brake fluid, windshield washer fluid, filter, etc… Carbon footprint related to material, production and EoL should be considered.

- Maintenance parts: This refers to the replacement of parts due to normal wear and tear, including components that are not designed to last the entire lifespan of a vehicle, such as the brakes, clutch, tires, starter battery etc...  The range of parts that need to be replaced during maintenance varies between different cars, depending on factors such as the driver's behaviour, road conditions and other variables (weather etc.). Hence, it is difficult for OEMs to recommend a frequency of exchange of these parts to the customer. The frequency of maintenance of such parts can only be estimated with acceptable probability through statistical analysis and a generic frequency of these parts shall be used for the purpose of carbon footprint estimation. For these parts recycling aspects must be considered and are same as that of original production parts.

Other repair or maintenance activities not included in the methodology are:

- Unexpected repairs (i.e., accident):  Looking at the challenge of estimating the consequences of road accidents and premature fails (for components expected to last for service life), the guideline does not address such cases. Furthermore, impact of such cases is not very significant compared to overall carbon footprint of a car.

- Cleaning operations: Effect of car wash may not be a significant factor in the calculation as it is a very small fraction of the overall carbon footprint of a car. Hence, the guideline does not address cleaning operations.

#### Maintenance carbon emission estimation

Since the carbon footprint related to maintenance occurs after the vehicle is placed on the market, it can only be calculated based on the known frequency specified by the vehicle manufacturer.

For Level 3 and Level 4, there are two possibilities (considered in a hierarchical order):

- List of maintenance parts/consumable and associated frequency provided by the manufacturer,

- List of maintenance parts/consumable not available

[ For Level 1 and Level 2, the approach for maintenance and consumables emissions estimation should follow point b) of the aforementioned list of possibilities.]

- Maintenance Data Available

If list of maintenance parts/consumable and associated frequency is provided by the manufacturer or parts supplier, then the following estimation shall be used.

As this will reflect the effort from the manufacturer to reduce carbon footprint of maintenance, the manufacturer or parts supplier should provide all the assumptions (list, frequency, carbon emission) used for carbon footprint estimation.

C maintenance= i=1nCEFi*fi.maintainence

Where,

i			Consumable/ maintenance parts;

fi.maintainence	Maintenance frequency of the consumable/maintenance parts defined;

CEFi  		Carbon emission factor of the consumable/maintenance parts as used for calculation of upstream emissions (kgCO2e).

Determination of list of consumables and parts

The following table provides a list of consumables and parts for guidance and shall not be considered as exhaustive. The manufacturer shall provide the list adapted to the powertrain and vehicle segment for which the carbon footprint is calculated. However, for Level 3 and Level 4, last column of the table indicates items responsible for the most significant potential impacts that shall be included if a replacement is needed (to be justified) in the considered lifetime of the vehicle.

Table 4.3.6-1 : List of consumables and maintenance parts

|  |  | Petrol | Diesel | CNG | NOVC-HEV | OVC-HEV | Pure EV | FCHV | OVC-FCHV | H2-ICE | Level 3/ Level 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consumables | Engine coolant |  |  |  |  |  | - | - | - |  |  |
|  | Engine lubricant |  |  |  |  |  | - | - | - |  |  |
|  | Screen wash |  |  |  |  |  |  |  |  |  |  |
|  | Brake fluids |  |  |  |  |  |  |  |  |  |  |
|  | AdBlue/Urea/Reagent | - |  | - | - | - | - | - | - | - | YES |
| Maintenance parts | Passenger air filter |  |  |  |  |  |  |  |  |  |  |
|  | Engine air/oil filter |  |  |  |  |  | - | - | - |  |  |
|  | Spark plug |  | - |  |  |  | - | - | - |  |  |
|  | Windshield wiper blades |  |  |  |  |  |  |  |  |  |  |
|  | Tyres |  |  |  |  |  |  |  |  |  | YES |
|  | Brake linings |  |  |  |  |  |  |  |  |  | YES |
|  | Brake discs |  |  |  |  |  |  |  |  |  |  |
|  | SLI battery |  |  |  |  |  |  |  |  |  | YES |
|  | Aftertreatment |  |  |  |  |  | - | - | - |  |  |

The impacts resulting from different maintenance items can vary greatly, with some items being less significant and others usually accounting for a significant proportion of overall impacts. It is therefore proposed that the following items should be mandatorily included (where relevant for a given vehicle powertrain type):

- Consumables: AdBlue/Urea, Refrigerants for HVAC (heating, ventilation and air conditioning) systems

- Maintenance and wear parts: Tyres, starter battery (i.e. 12V), brake pads, brake discs,], additional auxiliary batteries (if relevant for commercial vehicles – e.g. for temperature controlled payload areas).

- Notable parts to be considered:	traction/storage battery (i.e. for electric powertrain vehicles), [fuel cell stack (for fuel cell electric vehicles)],

For certain items, including mandatory items, replacements might not be needed in the vehicle’s typical operational lifetime based on the supplier assessment. In these cases, exclusions made on this basis should be justified.

Determination of Frequency

For most maintenance items frequency can be simply calculated by the following two approach:

Frequency by distance

Frequencydistance= Service life [km]Maintainence interval [km]

Frequency by duration

Frequencyduration= Service life [years]Maintainence interval [years]

In case the manufacturer/supplier recommends frequency by both distance and duration, both calculations should be performed, and most conservative frequency shall be considered for the carbon footprint estimation.

[ Based on consultations with experts, stakeholders, and manufacturers, there is no evidence to suggest that battery or fuel cell replacement is a common or routine occurrence. In fact, traction batteries and fuel cell systems are generally designed to last for the entire lifetime of the vehicle without the need for replacement.

However, given the significant impact of batteries and fuel cells on vehicle life cycle analysis, their inclusion in the study must be assessed and justified in cases involving non-standard circumstances. These may include niche applications, small battery sizes, low-durability battery chemistries, or other factors identified by the practitioner.

This Resolution proposes the following approaches to provide a structured hierarchy for determining the need for replacements of traction batteries and fuel cells, which could be applied across all types of LCA study (i.e. for general analyses and scientific studies, as well as product LCA or prospective LCA).]

[Methodology for calculating the need for traction battery replacement(s)]

[The following recommended prioritization is proposed for the underlying assumptions of traction battery replacement, with the choice of which option is most appropriate or feasible left to the practitioner (i.e. depending on the availability of data and objective of the study). The different options are listed in order of accuracy and preference. It is recognized that OEMs and battery suppliers will likely have more sophisticated battery performance modelling supporting their product development and specification. The most important point is that in all cases the study shall clearly indicate whether a battery replacement is included or not, and the basis/explanation for this assumption.

Table 4.3.6-2 : Prioritisation for traction battery replacement in order of accuracy and specificity (highest to lowest)

| Level | Traction Battery Deterioration Factor |
| --- | --- |
| Level 1 | Simple assumption on whether battery replacement is needed or not, including explanation for this. |
| Level 2 |  |
| Level 3 | Standardized methodology based on battery cycle life and lifetime operational activity (below). An explanation/justification for the result should be provided. |
| Level 4 | Optional, depending on availability OEM / supplier specific methodological/modelling approach to define the need for a battery replacement (or not) over the operational life of the vehicle. An explanation/justification for the result should be provided. |

Standardized Methodology

For vehicle traction batteries, the following approach for accounting for the frequency of energy storage replacement for Option 2, which is based on a combination of parameters including the anticipated battery cycle life (i.e. number full charge/discharge cycles).  This methodology also provides a dynamic link to the vehicle battery capacity and the lifetime activity/service life (as defined in the study Goal & Scope), which can be used in product LCA, prospective LCA or other more generic studies.

The methodology for determining the number of traction battery replacements is as follows (i.e. where a value of N > 1 means at least one replacement is likely to be needed):

N= E Average×A [Lifetime]C Battery usable×CL [Battery]

Where:

N = Total number of traction batteries needed over the vehicle lifetime

C [Battery usable] = usable (i.e. ‘net’) traction battery capacity in kWh

CL [Battery] = average battery cycle life – number of full charge/discharge cycles (within the usable capacity)

A [Lifetime] = vehicle lifetime activity (in km) *

E [Average] = vehicle average electrical energy consumption, in kWh per km

*As a sensitivity it is recommended to also explore the potential number of replacements needed based on the warrantied number of km for the battery (where this is present).

In the absence of manufacturer-specific data on the battery cycle life (parameter ‘CL’ above), then it is proposed to use a value of 2000 charge/discharge cycles, which is representative of a typical value for current technology.  Should the battery come with an expected calendar lifetime lower than the defined vehicle lifetime in the Goal & Scope of the LCA study, then a replacement will also be required.]

[Methodology for calculating the need for fuel cell system replacement(s)]

[Fuel cell durability(/life) is defined as the number operational hours to reach 10% degradation of the original fuel cell rated power (in kW).  The following general methodological approach is therefore proposed to determine the need for one or more fuel cell replacements over the service life of a vehicle using fuel cell based powertrain (i.e. an FCHV or FC-REEV powertrain), consistent with the similar methodology proposed for fuel cell efficiency degradation (see earlier Section 4.3.3.2).

(Recognising the potential for further development of knowledge in this area, it is also proposed to that should an OEM / supplier specific methodological approach be subsequently developed to define operational fuel cell efficiency loss and/or replacements, this would also be acceptable, if validated by an independent third party expert on fuel cells.)

Fuel cell lifetime max energy output (= FC [max energy]):

Lifetime max energy output (kWh) = Fuel cell durability (hrs)*Fuel cell average running power (kW)

Where:

Fuel cell average running power (kW) = maximum rated fuel cell power (kW) * average operation % of rated fuel cell power

Fuel cell vehicle lifetime energy requirement (energy output from fuel cell) (= FC [lifetime energy]):

Lifetime vehicle energy requirement fuel cell outputkWh=MJkm*Lifetime km*kWhMJ*fuel cell average efficiency (%)

Where:

MJ/km = hydrogen energy input to the vehicle, i.e. hydrogen energy consumption.

The number of fuel cells needed (and therefore the number replacements) is defined by the ratio of the vehicle’s lifetime energy requirements and the maximum fuel cell lifetime energy delivered:

Number of fuel cell replacements =roundup to integer FClifetime energyFCmax energy-1

]

- Maintenance Data not available

The maintenance parts/consumable are already included in the upstream emission as they are part of the vehicle before placing the vehicle on the market. Hence, if list of maintenance parts/consumables is not available then a fixed percentage of upstream emissions can be attributed to maintenance depending upon powertrain, service life, vehicle segment and region of use. These factors can be defined by each region through statistical analysis. For all passenger cars [light duty vehicles], it can be assumed that there is no variation within segments.

C maintenance= fmaintainence*C Material +C Production +C Recycling

Where,

fmaintainence 	Maintenance frequency as defined in table 2.2

C Material 		Carbon emissions of material acquisition stage,

C Production  	Carbon emissions of production stage

C Recycling 	Carbon emissions of recycling stage for maintenance parts, to be evaluated in line with Section 3.2.13.

[Table 4.3.6-3 : Default maintenance frequency as a function of powertrain

| Powertrain | Petrol | Diesel | CNG | NOVC-HEV | OVC-HEV | Pure EV | FCHV | OVC-FCHEV | H2-ICE | . . . |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fmaintenance | YY % | ZZ % |  |  | Under discussion |  |  |  |  |  |

]

## End of Life [SG5]

### Levels in disposal and recycling stage

### System boundaries

### Data collection and data types

### Scenario

#### Second life parts

#### EVL management out of sale region

#### Recycling process and technology

### Energy modelling [SG6]

[TBD]

### GHG calculation for each process

## Interpretation [?]

## Sensitivity Analysis [?]

## Reporting [?]

## Verification process [?]

Batteries for starting-lighting-ignition (SLI) and auxiliary applications only - does not apply to primary battery packs used for electric-drive powertrains

While the term "Deterioration factor" is commonly used for emissions certification, note that emissions deterioration may be expressed as a multiplicative term, an additive term, or a complex polynomial.

https://unece.org/sites/default/files/2023-01/ECE_TRANS_180a22e.pdf

As also previously implemented in (Ricardo et al., 2020) based on consultation with stakeholders.

FCH 2 JU - MAWP Key Performance Indicators (KPIs) - European Commission (europa.eu)