Inhalt

2	Preamble	2

3	Purpose	2

4	Scope	2

5	(Normative references – should be moved to the preamble)	2

5.1	Netherlands	2

5.2	OICA	2

6	Terms and definitions	3

6.1	Definition of fast charging	3

6.2	Definition of family criteria	4

6.2.1	Test family criteria from Netherlands	4

6.2.2	Test family criteria from OICA	4

6.2.3	Test family criteria from JRC	4

6.2.4	Parent family criteria from Netherlands	5

6.2.5	Parent family criteria from OICA	5

6.2.6	Other possible approaches on parent family	5

7	Abbrevations	5

8	Test cases	5

8.1	Overview of test cases from the Netherlands	5

8.2	Overview of test cases from OICA	5

8.3	Overview of test cases JRC	5

9	General requirements	6

9.1	Vehicle manufacturer specifications	6

9.1.1	OICA	6

9.1.2	Netherlands	6

9.2	Specifications and tests – test system requirements	6

9.2.1	Netherlands	7

9.2.2	OICA	7

9.2.3	JRC comments	8

10	Determination of …	9

10.1	Parameters proposed by OICA	9

10.2	JRC	9

11	KPI‘s	10

11.1	Netherlands	10

11.2	OICA	10

JRC: Discussion points

F&SC-04-03e document: Uniform provisions concerning the optimal charging power curve of light-duty electrified chargeable vehicles using direct current – Fastest Recharge Power Curve (FRPC)

# Preamble

# Purpose

From the 1st drafting meeting: The Fast and Smart Charging (F&SC) cluster aims at developing a provision concerning the charging power curve of light-duty vehicle electrified chargeable vehicles using direct current. The procedure to determine the power charging curve shall be both reproducible and repeatable, and representing real-world charging. This will provide consumers with reliable information on the FRPC and related KPIs.


OICA:

This Resolution provides a worldwide harmonized method to set and verify minimum performance requirements on DC fast charging of Pure Electric Vehicles (PEVs) and Off-vehicle Charging Hybrid Electric Vehicles (OVC-HEVs).

# Scope

OICA: This resolution applies to vehicles of categories 1-2 and 2, that have a technically permissible maximum laden mass not exceeding 3,855 kg, and to vehicles of category 1-1, that are PEV and OVC-HEV vehicles.

At the option of the Contracting Party, the scope may be limited to 3,500 kg for the relevant categories.

# (Normative references – should be moved to the preamble)

## Netherlands

GTR 15 (WLTP)/ UNR 154

GTR 22 (Battery durability LDV)

ISO 12906 (FRPC specific and complimentary source of requirements)

HDV Battery durability GRPE proposal, if applicable

## OICA

UNR 154 series 2 and 3 (WLTP)

GTR 22 (Battery durability LDV

ISO 12906

# Terms and definitions

## Definition of fast charging

Focus on DC-charging

Minimum power “fast charging” from a charging station point (s. Fig. 1 and 2)

Focus on what is possible in the vehicle:
"Fully charged, 100% state of charge (SOC), corresponds to the highest SOC which is achievable with normal operation of the vehicle", however, fast charging is typically terminated at 80% SoC due to battery durability aspects.

Norming factor: UBE on the vehicle (defined by WLTP and GTR22)

Scope: BEV and PHEV  tbd?

https://www.charin.global/media/pages/technology/knowledge-base/63d38591d7-1639130326/charin_dc_ccs_power_classes.pdf

Figure 1: From CharIn

Recharging systems | European Alternative Fuels Observatory

Figure 2: From EU-Com

## Definition of family criteria

### Test family criteria from Netherlands

Definition of vehicle family GTR 22, part B family as a basis

### Test family criteria from OICA

GTR22 Part B criteria for battery durability were used and adjusted for fast charging:

Battery and charging system design including thermal management and any other characteristics having a non-negligible influence on fast charging

(Declared maximum charging power (Value from OEM))

Type of battery (e.g., type of cell, including format, capacity (Ampere-hour), nominal voltage, nominal power  tbd by the drafting group) with focus on fast charging

Battery management system (BMS) with regards to fast charging monitoring and estimations

Operation strategy of all components influencing the fast charging

### Test family criteria from JRC

GTR No. 22 Part B family adjusted to consider the parameters related to the DC fast charging,

-any other characteristics having a non-negligible influence on fast charging

-BMS related to DC fast charging,

-thermal management of the battery, battery conditioning function? Vehicle with/without battery conditioning function in the same family?

In ISO/SAE 12906 optimizing features are in 6.2.1. General requirement, 6.3.2.2. Relocations (optimizing features not allowed), 6.3.2.3 Soaking phase (not allowed), 6.3.2.4 Preparation phase (optional)

(b)	Type of battery (dimensions, type of cell, including format and chemistry, capacity (Ampere-hour), nominal voltage, nominal power.

### Parent family criteria from Netherlands

Worst-case principle: test family member with expected worst performance

### Parent family criteria from OICA

In general worst-case principle

Definition of worst case may be related to second order characteristics like thermal management, HV-system (length of HV cables)

Remark: Could be very difficult to prove what is “worst case”

### Other possible approaches on parent family

GTR22: maximum energy consumption (witness testing, part of WLTP measurement cycle)

# Abbrevations

# Test cases

## Overview of test cases from the Netherlands

Charge power curve to cover the entire UBE spectrum from zero charge (0% SoC) to full charge (100% SoC)- FRPC to cover the full charging window from close to 0% to close to 100%

Consumer metric might focus on given charging window, eg. 10% to 80% or else

## Overview of test cases from OICA

Discuss charge power curve (handling, parameter, measurement, tolerances …) separately with more time - not part of the resolution, focus on 10-80% SoC  relevant for fast charging

Start Point and End Point for the fast charging procedure have to be defined

## Overview of test cases JRC

To agree on the test case requirements and customer metric post-processing and KPI.

Not all the DC HP charging stations will allow to reach automatically the full charged status of the battery  due to battery protection systems, and unplugging and plugging again the vehicle is needed to complete the charge at lower power with the same station.

In the draft HDV GTR a provision has been added to take into account this and give instruction on how to charge at full.

To consider in case to prescribe the reporting on how the full charge was reached (i.e. with/without interruption, ...).

# General requirements

## Vehicle manufacturer specifications

### OICA

Required charging values (current, voltage, power and their duration) to be provided by OEM’s see also ISO/SAE 6.2.1 Vehicle manufacturer specifications

Measurement set up, tolerances and accuracies see ISO/SAE 6.2.3 Measurement tolerances and accuracies

### Netherlands

Condition for charging

- Break-off criterion: same as UBE from GTR 15 / ISO

- When reached, the battery is nearly empty (close to 0% SoC)

- End-of-charge criterion: same as for UBE from GTR 15 / ISO

- When reached, the battery is nearly full (close to 100% SoC)

Pre-conditioning, soaking, preparation

- Align with GTR 15 / GTR22 as much as possible

- Where GTR15 / GTR22 are not comprehensive enough, follow ISO 12906

- Allow for active/passive battery preconditioning (flag to be recorded)

- Need for vehicle dyno for preparation phase?

  - Is it possible to do without a dyno if UN GTR 15 / GTR22/ISO is used as a basis?

  - Validation test to confirm if vehicle dyno is required

  - Implication of dyno usage:

    - Higher resources needed

    - Restrict potential application of FRPC resolution to OEMs / certification bodies /technical services

## Specifications and tests – test system requirements

1st Drafting meeting E-laad also posed the question whether a real market available charging station should be used during the testing or a more sophisticated but more controlled EVSE emulator should be used.
To specify this in the test system requirements can be an option

### Netherlands

EVSE specifications and requirements:

- Voltage: as recommended by vehicle manufacturer

- Current: [10%] more than max current from OEMs specs

Test system requirements:
- GTR15 where relevant (Annex 6, para 2.2.)
- Test location requirement from ISO 12906, para 6.3.2.5.

Sampling rate:
- [1]Hz / [20]Hz for energy (as ISO12906)

### OICA

Refer in general to ISO/SAE 6.3.2 Test procedure for test case Fast Charge

Start/stop conditions in ISO12906 chapter 6.3.2.5 for 10-80% SoC evaluation

Pre-cond/soaking/prep, focus on ISO12906 only
Conditioning on a dyno for charging possible, could be set as optional

EVSE requirements:
- Remove +10% current  not feasible for high charging current demand

Test system requirements:
-     Add ISO 6.3.2.3 soaking phase requirements

For test location requirements refer to ISO12906

Sampling rate: 
- 1Hz

### JRC comments

- Pre-conditioning, soaking, preparation:

- ISO preparation phase: optional

25-30km/h for a maximum 30 minutes or alternatively EV may be soaked for maximum 30 min at temp 0 -30 C

- Green NCAP: conditioning drive on dyno or on road for approx. 15 minutes

- UN GTR No 15:  pre-cond WLTC

- Allow for active/passive battery preconditioning (flag to be recorded)

- In ISO/SAE 12906

optimizing features 6.2.1. General requirement,

6.3.2.2. Relocations (optimizing features not allowed),

6.3.2.3 Soaking phase (not allowed),

6.3.2.4 Preparation phase (optional)

- To consider to prescribe the reporting on active/passive battery preconditioning and flag

- Need for vehicle dyno for preparation phase?

- Preparation phase prescribes different drive cycle in ISO/SAE, Green NCAP, GTR 15/22 etc…Dyno can also not be needed for preparation.

But:

- ISO/SAE 12906

It requires dyno for determining the DBE (discharged battery energy) versus SOC curve

This curve DBE vs SOC is needed to determine:

- the recharged usable battery energy within a reference time t_ref

- and the recharged electric range

For the recharged electric range calculation the EC_DC,WLTC is also needed as parameter. This is derived from the WLTP.

To discuss in the group based on KPI

Bidirectional power supply system may be used to deplete the battery during the preparation and to derive the curve DBE vs SOC. But the influence on the battery and systems during preparation is different. Moreover, for the curve DBE vs SOC an error is introduced.

- EVSE specifications and requirements:

- to consider to add provision on modular charging station, for charging efficiency related aspects

-Test system requirements:

- GTR 15 0.1 Hz Pressure, humidity, 0.033Hz temperature

- ISO 1 Hz SOC, Power, Energy depleting 20 Hz,

- Green NCAP , 1 Hz

- GTR 22, GTR 21

- Voltage, Current, 20 Hz during depleting, 0.033Hz during charging

- Room temperature 0.033Hz

- Time, ± 10 ms

- CAN bus data from vehicle?

- SOC monitor vehicle?

- Voltage, current, SOC, from charging station if possible?

- Voltage, current, SOC, man-in-the-middle or EVSE emulator?

- Moving average power window for the calculation of the maximum power?

- ISO the maximum charging power is the highest averaged power within a 30 s interval during the charging duration

- Ambient temperature,

- ISO 23C +/- 3 C on a 5 minutes running arithmetic average during soak with no systematic deviation from the set point,

- ISO 0-30C, 23C+/- 3 C recommended;  during charging phase, to resemble real-world conditions

# Determination of …

  - Parameters proposed by Netherlands

Power at the station (MP2 point of ISO 12906)

Energy: Station and vehicle/battery

SoC calculated: from energy at the battery and UBE

SoC declared: value on dashboard/ from on-board system
(SoC calculated from energy recharged and on-board SoC also collected)

Charging efficiency derived from total energy at the station vs. UBE (like ISO)

[Battery/cell] temperature

Activation flag for Battery pre-conditioning (heating/cooling active)

Others ?

## Parameters proposed by OICA

Time

Indicated SoC (declared SoC)

Electrical power

## JRC

# KPI‘s

## Netherlands

Consumer metric might focus on given charging window, eg. 10% to 80% or else
Consumer info metrics to be proposed later
Energy recharged to be compared to declared UBE as part of GTR 15/ISO

## OICA

@ temperature = 23±3°C (tolerance from ISO12906)

Charging time from 10-80% SoC

Maximum charging power

Discuss charge power curve (handling, parameter, measurement, tolerances …) separately with more time - not part of the resolution, focus on 10-80% SoC  relevant for fast charging