6.2.8.	Specifications on minimum and maximum sound level for AVAS sound

When tested under the conditions of Annex 3 paragraph 3.3.2, a vehicle which is equipped with an AVAS shall fulfil the requirements of Table 2a, Table 2b and Table 3.  The specifications for the maximum sound levels for vehicle speeds greater 0 km/h according to table 2a and 2b apply only to vehicle other than category M1 and N1. Vehicle of category M1 and N1 comply with the provisions of UN Regulation No. 51 and the specifications of table 2a and 2b of this Regulation for standstill and reverse driving.

The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type approval test set out in Annex 3 was carried out, shall not deviate from the test result in a significant manner.

In the speed range of Table 2a and Table 2b and when tested under the conditions of Annex 3 paragraph 3.3.2., a vehicle other than category M1 and N1 which is equipped with an AVAS, shall not emit an AVAS overall sound level of more than 75 dB(A), if driving in forward direction. This test can be stated by manufacturer declaration.

During measurement in reverse the maximum level requirement in the frontline of the vehicle for forward driving has to be fulfilled in addition. This can be stated by manufacturer declaration.

The sound levels measured and reported shall be mathematically rounded to the nearest integer value.