| ORG | Paragraph # | Type of comment1 | Original text (Consolidated version prepared by OICA) | Proposed change | Justification |
| --- | --- | --- | --- | --- | --- |
| CH | 2.23 | Ed | Table of symbols Lκj; dB(A); Annex 7; 3.5.; sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place. | Lκj; dB(A); Annex 7; 3.6.; sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place. | Due to renumbering |
| CH | 2.29 | Ed/te | “Exterior Sound Enhancement System (ESES)” means an active system that is installed on a vehicle for the purpose of producing exterior sound such as, but not limited to - horns according to UN Regulations No. 28, or - AVAS according to UN Regulation No. 138, or - Audible reverse warning device according to UN Regulation No. 165, or - sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit. " | “Exterior Sound Enhancement System (ESES)” means an active system installed on a vehicle to generate exterior sound that enhances vehicle audibility or character in a manner not regulated by other UN Regulations. This includes but is not limited to AVAS according to UN Regulation No. 138 in the optional speed range, and sound actuators either integrated into an exhaust silencing system or mounted as an individual unit. For the purpose of this Regulation, any system or function that is subject to applicable mandatory requirements under another UN Regulation shall not be considered an ESES. This includes, but is not limited to: (a) horns according to UN Regulations No. 28; (b) anti-theft according to UN Regulation No. 116; (c) Audible reverse warning device according to UN Regulation No. 165; (d) AVAS according to UN Regulation No. 138, in the mandatory speed range [+ a fade out area of 9.50 km/h]. - (Minimum sound according to UN Regulation No. 138, ) | CH solution (in blue): split the definition between AVAS and ESES according to the speed range Other good solution (without the blue part): divide the definition by optional and mandatory sounds regardless of the speed range. |
|  |  |  |  | Modes Every mode of the ESES shall comply with the requirements of this regulation and other UNR. If any mode of the ESES is not only subject to prevent component damages or mandatory minimum sound required by other applicable UN-Regulations, such as, but not limited to - UN Regulations No. 28, and - UN Regulation No. 138, and - UN Regulation No. 165, the ESES shall be capable of being muted by the driver to the “manufacturer recommended mode”, (also named “default mode” or “normal mode”) that shall only prevent component damages and comply with mandatory sound required by any other applicable UN Regulation. ESES - Selectable sound apart from AVAS | Proposals during meeting |
| OICA | 2.29. | Ed/te | “Exterior Sound Enhancement System (ESES)” means an active system that is installed on a vehicle for the purpose of producing exterior sound such as, but not limited to - horns according to UN Regulations No. 28, or - AVAS according to UN Regulation No. 138, or - Audible reverse warning device according to UN Regulation No. 165, or - sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit. " | “Exterior Sound Enhancement System (ESES)” means an active system that is installed on a vehicle for the purpose of producing exterior sound such as, but not limited to - horns according to UN Regulations No. 28, or - AVAS according to UN Regulation No. 138, or - Minimum sound according to UN Regulation No. 138, or - Audible reverse warning device according to UN Regulation No. 165, or - sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit. " | The reference to AVAS and UN Regulation No. 138 has been deleted, since there is the objective that during the next update of the Regulation it will focus on whole vehicle sound and the expression AVAS could be obsolete. |
| CH | 6.2.3. | te | … The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type-approval test set out in Annex 3 and Annex 7 was carried out, shall not deviate from the test result in a significant manner. Any ESES for the purpose of the exterior sound emission shall be operational during the type-approval test. Footnote: 1 See recommendations provided by informal document GRB-68-03 as guidance for technical interpretation. The document can be found in https://unece.org/documents-reference-only-0. | … The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type-approval test set out in Annex 3 and Annex 7 was carried out, shall not deviate from the test result in a significant manner.1 If the vehicle is equipped with an electric sound enhancement system ESES the ESES shall be active and in the same configuration and activation state during the type-approval test (including tests specified in Annex 7 to this Regulation subject to paragraph 3.1. of Annex 7 to this Regulation) and in real world operation. The vehicle equipped with an ESES shall continue to comply with all requirements and specifications of other UN Regulations (e.g. but not limited with the stationary sound requirements of UN regulation No. 138). The ESES shall be capable of being muted by the driver. In the muted state, the vehicle shall continue to comply with the applicable minimum sound emission requirements within the mandatory speed range as specified in UN Regulation No. 138, and the maximum limits set out in Annex 7 to this Regulation. The emitted sound shall progressively decrease at vehicle speeds between 20.00 km/h and 28.00 km/h and shall cease entirely at speeds above 28.50 km/h (with the sole margin of +1.00 km/h including CoP tests) while in the muted state." Every mode of the ESES shall comply with the requirements of this regulation. Any ESES shall be deactivated after each vehicle shutdown. Footnotes: 1 See recommendations provided by informal document GRB-68-03 in its latest version as guidance for technical interpretation. The document can be found in https://unece.org/documents-reference-only-0. [2 The sound content shall be alterable by the vehicle manufacturer only, shall not impair traffic safety and shall not be designed to cause deliberate disturbance. See recommendations provided by informal document #### as guidance for technical interpretation. Notwithstanding the provisions of this Regulation a Contracting Party may permanently, temporarily or conditionally prohibit specific sound content emitted by [ESES], or impose additional requirements for the content of [ESES].] | GRB-68-03 needs to be adapted in order to work with the new ASEP lines. “In the muted state… muted state.” Could be left out depending on the definition Requirements should be in the text and not in the footnote 2 |
|  | 6.2.3. |  |  | The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type-approval test set out in Annex 3 and Annex 7 was carried out, shall not deviate from the test result in a significant manner. | Proposal provided by HM Gerhard in the chat before end of meeting. |
|  |  |  |  | No testing of ASEP within speed range of 0-20 km/h | As a first step for fast adoption but acceleration needs to be considered. |
|  | UN-R 138 |  |  | 6.2.8. Specifications on minimum and maximum sound level for AVAS sound For vehicles of category M1 and N1 the maximum sound levels of UN R51 Annex 7 apply. For any other vehicle categories: When tested under the conditions of Annex 3 paragraph 3.3.2, a vehicle which is equipped with an AVAS shall fulfil the requirements of Table 2a, Table 2b and Table 3. | Not practical for the interim step as it would require a new series of amendments and transitional provisions. |
| COM | 6.2.3.1. | ge/te | The vehicle manufacturer shall not intentionally alter, adjust, or introduce any mechanical, electrical, thermal, or other device or procedure solely for the purpose of fulfilling the sound emission requirements as specified under this Regulation which is not operational during typical on-road operation. | The motorcycle vehicle manufacturer shall not use any device, procedure or software which would modify any noise related functional parameters of the vehicle for the purpose of passing the tests within this Regulation and which would result in increased noise emissions during typical on-road operation in the ASEP control range of Annex 7 paragraph 2.3, compared to during testing according to the provisions of this Regulation. | Update of the prohibition of defeat device clause/ alignment with the one of UNR 41 (updated during last GRBP, February 2025) |
| NL | New para. 6.4. | te |  | 6.4 Additional requirements for ESES 6.4.1 The non-mandatory ESES sound shall have a direct available “button” for the driver to be switched off completely. 6.4.2. ESES sound shall contribute to the detectability of the vehicle that it is mounted on 6.4.3 ESES sound shall not mask mandatory required sound of the vehicle 6.4.4 ESES sound shall not reduce the detection of other vehicles in traffic in such a way that other road users could be mis informed by the ESES. 6.4.5 ESES sound shall fulfil the description of document xxxxx (a document to describe allowed sound for ESES) | The test in the speed range of mandatory sound should show that the mandatory sound requirements are dominant in the SPL. 6.4.2.: The test in the speed range of mandatory sound should show that the mandatory sound requirements are dominant in the SPL. 6.4.4.,: This might be difficult, but this could be based on the assumption that were shared by OICA. A vehicle approaching at a higher speed should have a louder signal to warn, but it should not mask a vehicle that is in fact closer and will be aearlier at the point of the observer. 6.4.5.: For this we need an additional document for reference. In my opinion OICA should propose the content of the first version of this document to commit to a certain way forward with a sound that will contribute to the purposes that were shared in the meetings. Pleasant Detectability A certain level of signature Etc... |
| CH | 11.xx. | Te | Supplement [yy] does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement [yy] | Supplement [yy] does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement [yy] until [10] months after the entry into force of Supplement [yy]. | Subject to discussion. Has to be checked with the templates. |
| NL | 11.xx | ge | Supplement [yy] does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement [yy] |  | Is a supplement not to be used for new extensions? Existing Tas will remain valid, but extension should follow the supplement. |
| OICA | 11.xx | ed | Supplement [yy] does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement [yy] | "11.[xx19]. Supplement [yy11] does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement [yy11] |  |
| CH | Annex 1 – Appendix 1 1.1.10 | ge / te |  | Add: 1.1.10 External Sound Enhancement System, [Make, Part number, Place, Number of devices]: |  |
| CH | Annex 3, 2.2.3.3. | ge / ed | Active Sound Systems Exterior Sound Enhancement Systems (ESES) Any ESES, either for noise control, or sound enhancement, shall operate as foreseen by the vehicle manufacturer, and not be interfered with during the measurements. | Active Sound Systems Exterior Sound Enhancement Systems (ESES) Any ESES, either for noise control, or sound enhancement, shall operate as foreseen by the vehicle manufacturer, subject to paragraph 6.2.3. of this Regulation and Annex 7 to this Regulation, and not be interfered with during the measurements. |  |
| OICA | Annex 3, 3.1.2.1.4.3. |  | Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV) The gear selector position for forward driving shall be used. The acceleration value awot test shall be calculated as defined in paragraph 3.1.2.1.2.1. The achieved acceleration awot test shall be greater or equal to aurban. If possible, the manufacturer shall take measures to avoid an acceleration value awot test greater than 2.0 m/s². Table 1 in Appendix to Annex 3 provides examples for valid measures to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. The achieved acceleration awot_test , but not more than 2.0 m/s², is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead awot ref.[For Annex 7 calculations, the true achieved acceleration shall be used.] | Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV) The gear selector position for forward driving shall be used. The acceleration value awot test shall be calculated as defined in paragraph 3.1.2.1.2.1. The achieved acceleration awot test shall be greater or equal to aurban. If possible, the manufacturer shall take measures to avoid an acceleration value awot test greater than 2.0 m/s². Table 1 in Appendix to Annex 3 provides examples for valid measures to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. The achieved acceleration awot_test , but not more than 2.0 m/s², is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead awot ref.[For Annex 7 calculations, the true achieved acceleration shall be used.] | The use of the suitable acceleration value (limited to 2 m/s² or true achieved) for calculation in Annex 7 is moved to Annex 7 and there described |
| CH | Annex 7, 1.1. | ed | Vehicles with propulsion technologies other than combustion engines shall comply with Analysis method 136 (Slope assessment method) only. This shall apply as well to hybrid electric vehicles when tested without an operating combustion engine. Manufacturers of vehicles, according to this paragraph, shall provide a statement of compliance for ASEP, confirming that the vehicle complies with the specifications described in paragraph 3.5. of this Annex, when tested with any operation condition within the control range in paragraph 2.3. | Vehicles with propulsion technologies other than combustion engines shall comply with Analysis method 136 (Slope assessment method) only. This shall apply as well to hybrid electric vehicles when tested without an operating combustion engine. Manufacturers of vehicles, according to this paragraph, shall provide a statement of compliance for ASEP, confirming that the vehicle complies with the specifications described in paragraph 3.6. of this Annex, when tested with any operation condition within the control range in paragraph 2.3 | Could the content of footnote 36 not be added to the text itself? For readability this might be helpful. |
| OICA | Annex 7, 2.3. |  | … Vehicle speed VAA_ASEP: vAA ≥ 20 km/h Vehicle acceleration aWOT_ASEP: aWOT ≤ 5.0 m/s2 Engine speed nBB_ASEP: nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest The manufacturer shall take measures to achieve an acceleration aWOT_ASEP within the acceleration control range. Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. Vehicle speed VBB_ASEP: If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. | … Vehicle speed VAA_ASEP: vAA ≥ 20 km/h Vehicle acceleration aWOT_ASEP: aWOT ≤ 5.0 m/s2 Engine speed nBB_ASEP: nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest For vehicles subject to paragraph 1.1. of this Annex, the manufacturer shall take measures to achieve an acceleration aWOT_ASEP within the acceleration control range. Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. Vehicle speed VBB_ASEP: If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. |  |
|  | Annex 7, 3.1. |  | Determination of the anchor point The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows: In the case the test has been carried out with two gear ratios: Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i; nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio i reported from Annex 3; aanchor is the average of awot,rep of the 4 runs of the gear ratio i reported from Annex 3; In the case the test has been carried out in a single gear: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test; nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3. aanchor is the average of awot,rep of the 4 runs of the gear ratio selected for the test reported from Annex 3; aanchor is the determined acceleration and not restricted to 2.0 m/s² as provided from paragraph 3.1.2.1.4.3. Annex 3 for determination of the kp-factor." | Determination of the anchor point The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows: In the case the test has been carried out with two gear ratios: Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i; nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio i reported from Annex 3; aanchor is the average of awot,reptest of the 4 runs of the gear ratio i reported from Annex 3; In the case the test has been carried out in a single gear: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test; nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3. aanchor is the average of awot,rep of the 4 runs of the gear ratio selected for the test reported from Annex 3; aanchor is the determined acceleration and not restricted to 2.0 m/s² as provided from paragraph 3.1.2.1.4.3. Annex 3 for determination of the kp-factor.” In the case for vehicles subject to paragraph 1.1. of this Annex: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3, but limited to 60 km/h for further calculation. aanchor is the average acceleration awot test reported from Annex 3, as defined in paragraph 3.1.2.1.2. of Annex 3; aanchor is the measured acceleration and is not limited to 2.0 m/s² as provided from Annex 3 paragraph 3.1.2.1.4.3. for determination of the kp-factor.” | The vehicle speed at the anchor point is limited to 60 km/h. For some Type Approval Authorities and manufacturers, who do not want to limit the acceleration in Annex 3 WOT-test in regard to vehicle safety (no blocking of accelerator), the effect of moving the anchor point to higher speeds would lead to an unneeded burden. By limiting the speed this burden is also limited. |
| CH | Annex 7, 3.1. | te / ed | Determination of the anchor point The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows: In the case the test has been carried out with two gear ratios: Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i; nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio i reported from Annex 3; aanchor is the average of awot,rep of the 4 runs of the gear ratio i reported from Annex 3; In the case the test has been carried out in a single gear: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test; nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3. aanchor is the average of awot,rep of the 4 runs of the gear ratio selected for the test reported from Annex 3; aanchor is the determined acceleration and not restricted to 2.0 m/s² as provided from paragraph 3.1.2.1.4.3. Annex 3 for determination of the kp factor. | Determination of the anchor point The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows: In the case the test has been carried out with two gear ratios: Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i, measured with the vehicle in muted state; nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio i reported from Annex 3; aanchor is the average of awot,repref of the 4 runs of the gear ratio i reported from Annex 3; In the case the test has been carried out in a single gear: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test, measured with the vehicle in muted state; nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3. aanchor is the average of awot,repref of the 4 runs of the gear ratio selected for the test reported from Annex 3; aanchor is the determined acceleration and not restricted to 2.0 m/s² as provided from paragraph 3.1.2.1.4.3. Annex 3 for determination of the kp factor. |  |
| CH | Annex 7, 3.2.2. | ed | For vehicles subject to paragraph 1.1. of this annex, Slopeκ shall set to a constant value of 0,25 dB/km/h for further calculation. | For vehicles subject to paragraph 1.1. of this annex, Slopeκ shall be set to a constant value of 0,25 dB/(km/h) for further calculation. |  |
|  | Annex 7, 3.4. |  | Calculation of the sound level increase regarding performance For vehicles subject to paragraph 1.1. of this annex, the sound level ΔLacc,κj for measurement point j and gear ratio κ shall be calculated: For vBB test κ,j × awottest κ,j≤ vanchor× aanchor: ΔLacc κ,j =0 dB For vBB test κ,j × awottest κ,j> vanchor× aanchor: ΔLacc κ,j =[8dB] ×log10 vBB test κ,j ×awottest κ,j vanchor ×aanchor For all other vehicles the sound level ΔLacc,κj is set to 0 dB." | Calculation of the sound level increase regarding performance For vehicles subject to paragraph 1.1. of this annex, the sound level ΔLASEPacc,κj for measurement point j and gear ratio κ shall be calculated: For vBB, test, κ,j × awot, test, κ,j≤ vanchor× aanchor: ΔLASEPacc,κj =0 dB For vBB test κ,j × awot, test, κ,j> vanchor× aanchor: ΔLASEPacc,κ,j =[8dB] ×log10 vBB test κ,j ×awot,test, κ,j vanchor ×aanchor For all other vehicles the sound level ΔLASEPacc,κj is set to 0 dB." | To avoid misunderstandings the sound level increase regarding performance is renamed from Index acc to ASEP. So, the new variable is called Δ LASEP, since it temporary allows higher LASEP -Levels during performances higher than tested in Annex 3. |
| CH | Annex 7, 3.4. | ge | ΔLacc κ,j =[8dB] ×log10 vBB test κ,j ×awottest κ,j vanchor ×aanchor |  | [8db]:_ to be discussed: maybe 4? vBB test κ,j Is the vehicle speed not sufficiently covered under § 3.3.? |
|  | Annnex 7, 3.6. |  | Specifications Every individual sound measurement shall be evaluated. The sound level of every specified measurement point shall not exceed the limits given below: Lκj - ΔLacc κ,j ≤ LASEP_κ.j + x With: x = 3 dB(A) + limit value - Lurban for vehicle tested with non-locked transmission conditions x = 2 dB(A) + (limit value1 - Lurban), of Annex 3 for all other vehicles and only for vehicles - tested with non-locked transmission conditions given by multiple gear ratios or - having multiple electric propulsion sources or - having an Lcrs_rep greater than Lwot_rep of Annex 3: x = 3 dB + (limit value2 - Lurban) of Annex 3 If at any point the measured sound level at a point exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification." Footnotes: - As applicable for the approved type of vehicle - As applicable for the approved type of vehicle | 3.56. Specifications Every individual sound measurement shall be evaluated. The sound level of every specified measurement point shall not exceed the limits given below: Lκj - ΔLASEPacc, κ,j ≤ LASEP,κ.j + x With: x = 3 dB(A) + limit value - Lurban for vehicle tested with non-locked transmission conditions x = 2 dB(A) + (limit value1 - Lurban), of Annex 3 for all other vehicles and only for vehicles - tested with non-locked transmission conditions given by multiple gear ratios or - having multiple electric propulsion sources or - having an Lcrs_rep greater than Lwot_rep of Annex 3: x = 3 dB + (limit value2 - Lurbanurban) of Annex 3 If at any point the measured sound level at a point exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification." Footnotes: - As applicable for the approved type of vehicle - As applicable for the approved type of vehicle |  |
| CH | Annex 7, 3.6 | ge | Lκj - ΔLacc κ,j ≤ LASEP_κ.j + x […] and only for vehicles - tested with non-locked transmission conditions given by multiple gear ratios or - having multiple electric propulsion sources or - having an Lcrs_rep greater than Lwot_rep of Annex 3: x = 3 dB + (limit value - Lurban) of Annex 3 | Question: Is it really necessary to have both (x and ΔLacc κ,j )? Question: Is this (“having an Lcrs_rep greater than Lwot_rep of Annex 3”) a new exemption or was this already regulated somewhere else? x = 3 dB + (limit value3 - Lurban) of Annex 3 |  |
| CH | Annex 7, 3.6.1. | ge | If at any point the measured sound level exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification. | 3.6.1. If at any point the measured sound level at a point exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle not subject to paragraph 1.1. of this Annex is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification. For Vehicles subject to paragraph 1.1. of this Annex the measured sound levels at any point shall not exceed the limit in order to be in compliance with ASEP. | As ESES are freely controllable and we have at the moment two different sets of tolerances, those border lines should be complied with. |
| OICA | Annex 9 2.2. | ed | 2.2. “Exterior sound enhancement system" means a system that is installed to a vehicle for producing exterior sound, such as but not limited to sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit. | “2.2. "Exterior sound enhancement system" means a system that is installed to a vehicle for producing exterior sound, such as but not limited to sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit.” |  |

Additional documents provided by Switzerland in relation to UN-R 51:

SG-13-05-2025-07-30 proposal for a document for reference - CH.docx

SG-13-06-2025-07-30 Amendments to 138 - CH.docx

As applicable for the approved  type of vehicle