|  | E/ECE/324/Rev.1/Add.85/Rev.3 −E/ECE/TRANS/505/Rev.1/Add.85/Rev.3 |  |
| --- | --- | --- |
|  | SLR-74-11 | xx September 2025 |

Agreement

Concerning the Adoption of Harmonized Technical United Nations Regulations for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these United Nations Regulations*

(Revision 4, including the amendments which entered into force on 17 June 2025)

________________

Addendum 85: UN Regulation No. 86

Revision 3 – Amendment 4 / Revision 4

02 series of amendments – Date of entry into force: 22 June 2022 (Revision 3 – Amendment 4)

Supplement 1 to 02 series of amendments – Date of entry into force: 04 01 2023 (Revision 4 – Amendment 1)

Supplement 2 to 02 series of amendments – Date of entry into force: 17 06 2025 (Revision 4 – Amendment 3)

- usage of hyphenated “driving-beam” / “passing-beam” instead of “main beam” and “dipped beam” throughout the text, except in their definitions;  According to English editors “dipped beam”, “passing beam”, etc. should be without dash when beam is the main noun. However, if “dipped-beam”, “passing-beam”, etc. are used as an adjective for another noun, they should be with a dash, for example “dipped-beam headlamps”, “dipped-beam inclination”,

- Use the syntax “UN Regulation” whenever appropriate;above  In the text of a UN Regulation, it is enough to say “this Regulation” (instead of “this UN Regulation”)

- replace para. “above / below” with the relevant reference;

- assign numbers to Tables and Figures (like in Regs. 148, 149, 150);

- “retro-reflection”, “retro-reflector”, “side-marker”, “end-outline”  all with the hyphen

- “light signalling”  “light-signalling” with the hyphen

- “direction indicator lamp”  “direction-indicator lamp” with the hyphen (as per Par. 2.5.3.)

However, “direction indicators” remains unchanged

- always write “Annex to”;

- align written number in letters vs. number

- %  “per cent” (only once in Par. 6.19.7.6.)

- use scientific notation, as needed (e.g. luminous intensities)  see SLR-66-01 + lumens (not for lux)

- use “mm” instead of “m” unit, where suitable

- write units of measure (metres, lumens, candelas, lux, etc.) or their symbols (m, lm, cd, lx, etc.)

- Use figures in editable format, if available

- Delete “to this Regulation” after the reference to a specific Annex x, e.g. “Annex 3 to this Regulation”

- Replace footnote references to R.E.3 with new paragraph under Par. 2 (ECE/TRANS/WP.29/78/Rev.7)

- Reference to Rev.3 of 1958 Agreement in Device and Installation Regs.  Check CoP:

The conformity of production procedures shall comply with those set out in the Agreement, Schedule 1 (E/ECE/324-E/ECE/TRANS/505/Rev.3), with the following requirements :

- Check footnote repetitions (e.g. #11 and #16) and group them in one place

- Category vehicles / categories vehicles / vehicles in categories / vehicles of type, etc...  Vehicles of category(ies)… + delete “(off-road)”

- Color to colour, center to centre

- Indivisible between UN and Regulation, No. and the number, figure and units.

- Headlamps (6.1): remove some categories for light vehicles.

- Editorial alignment with UN Regulation No. 48.

- Same format of Clause 6. as in Regulation No. 48.

- Right reference of tables and figures.

- Ensure consistency in the number of the paragraphs referenced.

- Reference to R48 when referring to items defined in R48?

- Consistency of the 6.xx.4 Position between all functions

- Updated references to device regulation or simply in the 03 series of amendments?

Uniform provisions concerning the approval of agricultural or forestry vehicles with regard to the installation of lighting and light-signalling devices

This document is meant purely as documentation tool. The authentic and legal binding text is:

ECE/TRANS/WP.29/2024/92.

________________

UNITED NATIONS

UN Regulation No. 86

Uniform provisions concerning the approval of agricultural or forestry vehicles with regard to the installation of lighting and light-signalling devices

Contents

Page

Regulation

1.	Scope		4

2.	Definitions		4

3.	Application for approval		10

4.	Approval		11

5.	General specifications		12

6.	Individual specifications		18

7.	Modification and extension of approval of the vehicle type or of the installation of its lighting and light-signalling devices		48

8.	Conformity of production		48

9.	Penalties for non-conformity of production		48

10.	Production definitively discontinued		48

11.	Names and addresses of Technical Services responsible for conducting approval 
		tests, and of Type Approval Authorities		49

12.	Transitional provisions		49

Annexes

1	Communication		50

2	Examples of arrangements of approval marks		52

3	The relevant parts of Annex 3 of UN Regulation No. 48 apply		53

4	Visibility of lamps		54

5	Direction-indicator lamps – Geometric visibility		55

6	Signalling panels and signalling foils 		57

1.		Scope

This Regulation applies to vehicles of categories T, R and S, with regard to the installation of lighting and light-signalling devices.

2.	Definitions

For the purposes of this Regulation, the definitions given in the latest series of amendments to UN Regulation No. 48 in force at the time of application for type-approval shall apply, unless otherwise specified in this Regulation.

2.1.	Type definitions

2.1.1.	"Vehicle type with regard to the installation of lighting and light-signalling devices" means vehicles which do not differ in the essential respects mentioned in paragraphs 2.1.1.1. and 2.1.1.2.

The following are likewise considered not to be “vehicles of a different type”: vehicles

which differ within the meaning of paragraphs 2.1.1.1. and 2.1.1.2., but not in such a way as to entail a change in the kind, number, positioning and geometric visibility of the lamps prescribed for the vehicle type in question,

vehicles on which optional lamps are fitted or are absent, and

vehicles which are fitted with lamps, the position of which varies according to the direction of traffic in the country of registration.

2.1.1.1.	The dimensions and external shape of the vehicle;

2.1.1.2.	The number and positioning of the devices.

2.1.3.	The following are likewise considered not to be vehicles of a different type:

Vehicles which differ within the meaning of paragraphs 2.1.1. and 2.1.2. above, but not in such a way as to entail a change in the type, number, positioning and geometric visibility of the lamps prescribed for the vehicle type in question;

Vehicles on which optional lamps are fitted or are absent;

Vehicles which are fitted with lamps, the position of which varies according to the direction of traffic in the country of registration.

2.2.	Vehicle

2.2.1.	"Extreme outer edge" on either side of the vehicle means the plane parallel to the median longitudinal plane of the vehicle and touching its lateral outer edge, disregarding the projection:

2.2.1.1.	Of tyres near their point of contact with the ground, connections for tyre-pressure gauges and tyre inflating/deflating devices/ducts;

2.2.1.2.	Of any anti-skid devices mounted on the wheels;

2.2.1.3.	Of devices for indirect vision;

2.2.1.4.	Of side direction-indicator lamps, end-outline marker lamps, front and rear position lamps, parking lamps, side retro-reflectors and side-marker lamps;

2.2.1.5.	Of customs seals affixed to the vehicle and devices for securing and protecting such seals.

2.2.2.	Overall dimensions:

2.2.2.1.	"Overall width" means the distance between the two vertical planes defined in paragraph 2.2.1.;

2.2.2.2.	"Overall length" means the distance between the two vertical planes perpendicular to the median longitudinal plane of the vehicle and touching its front and rear outer edge, disregarding the projection:

(a)	Of devices for indirect vision;

(b)	Of end-outline marker lamps;

(c)	Of coupling devices, in the case of motorised vehicles.

2.3.	Lamps

2.3.1.	"Lamps of variable position" means lamps installed on the vehicle which can move in relation to the vehicle, without being detached.

2.3.2.	"Work lamp" means a device for illuminating a working area or process.

-

2.3.3.	"Signalling panel or signalling foil" means a device used to indicate to other road users the presence of a wide or long vehicle when viewed from the front, rear or side."

3.	Application for approval

3.1.	The application for approval of a vehicle type with regard to the installation of its lighting and light-signalling devices shall be submitted by the vehicle manufacturer or by his duly accredited representative.

3.2.	It shall be accompanied by the following documents and particulars in triplicate:

3.2.1.	A description of the vehicle type with regard to the items mentioned in paragraphs 2.1.1.1. to 2.1.1.2.; the vehicle type duly identified shall be specified;

3.2.2.	A list of the devices intended by the manufacturer to form the lighting and light-signalling assembly. The list may include several types of devices for each function. Each type shall be duly identified (component, type-approval mark, name of manufacturer, etc.), in addition, the list may include in respect of each function the additional annotation "or equivalent devices";

3.2.3.	A layout drawing of the lighting and light-signalling installation as a whole, showing the position of the various devices on the vehicle;

3.2.4.	If necessary, in order to verify the conformity to the prescriptions of the present Regulation, layout drawing(s) for each individual lamp showing the illuminating surface, the light-emitting surface, the axis of reference and the centre of reference. This information is not necessary in the case of the rear-registration plate illuminating lamp;

3.2.5.	A statement of the method used for the definition of the apparent surface.

3.2.6.	At the discretion of the manufacturer, a statement indicating whether lamps approved for and equipped with LED substitute light sources are allowed to be installed on the vehicle or not and, if this is allowed, which lamps.

3.3. 	An unladen vehicle fitted with a complete set of lighting and light-signalling equipment, as prescribed in paragraph 3.2.2., and representative of the vehicle type to be approved shall be submitted to the Technical Service responsible for conducting approval tests.

3.4.	The document provided in Annex 1 shall be attached to the type-approval documentation.

4. 		Approval

4.1. 	If the vehicle type submitted for approval pursuant to this Regulation meets the requirements of the Regulation in respect of all the devices specified in the list, approval of that vehicle type shall be granted.

4.2.	An approval number, in accordance with Schedule 4 of the 1958 Agreement (E/ECE/TRANS/505/Rev.3), shall be assigned to each approved vehicle type.

4.3. 	Notice of approval or of extension or refusal or withdrawal of approval or production definitively discontinued of a vehicle type pursuant to this Regulation shall be communicated to the Parties to the 1958 Agreement applying this Regulation, by means of a form conforming to the model in Annex 1.

4.4. 	There shall be affixed, conspicuously and in a readily accessible place specified on the approval form, to every vehicle conforming to a vehicle type-approved under this Regulation, an international approval mark consisting of:

4.4.1.	A circle surrounding the letter "E", followed by the distinguishing number of the country which has granted approval;

4.4.2.	The number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the circle prescribed in paragraph 4.4.1.

4.5.	If the vehicle conforms to a vehicle type-approved under one or more other Regulations annexed to the Agreement, in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 4.4.1. need not be repeated; in such a case the Regulation and approval numbers and the additional symbols of all the Regulations under which approval has been granted in the country which has granted approval under this Regulation shall be placed in vertical columns to the right of the symbol prescribed in paragraph 4.4.1.

4.6.	The approval mark shall be clearly legible and be indelible. The approval mark shall be placed close to or on the vehicle data plate affixed by the manufacturer.

4.7.	The approval mark shall be clearly legible and be indelible. The approval mark shall be placed close to or on the vehicle data plate affixed by the manufacturer.

4.8.	Annex 2 gives examples of the arrangements of approval marks.

*	Former titles of the Agreement:

Agreement concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958 (original version);

Agreement concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions, done at Geneva on 5 October 1995 (Revision 2).

Document ECE/TRANS/WP.29/78/Rev.[7]

The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 7, Annex 3 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html