|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 7. September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-seventh session

Geneva, 1–4 December 2025

Item Y of the provisional agenda

UN Regulation No. 134 (Hydrogen and fuel cell vehicles)

Proposal for supplement 3 to the 02 series of amendments to UN Regulation No. 134 (Hydrogen and fuel cell vehicles) *

Submitted by Germany on behalf of the GRSP Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Insert a new paragraph 0, to read:

"0.	Introduction

  - For supplement 3 to the 02 Series of Amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1 and Y1.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.".

Paragraph 1, footnote 2, amend to read:

"2	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), documentECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Insert new paragraphs 2.32 to 2.33:

"2.32.	"ADS feature of type 1 (ADSF-1)" means an ADS feature which includes an ADS fallback response requiring a fallback user

2.33.	“ADS feature of type 2 (ADSF-2)” means an ADS feature which does not include an ADS fallback response requiring a fallback user."

Paragraph 7.1.4.3.  amend to read:

“7.1.4.3.	If, during operation, a single failure results in a hydrogen concentration exceeding 3.0% by volume in air in the enclosed or semi-enclosed spaces of the vehicle, then a warning shall be provided or whilst the ADSF-2 is active, transmitted as a logic signal to the ADS (Paragraph 7.1.6.). If the hydrogen concentration exceeds 4.0% by volume in the air in the enclosed or semi-enclosed spaces of the vehicle, the main shut-off valve shall be closed to isolate the storage system. (Annex 5, Paragraph 3. test procedure).”

Paragraph 7.1.6.  amend to read:

“7.1.6.	Tell-tale Signal Warning to Driver or ADS

The warning shall be given by a visual signal or display text with the following properties:

(a) Visible to the driver while in the driver's designated seating position with the driver's seat belt fastened;

(b) Yellow in colour if the detection system malfunctions (e.g. circuit disconnection, short-circuit, sensor fault). It shall be red in compliance with section Paragraph 7.1.4.3;

(c) When illuminated, shall be visible to the driver under both daylight and night time driving conditions;

(d) Remains illuminated when 3.0% concentration or detection system malfunction exists and the ignition locking system is in the "On" ("Run") position or the propulsion system is activated.

Whilst an ADSF-2 is active, the warning shall be transmitted as a logic signal to the ADS.”

Annex 1 – Part 1 Model- III, Paragraph 0.4., footnote 2, amend to read:

"2	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), documentECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Annex 5, Paragraph 2. amend to read:

“2.	(...)

Prior to the crash impact, the sensors are located in the passenger and, luggage compartments of the vehicle as follows:

(a) At a distance within 250mm of the headliner above the driver's seat or near the top centre of the passenger compartment; for vehicles of category X, the most forward passenger seat shall be taken instead of the driver's seat;

(b) At a distance within 250mm of the floor in front of the rear (or rear most) seat in the passenger compartment;

(c) At a distance within 100mm of the top of luggage compartments within the vehicle that are not directly affected by the particular crash impact to be conducted.

(...)

This paragraph does not apply to vehicles of category Y.”

_________

II.		Justification

See paragraph 0.

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.