|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 7. September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-eighth session

Geneva, 1–4 December 2025

Item Y of the provisional agenda

UN Regulation No. 17 (Strength of seats, their anchorages and head restraints)

Proposal for supplement 1 to the 10 series of amendments, for supplement 2 to the 11 series and for supplement 1 to the 12 series of amendments to UN Regulation No. 17 (Strength of seats, their anchorages and head restraints)*

Submitted by Germany on behalf of the GRSP Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters. This document is superseding Informal Document GRSP-77-61.

- Proposal

x Insert a new paragraph 0., to read:

“0.	Introduction

0.1.	For supplement 1 to the 10 series, supplement 2 of the 11 series and supplement 1 to the 12 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2.	The Regulation was originally drafted for vehicles with driver and manual driving controls and it is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply."

Paragraph 1., amend to read:

“1.		Scope

This Regulation applies to:

(a)	Vehicles of categories M11 and N  with regard to the strength of seats and their anchorages and with regard to their head restraints;

(b)	Vehicles of categories M2 and M3 with regard to seats not covered by Regulation No. 80, in respect of the strength of seats and their anchorages, and in respect of their head restraints;

(c)	Vehicles of category M1 with regard to the design of the rear parts of seat backs and the design of devices intended to protect the occupants from the danger resulting from the displacement of luggage in a frontal impact.

It does not apply to vehicles with regard to side-facing or rearward-facing seats, or to any head restraint fitted to these seats, with the exception vehicles of category M2 and M3 of classes A and I, subject to the provisions of paragraph 5.1.1.

Vehicles of other categories may also be approved under this Regulation, in respect of the strength of seats and their anchorages, and in respect of their head restraints.

This Regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Annex 14, paragraph 3.1.7.8., amend to read:

“3.1.7.8.	The upper legs of the driver dummy shall rest against the seat cushion to the extent permitted by placement of the feet. Adjust the spacing of the legs so that the centreline of the knees and ankles is 200 mm (±10mm) apart and ensure that the knees are level.”

Annex 15, paragraph 1., amend to read:

“1.	Purpose

Procedures for folding or retracting head restraints in all designated seating positions equipped with head restraints, except the a driver’s designated seating position.”

_________

II.		Justification

See paragraph 0.

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.