Transmitted by VIAQ Chairs and Secretary                          Informal Document: VIAQ-32-09

Minutes (draft)

32nd Meeting of the GRPE Informal Working Group on VIAQ

| Date | Oct 13th 2025, 9.30 pm - 1.00 pm |
| --- | --- |
| Venue | Meeting Room H207, Palais des Nations, Geneva, Switzerland + Online (Teams) Meeting |
| Chairs | Chair: Dr. Andrey Kozlov (Russia) Vice-Chair: Ms. Inji Park (Korea) Secretary: Dr. Andreas Wehrmeier (OICA) |

- Welcome remarks.

Dr. Andrey Kozlov welcomed the participants to the meeting and thanked the participants for their contribution (see participants in Annex).

  - Review the minutes of the 31th meeting (VIAQ-31-10)

Dr. Andreas Wehrmeier presented the minutes of the 31st meeting. The minutes are approved with one change. The date of the meeting is corrected from 25th March 2024 to 25th March 2025.

  - Progress report (VIAQ-32-03)

Dr. Kozlov summarized the activities of the group in the progress report. He pointed out that the mandate of the group is terminated until November 2025 and the group will have to decide to finish the activities or to ask for prolongation at GRPE.

- Working items

2.1 Experiences with elaborated test method for VIAQ (VIAQ-32-04)

Mr. Markus Michael on behalf of CLEPA presented the results of a vehicle test according to VIAQ-31-08 document. He pointed out that they were basically able to follow the conditions of the test method, but the following points were not conformed.

- No new car was available.

- Data for temperature, pressure and humidity were not measured, but taken from online information.

- Average PM2.5 background concentration was below 15 µg/m³.

- Two different instruments for outdoor PM2.5 measurement were used for comparison.

- The pre-drive wasn’t done.

- They weren’t able to meet the required speed on the expressway due to traffic jams.

In their test drive around Stuttgart, they were able to measure concentration for PM2.5 between outside and cabin air

Mr. Michael concluded:

- average speed on expressway cannot be met in case of traffic jams;

- preconditioning is very time consuming and limits the number of results;

- time synchronization of measurement devices is crucial;

- logging of vehicle speed data from car was not possible in tunnels. GPS tracker can be an alternative, but difficult in tunnels;

- different PM2.5 detectors show very high deviation of test results, because the optical instruments are calibrated differently;

- there is no reference about good and bad results.

CLEPA is currently also doing test in India, which can be presented in a later stage.

2.2. Selection of HVAC Modes During Real-Road Driving Tests Based on Surveys and Experiments (VIAQ-32-05)

Prof. SungChul Seo on behalf of Korea Agency for Infrastructure Technology Advancement (KAIA) presented test results on the influence of HVAC settings on PM2.5, NO2 and CO2 concentrations in cabin air. He concluded that the HVAC settings have a significant influence on these pollutants in cabin air. In recirculation mode they found a fast rise in CO2 concentration, whereas PM2.5. and NO2 were very much depending on outside sources. He therefore recommended to include an option in the test method to also test HVAC condition in fresh air and recirculation mode.

Finally, Prof. Seo showed their new project to build an emission chamber, which can simulate outside air conditions and their ingress to vehicle cabin air.

2.3 Evaluation of “Draft Part IV of the Mutual Resolution (M.R.3) on Vehicle Interior Air Quality”. Measurements and recommendations by OICA. (VIAQ-32-06)

Dr. Jürgen Kranz on behalf of OICA presented results from their test according to the VIAQ-31-08 procedure. Like the experience from CLEPA the following conditions could not be met.

- Speed limits were not achieved.

- Background concentration of 15 µg/m³ in two different German cities were not achieved in mean value.

Dr. Kranz concluded from their measurements that:

- the test method gives a good guideline for measuring concentrations of PM2.5, NO2 and CO2 in cabin air;

- flushing time should be extended to 10 min to reduce CO2 level to background;

- PM2.5 background limit of 15 µg/m³ should be kept due to detection limits of the instruments, but the value cannot be achieved in many places in summer time, which will make testing of PM2.5 concentration impossible in these periods.

2.5 Draft Part IV of the Mutual Resolution (M.R.3) on Vehicle Interior Air Quality (VIAQ-32-07)

Considering the test and recommendations by CLEPA, OICA and KAIA the group decided the following amendments of the draft test procedure:

- Flushing time will be extended from 5 to 10 min.

- The length of the test run should be 25 km on urban roads and 25 km on expressways (deviation ±10%). The time for the full test run and exact milage should be documented. The speed should be monitored via vehicle reading or GPS logging.

- The standard HVAC Mode should stay “Auto Mode”.

- Measurement devices should be time synchronized within ±5 s.

- Calculation of test results should always be for the full test run.

- For substances concentration measurement it is essential that the same test instrument with equivalent calibration for inside and outside measurement is used. Only measurements with the same instrument can be compared.

The group also decided to add an Annex with optional test parameters that always must be clearly documented. Optional test parameters are:

- HVAC can be additionally operated in fresh air and recirculation mode.

- Older vehicle with mileage more than 15 000 km can be tested.

- Different from defined in subclause 9 outside air conditions need to be documented.

- Calculation of average concentrations of measured substances inside and outside the vehicle can additionally be done for parts of the test track like urban or expressway.

- Any other update of interior material emissions

No other topics are addressed

- Proposed agenda/topics for the next VIAQ IWG meeting

Dr. Kozlov will amend the document as decided and distribute to the group until end of October 2025. Members are then asked to review the documents and send feedback to Dr Kozlov.

Dr. Wehrmeier will schedule an online meeting by Mid November 2025 to review the comments and decide on the final document for submission to GRPE by end of November 2025.

After submitting the final test procedure, the group decided to pause the meeting activities and meet again in October 2026 for a review. Dr. Wehrmeier will then invite for an online meeting.

- Any other business

No points were raised.

- Next meeting schedule

Document feedback: Online meeting mid November 2025

Test procedure review: Online meeting October 2026

- Closing remarks

Dr. Kozlov thanked all participating parties for their contribution to create the three test methods for vehicle interior air quality. The group thanked the vice chair Inji Park and the secretary Andreas Wehrmeier for their work and gave a special thanks to the chair Andrey Kozlov for leading the work of this group to this final target.

Participants in on site:

Andrey Kozlov, Nami

Hyunwoo Lee,

C.R. Kim, Katri

Jaegyeong Lee, Katri

HYUNJUN JO

Andreas Wehrmeier, OICA

Francois Cuenot, UN

Su-Chui Seo, KAIA

Yang HoHye, KAIA

Jürgen Kranz, Audi

Vito Carlore, CLEPA

Participants online: