Inhalt

1	Open issues	3

1.1	Scope	3

1.2	Definition of fast charging	3

1.3	Family criteria (new)	3

1.4	SoC (high priority)	3

1.5	Recharged energy (high priority)	4

1.6	KPIs (high priority)	5

1.7	Preconditioning	5

1.8	Applicability	6

1.9	Test vehicle	7

1.10	Measurement tolerances and accuracies	7

2	Draft test procedure	7

3	Preamble	9

4	Purpose	9

5	Scope	9

6	(Normative references – should be moved to the preamble)	10

6.1	Netherlands	10

6.2	OICA	10

7	Terms and definitions	10

7.1	Definition of fast charging	10

7.2	Definition of family criteria	12

7.2.1	Test family criteria from Netherlands	12

7.2.2	Test family criteria from OICA	12

7.2.3	Test family criteria from JRC	12

7.2.4	Test family criteria from China	12

7.2.5	Parent family criteria from Netherlands	12

7.2.6	Parent family criteria from OICA	12

7.2.7	Other possible approaches on parent family	13

8	Abbrevations	13

9	Test cases	13

9.1	Overview of test cases from the Netherlands	13

9.2	Overview of test cases from OICA	13

9.3	China position on recharge curve	13

9.4	Overview of test cases JRC	14

10	General requirements	14

10.1	Vehicle manufacturer specifications	14

10.1.1	OICA	14

10.1.2	Netherlands	14

10.1.3	China	15

10.2	Specifications and tests – test system requirements	15

10.2.1	Netherlands	15

10.2.2	OICA	16

10.2.3	JRC comments	17

11	Determination of parameters	18

11.1	Parameters proposed by OICA	18

11.2	JRC	18

11.3	China	18

11.3.1	General comment on onboard parameters	18

11.3.2	SoC	18

11.3.3	Determination of UBE – Use of dyno?	19

12	PI‘s	19

12.1	Netherlands	19

12.2	OICA	19

12.3	KPI’s to consider (from drafting group coordinator) Charging efficiency (ISO) ….	19

PART A – Open issues and options for decisions

# Open issues

## Scope

Do we only cover DC charging or also AC?

For reference - The ToR of the F&SC cluster specifies under Task A : Development of a new WP.29 resolution on DC Fastest Recharge Power Curve (FRPC) of electric vehicles, hence the drafting team assumes that AC charging is excluded.

Since wireless charging is AC, this is also excluded from the scope.

BEV and PHEV however should be included according to the ToR.

Can the F&SC confirm these assumptions?

## Definition of fast charging

See chapter 6.1 of part A of this document.

OICA prefers to set a minimum power for DC fast charging to make clear what is understood as fast charging and what is not. This clarifies which vehicles are included or excluded from the scope.

Note that ISO has no specification on fast charging.

The sources in chapter 6.1 indicate a minimum power of 50 kW to label a charging cycle as fast charging.

Can the F&SC agree to set a minimum power for DC fast charging, and if yes, which value?

## Family criteria (new)

Proposal is to apply the worst-case principle, which means that the worst-case test vehicle is chosen to represent the family. The family itself is restricted by certain family criteria (yet to be determined, but including e.g. battery chemistry, capacity and pre-heating system). At the option of the manufacturer the family may be split into smaller subgroups.

Can the F&SC agree to the worst-case principle? The family criteria need further discussion within the drafting team, we will provide a proposal later.

## SoC (high priority)

Definition of the SOC start condition (SOC level and/or remaining range until the battery is empty)

Clear position is to take the SoC indicated to the driver, not the ‘technical’ SoC which may show additional battery margins that have been chosen by the manufacturer.

The issue is that the SoC is not a measurable parameter but a calculated one which is controlled by the manufacturer. It may be recalibrated while the vehicle is stationary or by degradation of the battery. This may lead to less than 70% of energy being recharged from an SoC of 10 to 80%. It also needs to be acknowledged that introducing an intentional offset in the SoC determination may bias the measured FRPC towards a better performance (overall, a charge from 8 to78% may be faster than a charge from 12 to 82%).

Does the F&SC accept the SoC signal as presented to the driver by the manufacturer as trustworthy, or is any kind of checking required to ensure that the reported SoC is correct?

How could SoC be verified/calibrated?

- Fast charge the vehicle from 10 to 80% SoC and continue to 100% to check if 20% SoC has been added

- Depleting battery to 0% and charging to 80%

- Do a normal charge at low power and compare recharged energy with SoC

- Fully charge the battery to 100%, deplete it to 10%, and then start fast charging from 10%

| Option | Pros | Cons |
| --- | --- | --- |
| Check if [≤20%] of UBE energy was added between [80 and 100%] SoC | This prevents a SoC offset to an interval [<10% to <80%] | Need to determine recharged energy from [10 to 80%] and from [80 to 100%], see issue 1.5 |
| Deplete battery to 0%, then charge to [80%] | The 0% SoC point is calibrated | No guarantee that [70%] of UBE was charged from [10-80%] SoC and/or if an SoC offset was applied (unless recharged energy is measured, see issue 1.5) |
| Apply a normal charge at low power, record SoC and recharged energy | The SoC vs. recharged energy curve prevents a SoC offset and ensures that [70%] energy is recharged from [10-80%] | Additional test burden to record the SoC vs. recharged energy. Need to determine discharged energy, see issue 1.5 |
| Fully charge battery to 100%, deplete to 10% | This ensures that the test starts at 10% SoC (no offset) | Test burden, the discharge may take a long time Need to determine discharged energy, see issue 1.5 |

OICA:

See ISO 12906, chapter 6.2.4:
The indicated SOC shall be obtained from the on-board measurement data of the EV or from the on-board indication.

NOTE The indicated SOC at any other interface available can be used for plausibility checks but can be subject to certain latencies.

NOTE: ISO does not check the SoC, yet a verification is added to observe if the SoC trajectory from 10% to 80% resulted in a recharged energy level of at least 69% (par. 6.3.2.5). This means that the recharged energy level has to be determined.

## Recharged energy (high priority)

Determination of recharged energy: measured power into the battery or DBE curve from the WLTP (See ISO)

OICA-comment: Is the determination of recharged energy necessary? > that depends on the decision regarding the KPI’s and on the verification if the recharged energy on the SoC trajectory of [10-80%] is at least [69%] of the UBE (See ISO par. 6.3.2.5)

Note: The UBE value is not declared in WLTP and not defined for PHEVs (this is likely to be amended soon in UNR 154).

Possibility is to deplete the battery via the bidirectional charging instead by driving on the dyno but this introduces an error and should therefore not be allowed. Discharging the battery should only be allowed by driving on the dyno while recording the energy versus SoC.

Does the F&SC see a need to measure recharged energy? This decision is interlinked with the issues of SoC determination (see issue 1.4) and KPI’s (see issue 1.6)

(Consideration from Iddo: At which locations do we measure energy?

## KPIs (high priority)

These are the KPI’s which are currently accepted by the drafting team:

Charging time from 10-80% SoC
Max charging power

Other options for KPIs that were mentioned during discussions:

- Recharged electric range (via recharged energy) in km after 10 min (covered by ISO

- Charging speed in average recharged distance per hour for the interval of [10-80%] SoC (not covered by ISO)

- Charging efficiency (covered by ISO)

- Recharged SoC in [10 minutes]

What does the F&SC see as information that is needed from a customer perspective? Should the information be limited to avoid information overflow, or complete to answer the needs of well-informed customers? Note that a decision on the KPI’s is urgently needed as this brings consequences to the test procedure, measured parameters, sample frequencies etc.

| KPI | Pros | Cons |
| --- | --- | --- |
| Recharged electric range (10 minutes) | KPI in ISO This KPI is better comparable as it includes the vehicle’s energy consumption | Need to determine recharged energy Based on WLTP energy consumption (not actual EC) |
| Average charging speed in recharged distance per hour over [10-80%] SoC interval | This KPI is better comparable as it includes the vehicle’s energy consumption | Need to determine recharged energy Based on WLTP energy consumption (not actual EC) KPI not in ISO |
| Charging efficiency in 10 minutes | KPI in ISO | Need to determine recharged energy |
| Charging efficiency (average) | Provides an overall efficiency rather than just the first 10 minutes | Need to determine recharged energy KPI not in ISO |
| Recharged SoC in 10 minutes | Relatively simple, no need to determine discharged energy SoC is a parameter the customer can better relate to than recharged energy | KPI not in ISO |

## Preconditioning

Use Case 1: Driving to fast charge facility with battery preconditioning on (reproducible, repeatable) According to the ISO procedure, the vehicle is preconditioned by a soak period of 12 hours at 23 degrees, followed by an optional phase of either 30 minutes driving between 25 and 30 km/h with battery preconditioning on (if installed) or a continued soak of 30 minutes between 0 and 30 degrees. According to the GreenNCAP procedure, the vehicle is soaked for 12 hours at 23 degrees, followed by a mandatory drive on the dyno on the WLTC middle and high phase or on the road for 15 minutes

Should the preconditioning phase consist of only soaking or soaking and driving? For maximum comparability the best option is to soak the vehicle prior to the test. Next, to honor the real-world use case of fast charging, we could also allow the vehicle to drive prior to the test and let the battery conditioning function allow the battery to be brought in a good state.

Does the F&SC support a preconditioning consisting of a soak of [12 hours] vehicle and then a mandatory drive the vehicle for [30 minutes]?

Note: it also needs discussion what this driving phase should look like. If we allow public chargers for this test procedure (see issue 1.8), the use of a dyno with a prescribed speed profile is not possible and the preconditioning speed should be low so it can be performed in the vicinity of the public charger (this is probably why the speed of 25-30 km/h was chosen for the ISO procedure). If we only test in the laboratory with a dyno, a speed profile could be applied or a higher constant speed.

For discussion: The test burden of soaking could optionally be left out if the manufacturer is convinced that the battery preconditioning will bring the battery at the optimum temperature for fast charging. This could be described in the test protocol as follows: “At the option of the manufacturer the soaking period may be omitted if the battery has a pre-warming system engaged during the preconditioning phase and the vehicle is not stored in a room above [23  degrees] in the [12 hours] prior to the preconditioning phase.”

## Applicability

Is the test executed in the lab or is it allowed to test outside (e.g. at a fast charger location)? In the latter case, a wider temperature range should be allowed (in ISO: 0 to 30 degrees) and a chassis dynamometer cannot be used for the execution of the test procedure.

Note: due to the high thermal inertia of the battery the wider temperature window will not cause any significant change of battery temperature in the 30 minutes following the soak period.

There are three options for the charging equipment:

- Public fast charger, and/or

- Fast charger in a lab environment, and/or

- Emulator in a lab

Does the F&SC allow public chargers to perform the test procedure or should this be done in a laboratory environment?

What lab equipment should be allowed for the FRPC test procedure?

Does the F&SC allow the use of an emulator or only a fast charger?

Note: once these decisions have been made, the drafting team will consider the requirements on that particular charging facility.

## Test vehicle

The run-in according to ISO12906 is to apply the run-in requirement of the applicable driving test (usually the WLTP procedure) or at least 300 km or the distance covered by one full charge. There is no maximum distance specified. The aim is that a relatively new vehicle is tested, not an in-use vehicle.

Should a maximum vehicle distance or battery cycle age be added? This should not be too strict as it then becomes difficult to obtain vehicles for testing. The maximum age could be based on the State of Certified Energy (SOCE) (UBE indicator) which refers to the drop-off with respect to the UBE at the beginning of life  (according to the UNR transposition of GTR 22, due to be adopted in October 2025). An eligible vehicle is proposed to show a value of at least [98] %.

Does the F&SC support adding an eligibility requirement for the test vehicle, for example a SOCE indication which shows that at least 98% of the original capacity is available?

## Measurement tolerances and accuracies

Sampling rate

Accuracy

Resolution

- See Table 2 ISO12906

This is dependent on the KPI’s (what to measure) and on the parameters that need to be measured. Without energy measurement we can have a lower sampling frequency (1 Hz) and with energy measurement it should be higher (10 Hz).

So first a decision is needed on the KPI’s, then the drafting team will prepare a proposal for the measured parameters including the sampling frequency, resolution and accuracy.

# Draft test procedure

This picture below is prepared by the JRC and meant to be used for discussion purposes, it is not the final test procedure.

PART B – Background information, normative references and stakeholder positions

JRC: Discussion points (proposal to start from – framework of discussion)

F&SC-04-03e document: Uniform provisions concerning the optimal charging power curve of light-duty electrified chargeable vehicles using direct current – Fastest Recharge Power Curve (FRPC)

# Preamble

# Purpose

From the 1st drafting meeting: The Fast and Smart Charging (F&SC) cluster aims at developing a provision concerning the charging power curve of light-duty vehicle electrified chargeable vehicles using direct current. The procedure to determine the power charging curve shall be both reproducible and repeatable, and representing real-world charging. This will provide consumers with reliable information on the FRPC and related KPIs.


OICA:

This Resolution provides a worldwide harmonized method to set and verify minimum performance requirements on DC fast charging of Pure Electric Vehicles (PEVs) and Off-vehicle Charging Hybrid Electric Vehicles (OVC-HEVs).

# Scope

OICA: This resolution applies to vehicles of categories 1-2 and 2, that have a technically permissible maximum laden mass not exceeding 3,855 kg, and to vehicles of category 1-1, that are PEV and OVC-HEV vehicles.

At the option of the Contracting Party, the scope may be limited to 3,500 kg for the relevant categories.

# (Normative references – should be moved to the preamble)

## Netherlands

GTR 15 (WLTP)/ UNR 154

GTR 22 (Battery durability LDV)

ISO 12906 (FRPC specific and complimentary source of requirements)

HDV Battery durability GRPE proposal, if applicable

## OICA

UNR 154 series 2 and 3 (WLTP)

GTR 22 (Battery durability LDV

ISO 12906

# Terms and definitions

## Definition of fast charging

Focus on DC-charging

Minimum power “fast charging” from a charging station point (s. Fig. 1 and 2):

  - P ≥ 50kW, no upper limit

“Fast charging” from vehicle view – tbd:

  - Fastest possible DC charging according to optimized charging conditions of the vehicle

  - Fastest possible DC charging according to selected charging conditions of the vehicle (e.g. precond on/off, battery-efficient charging, …)

Focus on what is possible in the vehicle:
"Fully charged, 100% state of charge (SOC), corresponds to the highest SOC which is achievable with normal operation of the vehicle", however, fast charging is typically terminated at 80% tbd SoC due to battery durability aspects.

Norming factor: UBE on the vehicle (defined by WLTP and GTR22)

Scope: BEV and PHEV

https://www.charin.global/media/pages/technology/knowledge-base/63d38591d7-1639130326/charin_dc_ccs_power_classes.pdf

Figure 1: From CharIn

Recharging systems | European Alternative Fuels Observatory

Figure 2: From EU-Com

## Definition of family criteria

### Test family criteria from Netherlands

Definition of vehicle family GTR 22, part B family as a basis

### Test family criteria from OICA

GTR22 Part B criteria for battery durability were used and adjusted for fast charging:

Battery and charging system design including thermal management and any other characteristics having a non-negligible influence on fast charging

(Declared maximum charging power (Value from OEM))

Type of battery (e.g., type of cell, including format, capacity (Ampere-hour), nominal voltage, nominal power  tbd by the drafting group) with focus on fast charging

Battery management system (BMS) with regards to fast charging monitoring and estimations

Operation strategy of all components influencing the fast charging

### Test family criteria from JRC

GTR No. 22 Part B family adjusted to consider the parameters related to the DC fast charging,

-any other characteristics having a non-negligible influence on fast charging

-BMS related to DC fast charging,

-thermal management of the battery, battery conditioning function? Vehicle with/without battery conditioning function in the same family?

In ISO/SAE 12906 optimizing features are in 6.2.1. General requirement, 6.3.2.2. Relocations (optimizing features not allowed), 6.3.2.3 Soaking phase (not allowed), 6.3.2.4 Preparation phase (optional)

(b)	Type of battery (dimensions, type of cell, including format and chemistry, capacity (Ampere-hour), nominal voltage, nominal power.

### Test family criteria from China

For the different-platform models

For the same-platform models

### Parent family criteria from Netherlands

Worst-case principle: test family member with expected worst performance

### Parent family criteria from OICA

In general worst-case principle

Definition of worst case may be related to second order characteristics like thermal management, HV-system (length of HV cables)

Remark: Could be very difficult to prove what is “worst case”

### Other possible approaches on parent family

GTR22: maximum energy consumption (witness testing, part of WLTP measurement cycle)

# Abbrevations

# Test cases

## Overview of test cases from the Netherlands

Charge power curve to cover the entire UBE spectrum from zero charge (0% SoC) to full charge (100% SoC)- FRPC to cover the full charging window from close to 0% to close to 100%

Consumer metric might focus on given charging window, eg. 10% to 80% or else

## Overview of test cases from OICA

Discuss charge power curve (handling, parameter, measurement, tolerances …) separately with more time - not part of the resolution, focus on 10-80% SoC  relevant for fast charging

Start Point and End Point for the fast charging procedure have to be defined

## China position on recharge curve

Is the full recharge curve measured (0-100%) or a limited one (e.g.20-80%)?
It is recommended to measure a 10-80% charging curve.

1.The ISO 12906 standard also measures a 10-80% charging curve.

2.Intercepting the charging curve of 10-80% from the charging curve of 0-100% is not an optimal curve.

3.Most users hardly discharge the vehicle to 0%. This situation does not occur frequently.

Figure 3: 10-80% vs. 0-100% SoC charging curves

## Overview of test cases JRC

To agree on the test case requirements and customer metric post-processing and KPI.

Not all the DC HP charging stations will allow to reach automatically the full charged status of the battery  due to battery protection systems, and unplugging and plugging again the vehicle is needed to complete the charge at lower power with the same station.

In the draft HDV GTR a provision has been added to take into account this and give instruction on how to charge at full.

To consider in case to prescribe the reporting on how the full charge was reached (i.e. with/without interruption, ...).

# General requirements

## Vehicle manufacturer specifications

### OICA

Required charging values (current, voltage, power and their duration) to be provided by OEM’s see also ISO/SAE 6.2.1 Vehicle manufacturer specifications

Measurement set up, tolerances and accuracies see ISO/SAE 6.2.3 Measurement tolerances and accuracies

### Netherlands

Condition for charging

- Break-off criterion: same as UBE from GTR 15 / ISO

- When reached, the battery is nearly empty (close to 0% SoC)

- End-of-charge criterion: same as for UBE from GTR 15 / ISO

- When reached, the battery is nearly full (close to 100% SoC)

Pre-conditioning, soaking, preparation

- Align with GTR 15 / GTR22 as much as possible

- Where GTR15 / GTR22 are not comprehensive enough, follow ISO 12906

- Allow for active/passive battery preconditioning (flag to be recorded)

- Need for vehicle dyno for preparation phase?

  - Is it possible to do without a dyno if UN GTR 15 / GTR22/ISO is used as a basis?

  - Validation test to confirm if vehicle dyno is required

  - Implication of dyno usage:

    - Higher resources needed

    - Restrict potential application of FRPC resolution to OEMs / certification bodies /technical services

### China

Measurement temperature 

What would be the appropriate temperature?

(Remark: Focus should be on battery temperature rather than outside temp)

| operating condition | China |  | Asia-Pacific |  |  |  | Europe |  | Middle East and Africa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Constant-temperature DC charging | 25±2 |  | 25±2 |  |  |  | 25±2 |  | 35±2 |
| High-temperature DC charging | 38±2 |  | 38±2 |  |  |  | 38±2 |  | 45/50±2 |

## Specifications and tests – test system requirements

1st Drafting meeting E-laad also posed the question whether a real market available charging station should be used during the testing or a more sophisticated but more controlled EVSE emulator should be used.
To specify this in the test system requirements can be an option

### Netherlands

EVSE specifications and requirements:

- Voltage: as recommended by vehicle manufacturer

- Current: [10%] more than max current from OEMs specs

Test system requirements:
- GTR15 where relevant (Annex 6, para 2.2.)
- Test location requirement from ISO 12906, para 6.3.2.5.

Sampling rate:
- [1]Hz / [20]Hz for energy (as ISO12906)

### OICA

Refer in general to ISO/SAE 6.3.2 Test procedure for test case Fast Charge

Start/stop conditions in ISO12906 chapter 6.3.2.5 for 10-80% SoC evaluation

Pre-cond/soaking/prep, focus on ISO12906 only
Conditioning on a dyno for charging possible, could be set as optional

EVSE requirements:
- Remove +10% current  not feasible for high charging current demand

Test system requirements:
-     Add ISO 6.3.2.3 soaking phase requirements

For test location requirements refer to ISO12906

Sampling rate: 
- 1Hz

### JRC comments

- Pre-conditioning, soaking, preparation:

- ISO preparation phase: optional

25-30km/h for a maximum 30 minutes or alternatively EV may be soaked for maximum 30 min at temp 0 -30 C

- Green NCAP: conditioning drive on dyno or on road for approx. 15 minutes

- UN GTR No 15:  pre-cond WLTC

- Allow for active/passive battery preconditioning (flag to be recorded)

- In ISO/SAE 12906

optimizing features 6.2.1. General requirement,

6.3.2.2. Relocations (optimizing features not allowed),

6.3.2.3 Soaking phase (not allowed),

6.3.2.4 Preparation phase (optional)

- To consider to prescribe the reporting on active/passive battery preconditioning and flag

- Need for vehicle dyno for preparation phase?

- Preparation phase prescribes different drive cycle in ISO/SAE, Green NCAP, GTR 15/22 etc…Dyno can also not be needed for preparation.

But:

- ISO/SAE 12906

It requires dyno for determining the DBE (discharged battery energy) versus SOC curve

This curve DBE vs SOC is needed to determine:

- the recharged usable battery energy within a reference time t_ref

- and the recharged electric range

For the recharged electric range calculation the EC_DC,WLTC is also needed as parameter. This is derived from the WLTP.

To discuss in the group based on KPI

Bidirectional power supply system may be used to deplete the battery during the preparation and to derive the curve DBE vs SOC. But the influence on the battery and systems during preparation is different. Moreover, for the curve DBE vs SOC an error is introduced.

- EVSE specifications and requirements:

- to consider to add provision on modular charging station, for charging efficiency related aspects

-Test system requirements:

- GTR 15 0.1 Hz Pressure, humidity, 0.033Hz temperature

- ISO 1 Hz SOC, Power, Energy depleting 20 Hz,

- Green NCAP , 1 Hz

- GTR 22, GTR 21

- Voltage, Current, 20 Hz during depleting, 0.033Hz during charging

- Room temperature 0.033Hz

- Time, ± 10 ms

- CAN bus data from vehicle?

- SOC monitor vehicle?

- Voltage, current, SOC, from charging station if possible?

- Voltage, current, SOC, man-in-the-middle or EVSE emulator?

- Moving average power window for the calculation of the maximum power?

- ISO the maximum charging power is the highest averaged power within a 30 s interval during the charging duration

- Ambient temperature,

- ISO 23C +/- 3 C on a 5 minutes running arithmetic average during soak with no systematic deviation from the set point,

- ISO 0-30C, 23C+/- 3 C recommended;  during charging phase, to resemble real-world conditions

# Determination of parameters

  - Parameters proposed by Netherlands

Power at the station (MP2 point of ISO 12906)

Energy: Station and vehicle/battery

SoC calculated: from energy at the battery and UBE

SoC declared: value on dashboard/ from on-board system
(SoC calculated from energy recharged and on-board SoC also collected)

Charging efficiency derived from total energy at the station vs. UBE (like ISO)

[Battery/cell] temperature

Activation flag for Battery pre-conditioning (heating/cooling active)

Others ?

## Parameters proposed by OICA

Time

Indicated SoC (declared SoC)

Electrical power

## JRC

## China

### General comment on onboard parameters

We believe that these on-board parameters can accepted as reliable parameters and do not need to be re-measured.

### SoC

Currently, the SOC is determined by the vehicle’s instrument display. Before testing, preprocessing is required, involving fully discharging and fully charging the vehicle to calibrate the accuracy of the SOC display.

### Determination of UBE – Use of dyno?

Is the UBE from the WLTP test procedure applied (UN R154) or is this established as part of the FRPC?

Is the use of a chassis dynamometer required?

The use of the UBE from the WLTP test procedure (UN R154) is not recommended

There is no need to use a dynamometer.

Figure 4: Use of dyno necessary?

# PI‘s

## Netherlands

Consumer metric might focus on given charging window, eg. 10% to 80% or else
Consumer info metrics to be proposed later
Energy recharged to be compared to declared UBE as part of GTR 15/ISO

## OICA

@ temperature = 23±3°C (tolerance from ISO12906)

Charging time from 10-80% SoC

Maximum charging power

Discuss charge power curve (handling, parameter, measurement, tolerances …) separately with more time - not part of the resolution, focus on 10-80% SoC  relevant for fast charging

## KPI’s to consider (from drafting group coordinator)
Charging efficiency (ISO) ….