Part 1 - Topics in square brackets

7.7.	[Over-the-air transmission of OBM data

7.7.1.	The manufacturer shall ensure that the contents of the OBM OTA outbox and other OBM data as required by part C of Annex 4 are transmitted over the air to servers under their control by taking cybersecurity measures in accordance with UN Regulation No 155.

7.7.2.	The manufacturer shall ensure that over-the-air data transmission of OBM data is performed by the OBM system as soon as connectivity conditions are appropriate.

7.7.3.	[The over-the-air transmission of OBM data may be delayed for vehicles being operated outside the territory of [a contracting party applying this regulation] until they are operated on the territory of [a contracting party applying this regulation] and the connectivity conditions are appropriate.] Filtering of data on contracting party level will be done by the OEM servers.

7.8.2.	The manufacturer shall ensure that the OBM system conforms to the standards specified in Appendix 1, paragraph 6.5.3. of UN Regulation No 154, Annex C5.

[Manufacturer shall use the most appropriate signals from the applicable standards for OBM parameters in Annex 4..]

7.9.	[Submission of anonymous OBM data

7.9.1.	The manufacturer shall collect all OBM data received over the air and submit it to the authority server, in anonymised form, using the infrastructure and common methods established to that effect.

7.9.2.	The manufacturer shall submit all OBM data received over the air in a calendar year to the authority server before the end of the second quarter of the year which follows the calendar year in which that data has been received.

7.9.3.	When the transmission of OBM data from a group of vehicles is impaired due to the obsolescence of over-the-air transmission hardware, the manufacturer shall notify the granting type-approval authority thereof without delay after the impairment.

7.9.4.	Upon request of a type-approval authority or the market surveillance authority, the manufacturer shall submit, in anonymised form, the OBM data, in their possession received over the air pertaining to specific OBM families or groups of vehicles with common family identifiers listed in part C of Annex 4.

This applies only to data which has not yet been transmitted to the authority's server.

7.9.5.	Authorities shall use anonymous OBM data submitted by manufacturers to support in-service conformity checks carried out in accordance with Paragraph 12.]

15.2.	By way of derogation OBM data submission shall only apply after the [availability of the authority server]. This OBM data transmission shall include the data collected in the previous two years.

Annex 1

OBM characteristics and information concerning the conduct of tests

If there are drawings, they shall be to an appropriate scale and show sufficient detail; they shall be presented in A4 format or folded to that format. Photographs, if any, shall show sufficient detail.

[

| 0 | GENERAL |
| --- | --- |
| 0.1. | Make (trade name of manufacturer): … |
| 0.2. | Vehicle type with regards to OBM: ... |
| 0.2.3. | Family identifiers: |
| 0.2.3.12. | OBM family(s): … |
| 0.4. | Category of vehicle (a): … |
| 0.8. | Name(s) and address(es) of assembly plant(s): … |
| 0.9. | Name and address of the manufacturer's representative (if any): … |
| 12.18 . | OBM System |
| 12.18.1. | Technical justification for the absence of manipulation devices and manipulation strategies in accordance with Annex 7. |
| 12.18.2. | Description of thresholds and parameters used to calculate the status of 'Aftertreatment operational status’. |
| 12.18.3. | Description of the functional operation characteristics OBM monitoring system. |
| 12.19. | EEEDWS system |
| 12.19.1 | Description of the functional operation characteristics EEEDWS system. |

]

Annex 1 - Appendix 1

Test Report

[

| 0 | GENERAL |
| --- | --- |
| 0.1. | Make (trade name of manufacturer): … |
| 0.2. | OBM Type: ... |
| 0.2.3. | Family identifiers: |
| 0.2.3.12. | OBM family(s): … |
| 0.4. | Category of vehicle (a): … |
| 0.5 | Manufacturer´s name and address: … |
| 0.6 | Technical Service responsible for carrying out the tests … |
| 0.7 | Date of test report … |
| 0.8 | Number of test report issued by that Technical Service … |
| 0.9 | Reason for extension … |
| 0.10 | Date of issue … |
| 0.11 | Last amendment from … |
|  | TEST REPORT |
| 1.1 | OBM system provides unrestricted access to OBM data |
| 1.2 | The EEEDWS has assigned the OBM monitoring status of NOx and particulate matter (PM) |
| 1.3 | OBM system has performed the evaluation of exhaust emissions of Nox at the end of the trip |

]

Annex 4

  - Prior to submission of OBM data to the authority server, the manufacturer shall append the family vehicle identifiers listed in Table 10.1 to enable authorities to determine the vehicle type, variant, applicable family designations and supplementary information applicable to the vehicle type-approval.

Table 10.1 Vehicle family identifiers

[

| Vehicle family identifier | Identifier |
| --- | --- |
| Vehicle type | Identifier of the vehicle type according to UN Regulation No. 154 |
| Vehicle variant | Identifier of the vehicle variant |
| Vehicle version | Identifier of vehicle version |
| Whole-vehicle type-approval code | Approval code of the vehicle type |
| PEMS family | Unique identifier for a grouping of vehicles with respect to PEMS |
| OBM family | Reserved |
| Part B Battery durability family identifier | Reserved |
| Date of manufacture | Date of manufacture of the vehicle |
| Worst case energy consumption of part B family | Reserved |
| Part A family identifier | Reserved |

]

If any family identifier is not applicable to the vehicle type ‘NONE’ shall be stated.

46

Annex 4 – Appendix 5

Battery durability parameters

C. Madec : Add-on for ACEA / OICA discussions

Annex 5

2.4.4.3.1	This option allows a countdown of distance once the inducement system starts. Engine restarts shall be prevented at the next new OBM trip after the vehicle has driven, with the engine running, a maximum distance of no more than the average driving range of one full tank of fuel since the activation of the repair inducement. Engine restarts initiated by the vehicle control system, such as start-stop systems, or engine restarts resulting from an engine stall condition shall be permitted after maximum distance has been exceeded. The average driving range of one full fuel tank shall be derived from the fuel consumption as determined from a Type 1 (4-phase WLTP  according to UN Regulation No. 154 test.

Annex 7

1.4.	In-service conformity checks may be performed by [Contracting parties] and recognised third parties.

4.4.	In-service conformity checks of OBM systems shall be conducted in parallel with the ISC tests for verifying compliance with tailpipe emission limits, as outlined in Annex 4 UN Regulation No. 83. The same emissions test (RDE according to UN Regulation No. 168 or Type 1 [4 phase WLTP according to UN RegulationNo. 154]) shall support both testing methodologies.

Annex 8

2.2	Where available, the EVP shall use [information from approval of the vehicle /documentation accompanying the vehicle].

4.2.4	EVP data shall be available in [English].

Annex 8 – Appendix 1

Environmental vehicle passport parameters

Table 1: General vehicle information

| Parameter | Identifier |
| --- | --- |
| Approval Certificate number | [Number of the approval certificate] |
| Date approval granted | [Issue date of the approval] |
| Make (Trade name of manufacturer) | [Code of the vehicle make] |
| Vehicle Identification Number (VIN) | [vehicle Identification Number] |
| Date of manufacture of the vehicle | [Date of manufacturing of the vehicle] |
| Commercial name | [Commercial name of the vehicle] |

Table 2: Vehicle family identifiers

| Parameter | Identifier |
| --- | --- |
| PEMS family identifier | [Unique identifier of the PEMS family] |
| OBM family identifier | Reserved |
| Battery durability family identifier | Reserved |

Table 3: Powertrain data

| Parameter | Identifier |
| --- | --- |
| Maximum net power (internal combustion engine) | [Maximum net power] |
| Maximum net power (electric motor) | [Maximum net power for electric engines] |

Table 4: Pollutant emissions data

| Parameter | Identifier |
| --- | --- |
| (WLTP highest values) CO, NOx, NMHC, THC, THC+NOx, Particles (mass), Particles (number) | [Values from WLTP Type 1 (4-phase WLTP according to UN Regulation No. 154)] |

Table 5: CO2 emissions /fuel consumption /electrical energy data

| Description | Identifier |
| --- | --- |
| CO2 emissions/fuel consumption/electric energy consumption, WLTP values (Low, Medium, High, Extra high, and Combined) | [Values from WLTP Type 1 ([4-phase WLTP according to UN Regulation No. 154])] |
| Electric range of pure electric vehicles Electric range | [Values from WLTP Type 1 ([4-phase WLTP] according to UN Regulation No. 154)] |
| Charge sustaining CO2 emissions/fuel consumption of OVC-HEV: WLTP values (Low, Medium, High, Extra high, City, Combined) | [Values from WLTP Type 1 ([4-phase WLTP according to UN Regulation No. 154])] |
| Charge depleting CO2 emissions/fuel consumption of OVC-HEV: WLTP values (Combined) |  |
| Electric consumption of OVC-HEV: WLTP values (Low, Medium, High, Extra high, City, Combined) |  |
| CO2 emissions/fuel consumption/electric consumption of OVC-HEV: WLTP values (Weighted, combined) |  |
| Equivalent range of OVC-HEV: Equivalent All electric Range | [Values from WLTP Type 1 ([4-phase WLTP according to UN Regulation No. 154])] |

Annex 9

[1.1.1.	If the display has no home or default screen (e.g. a display which opens always on the last screen displayed), with the approval of the Type Approval Authority, manufacturer may use another appropriate counting method for the five screen limit.]

Annex 10

2.2.	Manufacturers shall not introduce software or calibration updates that manipulate data related to sensors, fuel or electric energy consumption, electric range or battery durability. The handling of such data in the manner required by [this and other applicable regulations] shall not constitute manipulation.

2.5.	Manufacturers shall not manipulate data from the OBM systems or OBFCM devices before submission to authorities or dissemination to other parties. The handling of such data in the manner required by [this and other applicable regulations] shall not constitute manipulation.

Part 2 – Other topics

7.1.1.	The manufacturer shall ensure that the OBM systems perform the functions as laid down in paragraphs 7.2. to 7.8. by means of hardware and software installed in the vehicle for as long as the vehicle is in use.

[Requirements in 7.7 do not apply later than 20 years after the date of manufacturing of the vehicle.]

---------------

7.10.5.	In the case of a multi-stage type-approval, the manufacturer referred to in paragraphs 7.10.1 to 7.10.4 shall be understood as the manufacturer of the base vehicle, and the EVP data shall refer to the base vehicle.

---------------

SIA and therefore the working document uses “omitted” in several places. Not all signals can be omitted. Applies only to those, which are not in data structures.

---------------

Some paragraph numbers do not end with a “.”

Annex 4 2.3 ff

Annex 6

Annex 8

Annx 9 2.1ff

---------------

7.8.3	For paragraph 7.8. the deficiency provisions of UN Regulation 154, Annex C5 Section 4 apply.

Requirements relating to the type approval of On-board monitoring (OBM) system

4.1. A manufacturer may request to the Type Approval Authority that an OBM system be accepted for type approval even though the system contains one or more deficiencies such that the specific requirements of this regulation are not fully met. The Type Approval Authority may approve up to two separate components or systems with one or more deficiencies. When a manufacturer adopts specific conditions for misfire defined in paragraph 3.3.3.2.1. of this annex, these conditions shall not be considered as a deficiency.

4.2. In considering the request, the Type Approval Authority shall determine whether compliance with the requirements of this annex is infeasible or unreasonable. The Type Approval Authority shall take into consideration data from the manufacturer that details such factors as, but not limited to, technical feasibility, lead time and production cycles including phase-in or phase-out of engines or vehicle designs and programmed upgrades of computers, the extent to which the resultant OBM system will be effective in complying with the requirements of this Regulation and that the manufacturer has demonstrated an acceptable level of effort towards compliance with the requirements of this Regulation.

4.2.1. The Type Approval Authority shall not accept any deficiency request that includes the complete lack of a required diagnostic monitor or the lack of mandated recording and reporting of data related to a monitor.

4.2.2. The Type Approval Authority will not accept any deficiency request that does not respect the OBD thresholds set out in paragraph 6.8.2. of this Regulation.

4.3. In determining the identified order of deficiencies, deficiencies relating to paragraphs 3.3.3.1., 3.3.3.2. and 3.3.3.3. of this annex for positive ignition engines and paragraphs 3.3.4.1., 3.3.4.2. and 3.3.4.3. of this annex for compression-ignition engines shall be identified first.

4.4. Prior to or at the time of type approval, no deficiency shall be granted in respect of the requirements of paragraph 6.5., except paragraph 6.5.3.5., of Appendix 1 to this annex.

4.5. Deficiency period

4.5.1. A deficiency may be carried-over for a period of two years after the date of type-approval unless it can be adequately demonstrated that substantial vehicle hardware modifications and additional lead-time beyond two years would be necessary to correct the deficiency. In such a case, the deficiency may be carried-over for a period not exceeding three years.

4.5.2. A manufacturer may request that the Type Approval Authority grant a deficiency retrospectively when such a deficiency is discovered after the original type-approval. In this case, the deficiency may be carried-over for a period of two years after the date of notification to the Type Approval Authority unless it can be adequately demonstrated that substantial vehicle hardware modifications and additional lead-time beyond two years would be necessary to correct the deficiency. In such a case, the deficiency may be carried-over for a period not exceeding three years.

4.6. At the request of the manufacturer, a vehicle with an OBM system may be accepted for type-approval with regard to emissions, even though the system contains one or more deficiencies such that the specific requirements of this annex are not fully met, provided that the specific administrative provisions set out in section paragraph 3 of this annex are complied with. The Type Approval Authority shall notify its decision in granting a deficiency request to all other Contracting Parties to the 1958 Agreement applying this Regulation.

---------------