# October workshop – IWG DDADWS 9

Draft Minutes

### Monday, 13th October – Wednesday, 15th October 2025

### CLEPA main office, Brussels

Chair: 		Australia:	Dr. Linda Rasmussen

Secretariat:	OICA:		Mr. Simone Bambagioni

Participants:

Physical participation: Australia, CLEPA, EuroNCAP, JASIC, OICA (incl. FEBIAC, JAMA), Seeing Machines, UK, University of Birmingham, University of Toronto, UTAC.

Online participation: Canada, CLEPA, OICA (incl. Australia TIC), TÜV.

### Monday, 13th October 2025 – 13:00 to 17:00

13.00-13.30 – Welcome and introductions

Welcome by the chair Dr Linda Rasmussen, Director, Vehicle Standards section, Australian Government Department of Infrastructure, Transport, Regional Development, Communications, Sport and the Arts

The chair welcomed the group and noted that the strong emphasis on research for this workshop was due to the success of the first workshop. This provides a good basis for determining improvements in the 01 series of amendments to the DDAW regulation.

She also appreciated the recent GRSG outcome, highlighting satisfaction with the adoption of both the 00 series of ADDW and DDAW regulations, and thanking everybody for their good cooperation in achieving this outcome.

Approval of the reports of the previous sessions

Document:  DDADWS-08-17 (Secretary) Draft Minutes

The draft minutes of the previous session were approved with no objections.

Approval of the agenda

Document:  DADWS-09-02 (Chair) October Workshop - Draft Program

The draft agenda of the workshop was approved with no objections.

13.30-15.30 - Observer Rating of Drowsiness (ORD) – literature review and assessor training

Yohei Iwashita (JASIC) - Usefulness of ORD based on literature review

JASIC, on behalf of Japan, presented an overview of ORD’s usefulness based on a recent literature review.

Mr. Schramm (OICA) noted some uncertainty when using ORD to assess lower levels of drowsiness, specifically regarding how the ORD D1 level corresponds to KSS levels 1–5. He questioned whether this lack of clarity represented a limitation of the method. Japan agreed that more empirical research is necessary to confirm its reliability.

Mr. Bohdan (CLEPA) commented that a small loss of resolution at lower levels of the scale can be acceptable. He added that, unlike the KSS – which depends on the driver’s own perception and mood – ORD relies on external observation and is therefore more objective.

Mr. Vukovic (OICA) stated that keeping ORD as an alternative method only may be unnecessary, as ORD serves the same purpose as KSS. He suggested the regulation could allow either scale to be used equivalently, as chosen by the manufacturer. He also noted that KSS levels below 5 are rarely used in practice, while ORD tends to be more intuitive for assessors.

Dr. Lenne (Seeing Machines) expressed a more cautious view, noting that people who are drowsy do not always appear visibly drowsy, which can lead to inconsistency in observer ratings. He stated that some manufacturer’s systems would be disadvantaged if based on ORD.

Ms. Johansson (OICA) stressed the importance of specifying which scale is used, pointing out that other observer-based scales, such as the Observer Rated Sleepiness (ORS), also exist and may differ in their approach.

Australia raised a question about the information presented on a 2024 study which used EEG as a ground truth and for which 9 levels applied on the ORD scale, noting that this was also reported as having a very good concordance rate. He wondered why 9 ORD levels were used, but other testing and the Japanese proposal have 5 levels. It was answered that this was not a Japanese study, and that it would need to be further examined.

Prof. Anderson (University of Birmingham) noted that KSS correlates strongly to relevant factors such as sleep and crash risk, and asked Japan what evidence would be considered sufficient to confirm that ORD can actually predict drowsiness level e.g. ORD v’s a particular outcome or vehicle behaviour. She further stated that other research shows ORD was only able to correlate to a SDLP outcome approximately 40% of the time, and so questioned whether ORD should be further tested to show for example that it can accurately detect known drowsy drivers.

Ms. Johansson (OICA) stated that some studies that have been referred to used ORS in testing, not ORD, and therefore others need to be certain on which scale was used when referring to research.

Takashi Osaki (JAMA) - Practical methods of ORD measurements and assessor training

JAMA delivered a presentation on practical methods for conducting ORD measurements and for training assessors. It noted that the assessment of drowsiness includes continuous monitoring of the area around the eyes, and also the area around the mouth and the facial expression.

UK asked whether age influences the assessment results. JAMA responded that the studies conducted so far showed similar results across different age groups, suggesting that age does not significantly affect the method’s consistency.

15.30-16.00 Afternoon tea

Prof. Birsen Donmez (University of Toronto) gave a short presentation highlighting potential issues identified in the EU Regulation formulas for calculating acceptance criteria (in Annex 4 – Appendix 1 of the proposed UN Regulation). She noted that the formula used in paragraph 9.1. (b) might need to be reviewed, and/or the description provided by the text of this paragraph may need to be amended to reflect that the criteria applies to the true population rather than the test participants.

Australia confirmed that the formulas implemented in the UN regulation were carried across from the EU regulation, in accordance with the agreed approach for the 00 series of the regulation. He noted that the EC FAQ document for the EU regulation explained a late change to the formula in the EU regulation, but the origins of the performance requirement including intention would need to be confirmed by those involved in the development of the EU regulation. The chair asked for any input accordingly, with no comments being provided.

16.00-17.00 - DDAW System HMI and warning strategy for the driver (combination and escalation)

Peter Burns (Transport Canada) - The Anatomy of Effective Attention Warnings: Building a Framework Beyond Beeps and Icons

Canada presented the work “The Anatomy of Effective Attention Warnings: Building a Framework Beyond Beeps and Icons.”

One aspect raised by Mr. Burns was that a dedicated effort to achieve conformity regarding system warnings and HMI across UNECE regulations should be considered.

Mr. Yamashita (CLEPA) recalled that during the previous GRSG meeting, Korea had raised similar concerns about the lack of clear guidance on this topic. Canada agreed and suggested that developing a concept of “master-type warnings” and defining levels of persistence and customization could help improve system design.

Ms. Johansson (OICA) proposed exploring the possibility of combining drowsiness and distraction warnings, as distracted drivers are usually aware of their behavior or know they are not fit to drive. She was not in agreement with separate warning strategies for drowsiness and distraction. She also mentioned that a new ISO symbol for inattention, intended to cover both drowsiness and distraction, is under development (ISO WG5 on ISO 2575). She gave an example of a warning message that only relates to the high-level issue; e.g. “Focus on driving”. She also mentioned concern over the potential impact of changes in regulatory text on areas such as detection algorithms and prioritization of warnings.

Canada replied that distraction and drowsiness should be treated differently: distraction is a temporary state that can often be corrected after a single alert, whereas drowsiness is more persistent and requires further action by the driver.

A discussion followed between Canada and OICA about whether a single warning approach could work for both cases. Dr. Lenne emphasized the need to distinguish between transient and non-transient states. Referring to the idea that detection is the main challenge while warning design is comparatively “easy,” he argued the opposite — that the warning strategy for drowsiness is highly complex, especially in defining escalation logic and requirement for a recurring warning as opposed to a one-off event. Canada agreed, underlining that escalation is a crucial element missing from the current draft regulation and should be further developed.

UK reminded that UN R121 does not address several characteristics relevant to DDAW, such as warning location. He stressed the need to ensure consistency across related regulations, recalling that a previous GRSG attempt to standardize symbols had resulted in only six common ones due to differing interpretations. Canada noted that the desired outcome would be a standardized approach to setting requirements for system warnings.

Prof. Claire Anderson (University of Birmingham) stated that drowsiness and distraction are definitely very different problems, and proposed using feedback-based approaches like fitness trackers, providing drivers with constructive feedback on their attention state. Canada welcomed the idea, recognizing its potential to make driver alerts more engaging and effective.

### Tuesday, 14th October 2025 – 09:30 to 17:00

9.30-10.30 - Inclusion of microsleep detection by the DDAW system

Mike Lenne (Seeing Machines) - Incorporating microsleeps with drowsiness as a strategy to further reduce the risks of drowsy driving

Dr. Lenne (Seeing Machines) delivered his presentation outlining the importance of detecting microsleep episodes to reduce the risks of drowsy driving. He emphasized that the increase in drowsiness during transition from wakefulness to sleep is difficult to detect in comparison to detection of microsleeps.

OICA requested several clarifications on the data and charts used in the study.

Mr. Vukovic (OICA) suggested that microsleep-related incidents should be reported as a percentage of all crashes, to avoid giving a misleading impression of their relative frequency, and that countermeasures might need to focus on specific driver groups rather than being generalized to the entire driving population.

Replying to the UK’s question, Dr. Lenne clarified that, based on available studies, no significant gender-related differences had been identified. UK also raised a point about driver acceptance, noting that persistent or repeated warnings could become annoying or lose effectiveness over time. He asked whether acceptability had been evaluated, and about any consideration of the possible concerns of driver’s about being “spied on” by the monitoring system. Dr. Lenne replied that acceptability depends on the type of solution used, and that regulatory guidance could help define what information should be stored or discarded, especially to ensure privacy protection.

Ms. Maiwald (CLEPA) remarked that drowsiness typically precedes microsleep, suggesting that efforts should focus on improving early drowsiness detection to prevent microsleeps altogether, including because during an eye closure event of several seconds a lot can happen. Dr. Lenne agreed, noting that achieving reliable detection of drowsiness is very difficult and remains a gradual process. He emphasized that the key challenge is determining how to act effectively in the meantime, acknowledging that while prevention is ideal, parallel strategies can still be implemented to mitigate risk during system development and refinement.

10.30-11.00 Morning tea

11.00-12.00 - DDAW System monitoring strategy and warning delay performance

Clare Anderson (The University of Birmingham) - DDAW system monitoring strategy and warning delay performance

This presentation will consider aspects such as:

- Should an activated DDAW system continuously monitor the driver after providing the first warning, and subsequently continue to warn the driver, even though drowsiness is not typically a transient state (unlike distraction)?

- Would the implementation of a continuous monitoring strategy require considerations of factors such as likelihood of false positive warnings, factors influencing likely self-assessment by the driver including elapsed time driving when the warning is first given, the possibility that the driver may have been changed during a single trip?

- How long is too long for the delay between a driver becoming drowsy (KSS = 8) and the system detecting this drowsiness? For example, should the drowsiness regulation consider any data on the time-based safety risk increase for having an accident under continuous driving at a nominal drowsiness level?

Prof. Anderson presented the first of her two presentations at the workshop.

Following OICA’s request for clarifications, a short exchange took place between Prof. Anderson and OICA representatives to clarify several aspects:

- Near-crash events were recorded when the instructor intervened by pressing the brake pedal to avoid an imminent collision.

- Lane departures were defined as unintentional crossings of lane markings with at least two wheels.

- All feedback-based safety systems (such as lane-keeping and emergency braking) were deactivated to allow an independent assessment of the DDAW system.

OICA underlined that DDAW systems are intended to support, not replace, driver control, noting that many vehicles already include active assistance systems such as lane departure warning, lane-keeping, advanced emergency braking.

Note: allowing lane departures and near-crash events enables data recording about the state of the driver and their ability to safely operate the vehicle as a function of time and KSS level, as whenever the driver has responsibility for the overall driving task (including while ADAS are active), drowsiness is a road safety problem.

OICA also added that driver acceptability remains a key challenge, and stressed the need for DDAW systems to help change driver attitudes toward fatigue over time, rather than only influencing immediate driving behavior. Prof. Anderson agreed, noting that achieving both high accuracy and strong user acceptance is complex, since early warnings can increase false positives and reduce trust in the system.

Ms. Johansson (OICA) noted that her experience with professional drivers had showed different outcomes in that there was not high acceptability of alarms. She expressed concern about the possibility of a negative attitude towards immature DDAW systems resulting in a similar attitude towards other systems such as AEB, LDW. She also noted that requirements on PPV may not be easy to achieve in all cases.

UK pointed out that the newly adopted 00 series regulation still allows drivers to manually deactivate the warning system and asked whether this should be permitted. Prof. Anderson replied that, from a scientific safety perspective, deactivation should ideally not be allowed, although she recognized that regulators must balance safety with user autonomy.

Mr. Schmidt (TÜV) emphasized the need for more research involving everyday drivers under normal conditions, as most current validation studies are designed to induce fatigue. The regulation would need to be suitable for the broader general public including alert and non-professional drivers. Prof. Anderson noted that source studies did include participants with a range of prior sleep duration including up to 8 hours, but that more assessment would be needed to increase robustness.

Mr. Vukovic (OICA) asked how effective oculometry measures are for detecting microsleeps. Prof. Anderson responded that assessing microsleeps through eye metrics alone is very difficult. Laboratory studies typically use EEG to validate microsleep events, but such equipment is impractical in vehicles due to noise and motion artifacts. She explained that while long eye closures often overlap with microsleeps, they are not always equivalent, and further research is needed to quantify the relationship. Mr. Vukovic also asked whether studies showed what systems or technologies were better than others, and Prof. Anderson stated that there were no “silver bullets”.

In conclusion, Prof. Anderson confirmed that the study population consisted entirely of professional drivers. She noted that while the findings offer strong insight into fatigue behavior, future research should broaden participation to reflect the general driving population and everyday conditions. She also confirmed that the alarm types included auditory, visual and haptic.

12.00-13.00 Lunch

13.00-14.00 - Interventions with drowsy drivers

Birsen Donmez (The University of Toronto) - Introducing driving automation to assist in vehicle control and introducing a cognitive task to increase driver arousal

Prof. Birsen Donmez (University of Toronto) delivered her presentation focused on the interaction between drowsiness, automation, and intervention with cognitive tasks, examining how partial automation (SAE L2 and L3 systems) and cognitive tasks can influence driver alertness and fatigue.

Following CLEPA’s request for clarifications on how the L2 systems were activated, Prof. Donmez confirmed that both longitudinal and lateral control functions were active, effectively reducing the driver’s task load. She noted that this can lead to an unintended consequence: when automation assumes most driving functions, drivers tend to relax and become more prone to drowsiness, as observed in several real-world cases. She added that automation was likely abused in some cases, with drivers not sufficiently engaged or able to immediately perform the driving task.

Mr. Savona (UTAC) asked whether the system could still be effective at very high levels of drowsiness, to which Prof. Donmez replied that such cases were not included in the experiments. She conceded that the interventions were likely not sufficient for high drowsiness levels where warnings from the vehicle are needed, and instead is intended to delay the onset of fatigue.

14.00-15.00 - DDAW System test methodology for validation by the manufacturer

Clare Anderson (The University of Birmingham) - DDAW Test Methodology: Evidence-Based Best Practice – Part 2

This presentation will consider aspects such as:

- Should there be requirement for a minimum number of test runs per participant to ensure statistical significance, as opposed to being able to use a given participant once, but another participant multiple times?

- Should validation testing with human participants continue after the first True Positive warning is given, and if so what requirements would determine the extended testing procedure?

- How does the test bin change the outcome? How long should it be, and should the bins before/after the warning be considered, and why?

- How should positive and negative (fatigue) states be induced during the test procedure for validation?

Prof. Anderson delivered the second of her presentations.

CLEPA questioned whether the positive and negative assessments presented were determined according to the DDAW regulation or based solely on experimental data. Prof. Anderson explained that her analysis relied primarily on KSS as the ground truth and that the current regulatory framework had not yet been fully tested with the dataset, due to its complexity and multiple conditional statements.

Participants also recalled that when the regulation was originally drafted, some provisions had been shaped by practical experience and variability among drivers, acknowledging that individual reactions to drowsiness differ widely and that the existing structure represents an average approach.

A brief exchange between OICA and Prof. Anderson took place to clarify several points, such as the following:

- Drowsiness peaks during night driving are mainly due to a combination of circadian rhythm and prolonged wakefulness.

- The data of the study came from test track experiments using fully instrumented vehicles, supervised by safety instructors.

Ms. Johansson (OICA) further observed that the sequence of fatigue-related events (such as lane departures and microsleeps) can vary greatly between individuals, making it difficult to define a universal metric. Prof. Anderson agreed, emphasizing the importance of improving detection methods within the limits of current technology while continuing to explore earlier and more reliable fatigue indicators.

Australia firstly thanked Prof. Anderson for the presentation and noted that by addressing some specific questions posed in advance by Australia for consideration, it had provided very useful information to assist with developing the 01 series of amendments. He then provided a clarification regarding paragraph 4.1 of the DDAW test procedure (Annex 4 – Appendix 1), explaining that the requirement for a minimum of 10 events that are either true positive or false negative refers to the total number of events across all participants—not 10 per participant as appeared to have been inferred from the text of this paragraph in isolation. Each participant should continue testing until reaching the drowsiness threshold, thereby producing at least one true positive or one false negative result. Both Prof. Anderson and Prof. Donmez confirmed that this resolved some earlier confusion they had regarding this paragraph. CLEPA added that, according to the regulation, a KSS = 7 can be classified as a true positive, reinforcing the importance of consistent application of the defined thresholds when analyzing test data.

15.00-15.30 Afternoon tea

15.30-16.30 – Driver monitoring requirements in EuroNCAP

Adriano Palao (EuroNCAP) - Differences in requirements between EuroNCAP and the EU regulation for drowsiness, and the reasons for why EuroNCAP has different requirements

Mr. Palao of EuroNCAP delivered his presentation outlining the main components of the 2026 EuroNCAP protocol setting Driver State Monitoring system requirements for detection of driver drowsiness, and associated warnings and interventions. The presentation did not closely compare the protocol to the EU regulation. Useful additional information included:

- Systems using indirect means will continue to be eligible to score points where possible.

- The 2023-2025 campaign assessments comprised approximately 60% indirect types, 15% combined indirect and direct, 25% direct.

- There has been a recent increase in systems using direct means, with many not implemented well for validation performance.

- All OEMs used KSS to establish detection performance, with validation testing typically involving around 10 participants in a simulator or on a test track.

- Many OEMs simply submitted their approval against the DDAW regulation as evidence of the system performance, whereas the more detailed data and documentation was needed.

- Future milestones for EuroNCAP include to undertake testing of vehicles until the point where a driver falls asleep, thereby allowing assessment of the system’s ability to provide suitable warning well in advance of this point, as well as undertaking a 2,000 km on-road evaluation of vehicles to assess driver acceptance issues with system warnings.

Dr. Lenne (Seeing Machines) opened the discussion by highlighting that the topics addressed by EuroNCAP closely overlap with those discussed in the IWG including the objectives of industry and researchers. He also asked how EuroNCAP’s upcoming requirements might influence drowsiness detection strategies. Mr. Palao explained that, starting from July next year, all new vehicles will be equipped with driver monitoring cameras, primarily intended for distraction detection but with clear potential to support drowsiness assessment as well e.g. improving legacy indirect systems. He noted that when Euro NCAP began defining its protocol two years earlier, the goal was to anticipate future hardware availability and promote robust, forward-compatible systems.

UK raised a concern about the repeatability and consistency of EuroNCAP’s new on-road testing procedure, which involves driving vehicles for up to 2,000 km. Mr. Palao acknowledged that this represents a major shift: testing under real-world conditions will inevitably reduce repeatability and reproducibility, as driver behaviour, weather, and road environments cannot be fully controlled. However, EuroNCAP now works with a network of 13 accredited laboratories across Europe, ensuring a standardized framework for diverse conditions. He emphasized that testing is not conducted at night, though this remains an important limitation, and that expanding evaluation to public-road environments will be necessary in the future.

Following OICA’s question, EuroNCAP explained that KSS = 7 was chosen as the reference point for assessing drowsiness to maintain a consistent metric across programmes while acknowledging the scale’s limitations, but noting that other available measures were also subjective. Earlier warnings (e.g. at KSS = 6) were discussed in the past, but due to variability among drivers and systems, EuroNCAP decided to retain the KSS = 7 threshold until a more reliable alternative or test approach becomes available. He noted that some manufacturers were choosing to issue a warning at KSS = 6.

CLEPA observed that the EuroNCAP methodology aligns closely with discussions held within the IWG, especially in distinguishing between drowsiness, microsleep, and sleep. EuroNCAP confirmed that they are planning further on-road evaluation campaigns in the coming years and expressed openness to sharing collected data with the research community to support broader analysis.

Finally, Prof. Anderson asked what changes Mr. Palao would make to the KSS-based approach.

He replied that he would prefer to link driver states directly to risk, rewarding systems that detect drowsiness before critical events occur, rather than relying on subjective scales. While the KSS is not fundamentally problematic, he noted that its application remains inconsistent, and repeated tests across technical services would likely yield different results.

16.30-17.00 - Summary of workshop including an overview from Australia of topics to be discussed the next day – group discussion

The Chair remarked the importance of precision in the new and increasingly complex systems. Early detection of drowsiness is surely important, but if this annoys people it is not the ideal solution. Precision in how to deal with the detection is more and more crucial.

### Wednesday, 15th October 2025 – 09:30 to 13:00

09.30-10.30 - Group discussion led by Australia on opportunities for improvements in the 01 series (cont’d)

Kris Slattery (The Australian Government Department of Infrastructure, Transport, Regional Development, Communications, Sports and the Arts)

Topics:

> Activation, learning and deactivation

> System metrics and performance limitations

> System architecture

> System warning requirements and acknowledgement by the driver

> Drowsiness threshold and validation testing

The open discussion on opportunities for improvement in the 01 series of the DDAW regulation was introduced by Mr. Kris Slattery (Australia), who presented a structured overview of potential areas for enhancement. He emphasized that the document should be seen as a starting point for discussion, not as an exhaustive or prescriptive list. The goal was to stimulate reflection on how to improve performance, consistency, and usability of DDAW systems.

Mr. Slattery outlined five main topics for possible improvement:

- Activation, learning, and deactivation – including manual deactivation allowances, the need for separate deactivation controls for DDAW and ADDW, automatic reinstatement, speed range for activation, reset of the learning phase, and operating conditions.

- System metrics and performance limitations – including possible introduction of additional mandatory metrics to increase reliability and performance, limitations of systems using indirect means, and performance of alternate DDAW system.

- System architecture – consideration of a “backup” DDAW based on indirect means.

- Warning requirements and driver acknowledgement – including suppression logic, warning type and escalation, continuous monitoring after warning activation, and temporary failures.

- Drowsiness threshold and validation testing – including review of approach to warning thresholds, concealment of warnings during tests, treatment of false negatives, sensitivity calculation methods, and drowsiness rating assignment to test interval.

He then did an initial run through each of the sub-topics in each main topic, for the purpose of introducing the associated text from the current regulation and the types of considerations that are proposed to be needed against those requirements. He stated that there would not be time to discuss all of these sub-topics at the current workshop, so the full overview was provided before returning to concentrate on select sub-topics.

System metrics and performance limitations;

Metrics used by the DDAW system

Several comments were raised by the experts of OICA: Mr. Schramm cautioned that many proposed variables (e.g. time of day, driving duration, and traffic density) could influence drowsiness differently across individuals. He warned that incorporating personality-dependent factors into algorithms could make results inconsistent, in some cases better and in other cases worse. – Ms. Johansson noted an inconsistency between how EU regulations handle system activation/deactivation and the aim to enable early detection. She observed that striving for both high precision and early warnings could introduce contradictions, especially with indirect methods. – Mr. Vukovic sought clarification on whether the proposal implied mandatory data collection or system design requirements. Australia replied that the intent was simply to initiate discussion on whether such parameters could improve detection in edge cases, not to prescribe specific algorithms. Mr. Vukovic stressed the importance of including the industry perspective, noting that current systems often annoy drivers, who tend to deactivate them, undermining effectiveness. He suggested that the average driver journey is around 30 minutes, which is unlikely to cause the driver to be drowsy. He cautioned against tightening requirements prematurely, emphasizing the need to gain more field experience and consider diverse driver populations before introducing stricter provisions. He also suggested exploring alternative testing methods, such as the Observer Rating of Drowsiness (ORD), to complement current validation protocols.

Dr. Lenne (Seeing Machines) made an initial point based on the opening presentation overview, stating that a driver wearing a hat or sunglasses was previously a big issue for direct systems, but that it should not be now. He agreed that multiple contextual factors affect system performance but argued that the focus should shift from adding more inputs which could lead to problems, to refining the key performance metrics such as specificity and positive predictive value. He emphasized that “none of us want systems that annoy drivers”, and that improved metric definition and focus on the higher order outcome could better balance detection sensitivity and driver acceptance.

Australia remarked that the intent was not to constrain manufacturers in determining primary metrics and how those are used, but to consider whether new additional metrics could enhance safety in specific edge conditions. An example was given of a possible case where the ‘main algorithm’ was on the borderline of determining a drowsy driver for an extended period of time, and in addition to this the listed ‘extra’ metrics indicated that the driver had been driving for 5 hours continuously, mostly in freeway type conditions, and that the time was 3am - this could assist with providing an appropriate warning sooner. However, he agreed that focusing on overall system performance—particularly sensitivity and specificity—might be the most effective path forward.

Limitations of systems using indirect means

The discussion turned to how DDAW systems using indirect means perform when combined with DCAS (Driver Control Assistance Systems). Australia noted that while indirect DDAW systems rely on vehicle behavior, their effectiveness diminishes when DCAS assumes control of steering and speed. He proposed three options for consideration:

- Update the DCAS regulation (UN R171) to include explicit drowsiness monitoring requirements.

- Introduce a restriction in the DDAW 01 series, disallowing indirect means in vehicles equipped with DCAS.

- Define conditional operation limits for activation of a DCAS, such as time restrictions or drowsiness thresholds, for vehicles with DDAW systems relying solely on indirect means.

Mr. Rangaraj (OICA) responded that DCAS and DDAW/ADDW systems serve different purposes—the former controlling vehicle dynamics and the latter supporting driver awareness. He cautioned against overlinking the two and argued that R171 already contains sufficient safeguards, including hands-on-wheel and gaze-monitoring requirements. He emphasized the need for evidence-based discussion, supported by statistics, before making restrictive decisions, and noted that technological neutrality should be preserved.

UK supported further exploration of the direct vs. indirect distinction, pointing out that EU GSR2 already mandates driver monitoring systems in many vehicles. He encouraged the group to leverage that experience to improve drowsiness detection accuracy, while also addressing privacy and data handling consistently with existing frameworks. He emphasized the UK were not yet in a position to commit to decisions on points raised, without having time to work through the details.

Mr. Berghmans (OICA) confirmed that GRVA’s phase 3 discussions on DCAS are already considering how to integrate drowsiness detection, and alignment between the two groups would be beneficial.

Mr. Schmidt (TÜV) echoed the comment by Mr. Berghmans, stating that there is currently only a single mention of drowsiness in R171, with an expectation that paragraph 5.5.4.2.7. would be updated in future stages of work to incorporate requirements relating to driver drowsiness.

Ms. Maiwald (CLEPA) added that DCAS systems must monitor gaze direction, meaning that microsleeps linked to eye closure would already be able to be detected. Australia acknowledged this possibility, but highlighted that drowsiness without eye closure remains a safety gap, especially in handover phases where the driver might already be severely fatigued. The safety gap relates to situations where a driver meets the R171 requirements for hands-on-wheel and eyes-on-road, but has still become drowsy.

Mr. Savona (UTAC) agreed that future updates to R171 should take DDAW requirements into account for situations where a DCAS is active. Mr. Schmidt (TÜV) suggested continued collaboration with GRVA IWG on DCAS regulation for both DDAW and ADDW regulations, and noted that warning acceptance is currently an issue for all these regulations.

10.30-11.00 Morning tea

11.00-12.00 - Group discussion led by Australia on opportunities for improvements in the 01 series (cont’d)

Activation, learning, and deactivation;

Manual deactivation of DDAW system

It was reminded that the current provision allows only the deactivation of warnings, but not the entire DDAW system, as the system must continue learning from sensor data.

OICA cautioned that if regulations differentiate too rigidly between how drowsiness and distraction are detected or managed, it could lead to inconsistencies in system behavior and HMI design, potentially confusing drivers. They also supported allowing manual deactivation of the system, noting that other safety features, such as AEBS, can also be switched off. They argued that forcing differentiation between direct and indirect methods could compromise technology neutrality, and that the HMI layout should remain under manufacturer discretion, as space available for the HMI is limited and UN Regulations generally avoid prescribing interface details.

UK found it contradictory that a safety-critical system might allow warning deactivation. He questioned whether drivers would realistically re-enable alerts once disabled, suggesting instead that warnings become mandatory as drowsiness progresses. He also emphasized that privacy concerns linked to camera systems should be addressed through data protection measures, not by allowing full system shutdown.

Australia clarified that the existing rationale for allowing deactivation of warnings but not the entire system was to avoid driver annoyance while maintaining system learning for systems using indirect means. He also acknowledged that for direct camera-based systems, some drivers may wish to disable image capture entirely due to privacy concerns, and this would require the allowance for the entire system to be deactivated. Mandatory warnings at high levels of drowsiness are preferred but would need agreement.

Interruptions and reset of the learning phase

The group then discussed concerns about constant interruptions of the learning phase, which could prevent the system from functioning effectively. Australia presented a draft proposal (previously unadopted) to clarify how learning should resume after speed drops or interruptions, suggesting updates to paragraph 5.3.6.2 and Annex 3, para. 1.3. – He noted that direct systems are typically ready almost immediately, whereas indirect systems may require longer learning phases.

Mr. Rangaraj (OICA) stressed that EuroNCAP procedures should not be used as direct references for developing type-approval requirements, as the former represents consumer rating criteria, not compliance testing. He justified the 65 km/h lower limit by explaining that speeds below this threshold typically correspond to urban environments in Europe, where fatigue is less relevant and frequent alerts could unnecessarily irritate drivers. He proposed including a best-practice clause similar to the proposal shown, to reduce interruptions or any reset of the system learning phase before it has completed.

Mr. Hammond (OICA) added that some real-life cases—such as drivers stopping briefly for coffee or resting with the engine running—could require resetting the learning phase, since driver condition or the person driving may change.

System architecture;

‘Backup’ DDAW system functionality through indirect means

On system architecture, the group examined whether direct DDAW systems should be required to include a backup indirect method for scenarios where the primary system is impaired (e.g., tinted glasses, facial occlusion) or has been deactivated for privacy reasons. The proposal considered that new vehicles would likely have mandatory fitment of sensors allowing an indirect system to function.

OICA understood the intention of the proposal, but opposed making such redundancy mandatory, arguing that it would discourage manufacturers from adopting direct systems and undermine technology neutrality. Not all OEMs use EuroNCAP protocols, and it was suggested that not all vehicles are equipped with the sensors needed to support indirect methods. It was stated that it was not normal practice to have in-built redundancy.

Dr. Lenne (Seeing Machines) observed that all measurement approaches face operational limits—camera-based or otherwise—and that the regulation should focus less on the method and more on ensuring equivalent detection accuracy and driver acceptance.

Drowsiness threshold and validation testing;

Drowsiness threshold for system warnings

Finally, participants revisited the warning threshold, currently set at KSS = 8, with an optional manufacturer choice to warn earlier at KSS = 7.

Australia noted that KSS = 7 might still miss early hypo-vigilance, where attention and reaction times degrade before drivers perceive themselves as drowsy. An earlier threshold (e.g., KSS = 6) could be safer if combined with a graduated warning strategy to minimize annoyance, while on the opposite, a KSS = 8 could remain acceptable if supported by stronger warnings from the beginning.

The experts of OICA cautioned that warnings misaligned with the driver’s perception can feel intrusive and annoying, and underlined the need for validity and perceived relevance to sustain long-term behavioral change. They also emphasized that driver response improves when warnings align with perceived risk and that early, vague alerts risk being ignored or rejected.

UTAC supported approaches that raise driver self-awareness, suggesting progressive feedback (e.g., visual indicators) to help drivers recognize their own fatigue developing.

Ms. Larsson (OICA) concluded by appreciating Prof. Anderson’s research on eye closure and microsleep detection against KSS, the only published dataset currently available, and encouraged further studies to assess acceptance of earlier warnings, which are often viewed as intrusive.

12.00-12.15 - Small Series Exemption implementation outside the EU

Charan Rangaraj (OICA/CLEPA)

The discussion was introduced by Mr. Rangaraj (OICA), who presented the current EU framework for small series exemptions under Regulation (EU) 2018/858, as amended by 2022/2236.

- Scheme 1 covers vehicles produced in very limited numbers (up to 1500 units per year) and not derived from mass-produced models.

- Scheme 2 applies to other small series with tailored provisions.

In this context, Scheme 1 vehicles are currently exempt from DDAW requirements, whereas Scheme 2 foresees compliance by 2028 for all vehicles. However, an additional amendment just adopted by the EU in 2025 foresees the exemption, under the Scheme 2, for vehicles with a driver R-point less than 450 mm above ground level. Those are basically sports or ultra-low vehicles which face technical challenges due to camera positioning and visibility constraints. OICA’s proposal was therefore to introduce exemptions in the UN Regulation, to avoid repetitive and fragmented discussions with each national type-approval authority.

UK acknowledged the issue but argued that exemptions should remain a matter for individual contracting parties, as done in the EU and UK under their respective national schemes. He cautioned that embedding such provisions in a UN Regulation could prematurely restrict future technology options, noting that manufacturers might find viable solutions over time.

OICA replied that the intention was not to exempt all small series vehicles, but only those facing unavoidable technical limitations, such as camera placement in low-seated cars. He explained that the goal was to avoid repetitive bilateral negotiations with multiple national authorities, ensuring consistent treatment of identical vehicle types across markets.

Australia acknowledged the technical difficulties linked to current mass-produced sensors, which often cannot be adapted to low-profile sports cars with steep windscreen angles. However, he stated that he was confident technical solutions would emerge, and therefore it would be unwise to exclude these vehicles indefinitely from an international regulation’s scope.

12.15-13.00 Looking ahead

Extension of the mandate

The Chair recalled that, according to the ToR, the current mandate runs until October 2026, but that the group would likely require an additional year to complete the 01 series of amendments for both DDAW and ADDW.

OICA supported the proposal but questioned whether one additional year would be sufficient, considering that work on the Drowsiness 01 series would only begin at the end of 2025. The Chair confirmed that the priority would be to finalize the Drowsiness regulation first, before proceeding with the Distraction regulation, which is expected to require less effort. The group agreed to aim for completion by October 2027, avoiding a longer extension that could appear inefficient to WP.29.

The plan might be to submit an initial informal document to GRSG in October 2026, followed by updates in April 2027, and a formal proposal by July 2027 for adoption in October 2027. The total extension required for the IWG will depend in part on how the work for the 01 series of the ADDW regulation is scheduled, which is yet to be determined.

Future meetings

Proposed calendar for the next meetings:

- Meeting 10: 2025, 1 December (online)

- Meeting 11: 2026, 17 February (online)

- Meeting 12: 2026, week of GRSG-131 - 1 day (hybrid)

- Meeting 13: 2026, 3 June (online)

UK raised concerns about the possible five-day duration of the next GRSG session, which might prevent the IWG from meeting during that week. He proposed either more frequent shorter meetings or a dedicated multi-day session to review the text in detail, noting that such discussions are far more productive in person than online.

Ms. Jürss (OICA) agreed, stressing that the current plan of short online meetings would not suffice to handle the workload. She recommended either increasing the frequency of meetings or scheduling two-day sessions, possibly spread across consecutive days.

The Chair acknowledged this need and committed to circulating a revised meeting plan by email, seeking quick feedback from participants. A face-to-face meeting similar to the current workshop – after the week of April GRSG session – may also be considered.

UK and OICA both supported organizing an intensive 4–5-day meeting once a complete draft is available, to allow for full line-by-line review and consolidation of members’ positions.