|  | United Nations | ECE/TRANS/WP.29/xx/Rev.X |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General Xx Original: English |

| Submitted by the Secretariat | SCUNV-17-02 Agenda item 2 |
| --- | --- |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Resolution on the performance assessment of imported or exported used vehicles,

[Consolidated] Resolution on the inspection criteria for [the Import and Export of] Used Vehicles

The text reproduced below was prepared by the Informal Working Group on Safer and Cleaner Used Vehicles and was adopted by the World Forum for Harmonization of Vehicle Regulations (WP.29) at its xxx session on the basis of ECE/TRANS/WP.29/20XX/xx.

Resolution on the performance assessment of imported or exported used vehicles

[Consolidated] Resolution on the inspection criteria for [the Import and Export of] Used Vehicles

Contents

Page

Resolution

Preamble		x

Introduction		x

1.	Definitions		x

2.	Procedural Background		x

3.	Statement of technical rational and justification		xx

4.	Scope		xx

5.	Existing regulations and standards		xx

6.	Applying this consolidated resolution		xx

7.	Recommendations		xx

Annexes

1	General Provisions		xx

1	Used Vehicle Inspection Chapters and Groups		xx

2	Used Vehicle Inspection Method		xx

Preamble

The World Forum for Harmonization of Vehicle Regulations,

[to be updated further]

- At the 185th session of WP.29 held in November 2021, UNECE secretariat presented the latest activities of the UNRSF "Safer and Cleaner Used Vehicles for Africa" project and invited the World Forum to consider the establishment of an IWG to take a lead role in handling technical issues related to this topic.

- ECE/TRANS/WP.29/1161, paras.112 and 113 contains WP.29 decision to establish an Informal Working Group (IWG) to take a leading role in vehicle regulatory elements in the areas of safety and environmental protection for both new and used vehicles for countries in different economic situations. This decision will require the IWG to use existing data, research, as well as UN Regulations / UN GTRs/UN Rules and or equivalent National/Regional regulations and standards as a guidance to form further discussions, activities, and outcomes of this group.

The United Nations Member States gathering at the World Forum for Harmonization of Vehicle Regulations (WP.29),

_Responding_ to the concerns expressed by countries that predominantly import used vehicles, as highlighted during the 2017 session of the Inland Transport Committee,

_Acknowledging_ the findings of the International Energy Agency’s 2010 report regarding the environmental and safety implications of used vehicle imports,

Recognizing the calls to action and initiatives led by the United Nations Environment Programme (UNEP), including the Partnership for Clean Fuels and Vehicles, the Global Fuel Economy Initiative, the Climate and Clean Air Coalition, and the Electric Mobility Programme,

Aligning with the objectives of regional frameworks such as the ECOWAS resolution on vehicle imports and environmental standards,

Having considered the outcomes and recommendations of the Informal Working Group on Safer and Cleaner Used Vehicles,

Having reviewed existing national requirements governing the import and use of used vehicles in countries that enforce such regulations,

Recalling the harmonized vehicle performance assessment tools and periodic technical inspection frameworks developed by WP.29,

Emphasizing the importance of technology-neutral performance assessment criteria that are non-discriminatory with respect to the legal origin of exported vehicles, in order to prevent unnecessary trade barriers,

Recognizing the current lack of harmonization in registration documents and performance certification for vehicles undergoing import evaluation,

Have agreed to recommend the following principles and criteria for the assessment of vehicles that have been previously registered and operated in another country prior to export:

Introduction

1.		Definitions

All definitions to be identified and clearly stated in such a manner that all users of the document would understand the definition. (This could mean that we would have to slightly deviated from the normal explanation of the definition)

2.		Procedural Background

xx.	At the 185th session of WP.29 held in November 2021, UNECE secretariat presented the latest activities of the UNRSF "Safer and Cleaner Used Vehicles for Africa" project and invited the World Forum to consider the establishment of an IWG to take a lead role in handling technical issues related to this topic.

xx.	The IWG on SCUNV developed the above proposal and submitted it to WP.29 in the  at the xx session in xx MONTH YEAR as an informal document for consideration of the Contracting Parties.

xx.	The Consolidated Resolution (R.E. [X]) provides the provisions for the export and import of used vehicles.

3.		Statement of technical rational and justification

4.		Scope

xx.	This Resolution establishes minimum requirements for the inspection of Used Vehicles at Country of export and Country of import.

5.		Existing regulations and standards

6.		Applying this consolidated resolution

7.		Recommendations

Annex 1

I.	Purpose

1.	This document’s primary purpose is to provide an overview of items to be considered for inspection of Used Vehicles in order to determine the roadworthiness of that vehicle in case of transferring the vehicle at the point of origin as well as of destination. This document will be further developed by the IWG with elements such as recommended inspection methods and/or guidance on assessment of defects/shortcomings in accordance with their risk. The further development of the document may result in adding/removing specific elements.

II.	Used Vehicle Inspection Chapters and Groups

- Identification of the vehicle

  - Vehicle identification/chassis/serial number and main components as defined

- Braking equipment

  - Mechanical condition and operation

  - Service braking performance and efficiency

  - Secondary (emergency) braking performance and efficiency (if met by separate system)

  - Parking braking performance and efficiency

  - Endurance braking system performance

  - Anti-lock braking system (ABS) (if fitted)

  - Electronic brake system (EBS) (if fitted)

  - Brake fluid

- Steering

2.1. Mechanical condition

2.2. Steering wheel, column and handle bar

2.3. Steering play

2.4. Wheel alignment

2.5. Trailer steered axle turntable

2.6. Electronic Power Steering (EPS) (if fitted)

- Visibility

3.1. Field of vision

3.2. Condition of glass

3.3. Rear-view mirrors or devices

3.4.  Windscreen wipers

3.5. Windscreen washers

3.6  Demisting system

- Lamps, reflectors and electrical equipment

4.1. Headlamps

4.2. Front and rear position lamps, side marker lamps, end outline marker lamps and [daytime running lamps(when required)]

4.3. Stop Lamps

4.4. Direction indicator and hazard warning lamps

4.5. Front and rear fog lamps (when required)

4.6. Reversing lamps

4.7. Rear registration plate lamp

4.8. Retro-reflectors, conspicuity (retro reflecting) markings and rear marking plates

4.9. Tell-tales mandatory for lighting equipment

4.10. Electrical connections between towing vehicle and trailer or semi-trailer

4.11. Electrical wiring

4.12.  Battery(ies) (visual)

- Axles, wheels, tyres and suspension

5.1. Axles

5.2. Wheels and tyres

5.3. Suspension system

- Chassis and chassis attachments

6.1. Chassis or frame and attachments

6.2. Cab and bodywork

- Other (Safety) equipment

7.1. Safety-belts/buckles and restraint systems

7.2. Fire extinguisher (if required)

7.3. Locks and anti-theft device

7.4. Warning triangle (if required)1

7.5. First aid kit. (if required)1

7.6. Wheel chocks (wedges)

7.7. Audible warning device

7.8. Speedometer (Metric vs Imperial)

7.9. Speed limitation device (if fitted/required)1

7.10 Odometer if available

7.11 Electronic Stability Control (ESC) (if fitted/required)

- Supplementary tests for passenger-carrying vehicles categories busses and coaches

8.1. Doors

8.2.  Demisting and defrosting system

8.3. Ventilation & heating system

8.4. Seats

8.5. Interior lighting and if fitted, destination devices

8.6. Gangways, standing areas

8.7. Stairs and steps

8.8. Requirements regarding the transportation of children (if applicable)

8.9.Requirements regarding the transportation of persons with reduced mobility (if applicable)

- Environmental nuisances, pollutant emissions and other items related to the protection of the environment

9.1. Exhaust emissions

9.2. Noise emissions

9.3. Other items related to the protection of the environment

9.4. Electromagnetic interference suppression (Recommended)

- [Supplementary tests for battery/hybrid electric vehicles]

## I.	Purpose

1.	This document’s primary purpose is to provide an overview of items to be considered for inspection of Used Vehicles in order to determine the roadworthiness of that vehicle in case of transferring the vehicle at the point of origin as well as of destination. This document further provides recommended inspection methods and/or examples of defects/shortcomings related to the inspection item as well as supporting information. The further development of the document would include harmonized guidance on the suitability of the vehicle for exports/imports in accordance with risk related to identified defects/shortcomings.

2.	As this document is considered as a living document, future discussions may result in adding/removing specific elements.

3.	The IWG on SCUNV decided to split the document into two parts, Part I would cover Safety aspects and Part II would focus on the assessment of Environmental performance/components.

II.	Recommended Used Vehicle Inspection Method

The method of inspection set out in the annex shall be the minimum requirement.

Where a method of inspection is given as visual, it means that in addition to looking at the items, the inspector can also handle them, evaluate noise, etc.

Functional check means a verification of the operability of the related part/subsystem/system related to the inspection item(s).

Annex (Part I)

Minimum inspection requirements

The inspection shall cover at least the items listed below applicable to the vehicle in concern.

The inspections should be carried out using techniques and equipment currently available, and without the use of tools to dismantle or remove any part of the vehicle.

| Item | Recommended Method | Main Reasons for Rejection | Comments/Guidance |
| --- | --- | --- | --- |
| - Identification of the vehicle |  |  |  |
| 0.1. Vehicle identification chassis/ serial number | Visual inspection. | - Missing or cannot be found. - Incomplete, illegible. - Not in accordance with vehicle documents or records. - Allegedly tampered. - Chassis/serial number does not match the registration plate(s) if fitted, in accordance with documents and records | Title/Registration Document |
| - Braking equipment |  |  |  |
| - Mechanical condition and operation |  |  |  |
| 1.1.1. Service brake pedal/hand lever pivot | Visual inspection of the components while the braking system is operated. Note: Vehicles with power-assisted braking systems should be inspected with the engine switched off. | - Pivot too tight. - Excessive wear or play. |  |
| 1.1.2. Pedal/hand lever condition and travel of the brake operating device | Visual inspection of the components while the braking system is operated. Note: Vehicles with power-assisted braking systems should be inspected with the engine switched off. | - Excessive or insufficient reserve travel. - Brake control not releasing correctly. - Anti-slip provision on brake pedal missing, loose or worn smooth. |  |
| 1.1.3. Vacuum pump or compressor and reservoirs | Visual inspection of the components at normal working pressure. Check time required for vacuum or air pressure to reach safe working value and function of warning device, multi-circuit protection valve and pressure relief valve. | - Insufficient pressure/vacuum to give assistance for at least two brake applications after the warning device has operated (or gauge shows an unsafe reading). - Time (proposed < 60 secs) taken to build up air pressure/vacuum to safe working value not in accordance with the requirements1. - Multi-circuit protection valve or pressure relief valve not working. - Air leak causing a noticeable drop in pressure or audible air leaks. - External damage likely to affect the function of the braking system. | Applicable for Heavy Duty Vehicles (HDVs) |
| 1.1.4. Low pressure warning gauge or indicator | Functional check by changing pressure in the system | Malfunctioning or defective gauge or indicator. | Applicable for Heavy Duty Vehicles (HDVs) |
| 1.1.5. Hand operated brake control valve | Visual inspection of the components while the braking system is operated. | - Control cracked, damaged or excessively worn. - Control insecure on valve or valve insecure. - Loose connections or leaks in system. - Unsatisfactory operation. |  |
| 1.1.6. Parking brake activator, lever control, parking brake ratchet | Visual inspection of the components while the braking system is operated. | - Ratchet not holding correctly. - Excessive wear at lever pivot or in ratchet mechanism. - Excessive movement of lever indicating incorrect adjustment. - Activator missing, damaged or in-operative. |  |
| 1.1.7. Braking valves (foot valves, unloaders, governors) | Visual inspection of the components while the braking system is operated. | - Valve damaged or excessive air leak. - Excessive oil discharge from compressor. - Valve insecure or inadequately mounted. - Hydraulic fluid discharge or leak. |  |
| 1.1.8. Couplings for trailer brakes (electrical & pneumatic) | Disconnect and reconnect braking system coupling between towing vehicle and trailer. | - Tap or self-sealing valve defective. - Tap or valve insecure or inadequately mounted. - Excessive leaks. - Not functioning correctly | Applicable for Heavy Duty Vehicles (HDVs) |
| 1.1.9. Energy storage reservoir pressure tank | Visual inspection. | - Tank damaged, corroded or leaking. - Drain device inoperative. - Tank insecure or inadequately mounted. | May require pit or lift |
| 1.1.10. Brake servo units, master cylinder (hydraulic systems) | Visual inspection of the components while the braking system is operated. | - Defective or ineffective servo unit. - Master cylinder defective or leaking. - Master cylinder insecure. - Insufficient brake fluid. - Master cylinder reservoir cap missing. - Brake fluid warning light illuminated or defective. - Incorrect functioning of brake fluid level warning device. |  |
| 1.1.11. Rigid brake pipes | Visual inspection of the components while the braking system is operated. | - Risk of failure or fracture. - Pipes or connections leaking. - Pipes damaged or excessively corroded. - Pipes misplaced. | May require pit or lift |
| 1.1.12. Flexible brake hoses | Visual inspection of the components while the braking system is operated. | - Eminent risk of failure or fracture. - Hoses damaged, chafing, twisted or too short. - Hoses or connections leaking. - Hoses bulging under pressure. - Hoses porous. | May require pit or lift |
| 1.1.13. Brake linings and pads | Visual inspection. | - Lining or pad excessively worn. - Lining or pad contaminated (oil, grease etc.). - Lining or pad missing | May require pit or lift |
| 1.1.14. Brake drums, brake discs | Visual inspection. | - Drum or disc excessively worn, excessively scored, cracked, insecure or fractured. - Drum or disc contaminated (oil, grease, etc.) - Drum or disc missing. - Back plate insecure. | May require pit or lift |
| 1.1.15. Brake cables, rods, levers, linkages | Visual inspection of the components while the braking system is operated. | - Cable damaged or knotted. - Component excessively worn or corroded. - Cable, rod or joint insecure. - Cable guide defective. - Restriction to free movement of the braking system. - Abnormal movement of the levers/linkage indicating maladjustment or excessive wear. | May require pit or lift |
| 1.1.16. Brake actuators (including spring brakes or hydraulic cylinders) | Visual inspection of the components while the braking system is operated. | - Actuator cracked or damaged. - Actuator leaking. - Actuator insecure or inadequately mounted. - Actuator excessively corroded. - Insufficient or excessive travel of operating piston or diaphragm mechanism. - Dust cover missing or excessively damaged. | May require pit or lift |
| 1.1.17. Load sensing valve | Visual inspection of the components while the braking system is operated. | - Defective linkage. - Linkage incorrectly adjusted. - Valve seized or inoperative. - Valve missing. - Missing data plate. - Data illegible or not in accordance with requirements1. | May require pit or lift. |
| 1.1.18. Slack adjusters and indicators | Visual inspection. | - Adjuster damaged, seized or having abnormal movement, excessive wear or incorrect adjustment. - Adjuster defective. - Incorrectly installed or replaced. | May require pit or lift |
| 1.1.19. Endurance braking system (where fitted or required) | Visual inspection. | - Insecure connectors or mountings. - System obviously defective or missing. | Applicable for Heavy Duty Vehicles (HDVs) |
| 1.1.20. Automatic operation of trailer brakes | Disconnect brake coupling between towing vehicle and trailer. | Trailer brake does not apply automatically when coupling disconnected. | Applicable to HDVs |
| 1.1.21. Complete braking system | Visual inspection. | - Other system devices (e.g. anti-freeze pump, air dryer, etc.) damaged externally or excessively corroded in a way that adversely affects the braking system. - Leakage of air or anti-freeze. - Any component insecure or inadequately mounted. - Inappropriate repair or modification to any component. |  |
| 1.1.22. Test connections (where fitted or required) | Visual inspection. | - Missing. - Damaged, unusable or leaking. | Applicable for HDVs |
| - Service braking performance and efficiency |  |  |  |
| 1.2.1. Performance | During a test on a static brake testing machine or, if impossible, during a road test apply the brakes progressively up to maximum effort. | - Inadequate braking effort on one or more wheels. - Braking effort from any wheel is less than 70% of maximum effort recorded from the other wheel on the same axle. Or in the case of testing on the road, the vehicle deviates excessively from a straight line and comes out of a corridor 3 m wide . - No gradual variation in brake effort (grabbing). - Abnormal lag in brake operation of any wheel. - Excessive fluctuation of brake force during each complete wheel revolution. | Requires roller brake tester. It is possible to partially check on a road test |
| 1.2.2. Efficiency | Test with a static brake testing machine or, if one cannot be used for technical reasons, by a road test using a decelerometer. Vehicles or a trailer has to be inspected following the standards given by ISO 21069, or according to the requirements or equivalent methods. Laden vehicle braking system performance should be assessed by testing the vehicle laden, or by evaluation using a method based on extrapolation or by some other acceptable means. Note: The efficiency of overrun brakes can be fully tested on a static brake testing machine by use of a special device or partially tested by applying the parking brake. | Does not give at least the minimum figure/distance laid down in below. Deceleration: Passenger carrying vehicles 5.0 m/s2 Goods vehicles 4.5 m/s2 Equivalent brake efficiency: Passenger carrying vehicles 5.0 N/kg or 50% ratio brake force to vehicle mass Goods vehicles 4.5 N/kg or 45% ratio brake force to vehicle mass. Brake distance in metres: Speed; Deceleration. km/h; 4.5 m/s2; 5.0 m/s2. 30; 7.72; 6.94. 35; 10.50; 9.45. 40; 13.72; 12.35. 45; 17.36; 15.63. 50; 21.43; 19.29. | Requires roller brake tester or for road tests a decelerometer or indications of brake distances |
| - Secondary (emergency) braking performance and efficiency (if met by separate system) |  |  |  |
| 1.3.1. Performance | If the secondary braking system is separate from the service braking system, use the method specified in 1.2.1. | - Inadequate braking effort on one or more wheels. - Braking effort from any wheel is less than 70% of maximum effort recorded from another wheel on the same axle specified. Or in the case of testing on the road, the vehicle deviates excessively from a straight line comes out of a corridor 3 m wide. - No gradual variation in brake effort (grabbing). | Requires roller brake tester. It is possible to partially check on a road test. Extremely rare system |
| 1.3.2. Efficiency | If the secondary braking system is separate from the service braking system, use the method specified in 1.2.2. | Braking effort less than 50% of the service brake performance defined in section 1.2.2 in relation to the maximum authorized mass or, in the case of semi-trailers, to the sum of the authorized axel loads. | Requires roller brake tester or decelerometer |
| - Parking braking performance and efficiency |  |  |  |
| 1.4.1.Performance | Apply the brake during a test on a static brake testing machine and/or during a road test with a decelerometer. | Brake inoperative on one side or in the case of testing on the road, the vehicle deviates excessively from a straight line. | Requires roller brake tester or decelerometer |
| 1.4.2. Efficiency | Test with a static brake testing machine or by a road test using either an indicating or recording decelerometer or with the vehicle on a slope of known gradient. Goods vehicles should, if possible, be tested laden. | Does not give at least for all vehicles a braking ratio of 18% in relation to the maximum authorized mass, or, for motor vehicles, of 12% in relation to the maximum authorized combination mass of the vehicle, whichever is the greater. | Requires roller brake tester. decelerometer or slope of known gradient. |
| - Endurance braking system |  |  |  |
| 1.5. Endurance braking system | Visual inspection and, where possible, test whether the system functions. | System not functioning | No performance test Applicable to HDVs |
| 1.6. Anti-lock braking system (ABS) | Visual inspection and inspection of warning device. | - Warning device malfunctioning. - Warning device shows system malfunction. - Wheel speed sensors missing or damaged. - Wirings damaged. - Other components missing or damaged. | May require pit or lift |
| 1.7 Electronic brake system (EBS) | Visual inspection of warning device. | - Warning device malfunctioning. - Warning device shows system malfunction. |  |
| - Brake fluid |  |  |  |
| 1.8 Brake fluid | Visual inspection | - Brake fluid contaminated or sedimented. - Imminent risk of failure |  |
| - Steering |  |  |  |
| - Mechanical condition and operation |  |  |  |
| 2.1.1. Steering gear condition | With the vehicle over a pit or on a hoist and with the road wheels off the ground or on turn tables, rotate the steering wheel from lock to lock. Visual inspection of the operation of the steering gear. | - Roughness in operation of gear. - Sector shaft twisted or splines worn. - Excessive wear in sector shaft. - Excessive movement of sector shaft. - Leaking. | Requires pit or lift and jack |
| 2.1.2. Steering gear casing attachment | With vehicle on a pit or hoist and the weight of the vehicle road wheels on the ground, rotate steering / handlebar wheel clockwise and anticlockwise or using a specially adapted wheel play detector. Visual inspection of the attachment of gear casing to chassis. | - Steering gear casing not properly attached. - Elongated fixing holes in chassis. - Missing or fractured fixing bolts. - Steering gear casing fractured. | Requires pit or lift and play detector. Partially possible manually with a jack |
| 2.1.3. Steering linkage condition | With the vehicle over a pit or on a hoist and with the road wheel on ground, rock steering wheel clockwise and anti-clockwise or using a specially adapted wheel play detector. Visual inspection of steering components for wear, fractures and security. | - Relative movement between components which should be fixed. - Excessive wear at joints. - Fractures or deformation of any component. - Absence of locking devices. - Misalignment of components (e.g. track rod or drag link). - Inappropriate repair or modification. - Dust cover missing, damaged or severely deteriorated. | Requires pit or lift and play detector. |
| 2.1.4. Steering linkage operation | With the vehicle over a pit or on a hoist and with the road wheels on ground and the engine running (power steering), rotate steering wheel from lock to lock. Visual inspection of movement of linkages. | - Moving steering linkage fouling a fixed part of chassis. - Steering stops not operating or missing. | Requires pit or lift. |
| 2.1.5. Power steering | Check steering system for leaks and hydraulic fluid reservoir level (if visible). With the road wheels on ground and with the engine running, check that the power steering system is operating. | - Fluid leak. - Insufficient fluid. - Mechanism not working properly. - Mechanism fractured or insecure. - Misalignment or fouling of components. - Inappropriate repair or modification. - Cables/hoses damaged, excessively corroded. | May require pit or lift. Excessive noise due to e.g. air in the hydraulic system |
| - Steering wheel, column and handlebar |  |  |  |
| 2.2.1. Steering wheel condition | With the road wheels on the ground, rock steering wheel from side to side at right angles to column and apply slight downward and upward pressure. Visual inspection of play. | - Relative movement between steering wheel and column indicating looseness. - Absence of retaining device on steering wheel hub. - Fracture or looseness of steering wheel hub, rim or spokes. |  |
| 2.2.2. Steering column | With the vehicle over a pit or on a hoist and the mass of the vehicle on the ground, push and pull the steering wheel in line with column, push steering wheel in various directions at right angles to the column. Visual inspection of play, and condition of flexible couplings or universal joints. | - Excessive movement of centre of steering wheel up or down. - Excessive movement of top of column radially from axis of column. - Deteriorated flexible coupling. - Attachment defective. - Inappropriate repair or modification. | Requires pit or lift |
| - Steering play |  |  |  |
| 2.3. Steering play | With the vehicle over a pit or on a hoist, the mass of the vehicle on the road-wheels, the engine running for vehicles with power steering and with the road wheels in the straight-ahead position, lightly turn the steering wheel clockwise and anti-clockwise as far as possible without moving the road wheels. Visual inspection of free movement. | Free play in steering excessive (for example movement of a point on the rim exceeding one fifth of the diameter of the steering wheel or not in accordance with the requirements1. | Requires pit or lift |
| - Wheel alignment |  |  |  |
| 2.4. Wheel alignment (X)2 | Check alignment of steered wheels with suitable equipment. | Alignment not in accordance with vehicle manufacturer’s data or requirements1. | Requires wheel alignment plate |
| - Trailer steered axle turntable |  |  |  |
| 2.5. Trailer steered axle turntable | Visual inspection or using a specially adapted wheel play detector. | - Component damaged or cracked. - Excessive play. - Attachment defective. | Applicable to HDVs |
| - Electric power steering if fitted |  |  |  |
| 2.6. Electric power steering if fitted | Visual inspection and consistency check between the angle of the steering wheel and the angle of the wheels when switching on/off the engine, and/or using the electronic vehicle interface | - EPS malfunction indicator lamp (MIL) indicates any kind of failure of the system. - Inconsistency between the angle of the steering wheel and the angle of the wheels. |  |
| - Visibility |  |  |  |
| - Field of vision |  |  |  |
| 3.1. Field of vision | Visual inspection from driving seat. | Obstruction within driver’s field of view that materially affects his view in front or to the sides. |  |
| - Condition of glass |  |  |  |
| 3.2. Condition of glass | Visual inspection. | - Cracked or discoloured glass or transparent panel (if permitted). - Glass or transparent panel (including reflecting or tinted film) that does not comply with specifications in the requirements1. - Glass or transparent panel in unacceptable condition. | Markings on glass/transparent panels to be checked. |
| - Rear-view mirrors or devices |  |  |  |
| 3.3. Rear-view mirrors or devices | Visual inspection. | - Mirror or device missing or not fitted according to the requirements1. - Mirror or device inoperative, damaged, loose or insecure. |  |
| - Windscreen washers |  |  |  |
| 3.4. Windscreen wipers | Visual inspection and by operation. | - Wipers not operating or missing. - Wiper blade missing or obviously defective. |  |
| - Windscreen washers |  |  |  |
| 3.5. Windscreen washers | Visual inspection and by operation. | Washers not operating adequately. |  |
| - Demisting system |  |  |  |
| - Demisting system (X)2 | Visual inspection and by operation. | System inoperative or obviously defective. |  |
| - Lamp, reflectors and electrical equipment |  |  |  |
| - Headlamps |  |  |  |
| 4.1.1. Condition and operation | Visual inspection and by operation. | - Defective or missing light / light source. - Defective or missing projection system (reflector and lens). - Lamp not securely attached. |  |
| 4.1.2. Alignment | Determine the horizontal aim of each headlamp on dipped beam using a headlamp aiming device or a screen. | Aim of a headlamp not within limits laid down in the requirements1. | Requires a headlamp alignment tester or screen |
| 4.1.3. Switching | Visual inspection and by operation. | - Switch does not operate in accordance with the requirements1. (Number of headlamps illuminated at the same time) - Function of control device impaired. |  |
| 4.1.4.Compliance with requirements1 (X)2 | Visual inspection and by operation. | - Lamp, emitted colour, position or intensity not in accordance with the requirements1. - Products on lens or light source which obviously reduce light intensity or change emitted colour. - Light source and lamp not compatible |  |
| 4.1.5. Levelling devices (where mandatory) (X)2 | Visual inspection and by operation if possible. | - Device not operating. - Manual device cannot be operated from driver’s seat. |  |
| 4.1.6. Headlamp cleaning device (where mandatory) (X)2 | Visual inspection and by operation if possible. | Device not operating. |  |
| 4.2.1. Condition and operation | Visual inspection and by operation. | - Defective light source. - Defective lens. - Lamp not securely attached. |  |
| - Front and rear position lamps, side marker lamps, end outline marker lamps and [daytime running lamps (when required)] |  |  |
| 4.2.2. Switching | Visual inspection and by operation. | - Switch does not operate in accordance with the requirements1. - Function of control device impaired. |  |
| 4.2.3.Compliance with requirements1 | Visual inspection and by operation. | - Lamp, emitted colour, position or intensity not in accordance with the requirements1. - Products on lens or light source which reduce light intensity or change emitted colour. |  |
| - Stop lamps |  |  |  |
| 4.3.1. Condition and operation | Visual inspection and by operation. | - Defective light source. - Defective lens. - Lamp not securely attached. |  |
| 4.3.2. Switching | Visual inspection and by operation. | - Switch does not operate in accordance with the requirements1. - Function of control device impaired. |  |
| 4.3.3.Compliance with requirements1 | Visual inspection and by operation. | - Lamp, emitted colour, position or intensity not in accordance with the requirements1. |  |
| - Director indicator and hazard warning lamps |  |  |  |
| 4.4.1. Condition and operation | Visual inspection and by operation. | - Defective light source. - Defective lens. - Lamp not securely attached. |  |
| 4.4.2. Switching | Visual inspection and by operation. | Switch does not operate in accordance with the requirements1. |  |
| 4.4.3.Compliance with requirements1 | Visual inspection and by operation. | Lamp, emitted colour, position or intensity not in accordance with the requirements1. |  |
| 4.4.4. Flashing frequency | Visual inspection and by operation. | Rate of flashing not in accordance with the requirements1. |  |
| - Front and rear fog lamps (when required) |  |  |  |
| 4.5.1. Condition and operation | Visual inspection and by operation. | - Defective light source. - Defective lens. - Lamp not securely attached. |  |
| 4.5.2. Alignment (X)2 | By operation and using a headlamp aiming device. | - Front fog lamp out of horizontal alignment when the light pattern has cut-off line. | Requires a headlamp alignment tester or screen |
| 4.5.3. Switching | Visual inspection and by operation. | - Switch does not operate in accordance with the requirements1. |  |
| 4.5.4.Compliance with requirements1 | Visual inspection and by operation. | - Lamp, emitted colour, position or intensity not in accordance with the requirements1. - System does not operate in accordance with the requirements1. |  |
| - Reversing lamps |  |  |  |
| 4.6.1. Condition and operation | Visual inspection and by operation. | - Defective light source. - Defective lens. - Lamp not securely attached. |  |
| 4.6.2.Compliance with requirements1 | Visual inspection and by operation. | - Lamp, emitted colour, position or intensity not in accordance with the requirements1. - System does not operate in accordance with the requirements1. |  |
| 4.6.3. Switching | Visual inspection and by operation. | - Switch does not operate in accordance with the requirements1. |  |
| - Rear registration plate lamp |  |  |  |
| 4.7.1. Condition and operation | Visual inspection and by operation. | - Lamp throwing direct light to the rear. - Defective light source. - Lamp not securely attached. |  |
| 4.7.2.Compliance with requirements1 | Visual inspection and by operation. | System does not operate in accordance with the requirements1. |  |
| - Retro-reflectors, conspicuity (retro reflecting) markings and rear marking plates |  |  |  |
| 4.8.1. Condition | Visual inspection. | - Reflecting equipment defective or damaged. - Reflector not securely attached. |  |
| 4.8.2.Compliance with requirements1 | Visual inspection. | - Device reflected colour or position not in accordance with the requirements1. |  |
| - Tell-tales mandatory for lighting equipment |  |  |  |
| 4.9.1. Condition and operation | Visual inspection and by operation. | Not operating. |  |
| 4.9.2.Compliance with requirements1 | Visual inspection and by operation. | Not in accordance with the requirements1. |  |
| - Electrical connections between towing vehicle and trailer or semi-trailer |  |  |  |
| 4.10. Electrical connections between towing vehicle and trailer or semi-trailer | Visual inspection: if possible examine the electrical continuity of the connection. | - Fixed components not securely attached. - Damaged or deteriorated insulation. - Trailer or towing vehicle electrical connections not functioning correctly. |  |
| - Electrical wiring |  |  |  |
| 4.11. Electrical wiring | Visual inspection with vehicle over a pit or on a hoist, including inside the engine compartment in some cases. | - Wiring insecure or not adequately secured. - Wiring deteriorated - Damaged or deteriorated insulation. |  |
| - Battery |  |  |  |
| 4.12. Battery | Visual inspection. | - Insecure - Fixation. - Leaking. - Defective switch (if required). - Defective fuses (if required). - Inappropriate ventilation (if required). |  |
| - Axels, wheels, tyres and suspension |  |  |  |
| - Axels |  |  |  |
| 5.1.1. Axles | Visual inspection with vehicle over a pit or on a hoist. Wheel play detectors may be used and are recommended for vehicles over 3.5 tonnes gross vehicle mass (GVM). | - Axle fractured or deformed. - Insecure fixing to vehicle. - Inappropriate repair or modification. | Requires pit or lift. May require play detector. |
| 5.1.2. Stub axles | Visual inspection with vehicle over a pit or on a hoist. Use of wheel play detectors is recommended. Apply a vertical or lateral force to each wheel and note the amount of movement between the axle beam and stub axle. | - Stub axle fractured. - Excessive wear in the swivel pin and/or bushes. - Excessive movement between stub axle and axle beam. - Stub axle pin loose in axle. | Requires pit or lift. May require play detector. |
| 5.1.3. Wheel bearings | Visual inspection with the vehicle over a pit or on a hoist. Wheel play detectors may be used and are recommended for vehicles over 3.5 tonnes GVM. Rock the wheel or apply a lateral force to each wheel and note the amount of upward movement of the wheel relative to the stub axle. | - Excessive play in a wheel bearing. - Wheel bearing too tight, jammed. | Requires pit or lift. May require play detector. |
| - Wheels and tyres |  |  |  |
| 5.2.1. Road wheel hub | Visual inspection. | - Any wheel nuts or studs missing or loose. - Hub worn or damaged. |  |
| 5.2.2. Wheels | Visual inspection of both sides of each wheel with vehicle over a pit or on a hoist. | - Any fracture or welding defect. - Tyre retaining rings not properly fitted. - Wheel badly distorted or worn. - Wheel size or type not in accordance with the requirements1 and effecting road safety. |  |
| 5.2.3. Tyres | Visual inspection of the entire tyre by either rotating the road wheel with it off the ground and the vehicle over a pit or on a hoist, or by rolling the vehicle backwards and forwards over a pit. | - Tyre size, load capacity, approval mark or speed rating not in accordance with the requirements1 and effecting road safety. - Tyres on same axle or on twin wheels of different sizes. - Tyres on same axle of different construction (radial / cross-ply). - Any serious damage or cut to tyre. - Tyre tread depth not in accordance with the requirements1. - Tyre rubbing against other components. - Re-grooved tyres not in accordance with requirements1. - Air pressure monitoring system malfunctioning or obviously inoperative. | Requires pit or lift. May require depth gauge. |
| - Suspension |  |  |  |
| 5.3.1. Springs and stabilizer | Visual inspection with vehicle over a pit or on a hoist. The use of wheel play detectors is recommended. | - Insecure attachment of springs to chassis or axle. - A damaged or fractured spring component. - Spring missing. - Inappropriate repair or modification. | Requires pit or lift. May require play detector. |
| 5.3.2. Shock absorbers | Visual inspection with vehicle over a pit or on a hoist or using special equipment, if available. | - Insecure attachment of shock absorbers to chassis or axle. - Damaged shock absorber. | Requires pit or lift. |
| 5.3.2.1. Efficiency testing of damping (X)2 | Use special equipment and compare left /right differences and/or absolute values given by manufacturers. | - Significant difference between left and right. - Given minimum values not reached. | Requires suspension test bench. |
| 5.3.3. Torque tubes, radius arms, wishbones and suspension arms | Visual inspection with vehicle over a pit or on a hoist. Wheel play detectors may be used and are recommended for vehicles over 3.5 tonnes GVM. | - Insecure attachment of component to chassis or axle. - A damaged, fractured or excessively corroded component. - Inappropriate repair or modification. | Requires pit or lift. May require play detector. |
| 5.3.4. Suspension joints | Visual inspection with vehicle over a pit or on a hoist. Wheel play detectors may be used and are recommended for vehicles over 3.5 tonnes GVM. | - Excessive wear in swivel pin and/or bushes or at suspension joints. - Dust cover missing or severely deteriorated. | Requires pit or lift. May require play detector. |
| 5.3.5. Air suspension | Visual inspection. | - System inoperable. - Any component damaged, modified or deteriorated in a way that would adversely affect the functioning of the system. - Audible system leakage. | Requires pit or lift. May require play detector. |
| - Chassis and chassis attachments |  |  |  |
| - Chassis or frame and attachments |  |  |  |
| 6.1.1. General condition | Visual inspection with vehicle over a pit or on a hoist. | - Fracture or deformation of any side or cross member. - Insecurity of strengthening plates or fastenings. - Excessive corrosion which affects the rigidity of the assembly. - Inappropriate repair or modification | Requires pit or lift. |
| 6.1.2. Exhaust pipes and silencers | Visual inspection with vehicle over a pit or on a hoist. | - Insecure or leaking exhaust system. - Fumes entering cab or passengers’ compartment. | Requires pit or lift. |
| 6.1.3. Fuel tank and pipes (including heating fuel tank and pipes) | Visual inspection with vehicle over a pit or on a hoist, use of leak detecting devices in case of LPG/CNG systems. | - Insecure tank or pipes. - Leaking fuel or missing or ineffective filler cap. - Damaged or chafed pipes. - Fuel stopcock (if required) not operating correctly. - Fire risk due to - Leaking fuel - Fuel tank or exhaust improperly shielded. - Engine compartment condition. - LPG/CNG system not in accordance with requirements1. | Requires pit or lift. Requires a gas detector for LPG/CNG. |
| 6.1.4. Bumpers, lateral protection and rear underrun devices | Visual inspection. | - Looseness or damage likely to cause injury. - Device obviously not in compliance with the requirements1. |  |
| 6.1.5. Spare wheel carrier (if fitted) | Visual inspection. | - Carrier not in proper condition. - Carrier fractured or insecure. - A spare wheel not securely fixed in carrier and likely to fall off. |  |
| 6.1.6. Coupling mechanisms and towing equipment | Visual inspection for wear and correct operation with special attention to any safety device fitted and /or use of measuring gauge. | - Component damaged, defective or cracked. - Excessive wear in a component. - Attachment defective. - Any safety device missing or not operating correctly. - Any indicator not working. - Inappropriate repair or modification. | May require measuring gauge. |
| 6.1.7.Transmission | Visual inspection. | - Loose or missing securing bolts. - Excessive wear in transmission shaft bearings. - Excessive wear in universal joints. - Deteriorated flexible couplings. - A damaged or bent shaft. - Bearing housing fractured or insecure. - Dust cover missing or severely deteriorated. - Illegal power-train modification. |  |
| 6.1.8. Engine mountings | Visual inspection not necessarily on a pit or hoist. | Deteriorated, loose or fractured mountings. |  |
| 6.1.9 Engine performance | Visual inspection. | - Control unit illegal modified. - Illegal engine and/or power-train modification. |  |
| - Cab and body work |  |  |  |
| 6.2.1. Condition | Visual inspection. | - A loose or damaged panel or part likely to cause injury. - Insecure body pillar. - Permitting entry of engine or exhaust fumes. - Inappropriate repair or modification. | Requires pit or lift. |
| 6.2.2. Mounting | Visual inspection over a pit or on a hoist. | - Body or cab insecure. - Body/cab obviously not located squarely on chassis. - Insecure or missing fixing of body/cab to chassis or cross members. - Excessive corrosion at fixing points on integral bodies. | Requires pit or lift. |
| 6.2.3. Doors and door catches | Visual inspection. | - A door will not open or close properly. - A door likely to open inadvertently or one that will not remain closed. - Door, hinges, catches and/or pillar missing, loose or deteriorated. |  |
| 6.2.4. Floor | Visual inspection over a pit or on a hoist. | Floor insecure or badly deteriorated |  |
| 6.2.5. Driver’s seat | Visual inspection. | - A loose seat or seat with defective structure. - Adjustment mechanism not functioning correctly. |  |
| 6.2.6. Other seats | Visual inspection. | - Seats in defective condition or insecure. - Seats fitted not in accordance with requirements1. |  |
| 6.2.7. Driving controls | Visual inspection and by operation. | Any control necessary for the safe operation of the vehicle not functioning correctly. |  |
| 6.2.8. Cab steps | Visual inspection. | - Step or step ring insecure. - Step or ring in a condition likely to cause injury to users. |  |
| 6.2.9. Other interior and exterior fittings and equipment (including registration plates if fitted) | Visual inspection. | - Attachment of other fitting or equipment defective. - Other fitting or equipment not in accordance with the requirements1. - Leaking hydraulic equipment. |  |
| 6.2.10. Mudguards (wings), spray suppression devices | Visual inspection. | - Missing, loose or badly corroded. - Insufficient clearance to road wheel. - Not in accordance with the requirements1. |  |
| - Other (safety) equipment |  |  |  |
| - Safety belts/buckles and restraining systems |  |  |  |
| 7.1.1. Security of safety-belts/buckles mounting | Visual inspection. | - Anchorage point badly deteriorated. - Anchorage loose. |  |
| 7.1.2. Condition of safety-belts/buckles. | Visual inspection and by operation. | - Mandatory safety-belt missing or not fitted. - Safety-belt damaged. - Safety-belt not in accordance with the requirements2. - Safety-belt buckle damaged or not functioning correctly. - Safety-belt retractor damaged or not functioning correctly. |  |
| 7.1.3. Safety belt load limiter | Visual inspection. | Load limiter missing or not suitable for vehicle. |  |
| 7.1.4. Safety belt pre-tensioners | Visual inspection. | Pre-tensioner missing or not suitable for vehicle. |  |
| - Lock and anti-theft device |  |  |  |
| 7.2. Locks and anti-theft device | Visual inspection and by operation. | - Device not functioning to prevent vehicle being driven. - Defective or inadvertently locking or blocking |  |
| - Audible warning device |  |  |  |
| 7.3. Audible warning device | Visual inspection and by operation. | - Not working. - Control insecure. - Not in accordance with the requirements1. |  |
| - Speedometer |  |  |  |
| 7.4.Speedometer | Visual inspection or by operation during road test or by electronically means. | - Not fitted in accordance with the requirements1. - Not operational. - Not capable of being illuminated. | The inspector may not know the country and destination and local requirements. |
| - Speedometer |  |  |  |
| 7.5. Odometer if available (X)2 | Visual inspection. | - Obviously manipulated (fraud). - Obviously inoperative. |  |
| - Electronic Stability Control (ESC) if fitted/required |  |  |  |
| 7.6. Electronic Stability Control (ESC) if fitted/required | Visual inspection, and/or using electronic interface | - Wheel speed sensors missing or damaged - Wiring damaged - Other components missing or damaged - Switch damaged or not working correctly - ESC MIL indicates any kind of failure of the system |  |
| - Supplementary tests for passenger-carrying vehicles categories – busses and coaches |  |  |  |
| - Doors |  |  |  |
| 8.1.1. Entrance and exit doors | Visual inspection and by operation. | - Defective operation. - Deteriorated condition. - Defective emergency control. - Remote control of doors or warning devices defective. - Not in accordance with the requirements1. |  |
| 8.1.2. Emergency exits | Visual inspection and by operation (where appropriate). | - Defective operation. - Emergency exits signs missing or illegible. - Missing hammer to break glass. - Not in accordance with requirements1. |  |
| - Demisting and defrosting system |  |  |  |
| 8.2. Demisting and defrosting system (X)2 | Visual inspection and by operation. | - Not operating correctly. - Emission of toxic or exhaust gases into driver’s or passenger compartment. - Defective defrosting (if compulsory). |  |
| - Ventilation and heating system |  |  |  |
| 8.3 Ventilation & heating system (X)2 | Visual inspection and by operation. | - Defective operation - Emission of toxic or exhaust gases into driver’s or passenger compartment |  |
| - Seats |  |  |  |
| 8.4.1 Passenger seats (including seats for accompanying personnel) | Visual inspection. | - Seats in defective condition or insecure. - Folding seats (if allowed) not working automatically. - Not in accordance with the requirements1. |  |
| 8.4.2. Driver’s seat (additional requirements) | Visual inspection. | - Defective special devices such as anti-glare shield or anti-dazzle screen - Protection for driver insecure or not in accordance with requirements1. |  |
| - Interior lighting and if fitted, destination devices |  |  |  |
| 8.5. Interior lighting and destination devices (X)2 | Visual inspection and by operation. | Device defective or not in accordance with requirements1. |  |
| - Gangways, standing areas |  |  |  |
| 8.6. Gangways, standing areas | Visual inspection. | - Insecure floor. - Defective rails or grab handles. - Not in accordance with the requirements1. |  |
| - Stairs and steps |  |  |  |
| 8.7. Stairs and steps | Visual inspection and by operation (where appropriate). | - Deteriorated or damaged condition. - Retractable steps not operating correctly. - Not in accordance with requirements1. |  |
| - Requirements regarding the transportation of children (if applicable) |  |  |  |
| 8.8.1. Doors | Visual inspection. | Protection of doors not in accordance with the requirements1/ regarding this form of transport. |  |
| 8.8.2. Signalling and special equipment | Visual inspection. | Signalling or special equipment absent or not in accordance with requirements1. |  |
| - Requirements regarding the transportation of persons with reduced mobility (if applicable) |  |  |  |
| 8.9.1. Doors, ramps and lifts | Visual inspection and by operation. | - Defective operation. - Deteriorated condition. - Defective control(s). - Defective warning device(s). - Not in accordance with the requirements1. |  |
| 8.9.2. Wheelchair fixings | Visual inspection and by operation if appropriate. | - Defective operation. - Deteriorated condition. - Defective control(s). - Not in accordance with the requirements1. |  |
| 8.9.3. Signalling and special equipment | Visual inspection. | Signalling or special equipment absent or not in accordance with requirements1. |  |
| For Chapter 9 on Emissions see Part II |  |  |  |

| Item | Recommended Method | Main Reasons for Rejection |  |  | Comments/Guidance |
| --- | --- | --- | --- | --- | --- |
| - Additional tests for battery hybrid and electric vehicles |  |  |  |  |  |  |
| - Hazard marking |  |  |  |  |  |  |
| 10.1. Electric Vehicle and Hybrid Electric Vehicle’s electrical hazard marking as defined by UN Regulation No. 100, UN GTR No. 20 if required/fitted) | Visual inspection |  | - Missing or cannot be found - Incomplete or illegible - Not in accordance with vehicle documents or records |  |  |  |
| - Electric regenerative braking system |  |  |  |  |  |  |
| 10.2. Electric regenerative braking system | Visual inspection |  | (a) Components missing, damaged or corroded (b) Warning device malfunctioning - Warning device shows system malfunction |  |  |  |
| - Low voltage wiring |  |  |  |  |  |  |
| - Electric power train |  |  |  |  |  |  |
| 10.4.1. Residual Energy Storage System (RESS), e.g. Traction battery(ies) | Visual inspection with vehicle over a pit or on a hoist, including inside the engine compartment when appropriated |  | (a) Not in accordance with requirements (b) Insecure or not adequately secured (c) Insecure or not adequately secured with immediate risk of falling, short-circuit or chock hazard (d) Damaged or corroded components (e) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (f) Leaking (g) Shields not in place or damaged (h) Shields not in place or damaged with immediate risk of falling, short-circuit or chock hazard (i) Damaged or deteriorated electrical insulation (j) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard |  |  |  |
| 10.4.2. RESS management system if fitted / required, e.g. range information, state of charge indicator, battery thermal control. | Visual inspection when possible |  | (a) Not in accordance with requirements (b) Components missing or damaged (c) Components missing or damaged with immediate risk of falling, short-circuit or chock hazard (d) Warning device malfunctioning (e) Warning device shows system malfunction (f) Warning device shows critical system malfunction (g) Operation of RESS ventilation / cooling system impaired, e.g. blocking of ventilation holes, ducts, fluid leaks |  |  |  |
| 10.4.3. Electronic converters, motor and change control and wiring harness and connectors | Visual inspection with vehicle over a pit or on a hoist, including inside the engine compartment when appropriated |  | (a) Not in accordance with requirements (b) Insecure or not adequately secured (c) Insecure or not adequately secured with immediate risk of falling, short-circuit or chock hazard (d) Damaged or corroded components (e) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (f) Shields not in place or damaged (g) Shields not in place or damaged with immediate risk of falling, short-circuit or chock hazard (h) Damaged or deteriorated electrical insulation (i) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard |  |  |  |
| 10.4.4. Traction motor(s) | Visual inspection with vehicle over a pit or on a hoist, including inside the engine compartment when appropriated |  | (a) Not in accordance with requirements (b) Insecure or not adequately secured (c) Insecure or not adequately secured with immediate risk of falling, short-circuit or chock hazard (d) Damaged or corroded components (e) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (f) Shields not in place or damaged (g) Shields not in place or damaged with immediate risk of falling, short-circuit or chock hazard (h) Damaged or deteriorated electrical insulation (i) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard |  |  |  |
| 10.4.5. Auxiliary power equipment, e.g. heating, defrosting | Visual inspection with vehicle over a pit or on a hoist, including inside the engine compartment when appropriated |  | (a) Not in accordance with requirements (b) Insecure or not adequately secured (c) Insecure or not adequately secured with immediate risk of falling, short-circuit or chock hazard (d) Damaged or corroded components (e) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (f) Shields not in place or damaged (g) Shields not in place or damaged with immediate risk of falling, short-circuit or chock hazard (h) Damaged or deteriorated electrical insulation (i) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard |  |  |  |
| 10.4.6. Service disconnect device | Visual inspection and voltage absence check, where possible without disassembling |  | (a) Insecure or not adequately secured (b) Insecure or not adequately secured with imminent risk of short-circuit or chock hazard (c) Damaged or corroded components (d) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (e) Shields not in place or damaged (f) Shields not in place or damaged with immediate risk of falling, short-circuit or chock hazard (g) Damaged or deteriorated electrical insulation (h) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard (i) Voltage presence |  |  |  |
| 10.4.7. “Active driving possible mode” indicator and associated information signal if driver leaves vehicle in active driving possible mode if fitted / required | Visual inspection and by operation if possible |  | (a) Indicator / information signal not fitted in accordance with the Requirements (b) Indicator / information signal not functioning correctly |  |  |  |
| 10.4.8. “State of drive direction” indicator if fitted / required | Visual inspection and by operation |  | (a) Indicator not fitted in accordance with the requirements (b) Indicator not functioning correctly |  |  |  |
| 10.4.9. RESS external charging system if fitted. Charging cable(s) if fitted and if possible | Visual inspection if possible |  | (a) Not in accordance with requirements (b) Damaged or corroded components (c) Damaged or corroded components with immediate risk of falling, short-circuit or chock hazard (d) Damaged or deteriorated electrical insulation (e) Damaged or deteriorated electrical insulation with immediate risk of falling, short-circuit or chock hazard |  |  |  |

1	"Requirements" are laid down by type-approval/certification requirements at the date of first registration, first entry into service or at time of manufacture  as well as retrofitting obligations or national legislation.

2	"(X)" Identifies items which are related to the condition of the vehicle and its suitability for use on the road, but which are not considered essential in a [periodic] [export/import] inspection.

Annex (Part II)

(forthcoming)

To be replaced once number determined

Main purpose of the inspection is related to the assessment if the current condition may pose a risk to road safety. The presence/non presence would only be checked if there is a legal requirement of having them in the jurisdiction where the vehicle is registered/used.

Chapter 10 is still under discussion by the IWG, but the IWG recognises the importance of this item and will keep it on the list pending more discussions.