Discussion paper – Trailer Reversing System Indication by Truck HMI

Problem Statement

Today’s scope of UN ECE Regulation No. 158 applies only to vehicle categories M and N while not considering trailers (category O (especially O3, O4)). Since mostly tractors are driving with a trailer attached, the truck reversing system (camera-monitor or detection system) is either switched off due to the presence of the trailer or it only shows / detects the attached trailer but not the area behind the trailer which is relevant during reversing the truck-trailer combination; hence, there is no useful assistance for the driver in these scenarios.

Since more than 20 years there are reversing assistance systems available for trailers.

Due to missing standardization in the past, the data transfer from trailer reversing aids to the truck was not possible, so these systems were designed to work independent from the truck which results in a poor distance indication to the driver, e.g. by flashing of the trailers end-outline marker lamps or actuation of a trailer-mounted beeper, if an obstacle is detected instead of a clear indication by the trucks HMI / dashboard in the driver’s cabin.

Proposal

Thanks to the update of the ISO 11992-2:[2026] that describes the Interchange of digital information between trucks and trailers, a standardized message to transfer distance data from a trailer’s reversing system to the truck is now available.

The distance of an obstacle behind the trailer can be transferred from the towed to the towing vehicle by using the parameter “APP – Rear obstacle distance” acc. ISO 11992-2:[2026].

Besides the distance, also the status of the reversing system can be transferred by using the parameter “Loading ramp approach assistance status” acc. ISO 11992-2:2014.

It is essential to ensure the driver’s awareness of vulnerable road users behind vehicles, when reversing, including for truck-trailer combinations. To encourage deployment of reversing detection on trailers, provisions are necessary to ensure a harmonized communication between trucks and trailers.

Proposal for an amendment of UN Regulation No. 158

“ 17.1     System activation

The system shall be activated when the backing event starts. If proper functioning cannot be affected, either the system shall automatically shut off or the driver shall be able to deactivate the system manually.

The detection system shall remain active as long as the vehicle direction selector is in the reverse position.

In case the vehicle can detect coupling with a coupling device, the system shall provide the information signal from the towed vehicle, if available. If no information from the coupled vehicle is available - the system may be switched off.

The detection system information of a coupled vehicle shall be deemed available, when

- Parameter “Loading ramp approach assistance status” is transmitted by the towed vehicle in accordance with ISO 11992-2 with value 01b (loading ramp approach assistance active) and

- Parameter “APP – Rear obstacle distance” is transmitted by the towed vehicle in accordance with ISO 11992-2 with value in the range of [0x00; 0xFA] (corresponds to [0;500cm]).”

Justifications

The ISO 11992-2 is the standardized interface between truck and trailer and mandated anyhow by UN Regulation No. 13 regarding their braking system.

By using this same interface to transfer the detection system information of a trailer, no additional hardware is needed.

By mandating via the UN regulation No. 158, the presentation of the detection system information from a trailer by the truck it will be achieved that reversing system information of a trailer is presented to driver, if the trailer is fitted with such system. This will result in a consistent driver reversing assistance with and without a coupled trailer.

Lastly, the proposal is compliant to the results of the VRU Proxi Task Force to not yet extend the scope of UN regulation No. 158 to the vehicle category O but paves the way to a future extension. And for today’s trailers it ensures better driver assistance if they are fitted with detection systems and encourages manufacturers to install these systems.