| ORG | Paragraph # | Type of comment1 | Original text (Consolidated version prepared by OICA) | Proposed change | Comments / Justification |
| --- | --- | --- | --- | --- | --- |
| France | Contents | Ge/Ed | 6 Provisions for a label for liquefied hydrogen vehicles of categories M2/M3 and N2 /N3……..43 | 6 Provisions for a label for liquefied hydrogen vehicles of categories M2/M3 and N2 /N3……..43 7 Periodic technical inspections | As the scope of this regulation is limited to M2/M3 and N2/N3 vehicles, there is no need to mention those categories here. Add a line to the table of contents related to Periodic technical inspection |
| TMC | 0.1 | Ge |  | 0.1.1. This is a transposition of paragraph 7. of GTR 13, which has been adopted as optional for the Contracting Parties, into a UN Regulation. Due to the introduction of hydrogen propelled vehicles in UN-R 105 on the construction of vehicles for the transport of dangerous goods, establishment of a UN Regulation incorporating appropriate administrative provision for type approval has been desired. 0.1.2. It was not intended to introduce new technical requirements deviated from those established in GTR 13, except for adjustment of deficiencies or clarifications considered as indispensable for facilitating the type approval of the related products. 0.1.3. It was decided to develop a new UN Regulation for vehicles and systems fuelled with liquefied hydrogen rather than incorporating them into UN Regulation No. 134, to allow the Contracting Parties applying UN Regulation No. 134 not to apply the new UN Regulation until such Contracting Parties will complete necessary examinations to introduce new technology into their respective national regulatory framework 0.1.4. The test procedures for the verification of post-crash hydrogen leakage and concentration given in GTR 13 were deemed premature and not ready to use for type approval regulation. 0.1.5. On the other hand, current focus of the application of liquefied hydrogen is given to heavy duty vehicles (i.e. categories M2, M3, N2 & N3) to which vehicle crash tests are not applied in the most of Contracting Parties. Therefore, the scope of this Regulation is limited to heavy duty vehicles and the post-crash requirements are not adopted in this Regulation. 0.1.6. Hydrogen storage technology is still evolving. Cryo-compressed hydrogen storage is one of such new technologies that stores hydrogen at very low temperature and relatively high pressure. Since paragraph 7. of GTR 13 was developed without taking any account for such a technology, this UN Regulation does not apply to cryo-compressed hydrogen storage. The definition of liquefied hydrogen is considered to determine the boundary of liquefied hydrogen and cryo-compressed hydrogen. 0.1.6 This Regulation does not apply to cryo-compressed or compressed hydrogen systems. 0.1.7. It is expected that the next phase of GTR 13 will address the missing items such as post-crash requirements of vehicles with a liquefied hydrogen storage system, cryo-compresses hydrogen storage system, etc. and will accommodate the updated technical requirements being developed at ISO/TC 197. | TMC: Introduction to be prepared DT: I don‘t think this is useful. Liquid hydrogen automatically excludes ccH2. Otherwise we would also have to add such a note in UN R134 as in many aspects ccH2 is closer to CGH2 than to LH2 being compressed gaseous hydrogen at very low temperatures 12.08.2025 Group: If really needed reduce the text to just exclude cCH2 |
| TMNA | 0.1.7. | Ge/Ed | 0.1.7. It is expected that the next phase of GTR 13 will address the missing items such as post-crash requirements of vehicles with a liquefied hydrogen storage system, cryo-compresses hydrogen storage system, etc. and will accommodate the updated technical requirements being developed at ISO/TC 197. | 0.1.7. It is expected that the next phase of GTR 13 will address the missing items such as post-crash requirements of hydrogen fuelled vehicles. liquefied hydrogen storage system, cryo-compressed hydrogen storage system, etc. and will accommodate the updated technical requirements being developed at ISO/TC 197. | TMNA Suggest simplified wording and include all fuel types as “hydrogen fuelled.” Should probably confirm if post-crash requirements are being developed at TC197 before including this in the regulation. |
| Pilots / Group | 0 |  | Include introduction on sLH2 |  | DT: No, from a storage homologation perspective, sLH2 is not relevant. sLH2 describes the fuelling process |
| TMC | 2.1 | Ge | 2.1. "Boil-off system" means a system that... |  | TMC: Definition of “boil-off system” will be necessary but should be technology neutral. Forvia: Agree definitions are needed here. However, the generic term „system“ is not precise enough in this context regarding the included functions: does a Boil-off system only control the inner container pressure or is a conversion also included? I would propose to split the definition and harmonize to ISO: boil-off valve: valve that vents the excess hydrogen gas pressure (boil-off gas) at a setting below the container’s first pressure relief valve setting boil-off converter: system that converts boil-off hydrogen to water and heat to prevent hydrogen discharge and associated risks of generating flammable atmospheres around the vehicle 12.08.2025 Group: Agree that more precise definitions are needed. Added 2.1 Boil-off converter and 2.2 Boil-off valve |
| TMC | 2.7 | Ge | “Date of manufacture” (of a liquefied hydrogen container) means the date (month and year) of the final inspection test carried out during manufacture. | “Date of manufacture” (of a liquefied hydrogen container) means the date (month and year) of the final inspection test carried out during manufacture. | TMC: In GTR13 Phase 2, it is allowed to use final inspection date as the date of manufacturer. For LHSS, proof pressure test is conducted during the manufacturing process but not on completed product. 12.08.2025 Group: Agree with TMC, accept proposed change. |
| Forvia | 2.14. | Ge | 2.14. “Liquefied hydrogen” means … look at ISO definition ISO 14687:2019(en) Hydrogen fuel quality — Product specification 3.15 liquid hydrogen – hydrogen that has been liquefied, i.e. brought to a liquid state. |  | Forvia: Equivalent to ISO. Propose to keep it as adding more would rather confuse and probably trigger more discussions. For sLH2 maybe we can simply add a note that subcooled is considered to be included in LH2. DT: Supporting proposal from Richard. No reference to sLH2 needed. sLH2 tanks have been homologated according to the text of this document without deviation. The tanks themselves are not different 12.08.2025 Group: Can we use the word liquefied to define liquid H2 or do we need to specify temperature? No definition of compressed H2 in R 134, so no definition for liquid needed. Therefore, delete proposal. |
| TMC | 2.14 | Ge | "Liquefied Hydrogen Storage System (LHSS)” means liquefied hydrogen storage container(s), PRDs, shut-off device, a boil-off system and the interconnection piping (if any) and fittings between the above components | "Liquefied Hydrogen Storage System (LHSS)” means liquefied hydrogen storage container(s), PRDs, shut-off device, a boil-off system and the interconnection piping (if any) and fittings between the above components. | TMC: The treatment system for boil-boil off hydrogen should not be a part of LHSS since it is outside of the boundary (para.5). Forvia: I would make this „boil-off valve“ DT: Ok, except if we consider it convered by the PRDs 12.08.2025 Group: Modify definition “boil-off system” to “boil-off valve” |
| TMC | 2.17 | Ge | "Maximum Fuelling Pressure (MFP)" means the maximum pressure applied to compressed system during fuelling. The maximum fuelling pressure is 125 per cent of the Nominal Working Pressure. | "Maximum Fuelling Pressure (MFP)" means the maximum pressure applied to a LHSS during fuelling defined by the manufacturer. Add “defined by manufacturer” | 26.08.2025 Group: Check ISO definition Agree to use ISO definition Maximum dispensing system pressure expected during normal (fault-free) vehicle fuelling |
| TMC | 2.20 | Ge | "Nominal Working Pressure (NWP)" means the gauge pressure that characterizes typical operation of a system. ] | "Nominal Working Pressure (NWP)" means the gauge pressure that characterizes typical operation of a system. ] | TMC: Different from CHSS. This pressure will be the pressure downstream of the pressure regulator DT: Delete, there is one occurence remaining that should be turned to MAWP 12.08.2025 Group: Agree to delete |
| Pilots / Group | 2.19 | Ge | "Pressure Relief Device (PRD)" means a device that, when activated under specified performance conditions, is used to release hydrogen from a pressurized system and thereby prevent failure of the system. | "Pressure Relief Device (PRD)" means a device that, when activated under specified performance conditions, is used to release hydrogen from a pressurized system and thereby prevent failure of the system. It can include first and second pressure relief devices as a reclosing or non-reclosing type (e.g., safety relief valve and burst disc). | 12.08.2025 Group: First and second PRDs to differenciate between „primary“ and „secondary“ primary closure devices, consistent with ISO 13985. Both the first and second PRDs are considered primary closure devices. Instances of “secondary” will be changed to “second.” |
| Pilots / Group | 2.xx | Ge | Do we need a definition for container attachments? |  | DT: Would say no container attachment needed for now, postpone to GTR13 Phase 3 if we realize that the same differenciation is needed as for CGH2 12.08.2025 Group: Agree no need. |
| TMC | 2.25 | Te | “Type of hydrogen storage system” … (cb) the nominal maximum allowable working pressure (NWPMAWP); |  | TMC: MAWP might be better to specify the LHSS characteristics. NWP will be the system pressure after pressure regulator that does not belong to LHSS. DT: MAWP is fine. NWP is very specific to CHG2 with the reference pressure at 15°C 12.08.2025 Group: Delete compressed gas. Agree NWP should be MAWP. |
| TMC | 2.26 | Te | "Type of specific components of hydrogen storage system" means a component or an assembly of components which do not differ significantly in such essential aspects as: (a) the manufacturer's trade name or mark; (b) the sort of component: (T)PRD, check-valve or shut-off device, and (c) the structure, materials and essential characteristics. | "Type of specific components of hydrogen storage system" means a component or an assembly of components which do not differ significantly in such essential aspects as: (a) the manufacturer's trade name or mark; (b) the sort of component: (T)PRD (boil-off valve, safety relief valve or burst disc), check-valve or shut-off valvedevice, and (c) the structure, materials and essential characteristics. | TMC: Typically, primary PRD is safety relief valve and secondary PRD is burst disc. DT: ISO 13985 is preferring first and second PRD rather than primary and secondary. Check the whole document if agreed 12.08.2025 Group: Switch to first and second PRD, consistent with ISO 13985. |
| TMC | 4.4.2. | Ge | the number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the circle prescribed in paragraph 4.4.1. | the number of this Regulation, followed by the letter "R", a dash and the approval number six digit number, of which the first two digits indicate the series of amendments to this Regulation to which the approval is granted and the subsequent four digits indicates section 3 of the type approval number granted, to the right of the circle prescribed in paragraph 4.4.1. | TMC: This is not correct (although this text is standard text for the most of UNR.) 12.08.2025 Group: Agree |
| TMC | 5. |  | This part specifies the requirements for the liquefied hydrogen storage system. The boundaries of the LHSS are defined by the interfaces which can isolate the stored liquefied (and/or gaseous) hydrogen from the remainder of the fuel system and the environment. All components located within this boundary are subject to the requirements defined in this paragraph. - Figure 1 shows a typical LHSS. The closure devices shall include the following functions, which may be combined: - Automatic shut-off device; - Boil-off system; (b) Pressure Relief Device (PRD). | This part specifies the requirements for the liquefied hydrogen storage system. The boundaries of the LHSS are defined by the interfaces which can isolate the stored liquefied (and/or gaseous) hydrogen from the remainder of the fuel system and the environment. All components located within this boundary are subject to the requirements defined in this paragraph. - Figure 1 shows a typical LHSS. The closure devices shall include the following functions, which may be combined: - Automatic sShut-off device; - Boil-off system; (b) Pressure Relief Device (PRD). | TMC: The valve for boil-off system and pressure relief device for vacuum jacket are considered as “specific component” subject to type approval? 12.08.2025 Group: Group accepts change to (a). For (b), Group requires more discussion re: boil-off valve function or boil-off control. Should be consistent with the definitions. (c) Agree to change to Pressure Relief function 25.08.2025 Group: Group accepts change to functions instead of devices. Also add language that the functions are minimally specified. |
| Pilots / Group | 5 Figure 1 |  |  |  | TMC: In Figure 1, “Vacuum Jacket Pressure Relief Device” is described. I understand that this device is not a scope of Part II component approval and therefore should not use “Pressure Relief Device” or delete this device from the Figure 1. 12.08.2025 Group: Add label to boil-off valve Change boil-off system to boil-off converter Delete Vacuum Jacket PRD and change drawing |
| France | 5 Figure 1 | ed |  |  | Could be useful to mention which one is the 1st PRD and which is the 2nd PRD on figure 1 |
| TMC | 5 Figure 1 Note | Ge | Note: The boil-off system is not required but can be used for the test in paragraph 5.2.2. | Note: The boil-off system is not a part of LHSS but may be used for the test in paragraph 5.2.2., at the manufacturer’s discretion. | TMC: Further clarification DT: Take out of the note if there is a requirement and use boil-off converter 12.08.2025 Group: Delete note, the text in para 5.2.2. already allows for boil-off system. |
| TMC | 5 Table 1 | Ge | Table 1 Overview of performance requirements Paragraph 5.1. Verification of baseline metrics 5.1.1. Proof pressure 5.1.2. Baseline initial burst pressure, performed on the inner container 5.1.3. Baseline Pressure cycle life. Paragraph 5.2. Verification of expected on-road performance 5.2.1. Boil-off 5.2.2. Leak 5.2.3. Vacuum loss. Paragraph 5.3. Verification for service-terminating conditions: Bonfire test. Paragraph 5.2.4. Requirements for pressure relief device and shut-off device. | Table 1 Overview of performance requirements Paragraph 5.1. Verification of baseline metrics 5.1.1. Proof pressure 5.1.2. Baseline initial burst pressure, performed on the inner container 5.1.3. Baseline Pressure cycle life. Paragraph 5.2. Verification of expected on-road performance 5.2.1. Boil-off 5.2.2. Boil-off hydrogen emissions Leak 5.2.3. Vacuum loss. Paragraph 5.3. Verification for service-terminating conditions: Bonfire test. Paragraph 5.2.4. Requirements for pressure relief device and shut-off device. | Pilots: Correct once the title has been finalized Group 26.08.25 Change 5.2.2. to Boil-off discharge rate |
| TMC | 5.1.1 | Ge | A system is pressurized to a pressure ptest ≥ 1.3 * (MAWP + 0.1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | The inner container and the pipe work situated between the inner container and the outer jacket A system is pressurized to a pressure ptest ≥ 1.3 * (MAWP (in MPa) + 0.1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | TMC: The test object should be clarified. 12.08.2025 Group: Agree |
| France | 5.1.1 | ed | The inner container and the pipe work situated between the inner container and the outer jacket A system is pressurized to a pressure ptest ≥ 1.3 * (MAWP (in MPa) + 0.1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | The inner container and the pipe work situated between the inner container and the outer jacket A system is pressurized to a pressure ptest ≥ 1.3 * (MAWP (in MPa) + 0.1 MPa) 130 per cent (MAWP + 0,1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | Prefer the following wording : “130 per cent (MAWP + 0,1 MPa)“ which is consistent to the wording used for vacuum loss test (Annex 3 §2.3) and Verification test for service-terminating performance due to fire (Annex 3 § 3.2) |
| TMC | 5.1.3 | Ge | When using metallic containers and/or metallic vacuum jackets, the manufacturer shall either provide a calculation in order to demonstrate that the container is designed according to current regional legislation or accepted standards (e.g. in US the ASME Boiler and Pressure Vessel Code, in Europe EN 1251-1:2000 and EN 1251-2:2000 and in all other countries an applicable regulation for the design of metallic pressure containers), or define and perform suitable tests (including Annex 3, paragraph 1.3.) that prove the same level of safety compared to a design supported by calculation according to accepted standards. For non-metallic containers and/or vacuum jackets, in addition to the tests in Annex 3, paragraph 1.3., suitable tests shall be designed by the manufacturer to prove the same level of safety as a metallic container. | When using metallic containers [and/or metallic vacuum jackets], the manufacturer shall either provide a calculation in order to demonstrate that the container is designed according to current regional legislation or accepted standards (e.g. in US the ASME Boiler and Pressure Vessel Code, in Europe EN 1251-1:2000 and EN 1251-2:2000 and in all other countries an applicable regulation for the design of metallic pressure containers), or define and perform suitable tests (including Annex 3, paragraph 1.3.) that prove the same level of safety compared to a design supported by calculation according to accepted standards. [For non-metallic containers [and/or non-metallic vacuum jackets], in addition to the tests in Annex 3, paragraph 1.3., suitable tests shall be designed by the manufacturer to prove the same level of safety as a metallic container.] | TMC: Does the material of vacuum jacket critical for cycle life? “and/or” is not appropriate. “and” will be correct. DT: Probably that and is fine. It is indeed not seeing pressure cycles over its life time, but may at least need a design to withstand vacuum TMC: What is exactly required? Is this practicable for type approval? DT: The requirement is to use one of this standard to qualify the pressure bearing container as a pressure vessel. Standard pressure vessels TMC: Limitation of metallic container to be considered. 12.08.2025 Group: Keep as it is. |
| TMC | 5.2.2 | Ge | Leak After the boil-off test in paragraph 2.1. of Annex 3, the LHSS is kept at boil-off pressure and the total discharge rate due to leakage shall be measured in accordance with the test procedure in Annex 3, paragraph 2.2. The maximum allowable discharge from the hydrogen storage system is R*150 Nml/min where R = (Vwidth+1)*(Vheight+0.5)*(Vlength+1)/30.4 and Vwidth, Vheight, Vlength are the vehicle width, height, length (m), respectively. | Boil-off hydrogen emissions After the boil-off test in paragraph 2.1. of Annex 3, the LHSS is kept at boil-off pressure and the total discharge rate due to leakage boil-off shall be measured in accordance with the test procedure in Annex 3, paragraph 2.2. The boil-off system recommended by the LHSS manufacturer may be installed for this test if requested by the manufacturer. The maximum allowable discharge from the hydrogen storage system is R*150 Nml/min where R = (Vwidth+1)*(Vheight+0.5)*(Vlength+1)/30.4 and Vwidth, Vheight, Vlength are the vehicle width, height, length (m), respectively. In case that the vehicle dimensions are not determined, the LHSS manufacturer shall specify the minimum values of Vwidth, Vheight and Vlength, The LHSS manufacturer shall provide the instruction to vehicle manufacturers for the installation of the LHSS that contains: (a) Necessary specifications of the boil-off system; and (b) Minimum dimensions of the vehicle. New proposal by Forvia & DT (14.08.2025) 5.2.2 Boil-off discharge rate After the boil-off test in paragraph 2.1. of Annex 3, the LHSS is kept at boil-off pressure and the total discharge rate due to boil-off shall be measured in accordance with the test procedure in Annex 3, paragraph 2.2. The boil-off system recommended by the LHSS manufacturer may be installed for this test if requested by the manufacturer. The maximum allowable discharge from the hydrogen storage system is R*150 Nml/min where R = (Vwidth+1)*(Vheight+0.5)*(Vlength+1)/30.4 and Vwidth, Vheight, Vlength are the vehicle width, height, length (m), respectively. | TMC: For clarity. Boil-off system will be installed by the vehicle manufacturer, not LHSS manufacturer. Forvia: I would propose to keep it open „manufacturer“. A LHSS may be sold together with a suitable boil-off converter OR it could also be attached by the vehicle manufacturer. DT: Not sure it is even needed. I am checking internally but this boil-off rate may be given independently from the boil-off converter, rather reflecting the insulation quality of the storage TMC: Need administrative provisions for independent LHSS manufacturer. |
|  |  |  |  |  | Forvia: I have quite a concern here: Without specifying the maximum number of tanks to be installed in a vehicle this information is incomplete and potentially misleading … I would propose to rather only include the measured leakage value for the LHSS according to these test conditions in the instructions. It is then up to the vehicle manufacturer to determine if this is sufficient for the vehicle tanking service conditions and the number LHSS to be installed in the vehicle into account, or to possibly apply modifications to vehicle installation to meet the needed overall vehicle leakage limits. DT: Good proposal 12.08.2025 Group: Forvia to propose language for test |
| TMC | 5.2.2 | Ge |  |  | TMC: At the last OICA/CLEPA call, we concluded that the “Leak” test of paragraph 5.2.2. after boil-off test should be the measurement of the amount of hydrogen emitted from boil-off system (after conversion). But the test procedure seems to be not appropriate for such a measurement since external gas source with pressure is used. We have to find out a pragmatic solution for this. Some idea could be: - Remove this requirement from Part I (LHSS approval) and include somewhat general requirement in Part III (vehicle approval). - Remove this requirement from Part I (LHSS approval) and include additional test and requirement in Part III (vehicle approval) based on the exhaust system requirements. We need the inputs from companies experienced in LHSS system/vehicle. Group 25.08.25 (Square bracket language stays and will be modified later with informal) 5.2.2 Boil-off discharge rate After the boil-off test in paragraph 2.1. of Annex 3, the LHSS is kept at boil-off pressure and the total discharge rate due to boil-off shall be measured in accordance with the test procedure in Annex 3, paragraph 2.2. The boil-off system recommended by the LHSS manufacturer may be installed for this test if requested by the manufacturer. [The maximum allowable discharge from the hydrogen storage system is R*150 Nml/min where R = (Vwidth+1)*(Vheight+0.5)*(Vlength+1)/30.4 and Vwidth, Vheight, Vlength are the vehicle width, height, length (m), respectively.] |
| France | 7.1.1.3 | Ge | The fuelling receptacle shall not be mounted within the external energy absorbing elements of the vehicle (e.g. bumper) and shall not be installed in such a way that access for refilling shall not be required in the passenger compartment, luggage compartment, or in any other unventilated compartment. in the passenger compartment, luggage compartment and other places where hydrogen gas could accumulate and where ventilation is not sufficient. Test procedure is by visual inspection. |  | Before modification, this paragraph prohibited the installation of the fuelling receptacle in the passenger compartment. After modification, the access for refilling shall not be required in the passenger compartment but nothing forbid the installation of the fuelling receptacle in the passenger compartment anymore. Need further explanations. |
|  | 7.1.1.4 |  | If appropriate, the geometry of the fuelling receptacle of liquefied hydrogen gas vehicles may be at the manufacturer’s discretion and in agreement with the technical service in absence of a standard. |  | TMC: Not enforceable Forvia: Would propose to delete for now and add with reference to the standard when published in later amendment. DT: Remove and introduce later amendment when ISO 25578 published 01/2028 25.08.26 Group: Agree to delete current text, add reference to ISO 25578 in introduction |
| TMC | New para. 7.1.2 | Ed |  | 7.1.2. LHSS installation LHSS shall be installed in accordance with the instruction provided by the LHSS manufacturer. | TMC: Need this kind of provision to ensure proper installation of LHSS. Forvia: I have quite a concern here: Without specifying the maximum number of tanks to be installed in a vehicle this information is incomplete and potentially misleading … I would propose to rather only include the measured leakage value for the LHSS according to these test conditions in the instructions. It is then up to the vehicle manufacturer to determine if this is sufficient for the vehicle tanking service conditions and the number LHSS to be installed in the vehicle into account, or to possibly apply modifications to vehicle installation to meet the needed overall vehicle leakage limits. DT: Do we have the same wording in UNR134, if not we should be more specific here 26.08.25 Group: Agree to below language 7.1.2. LHSS installation The LHSS shall be installed in accordance with the instructions provided by the manufacturer of the LHSS, and in conformity with the description provided in Annex 1, Part 2 to this Regulation |
| TMC | 7.2.2.1 | Te | Pressure relief systems The hydrogen gas discharge from pressure relief devices shall not be directed: (a) towards exposed electrical terminals, exposed electrical switches or other ignition sources; (b) into or towards the vehicle passenger or luggage compartments; (c) into or towards any vehicle wheel housing; and (d) towards hydrogen storage containers. |  | TMC: In GTR13 Amendment 1, the relevant paragraph only applies to TPRD. The relevant provision of GTR13 phase 1is introduced. 25.08.25 Group: Adopt GTR13 phase 2 language Pressure relief systems The hydrogen gas discharge from pressure relief systems shall be directed such that the hydrogen exhaust does not impinge upon: (i) enclosed or semi-enclosed spaces; (ii) any vehicle wheel housing; (iii) hydrogen gas containers; (iv) the vehicle’s REESS |
| TMC | 7.2.3.1 |  | Hydrogen gas discharge, leakage and/or permeation from the vehicle fuel system shall not directly vent into the passenger compartment, luggage compartment, or to any enclosed or semi-enclosed spaces within the vehicle that contains unprotected ignition sources. |  | TMC: Cargo compartment is not defined, but the enclosed cargo space for vans and trucks are considered as enclosed or semi-enclosed space. Group 25.08.25 – Aligns with GTR13 Phase 2 |
| TMC | 7.3 | Ed | Container retention LHSS shall be subject to the relevant accelerations in compliance with the acceleration corridors which are specified in Tables 2 to 3 in both positive and negative directions. The storage container(s) shall remain attached to the vehicle at a minimum of one attachment point. … | Container LHSS retention LHSS shall be subject to the relevant accelerations in compliance with the acceleration corridors which are specified in Tables 2 to 3 in both positive and negative directions. The storage container(s)LHSS shall remain attached to the vehicle at a minimum of one attachment point. … | TMC: Container --> LHSS. Fixture will be applied to vacuum jacket (not the inner container). |
| TMC | Annex 1 – Part 1 Model – I 0. | Ge | 0. General |  | TMC: Minimum dimensions of vehicle may need to be specified for compliance with 5.2.2. Forvia: I have a concern here. If anything I would propose adding the leakage measured during testing. |
| Forvia | Annex 1 – Part 1 Model – I 0. | Ge | 3.9.2.4. Nominal working pressure(s): … 3.9.4.8. Nominal working pressure(s): |  | Forvia: Propose to change to MAWP. DT: Remove NWP, MAWP is already included 12.08.2025 Group: Delete compressed gas. Agree NWP should be MAWP. |
| Forvia | Annex 1 – Part 1 Model – II | Ge | 3.9.2.4. Nominal working pressure(s): … 3.9.3.8. Nominal working pressure(s): |  | Forvia: Propose to change to MAWP. 12.08.2025 Group: Delete compressed gas. Agree NWP should be MAWP. |
| TMC | Annex 1 – Part 1 Model – III 2. | Ge | 2. MASSES AND DIMENSIONS (in kg and mm) (Refer to drawing where applicable) 2.4. Range of vehicle dimensions (overall) 2.4.1. For chassis without bodywork 2.4.1.1.2. Minimum permissible length: 2.4.1.2.2. Minimum permissible width: 2.4.1.3. Height (in running order) (for suspensions adjustable for height, indicate normal running position): 2.4.1.3.1. Maximum permissible height: 2.4.2. For chassis with bodywork 2.4.2.1. Length: 2.4.2.2. Width: 2.4.2.3. Height (in running order) (for suspensions adjustable for height, indicate normal running position): |  | TMC: Need to check compliance to paragraph 5.2.2. |
| France | Annex 1 Part 2 | ed | 1. Trademark: 2. Type and trade names: 3. Name and address of manufacturer: 4. If applicable, name and address of manufacturer’s representative: 5. Brief description of vehicle/component:2 6. Date of submission of vehicle/component for approval:2 7. Technical Service performing the approval tests: 8. Date of report issued by that Service: 9. Number of report issued by that Service: 10. Approval with regard to the safety-related performance of hydrogen-fuelled vehicles is granted/refused: 2 11. Place: 12. Date: 13. Signature: 14. The information document annexed to this communication: 15. Any remarks: | 1. Trademark: 2. Type and trade names: 3. Name and address of manufacturer: 4. If applicable, name and address of manufacturer’s representative: 5. Brief description of vehicle/component: 6. Installation restrictions applicable to the LHSS as described in paragraph 5.2.2.: 6. 7. Date of submission of vehicle/component for approval:2 7. 8. Technical Service performing the approval tests: … | In the boil off discharge rate test (§ 5.2.2), the max allowable discharge from the hydrogen storage system depends on the width, height and length of the vehicle in which the LHSS will be installed. Then, not every LHSS will be able to fit into every car. Installation restriction are then needed. |
| France | Annex 3 1.1.1 | ed | (a) For metallic containers, either ptest is equal to or greater than the maximum pressure of the inner container during fault management (as determined in Annex 3, paragraph 2.3.) or the manufacturer proves by calculation that at the maximum pressure of the inner container during fault management no yield occurs; [(b) For non-metallic containers, ptest is equal to or greater than the maximum pressure of the inner container during fault management (as determined in Annex 3, paragraph 2.3.).] | (a) For metallic containers, either ptest is equal to or greater than the maximum pressure of the inner container during fault management vacuum loss test (as determined in Annex 3, paragraph 2.3.) or the manufacturer proves by calculation that at the maximum pressure of the inner container during fault management no yield occurs; [(b) For non-metallic containers, ptest is equal to or greater than the maximum pressure of the inner container during fault management vacuum loss test (as determined in Annex 3, paragraph 2.3.).] | Why don’t we refer to vacuum loss test here instead of fault management ? Seems to be more comprehensible to refer to vaccum loss test directly here. |
| TMC | Annex 3 1.1.1 | Te | [(b) For non-metallic containers, ptest is equal to or greater than the maximum pressure of the inner container during fault management (as determined in Annex 3, paragraph 2.3.).] |  | TMC: Limitation to metallic container to be considered. Group 25.08.25 – Keep in square brackets. |
| France | Annex 3 1.2 | ed | For steel inner containers, the test is passed successfully if at least one of the two passing criteria described in paragraph 5.1.2. of this Regulation is fulfilled. | For steel inner containers, the test is passed successfully if at least one of the two the passing criteria described in paragraph 5.1.2. of this Regulation is fulfilled. | There is only one passing criteria described in § 5.1.2: burst pressure shall be equal to the burst pressure used for the mechanical calculations. (with 2 methods to determine the burst pressure used for the mechanical calculations for steel containers) |
| France | Annex 3 1.2 | ed | For inner containers made out of an aluminium alloy or other material, a passing criterion shall be defined which guarantees at least the same level of safety compared to steel inner containers. | For inner containers made out of an aluminium alloy or other metallic material, a passing criterion shall be defined which guarantees at least the same level of safety compared to steel inner containers. | Limitation to metallic container to be considered. Otherwise, this part should be in SQB if non metallic containers are considered here. |
| TMC | Annex 3 2.2 |  | An LHSS shall be free from leakage through stem or body seals or other joints and shall not show evidence of porosity in casting when tested as described in Annex 4, paragraph 3.3. at any gas pressure between zero and its MAWP. The test shall be performed on the same LHSS at the following conditions: (a) At ambient temperature; (b) At the minimum operating temperature or at liquid nitrogen temperature after sufficient conditioning time at this temperature to ensure thermal stability; (c) At the maximum operating temperature after sufficient conditioning time at this temperature to ensure thermal stability. During this test, the LHSS under test shall be connected to a source of gas pressure. A positive shut-off device and a pressure gauge having a pressure range of not less than 150 per cent and not more than 200 per cent of the test pressure shall be installed in the pressure supply piping; the accuracy of the gauge shall be 1 per cent of the pressure range. The pressure gauge shall be installed between the positive shut-off device and the sample under test. Throughout the test, the sample shall be tested for leakage, with a surface-active agent without formation of bubbles or measured with a leakage rate less than 216 Nml/hr. [The test shall be conducted according to the procedure described in Annex 4, paragraph 2.] |  | TMC: Paragraph 3.3 applies to shut-off device. Is this mean PRD and container are not subject to this provision? Insert the test procedure of Annex 4, paragraph 2 with necessary clarifications, such as the operating temperature. This test procedure seems to be not appropriate for measuring the boil-off hydrogen emission because external gas source is used. Need to consider a pragmatic solution. Forvia: Agree: the LHSS is filled and conditions with LH2, so there is not need to use test gas. AND Additionally conducting this test at ambient external temperatures is sufficient as the closures have already been tested as a component at higher temperatures and any component beging subjected to cold LH2 temperatures in service will be subjected to these temperatures in this system test. |
| Forvia / DT | Annex 3 2.2 |  | An LHSS shall be free from leakage through stem or body seals or other joints and shall not show evidence of porosity in casting when tested as described in Annex 4, paragraph 3.3. at any gas pressure between zero and its MAWP. The test shall be performed on the same LHSS at the following conditions: (a) At ambient temperature; (b) At the minimum operating temperature or at liquid nitrogen temperature after sufficient conditioning time at this temperature to ensure thermal stability; (c) At the maximum operating temperature after sufficient conditioning time at this temperature to ensure thermal stability. During this test, the LHSS under test shall be connected to a source of gas pressure. A positive shut-off device and a pressure gauge having a pressure range of not less than 150 per cent and not more than 200 per cent of the test pressure shall be installed in the pressure supply piping; the accuracy of the gauge shall be 1 per cent of the pressure range. The pressure gauge shall be installed between the positive shut-off device and the sample under test. Throughout the test, the sample shall be tested for leakage, with a surface-active agent without formation of bubbles or measured with a leakage rate less than 216 Nml/hr. | - Boil-off discharge rate 2.2.1 The boil-off discharge rate test shall be performed directly after the boil-off test in paragraph 2.1 under the same conditions. In case a boil-off converter is used it shall be preconditioned until the nominal operating conditions are reached before conducting the measurement. The inner container pressure is kept at boil-off pressure during the entire test. The hydrogen boil-off discharge rate shall be measured by an appropriate method e.g., suitable hydrogen detector and mass flow meter, at the outlet of the boil-off valve, the boil-off vent line or the boil-off converter (if used) as applicable. The test shall at least last for 1 hour after stabilisation. 2.2.2 The boil-off discharge rate shall be recorded during the entire test. The test maximum boil-off discharge rate measured shall not exceed the value specified in paragraph 5.2.2. | Forvia & DT proposal (14.08.2025) Group 25.08.25 2.2.1 The boil-off discharge rate test shall be performed directly after the boil-off test in paragraph 2.1 under the same conditions. In case a boil-off converter is used it shall be preconditioned until the nominal operating conditions are reached before conducting the measurement. The inner container pressure is kept at boil-off pressure during the entire test. The hydrogen boil-off discharge rate shall be measured by an appropriate method e.g., suitable hydrogen detector and mass flow meter, at the outlet of the boil-off valve, the boil-off vent line or the boil-off converter (if used) as applicable. The test shall at least last for 1 hour after stabilisation. 2.2.2 The boil-off discharge rate shall be recorded during the entire test. |
| Pilots / Group | Annex 3 2.3.2 | te | The pressure of the inner container and the vacuum jacket is recorded or written during the entire test. The opening pressure of the first safety device is recorded or written. The first part of test is passed if the following requirements are fulfilled: (a) The first pressure relief device opens ≤ below or at MAWP and limits the pressure to not more than 110 per cent of the MAWP; (b) The first pressure relief device does not open close at pressure above MAWP ; (c) The secondary pressure relief device does not open during the entire test. |  | Shall we define the maximum margin below MAWP? Baptiste Ravinel to check Forvia: I would rather oppose to this change and propose to better delete (b): The initial requirement menas „not open“. However, I would rather see this redundant to (a), so we don‘t need this and deleting would actually rather an editorial change. I would oppose to change to „not close“ as PRVs in fact are reclosing devices that can practically entere a chattering mode (frequent opening and closing). When such chattering occurs the pressure could likely be above MAWP (and frequently changing -> no guidance on how to measure is included...) and this could be misinterpreted as „closing“. DT: B could be removed, it is just stating stronger what is already written in a) Group 25.08.25 The pressure of the inner container and the vacuum jacket is recorded or written during the entire test. The opening pressure of the first safety device is recorded or written. The first part of test is passed if the following requirements are fulfilled: (a) The first pressure relief device opens ≤ MAWP and limits the pressure to not more than 110 per cent of the MAWP; (b) The second pressure relief device does not open during the entire test. |
| TMC | Annex 3 2.3.3 | Te | (e) The test is terminated when the second pressure relief device does not open any more even if the pressure of the container exceeds the value specified in paragraph 2.3.4. |  | TMC: Clarify this is fail or void case. DT: At this stage it is only the description of the test, there is no pass criteria Forvia: Disagree, this would allow the PRV to NOT fulfill it‘s function in this test e.g., being jammed in closed position. =>propose to delete the added text. Question: is there a concern to confuse a leakage somewhere around the setpoint as opening? As a terminating condition the PRVs must stay closed, or potentially one could also thing of requiring the pressure to be constant a certain time, but this constant value needs to be below the PRVs setpoint (please note the PRVs usual behavior is to have a re-closing pressure below ist setpoint (hysteresis), see inserted picture...). Group 25.08.25, delete (e) 2.3.3. After passing the first part, the test shall be repeated subsequently to re-generation of the vacuum and cool-down of the container as described above. (a) The vacuum is re-generated to a value specified by the manufacturer. The vacuum shall be maintained at least 24 hours. The vacuum pump may stay connected until the time directly before the start of the vacuum loss; (b) The second part of the vacuum loss test is conducted with a completely cooled-down container (according to the procedure in Annex 3, paragraph 2.1.); (c) The container is filled to the specified maximum filling level; (d) The line downstream the first pressure relief device is blocked and the vacuum enclosure is flooded with air at an even rate to atmospheric pressure; |
| TMC | Annex 3 2.3.4 | Ge | The pressure of the inner container and the vacuum jacket is recorded during the entire test. For steel containers the second part of the test is passed if the secondary pressure relief device does not open below 110 per cent of the set pressure of the first pressure relief device and limits the pressure in the container to a maximum 136 per cent of the MAWP if a safety valve is used, or, 150 per cent of the MAWP if a burst disc is used as the secondary pressure relief device. [For other container materials, an equivalent level of safety shall be demonstrated.] |  | TMC: Is this requirement specific to material? Forvia: I guess the idea may have been that the initial „level of safety“ was considered to be linked to the yield strength of metallic materials and this again being linked to the minimum burst pressure requirements. I would propose to keep for the time being. However, in case a general limitation to metallic materials is decided, this would in fact be obsolete could be deleted. 26.08.25 – Requires CP input. Discussions for materials are deferred to GTR13 Phase 3. Keep in square brackets but strikethrough. [For other container materials, an equivalent level of safety shall be demonstrated.] |
| TMC | Annex 3 3.3 | Ge | … (b) (b) If a burst disc is used outside the vacuum area as secondary pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP of the inner container; (c) (c) If a burst disc is used inside the vacuum area as secondary pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP + 0.1 MPa of the inner container. … |  | TMC: Need clarification what is inside/outside. Forvia: I guess the point to differentiate here is if the burst disc reliefs into the vacuum space or if it externally reliefs. Would this be clearer? Change proposal: „(b) If a burst disc with a relief external to the vacuum space is used outside the vacuum area as secondary pressure relief device, [...] (c)If a burst disc with a relief internal to the vacuum space is used inside the vacuum area as secondary pressure relief device, [...]” DT: The only difference is accounting for the relative prerssure Group 25.08.25 (b) If (b) If a burst disc with a relief external to the vacuum space is used as second pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP of the inner container; (c) (c) If a burst disc with a relief internal to the vacuum space is used as second pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP + 0.1 MPa of the inner container. |
| TMC | Annex 4 2. |  | External leakage test A component shall be free from leakage through stem or body seals or other joints and shall not show evidence of porosity in casting when tested as described in Annex 4, paragraph 3.3. at any gas pressure between zero and its MAWP. … |  | TMC: Paragraph 3.3 applies to shut-off device. Is this mean PRD and container are not subject to this provision? Forvia: These tests are for PRDs and shut-off devices, see first para in annex 4. However, seeing this there is an issue with the test pressure for PRVs, that would open at 110%MAWP, see comment below. MAWP will not work for PRVs. PRVs are usually considered to be leak tight up to 80% of their set-pressure: Group 25.08.25 A component shall be free from leakage through stem or body seals or other joints and shall not show evidence of porosity in casting when tested as described in Annex 4, paragraph 3 [at any gas pressure between zero and its MAWP.] During this test, the equipment under test shall be connected to a source of gas pressure. The PRV outlet shall be plugged. A positive shut-off device and a pressure gauge having a pressure range of not less than 150 per cent and not more than 200 per cent of the test pressure shall be installed in the pressure supply piping; the accuracy of the gauge shall be 1 per cent of the pressure range. The pressure gauge shall be installed between the positive shut-off device and the sample under test. 25.08.26 – Post-meeting – Changed for clarity: During this test, the equipment under test shall be connected to a source of gas pressure. If a component has a pressure relieving function, that function shall be disabled during the test. If a component has a pressure relieving function, that function shall be disabled during the test. *“The PRV outlet shall be plugged“ should also be added to the pressure cycle test. Delete because Endurance Test is a component test only. |
| Pilots / Group | Annex 4 3.6 | Ge | The component shall be operated through 2 per cent of the total cycles at the minimum material temperature (-40 °C to +85 °C) but not less than the temperature of liquid nitrogen after sufficient conditioning time at this temperature to ensure thermal stability and at the MAWP of the component. The component shall comply with Annex 4, paragraphs 2. and 9. at the appropriate minimum material temperature (-40 °C to +85 °C) at the completion of the low temperature cycles. |  | Is the temperature range correct? Group 25.08.25 – Ambient temp ranges are same as GTR13 |
| France | Annex 6 | Ed | Annex 6 Provisions for a label for liquefied hydrogen vehicles of categories M2/N2 and M3/N3. | Annex 6 Provisions for a label for liquefied hydrogen vehicles of categories M2/N2 and M3/N3. | As the scope of this regulation is limited to M2/M3 and N2/N3 vehicles, there is no need to mention those categories here. |
| Pilots / Group | Annex 6 | Ge | Background: white or white reflecting Border: Light-blue, RGB code 0, 176, 240 or similar Letters: Light-blue, RGB code 0, 176, 240 or similar |  | Tolerances for approval? DT: Align with UN R134 Group 25.08.25 – Agree. Need “or similar” language, will not align with R134 |
| France | Annex 7 | Ge | / | Annex 7 Periodic technical inspection Periodic technical inspection means the inspection of vehicles performed at specified intervals according to national regulations. Guidelines for the visual inspection of each hydrogen system [during its service life] shall be provided by the hydrogen system manufacturer on the basis of use under service conditions specified herein. Each hydrogen system shall be visually inspected at least every 48 months after the date of its entry into service on the vehicle (vehicle registration), and at the time of any reinstallation, for external damage and deterioration. The visual inspection shall be performed in accordance with the manufacturer's specifications: hydrogen system without a label containing mandatory information or with labels containing mandatory information that are illegible in any way shall be removed from service. If the hydrogen system can be positively identified by the manufacturer and serial number, a replacement label may be applied, allowing the hydrogen system to remain in service. | PTI requirements for LHSS Service life is in SQB because there is no information regarding service life of the LHSS in the regulation. |