8.3.5.1.	Future Changes in Energy Mix

“The energy mix change considered for the use stage shall be based on the latest available dynamic scenario, in order of preference:

1. Official, government-published energy mix or emission factors (where these are consistent with the defined scope) based on currently implemented policy for a country or geographical region of interest

2. If the above government-published energy mix or emissions factors are not available, use the Stated Policies Scenario (STEPS) from the most recent IEA WEO report, with the inclusion of the Sustainable Development Scenario (SDS)  as a sensitivity (or equivalent scenario taking into account international commitments under the UNFCCC)

3. For regions without official government-published energy consumption or emission factors, or for which IEA WEO data is unavailable, dispatch modelling or the use of peer-reviewed published data based upon dispatch modeling may be used if it reflects implement policy.  Any such analysis should comprise a sensitivity consistent with the goals of IEA WEO SDS.

The selected scenario should be communicated and justified. The detailed calculations of the fuel or grid mix composition and average representative fuel or grid mix composition over the full service life of the vehicle should be communicated.

A static scenario may be used when a dynamic scenario is not available for the geographical region of interest, in alignment for all energy pathways. The reference year shall be reported.

(Note) Dispatch modelling:  An electric power sector dispatch model is a mathematical representation of how sources of electricity generation are chosen and scheduled (i.e., “dispatched”) to meet a given electricity demand. It essentially models the decision-making process of electricity system operators and is capable of making projections of future generation and future emissions given cost constraints, environmental policy and emissions constraints, and constraints related to resource adequacy. Dispatch models also include representations of individual electricity generation units (EGU) and regional interconnection of EGUs as part of regional, national, and international electricity grids.