Proposal for supplement [8] to the 01 Series of Amendments to UN Regulation No. 121 (Identification of Controls, Tell-Tales and Indicators)

Submitted by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening*

The text reproduced below was prepared by the Task Force on Automated Vehicles Regulatory Screening (TF-AVRS), with the aim to amend and supersede document ECE/TRANS/GRSG/2025/34. The modifications to ECE/TRANS/GRSG/2025/34 are marked in (bold) red for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraphs 0. and 0.1. to 0.1.2., to read:

"0.		Introduction

0.1.		Supplement [8] to the 01 series of amendments is introduced to take into account vehicles of categories X and Y, as well as vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an Automated Driving System Feature of Type 2 (ADSF-2). (ADS) 1

0.1.1. 	The scope is updated to reflect that this Regulation does not apply to vehicles of category Y, since no occupants are inside the vehicle under any circumstances.

0.1.2. 		Paragraph 2.1. is updated since, some controls listed in table 1, may be operated by a passenger, e.g. power window lock (item no. 14), air conditioning, heating system and fan (item nos. 33, 38, 39).

0.1.3.	Paragraphs 5.7.1. and 5.7.2. are added to be able to clearly distinguish between the lack of responsibilities of the occupants in the vehicle, when an ADS feature of type 1 and 2 is active.

0.1.2.	The scope is updated to reflect that this Regulation does not apply to vehicles of category X, which have no manual mode that is intended to be operated on-board the vehicle.

UN R121 does not have any speed-related provisions which means that vehicles with a max. speed of 6 km/h operated by a driver are within the scope of this Regulation.”

Paragraph 1. - Scope, amend to read:

"1.		Scope

This Regulation applies to vehicles of categories M and N 1 except vehicles of category Y 1 as well vehicles of category X without a manual mode that is intended to be operated on-board the vehicle.  It specifies requirements for the location, identification, colour, and illumination of motor vehicle hand controls, tell-tales and indicators. It is designed to ensure the accessibility and visibility of vehicle controls, tell-tales and indicators, and to facilitate their selection under daylight and night-time conditions, in order to reduce the safety hazards caused by the diversion of the driver's attention from the driving task and by mistakes in selecting controls."

Paragraph 1, footnote 1, amend to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.38, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 4.4.1, footnote 2, amend to read:

"2	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 38, Annex 3 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 2.1., amend to read:

"2.1.		"Control" means that hand-operated part of a device that enables the driver or passenger to bring about a change in the state or functioning of a vehicle or vehicle’s subsystem"

Insert new paragraphs 2.10. and 2.11., to read:

"2.10.	"ADS feature of type 1 (ADSF-1)" means an ADS feature which includes an ADS fallback response requiring a fallback user

2.11.	"ADS feature of type 2 (ADSF-2)" means an ADS feature which does not include an ADS fallback response requiring a fallback user."

Paragraph 5.1.4., amend to read:

"5.1.4.		Notwithstanding paragraphs 5.1.1., 5.1.2. and 5.1.3. the tell-tale for "passenger air bag off", if fitted, shall be located within the interior of the vehicle and forward of and above the design H-point of both the driver's seat (if applicable) and the front passenger(s)' seat in their forward most seating positions.  The tell-tale which alerts front seat occupants that the passenger air bag is switched off shall be visible to the driver (if applicable) and front passenger(s) under all driving conditions."

Paragraph 5.2.7., amend to read:

"5.2.7.		The identification of the following need not appear to the driver or passenger perceptually upright:"

Insert new paragraph 5.7. to 5.7.2. and footnote 3, to read:

"5.7.	Special provisions for vehicles equipped with an ADS

5.7.1.	While an ADSF-2 is active, the occupants in the vehicle shall not be informed by tell-tales and indicators that are related to DDT 3 unless it serves a purely informative purpose (e.g. displaying the vehicle’s speed).

5.7.2.	While an ADSF-1 is active, displaying tell-tales and indicators that are related to DDT is optional and subject to the safety concept of ADS. [Such tell-tales and indicators shall (re-)appear at the latest [500 ms] after ADS initiates or receives the request to start the process of transferring DDT back to the driver"

"3	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Annex 1, insert new paragraphs 10.1. and 10.2., to read:

"10.	Comments

10.1.	Vehicle is equipped with ADSF-1 ? Yes/No/Optional

10.2.	Vehicle is equipped with ADSF-2 ? Yes/No/Optional"

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.