Proposal for amendment X to the XX series of amendments to UN Regulation No. 135 (Pole Side Impact (PSI))

The text reproduced below was prepared by the expert from Republic of Korea on behalf of the Ad Hoc Group on Emergency Door Opening, to make an amendment to resolve issues regarding extrication. The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert a new paragraph 5.4.3., to read:

“5.4.3.	After the impact, it shall be possible without the use of tools, except for those necessary to support the weight of the dummy:”

5.4.3.1.	To open at least one door per row of seats. Where there is no such door, it shall be possible to allow the evacuation of all the occupants by activating the displacement system of seats, if necessary. In case no displacement system is available for the evacuation of a rear seated occupant, it shall be shown that a 50th percentile manikin can be evacuated without the use of any devices to support its weight and any other tools. This is not applicable to convertibles where the top can be easily opened to allow the evacuation of the occupants.

For vehicles of category N1 this evacuation may be done via an emergency window if this window can be easily opened, but if tools are necessary, (e.g. for breaking the window) these tools shall then be provided by the manufacturer and shall be visible and located in close proximity to that emergency window.

This shall be assessed for all configurations or worst-case configuration for number of doors on each side of the vehicle and for both left-hand drive and right-hand drive vehicles, when applicable.”

5.4.3.2.	To release the dummy from their restraint system which, if locked, shall be capable of being released by a maximum force of 100 N on the centre of the release control;”

5.4.3.3.	To remove the dummy from the vehicle without adjustment of the seats.”

II.	Justification

- GRSP experts interested in occupant extrication after crashes formed Ad Hoc Group and initiated the discussion for developing the amendments of vehicle regulations in the group.

- This amendment aims to enhance vehicle safety by revising and harmonizing requirements in relevant vehicle regulations related to safety belt unbuckling after the impact, which is one of the issues discussed in EDO Ad Hoc Group.