EDO (Emergency Door Opening) – 3rd meeting

Date: 10 November 2025 09:00 - 13:00 CET

Venue: Online

Participants: AU, CA, CLEPA, CN, DE, ES, JP, KR, NL, OICA, SE, UDV

# Agenda:

- Welcome

- Adoption of the Agenda

Proposed addition: Requirements in China

Agenda adopted with the addition.

- Adoption of the Meeting Minutes of the 2nd Meeting

The meeting minutes were adopted.

- Amendments to Regulations

    - Submergence

Document:

- EDO-03-08e.docx (NED)

NL introduced a proposal to amend UN Regulation No. 21. It adds provisions to ensure operability of windows during vehicle submergence. Tests are proposed either on vehicle or component level. Although a specific test procedure is not yet available, a documentation by the manufacturer was seen insufficient. ES believes that a vehicle test is not suitable for a Technical Service and in consequence a detailed test procedure for the components would be needed.

OICA will need to have a deeper look into the proposal as there are some concerns on the handling of such a component test. A completely submerged door might not represent the real vehicle behaviour.

JP mentioning that R21 scope is only M1, but N1 should probably have the same requirements.

Conclusion:

- Proposal needs further consideration.

- Any feedback will be discussed at a next meeting.

    - Unbuckling

Documents:

    - EDO-03-04e.docx (KOR) Draft_Amendment_of_UN_R94_Regarding_Belt_Unlatching

    - EDO-03-05e.docx (KOR) Draft_Amendment_of_UN_R95_Regarding_Belt_Unlatching

    - EDO-03-06e.docx (KOR) Draft_Amendment_of_UN_R135_Regarding_Belt_Unlatching

    - EDO-03-07e.docx (KOR) Draft_Amendment_of_UN_R137_Regarding_Belt_Unlatching

- EDO-03-09e.pptx (KOR)

EDO-03-04 on UN R94 was introduced by KR. It was explained that there are some misalignments in the crash regulations when it comes to the evacuation procedure. An overview was introduced (EDO-03-09e) to the group. It was highlighted that the allowed max. force to unbuckle an occupant differs and UN Regulation No. 135 doesn’t contain any evacuation provisions. KR hence proposing a harmonization. DE wondered if there might be valid reasons why the requirements in the regulations differ from each another. The other proposals (EDO-03-05 to EDO-03-07) were introduced.

During the following discussion these points have been identified for further consideration:

- R135 missing evacuation provisions might be based on GTR 14. The preamble states that these provision should be discussed during a phase 2 of the GTR;

- Different max. force limits might be related to different post crash conditions (take into account the specificities of a frontal and a side crash);

- The evacuation procedure of a 50th percentile dummy in the second seat row is unclear, when there is no such dummy present during the mandatory test;

- Crash regulations are mandated in a package and hence it might not be necessary to mandate testing the evacuation in every Regulation;

- Proposed amendments to UN R94 seem to ease current requirements;

- Evaluate the necessity to add basic requirement on door opening and evacuation to UN R153 (rear impact).

Conclusion:

- Further review/discussion is needed.

    - Door opening requirements after crash tests

  - JASIC comments

  - Clarification on existing requirements (voltage drop)

Document:

    - EDO-03-03e.pdf (JASIC) Comments on EDO-02-04 KR Proposals

Document was introduced by JASIC. Concerns have been already addressed by the presentation of KR (see 4.2).

ES would like to further review the provisions in the crash regulations with respect to a voltage drop. In most of the times there are no voltage drop perceived when testing according to a UN Regulation. The existing requirement doesn’t seem to consider the issue discussed within this group.

Conclusion:

- Post crash requirements on door opening need further consideration.

    - Proposed amendments to UN Regulation No. 11

Document:

  - EDO-03-02e.pdf (JASIC) Comments on EDO-02-02 Proposed Amendments to UN R11

Document was introduced by JP.

Para. 5.3.3.: DE confirmed that the proposal shall be technology neutral. Requirements on “loss of electrical power” (1st sentence) shall be applicable at any time.

JP explaining that from their point of view, “manually intuitively’” would also allow pushing a handle, which then pops out for manual operation.

Para 6.3.4.1.: Already discussed that this para needs improvements.

Conclusion:

- Further improvements needed.

    - Identification of „Mechanical Emergency Release Handles“ (vehicle interior)

Document:

- EDO-03-10e.pdf (UDV)

UDV introduced a presentation on identification of backup solutions to open the door from the interior, see EDO-03-10e.

CA stressing that occupants are shocked after a collision and may act differently than normal. Even if there is an alternative for opening the concern is that because of the shock occupants keep on trying the “normal” way to open the door.

NL is fully in line with the proposal and CA’s statement. Accessibility should be considered (possible blockage). Main question seems to be, if a “emergency release device” would be allowed. From their point of view, electrical openings should be operable even after a loss of main power.

CN pointing at the standard they are drafting, which already contains many aspects of the presentation.

OICA explained that the main goal is to ensure that every occupant is able to leave the vehicle and some ideas shown might not be achievable.

CLEPA mentioned again the child lock systems which intentionally prevents the opening of the door from the interior.

SE supporting the positions of NL and CA. Child lock needs to be considered during future discussions.

Conclusion:

- Further discussion needed.

- Requirements in China

KR asking about the current status of the Chinese standard. Differences to the approach of this group should be identified.

CN explaining that it is currently not published. For the next meeting / GRSP they can prepare a presentation about the upcoming standard.

- Possible Feedback to GRSP

  - Identified topics for improvement

  - Proposed way forward on

    - establishing a TF / IWG;

    - addressing ‘58 or ‘98 agreement.

  - Drafting Terms of Reference

Document:

- EDO-03-11e.docx (GER)

DE provided a draft about the priorities as discussed. The document was amended during the meeting. The group agreed to the priorities.

Way forward discussion:

- Propose to GRSP to start a TF with ToR. If WP.29 sees the need to have an IWG then it can be easily transformed.

- ToR will be drafted and circulated in the group (commenting phase). Then it will be sent to GRSP.

- CATARC/CN shall be involved in the discussion for harmonization reasons.

During the discussion of including GTRs it was suggested that CN should introduce their GB standard at GRSP. The group worked on technology neutral requirements, but it seems the upcoming GB standard defines specific designs. Question was raised how to ensure (future) alignment.

A presentation to GRSP will be prepared, but no containing any specific amendments to Regulations (maybe only excerpts).

- Next Steps

  - Draft Tor and presentation will be shared within the group and then sent to GRSP

  - Discussion at GRSP to start a TF / IWG

  - In-person meeting after GRSP in Germany (3. – 6. Feb. 2026 Berlin t.b.c)