6.2.	Test procedure

Table A specifies the various test requirements for type approval of a vehicle.

| Table A |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application of test requirements for type-approval and extensions |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle category | Vehicles with positive ignition engines including hybrids |  |  |  |  |  |  |  | Vehicles with compression ignition engines including hybrids |  | Pure electric vehicles | Hydrogen fuel cell vehicles |
|  | Mono fuel |  |  |  | Bi-fuel3 |  |  | Flex-fuel3 | Mono fuel |  |  |  |
| Reference fuel | Petrol | LPG | NG/ Biomethane | Hydrogen (ICE) | Petrol | Petrol | Petrol | Petrol | Diesel | Petrol | — | Hydrogen (Fuel Cell) |
|  |  |  |  |  | LPG | NG/ Biomethane | Hydrogen (ICE) 4 | Ethanol (E85) |  |  |  |  |
| Type 1 test (for applicability of measured components to fuels and vehicle technology and therefore measurement procedures, see Table 1A and Table 1B) (limits) | Yes | Yes5 | Yes5 | Yes4 | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes | Yes | — | — |
| ATCT 1 (14°C test) | Yes | Yes | Yes | Yes4 | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes | Yes | — | — |
| Evaporative emissions (Type 4 test) | Yes | Yes 6 | Yes 6 | — | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | — | Yes | — | — |
| Durability1 (Type 5 test) | Yes | Yes | Yes | Yes | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes | Yes | — | — |
| OBD1 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | — | — |
| OBFCM | Yes | Yes7 | Yes7 | — | — | — | — | Yes (both fuels) | Yes | Yes | Yes9 | Yes7 |
| CO2 emissions, fuel consumption, electric energy consumption and electric range | Yes | Yes | Yes | Yes2 | Yes (both fuels) | Yes (both fuels) | Yes (petrol), Yes2 (hydrogen) | Yes (both fuels) | Yes | Yes | Yes | Yes8 |
| Low temperature electric range test | — | — | — | — | — | — | — | — | — | — | Yes | — |

1 	Declaration of compliance by the vehicle manufacturer at type-approval.

2 	Only fuel consumption shall be determined when the vehicle is running on hydrogen.

3 	When a bi-fuel vehicle is combined with a flex fuel vehicle, both test requirements are applicable.

4 	Only NOx emissions shall be determined when the vehicle is running on hydrogen.

5 	For Level 1A and Level 2 only - Particulate mass and particle number limits and respective measurement procedures shall not apply. For Level 1B only - In the case that a mono-fuel gas vehicle has a petrol tank it shall also be tested using the applicable petrol reference fuel

6 	For Level 1B, if the mono-fuel gas vehicle does have a petrol tank “Yes”, if the mono-fuel gas vehicle does not have a petrol tank “—“, For Level 1A and Level 2 “—“

7	[For Level 1B and Level 2 only]

8	CO2 emissions do not need to be measured

9	See transitional provisions in Paragraph 12 for Level 1A and Level 2 only.

5.11.1.

The manufacturer shall ensure that the following vehicles are equipped with a device for determining, storing and making available data on the quantity of fuel and/or electric energy used for the operation of the vehicle:

(a)	pure ICE and Not-Off-Vehicle Charging Hybrid Electric vehicles (NOVC-HEVs) powered exclusively by mineral diesel, biodiesel, petrol, ethanol or any combination of these fuels;

(b)	Off-Vehicle Charging Hybrid Electric Vehicles (OVC-HEVs) powered by electricity and any of the fuels mentioned in point (a);

(c)	Pure Electric Vehicles (PEVs).

For Level 1B and Level 2 only:

(d)	Not Off-Vehicle Charging Fuel Cell Hybrid Vehicles (	NOVC-FCHV), Off-Vehicle Charging Fuel Cell Hybrid Vehicles (OVC-FCHV) and Mono-fuel gas (except Hydrogen) vehicles.

[(c)	NOVC-FCHV, OVC-FCHV (tbc), PEV, CNG, LNG, LPG.]

5.11.2.	The device for monitoring the consumption of fuel and/or electric energy shall comply with the requirements laid down in Appendix 5.

8.		Conformity of production (COP)

8.1.	Every vehicle produced under a type approval according to this Regulation shall conform with regard to the vehicle type approved. The conformity of production procedures shall comply with those set out in the 1958 Agreement, Schedule 1 (E/ECE/TRANS/505/Rev.3), with the following requirements:

8.1.1.	The manufacturer shall implement adequate arrangements and documented control plans and carry-out, at intervals specified in this Regulation, the necessary tests to verify continued conformity with the approved type. The manufacturer shall obtain agreement for these arrangements and control plans from the responsible authority. The responsible authority shall perform audits at specific intervals. This audit shall include production and test facilities as part of the product conformity and continued verification arrangements. Where necessary the responsible authority may require additional tests to be conducted.

8.1.2.	The manufacturer shall check the conformity of production by conducting the appropriate tests in accordance with Table 8/1 and Table 8/2 and with the OBD requirements, where applicable according to Table A in paragraph 6.

The specific procedures for conformity of production are set out in paragraphs 8.2. to 8.4. and Appendices 1 to 4.

This paragraph is applicable to Level 1B and Level 2 only;

In addition, the manufacturer shall check the accessibility of the parameters listed in paragraph 3. of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 in accordance with paragraph 5.1. of Appendix 5.

Table 8/1

Type 1 Applicable Type-1 CoP requirements for the different types of vehicle

| Type of vehicle | Criteria emissions | CO2 emissions | Fuel Efficiency | Electric energy consumption | [OBFCM accuracy |
| --- | --- | --- | --- | --- | --- |
| Pure ICE | yes | Level 1A and Level 2 as applicable | Level 1B and Level 2 as applicable | Not Applicable | yes |
| NOVC-HEV | yes | Level 1A and Level 2 as applicable | Level 1B and Level 2 as applicable | Not Applicable | yes |
| OVC-HEV | yes: CD(1) and CS | Level 1A and Level 2 as applicable: CS only | Level 1B and Level 2 as applicable: CS only | yes: CD only | yes: CS only(2) |
| PEV | Not Applicable | Not Applicable | Not Applicable | Yes | Level 1A and Level 2 |
| NOVC-FCHV | Not Applicable | Not Applicable | Exempted | Not Applicable | Not Applicable |
| OVC-FCHV | Not Applicable | Not Applicable | Exempted | Exempted | Not Applicable] |

(1)  Only if there is combustion engine operation during a valid CD Type 1 test for CoP verification 
(2)   For Level 1A and Level 2 only: in addition to assessing the fuel consumption during CS, the accuracy of the vehicle energy charged is assessed in accordance with paragraph 3.4. of Appendix 2

12.		Transitional and special provisions

12.1.

This paragraph is applicable for Level 1A and Level 2 only

The obligation to preserve the values of the lifetime counters defined in paragraph 3.2 (p) and (q) and paragraph 3.3 of Appendix 5 to this Regulation shall apply as of 1 January 2030.

Notwithstanding the first subparagraph, at the request of the manufacturer and with the approval of the approval authority, a type-approval may be granted prior to 1 January 2030 for vehicles which comply with the obligation to preserve the values of the lifetime counters defined in paragraph 3.2. (p) and (q) and paragraph 3.3. of Appendix 5 to this Regulation and the provisions of paragraph 3 of Appendix 5.

As of 1 January 2030, Contracting Parties applying this Regulation shall not be obliged to accept type approvals for vehicles that do not comply with the provisions of paragraph 3 of Appendix 5 to this Regulation.

Appendix 2

Verification of conformity of production for Type 1 test—statistical method

- Xtests≤A-tP1,i+tP2,i∙s

B+tP1,i+tP2,i∙s≤ Xtests

- Xtests>A+tF1,i-tF2∙s

B-tF1,i-tF2∙s >Xtests

A-tP1,i+tP2,i∙s<Xtests≤A+tF1,i-tF2∙s

B-tF1,i-tF2∙s≤Xtests<B+tP1,i+tP2,i∙s

3.4.	Accuracy of the OBFCM device

[The conformity of production of OBFCM devices as defined in paragraph 4.2. of Appendix 5 shall be evaluated as follows:]

(1)	Applicable vehicles

| Vehicles referred in paragraph 5.11.1. to this Regulation | Level 1A | Level 1B | Level 2 |
| --- | --- | --- | --- |
| (a) | yes | yes | yes |
| (b) | yes | NA | yes |
| (c) | yes | NA | yes |
| (d) | NA | LPG only | LPG only |

(2)	For each single test i performed for the purposes of paragraph 3. of this appendix the value xi shall be set equal to:

1 / (1 - Accuracy)

where the Accuracy of the OBFCM device shall be determined in accordance with paragraph 4.2. of Appendix 5.

For Level 1A and Level 2 only

For PEVs and OVC-HEVs the accuracy regarding the vehicle energy charged shall be calculated using Vehicle_energy_chargedREESS_charging, measured at the mains during the application of a normal charge of the preconditioning as defined in paragraph 2.2.3 and 3.1.2 of Appendix 4 to Annex B8, and Vehicle_energy_chargedOBFCM, determined for the same normal charge using the differentials of the parameter "Total grid energy into the vehicle (lifetime) (kWh)".

(3)	This paragraph is applicable to Level 1A and 4-phase WLTP test in Level 2 only;

The conformity of production of the OBFCM devices shall be evaluated according to the requirements of paragraph 3.3.1., but applying a factor A value of 1.0526.

If for the last test N performed for the purposes of paragraph 3. the decision (iii) of paragraph 3.3.1. with regard to the conformity of production of the OBFCM devices is reached, the sequence of tests shall be continued until a final decision (i) or (ii) of paragraph 3.3.1. is reached.

The Type Approval authority shall keep a record of the determined accuracies of OBFCM device at each test as well as the decision according to paragraph 3.3.1. after each test.

[(4)  	This paragraph is applicable to Level 1B and 3-phase WLTP test in Level 2 only;

The conformity of production of the OBFCM devices shall be evaluated according to the requirements of paragraph 3.3.1. added a partial expression to the original formula (i),(ii),(iii), but applying a factor A value of 1.0526.and B value of 0.9524.

- Pass the family if Xtests≤A-tP1,i+tP2,i∙s

and       B+tP1,i+tP2,i∙s≤ Xtests

- Fail the family if Xtests>A+tF1,i-tF2∙s

or       B-tF1,i-tF2∙s >Xtests

- Take another measurement if:

A-tP1,i+tP2,i∙s<Xtests≤A+tF1,i-tF2∙s

or    B-tF1,i-tF2∙s≤Xtests<B+tP1,i+tP2,i∙s

If for the last test N performed for the purposes of paragraph 3.3.2.1. the decision (iii) of paragraph 3.4.(4) with regard to the conformity of production of the OBFCM devices is reached, the sequence of tests shall be continued until a final decision (i) or (ii) of paragraph 3.4.(4). is reached.]

The Type Approval authority shall keep a record of the determined accuracies of OBFCM device at each test as well as the decision according to paragraph 3.3.1. after each test.]

Appendix 5

Devices for monitoring on board the vehicle the consumption of fuel and/or electric energy

1.	Introduction

This appendix sets out the definitions and requirements applicable to the devices for monitoring on board the vehicle the consumption of fuel and/or electric energy.

2.	Definitions

In addition to the definitions in Paragraph 3 of this Regulation, the following definitions apply.

2.1.	"On-board Fuel and/or Energy Consumption Monitoring Device" ("OBFCM device") means any element of design, either software and/or hardware, which senses and uses vehicle, engine, fuel and/or electric energy parameters to determine and make available at least the information laid down in paragraph 3 of this appendix, and store the lifetime values on board the vehicle.

2.2.	"Lifetime" value of a certain quantity determined and stored at a time t shall be the values of this quantity accumulated since the completion of production of the vehicle until time t.

2.3.	"Engine fuel rate" means the amount of fuel injected into the engine per unit of time. It does not include fuel injected directly into the pollution control device.

2.4.	"Vehicle fuel rate" means the amount of fuel injected into the engine and directly into the pollution control device per unit of time. It does not include the fuel used by a fuel operated heater.

2.5.	"Total Fuel Consumed (lifetime)" means the accumulation of the calculated amount of fuel injected into the engine and the calculated amount of fuel injected directly into the pollution control device. It does not include the fuel used by a fuel operated heater.

2.6.	"Total Distance Travelled (lifetime)" means the accumulation of the distance travelled using the same data source that the vehicle odometer uses.

2.7	"Total grid energy into the battery (lifetime)" means the accumulation of the calculated amount of electric energy flowing into the battery when the vehicle is connected to an external power supply and the engine is turned off. It shall not include electrical losses between the external power source and the battery.

2.8.	"Charge-sustaining operation" means, for OVC-HEVs, the state of vehicle operation when the REESS state of charge (SOC) may fluctuate but the intent of the vehicle control system is to maintain, on average, the current state of charge.

2.9.	"Charge-depleting operation" means, for OVC-HEVs, the state of vehicle operation when the current REESS SOC is higher than the charge-sustaining target SOC value and, while it may fluctuate, the intent of the vehicle control system is to deplete the SOC from a higher level down to the charge-sustaining target SOC value.

2.10.	"Driver-selectable charge-increasing operation" means, for OVC-HEVs, the operating condition in which the driver has selected a mode of operation, with the intention to increase the REESS SOC.

2.11.		"Total grid energy into the vehicle (lifetime)" means the accumulation of the electric energy flowing into the vehicle from an external power supply connected via any charging interface the vehicle is equipped with.

2.12.	“Total grid energy into the vehicle from off-board AC charging (lifetime)” means the accumulation of the electric energy flowing into the vehicle from an AC external power supply connected via any charging interface the vehicle is equipped with.

2.13.	"Full charging event" means a full charge of the battery after break-off criterion is reached until the end-of-charge criterion is reached, as set out in the Type 1 test procedure.

2.14.	"Total gaseous fuel consumed (lifetime)" means the accumulation of the calculated amount of fuel injected into the engine and the calculated amount of fuel injected directly into the pollution control device, in kilograms. It does not include the fuel used by a fuel operated heater.

2.15.	"Total Fuel Cell Fuel Consumed (lifetime)" means the accumulation of the calculated amount of fuel injected into the fuel cell in kilograms."

2.16.	"Energy consumption rate" means the amount of energy consumed for vehicle propulsion per unit of time.

[2.17.	"Vehicle Identification Number" means Vehicle identification number (VIN) prescribed in ISO 3779, chassis number or those equivalent to these]

2.18.	"Lifetime value retention status" means the status "0" in which Lifetime values are preserved as specified in paragraph 5.2. and the status "1" in which Lifetime values are no longer preserved as specified in paragraph 5.5.

3.	Information to be determined, stored and made available

The OBFCM device shall determine at least the following parameters and store the lifetime values on board the vehicle. The parameters shall be calculated and scaled according to the standards referred to in paragraph 6.5.3.2. (a) or (e) or (f) or (g) of Appendix 1 to Annex C5 and shall be made available as signals through the serial port connector referred to in paragraph 6.5.3.2. (c) of Appendix 1 to Annex C5.

3.1.	For pure ICE and NOVC-HEVs powered exclusively by mineral diesel, biodiesel, petrol, ethanol or any combination of those fuels, and for Level 1B and Level 2 only LPG:

(a)	Total fuel consumed (lifetime) (litres);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Engine fuel rate (grams/second);

(d)	Engine fuel rate (litres/hour);

(e)	Vehicle fuel rate (grams/second);

(f)	Vehicle speed (kilometres/hour);

[(g)	Lifetime value retention status;]

For Level 1B and Level 2 only:

(h)	Vehicle Identification Number.

3.2.	For OVC-HEVs:

(a)	Total fuel consumed (lifetime) (litres);

(b)	Total fuel consumed in charge-depleting operation (lifetime) (litres);

(c)	Total fuel consumed in driver-selectable charge-increasing operation (lifetime) (litres);

(d)	Total distance travelled (lifetime) (kilometres);

(e)	Total distance travelled in charge-depleting operation with engine off (lifetime) (kilometres);

(f)	Total distance travelled in charge-depleting operation with engine running (lifetime) (kilometres);

(g)	Total distance travelled in driver-selectable charge-increasing operation (lifetime) (kilometres);

(h)	Engine fuel rate (grams/second);

(i)	Engine fuel rate (litres/hour);

(j)	Vehicle fuel rate (grams/second);

(k)	Vehicle speed (kilometres/hour);

(l)	Total grid energy into the battery (lifetime) (kWh);

(m)	Total discharge energy in V2X (lifetime) (kWh), [if applicable];

(n)	Total discharge energy for non-traction purposes (lifetime) (kWh), if applicable;

[(o)	Lifetime value retention status;]

(s)	Battery SOCE (%);

(t)	Battery SOCR (%);For Level 1A and Level 2 only:

(p)	Total grid energy into the vehicle (lifetime) (kWh);

(q)	Total grid energy into the vehicle from off-board AC charging (lifetime) (kWh);

[

For Level 1B and Level 2 only:

(r)	Energy consumption rate per second (Wh/second);

(t)	Date of manufacture of the vehicle [applicable to Level 1B and Level 2 only];

[(u)	Elapsed time since last charged by more than 50 per cent SOC swing [days];]

[(v)	Average battery temperature while propulsion system is active, during charging and (if equipped) during non-usage of the vehicles (i.e. non-propulsion system active, non-charging);]

(s)	Vehicle Identification Number.]

3.3.	For PEVs:

(a)	Total distance travelled (lifetime) (kilometres);

(b)	Total grid energy into the battery (lifetime) (kWh);

(c)	Vehicle speed (kilometres/hour);

(d)	REESS Current (A);

(e)	REESS Voltage (V);

(f)	REESS state of charge (%).

[(g)	Energy throughput (lifetime) (kWh);]

(g)	Total discharge energy in V2X (lifetime) (kWh) [if applicabe];

(h)	Total discharge energy for non-traction purposes (lifetime) (kWh), if applicable;

[(i)	Lifetime value retention status;]

[(r)	Battery SOCE (%);

(s)	Battery SOCR (%);]For Level 1A and Level 2 only:

(j)	Total grid energy into the vehicle (lifetime) (kWh);

(k)	Total grid energy into the vehicle from off-board AC charging (lifetime) (kWh);

[

For Level 1B and Level 2 only:

(l)	Energy consumption rate per second (Wh/s);

(m)	Vehicle Identification Number.]

3.4.	This paragraph is applicable to Level 1B and Level 2 only

For NOVC-FCHVs:

(a)	Total fuel consumed (lifetime) (kilograms);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Vehicle fuel rate (grams/second);

(d)	Vehicle speed (kilometres/hour);

(e)	Vehicle Identification Number;

[(f)	Lifetime value retention status.]

3.5.	This paragraph is applicable to Level 1B and Level 2 only

For OVC-FCHVs:

(a)	Total fuel consumed (lifetime) (kilograms);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Total grid energy into the battery (lifetime) (kWh);

(d)	Vehicle fuel rate (grams/second);

(e)	Vehicle speed (kilometres/hour);

(f)	Energy consumption rate per second (Wh/ second);

(g)	Energy throughput (lifetime) (kWh);

(h)	Total discharge energy in V2X (lifetime) (kWh);

(i)	Vehicle Identification Number;

[(j)	Lifetime value retention status.]

3.6.	This paragraph is applicable to Level 1B and Level 2 only

For mono-fuel gas (except LPG and Hydrogen) vehicles:

(a)	Total gaseous fuel consumed (lifetime) (kilograms);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Engine fuel rate (grams/second);

(e)	Vehicle fuel rate (grams/second);

(f)	Vehicle speed (kilometres/hour);

(g)	Vehicle Identification Number;

[(h)	Lifetime value retention status.]

4.	Accuracy

4.1.	With regard to the information specified in paragraph 3., the manufacturer shall ensure that the OBFCM device provides the most accurate values that can be achieved by the measurement and calculation system of the engine control unit.

4.2.	Notwithstanding paragraph 4.1., the manufacturer shall ensure that the accuracy is higher than - 0.05 and lower than 0.05 calculated with three decimals using the following formula:

For pure ICEs and NOVC-HEVs powered exclusively by mineral diesel, biodiesel, petrol, ethanol or any combination of those fuels, for OVC-HEVs,  and for LPG:

(LPG is applicable to  Level 1B and Level 2 only)

:

Where:

Fuel_ConsumedWLTP (litres)		is the fuel consumption determined at the first test carried out in accordance with paragraph 1.2. of Annex B6, calculated in accordance with paragraph 6. of Annex B7, using emission results over the total cycle before applying corrections (output of step 2 in Table A7/1 of Annex B7), multiplied by the actual distance driven and divided by 100. For OVC-HEVs the charge-sustaining Type 1 test shall be used.

Fuel_ConsumedOBFCM (litres)	is the fuel consumption determined for the same test using the differentials of the parameter ‘Total fuel consumed (lifetime)’ as provided by the OBFCM device.

For OVC-HEVs the charge-sustaining Type 1 test shall be used.

For PEVs, and OVC-HEVs:

(PEV and OVC-HEV are applicable to Level 1A and Level 2 only)

Accuracy=Vehicle_energy_chargedREESS charging-Vehicle_energy_chargedOBFCMVehicle_energy_chargedREESS charging

Where:

Vehicle_energy_chargedREESS_charging (kWh) 	is total energy obtained from measuring the full charging event in type-1 test according to paragraph 3.4.4.3. of Annex B8.

Vehicle_energy_chargedOBFCM (kWh) 	is the total energy as the differential in the "Total electric energy into the vehicle (lifetime) (kWh)" for the same full charging event.

Accuracy=Fuel_ConsumedWLTP-Fuel_ConsumedOBFCMFuel_ConsumedWLTP

4.2.1.	If the accuracy requirements set out in paragraph 4.2. are not met, the accuracy shall be recalculated for subsequent Type 1 tests performed in accordance with paragraph 1.2. of Annex B6, in accordance with the formulae in paragraph 4.2., using

For Level 1B: the fuel consumed, determined and accumulated over all performed tests

For Level 1A and Level 2: the fuel consumed and/or the vehicle energy charged (as applicable) determined and accumulated over all performed tests

The accuracy requirement shall be deemed to be fulfilled once the accuracy is higher than - 0.05 and lower than 0.05.

4.2.2.	If the accuracy requirements set out in paragraph 4.2.1. are not met following the subsequent tests pursuant to this point, additional tests may be performed for the purpose of determining the accuracy, however, the total number of tests shall not exceed three tests for a vehicle tested without using the interpolation method (vehicle H), and six tests for a vehicle tested using the interpolation method (three tests for vehicle H and three tests for vehicle L). The accuracy shall be recalculated for the additional subsequent Type 1 tests in accordance with the formulae in paragraph 4.2., using:

For Level 1B: the fuel consumed, determined and accumulated over all performed tests.

For Level 1A and Level 2: the fuel consumed and/or the vehicle energy charged (as applicable), determined and accumulated over all performed tests.

The requirement shall be deemed to be fulfilled once the accuracy is higher than - 0.05 and lower than 0.05. Where the tests have been performed only for the purpose of determining the accuracy of the OBFCM device, the results of the additional tests shall not be taken into account for any other purposes.

5.	Access to the information provided by the OBFCM device

5.1.	The OBFCM device shall provide for standardised and unrestricted access of the information specified in paragraph 3. and shall conform to the standards referred to in paragraphs 6.5.3.1. (a) and 6.5.3.2. (a) or (e) or (f) or (g) of Appendix 1 to Annex C5.

5.2.	By way of exemption from the reset conditions specified in the standards referred to in paragraph 5.1. and notwithstanding the requirements of  paragraph 5.4., once the vehicle has entered into service the values of the lifetime counters shall be preserved.

5.3.	The values of the lifetime counters may be reset only for those vehicles for which the memory type of the engine control unit is unable to preserve data when not powered by electricity. For those vehicles the values may be reset simultaneously only in the case the battery is disconnected from the vehicle.

5.4.	In the case of malfunctioning affecting the values of the lifetime counters, or replacement of the engine control unit, the counters may be reset simultaneously to ensure that the values remain fully synchronised.

5.5.

In cases where the lifetime values are no longer preserved notwithstanding the provisions of paragraph 5.2., the fact that they are no longer preserved shall be recorded in the  relevant control unit and the record concerned shall not be deleted easily.]

5.6.	This paragraph is applicable to Level 1B and Level 2 only:

Manufacturers shall provide functions to deter modifications to on-board fuel and power consumption measurement devices other than modifications approved by the manufacturer. Manufacturers shall permit modifications when such modifications are necessary for vehicle diagnosis, maintenance, inspection, retrofitting or repair. Removable calibration memory chips shall be embedded, contained in a sealed container or protected by an electronic algorithm and shall not be altered without specialized tools or procedures. Vehicle manufacturers that use programmable computer code systems, such as electrically erasable and programmable read-only memory, EEPROMs, etc., shall deter unauthorized reprogramming. Automobile manufacturers shall employ robust tamper-resistant measures and write-protection features that require electronic access to an off-site computer maintained by the automobile manufacturer.

2.1.1.5. Electric energy consumption (if applicable)

2.1.1.5.1. Electric energy consumption of OVC-HEVs and OVC-FCHVs (as applicable)

2.1.1.5.1.1. Recharged electric energy (EAC)

|  | 4 phases cycle | 3 phases cycle |
| --- | --- | --- |
| EAC (Wh) |  |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2 (p) or paragraph 3.3 (j) of Appendix 5 to this Regulation  is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (10) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (10) | x.xxx(11) |
| Accuracy vehicle energy charged (10) | x.xxx(11) |

(10)	in accordance with Appendix 5 to this Regulation

(11)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

2.1.1.5.1.2.	Electric energy consumption (EC)

Test 1a – Results after 4 phase cycle

| EC (Wh/km) | Low | Medium | High | Extra High | City | Combined |
| --- | --- | --- | --- | --- | --- | --- |
| Final values EC |  |  |  |  |  |  |

Test 1b – Results after 3 phase cycle

| EC (Wh/km) | Low | Medium | High | Combined |
| --- | --- | --- | --- | --- |
| Final values EC |  |  |  |  |

2.1.1.5.1.3.	UF-weighted charge-depleting electric energy consumption

Test 1

| ECAC,CD (Wh/km) | Combined |
| --- | --- |
| Calculated value ECAC,CD |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion (if applicable)

| ECAC,CD (Wh/km) | Combined |
| --- | --- |
| Averaging ECAC,CD |  |
| Final value |  |

2.1.1.5.1.4.	UF-weighted electric energy consumption

Test 1

| ECAC,weighted (Wh) | Combined |
| --- | --- |
| Calculated value ECAC,weighted |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion (if applicable)

| ECAC,weighted (Wh/km) | Combined |
| --- | --- |
| Averaging ECAC,weighted |  |
| Final value |  |

2.1.1.5.1.5. Information for COP

|  | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- |
| Electric energy consumption (Wh/km) ECDC,CD,COP or ECAC,CD,COP (as applicable) |  |  |
| AFEC,AC,CD (as applicable) |  |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2 (p) or paragraph 3.3 (j) of Appendix 5 to this Regulation  is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (12) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (13) | x.xxx(14) |
| Accuracy vehicle energy charged (13) | x.xxx(14) |

(12)	in accordance with paragraph 3.4. of Appendix 2 to this Regulation

(13)	in accordance with Appendix 5 to this Regulation

(14)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

2.1.1.5.2. Electric energy consumption of PEVs (if applicable)

Test 1

| EAC(Wh) |  |
| --- | --- |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2 (p) or paragraph 3.3 (j) of Appendix 5 to this Regulation  is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (15) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (15) | x.xxx(16) |
| Accuracy vehicle energy charged (15) | x.xxx(16) |

(15)	in accordance with Appendix 5 to this Regulation

(16)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

Test 1a – Results after 4 phase cycle

| EC (Wh/km) | Low | Medium | High | Extra High | City | Combined |
| --- | --- | --- | --- | --- | --- | --- |
| Calculated values EC |  |  |  |  |  |  |
| Declared value |  |  |  |  | - |  |

Test 1b – Results after 3 phase cycle

| EC (Wh/km) | Low | Medium | High | Combined |
| --- | --- | --- | --- | --- |
| Calculated values EC |  |  |  |  |
| Declared value |  |  |  |  |

Tests 2a and 2b  (if applicable)

Record test results in accordance with the table of Test 1

Tests 3a and 3b –

Record test results in accordance with the table of Test 1

Conclusion after 4 phase cycle

| EC (Wh/km) | Low 4 phase cycle | Medium 4 phase cycle | High 4 phase cycle | Extra High 4 phase cycle | City 4 phase cycle | Combined 4 phase cycle |
| --- | --- | --- | --- | --- | --- | --- |
| Averaging EC |  |  |  |  |  |  |
| Final values EC |  |  |  |  |  |  |

Conclusion after 3 phase cycle

| EC (Wh/km) | Low 3 phase cycle | Medium 3 phase cycle | High 3 phase cycle | Combined 3 phase cycle |
| --- | --- | --- | --- | --- |
| Averaging EC |  |  |  |  |
| Final values EC |  |  |  |  |

Information for COP - Results after 4 Phase cycle (For Level 1A - if applicable)

|  | Combined 4 phase cycle |
| --- | --- |
| Electric Energy Consumption (Wh/km) ECDC,COP |  |
| AFEC |  |

Information for COP - Results after 3 Phase cycle (For Level 1B - if applicable)

|  | Combined 3 phase cycle |
| --- | --- |
| Electric Energy Consumption (Wh/km) ECDC,COP |  |
| AFEC |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2 (p) or paragraph 3.3 (j) of Appendix 5 to this Regulation is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (17) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (18) | x.xxx(19) |
| Accuracy vehicle energy charged (18) | x.xxx(19) |

(17)	in accordance with paragraph 3.4. of Appendix 2 to this Regulation

(18)	in accordance with Appendix 5 to this Regulation

(19)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

Annex B6

Type 1 test procedures and test conditions

2.6.8.5.	OBFCM data recording and storing

2.6.8.5.1.	This paragraph is applicable to Level 1A and 4-phase WLTP test in Level 2 only

During the Type 1 test, the following parameters referred in Appendix 5 of this Regulation shall be recorded and saved (1 Hz sampling frequency) by the testing lab and shall be made available by the approval authority if requested by a regional authority:

(a)	Engine fuel rate (grams/second);

(b)	Engine fuel rate (litres/hour);

(c)	Vehicle fuel rate (grams/second).

[2.6.8.6.	OBFCM data accessibility

This paragraph is applicable to Level 1B and Level 2 only

Technical Service shall check the accessibility of the parameters listed in paragraph 3 of Appendix 5 to this Regulation in accordance with paragraph 5.1. of Appendix 5.]

[Annex B8 (add the following paragraph):

3.4.4.4.	  OBFCM data recording and storing

This paragraph is applicable to Level 1A and 4-phase WLTP test in Level 2 only

During the Type 1 test, the following parameters referred in Appendix 5 of 
 this Regulation shall be recorded and saved (1 Hz sampling frequency) by 
 the testing lab and shall be made available by the approval authority if 
 requested by a regional authority:

(a)	 REESS Voltage (V);

(b)	REESS Current (A);"

3.4.4.5.	  OBFCM data accessibility

This paragraph is applicable to Level 1B and Level 2 only

Technical Service shall check the accessibility of the parameters listed in paragraph 3 of Appendix 5 to this Regulation in accordance with paragraph 5.1. of Appendix 5.]

Annex C5 - Appendix 1

6.5.3.2.	Standards used for the transmission of OBD relevant information:

(a) 	ISO 15031-5 "Road vehicles - communication between vehicles and external test equipment for emissions-related diagnostics – Part 5: Emissions-related diagnostic services", dated 1 April 2011 or SAE J1979 dated 23 February 2012;

(b) 	ISO 15031-4 "Road vehicles – Communication between vehicle and external test equipment for emissions related diagnostics – Part 4: External test equipment", dated 1 June 2005 or SAE J1978 dated 30 April 2002;

(c) 	ISO 15031-3 "Road vehicles – Communication between vehicle and external test equipment for emissions related diagnostics Part 3: Diagnostic connector and related electrical circuits: specification and use", dated 1 July 2004 or SAE J 1962 dated 26 July 2012;

(d) 	ISO 15031-6 "Road vehicles – Communication between vehicle and external test equipment for emissions related diagnostics – Part 6: Diagnostic trouble code definitions", dated 13 August 2010 or SAE J2012 dated 07 March 2013;

(e) 	ISO 27145 "Road vehicles – Implementation of World-Wide Harmonized On-Board Diagnostics (WWH-OBD)" dated 2012-08-15 with the restriction, that only the standard specified in paragraph 6.5.3.1.(a) may be used as a data link;

(f)	SAE J 1979-2 "E/E Diagnostic Test Modes: OBDonUDS", April 2021.

(g) 	SAE J 1979-3 “E/E Diagnostic Test Modes: Zero Emission Vehicle Propulsion Systems on UDS (ZEVonUDS)”, October 2023.

The standards (e), (f) or (g) may be used as an option instead of (a).