|  | United Nations | ECE/TRANS/WP.29/GRPE/2025/23 |  |
| --- | --- | --- | --- |
|  | Informal document GRPE-93-XX 93rd GRPE, 14-17 October 2025 Agenda item 3(a) Updates to Working Document GRPE/2025/24 are shown using tracked changes Economic and Social Council |  | Distr.: General 4 August 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety-third session

Geneva, 14–17 October 2025

Item 3(a) of the provisional agenda

UN Regulation No. 143 (Heavy-Duty Dual Fuel Retrofit Systems)

Proposal for supplement 2 to the original version of UN Regulation No. 143 (Heavy-Duty Dual Fuel Retrofit Systems)

Submitted by the Task Force on Automated Vehicles Regulations Screening (AVRS)*

The text reproduced below was prepared by the Task Force on Automated Vehicles Regulations Screening (AVRS). This document aims to allow type approval of vehicles with Automated Driving Systems. The modifications to the current text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 1.1, Footnote 1, amend to read:

“1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.) (document ECE/TRANS/WP.29/78/Rev.48) https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 4.4.1., amend to read:

"4.4.1.	Dual-fuel operating mode indicator

A dual-fuel engine retrofit system shall have a visual indicator indicating to the driver (if applicable) the mode under which the engine operates (dual-fuel mode or diesel mode), with the following requirements:

(a) The characteristics and the location of this indicator are left to the discretion of the engine retrofit system manufacturer and may be part of an already existing visual indication system.

(b) This indicator may be completed by a message display. The system used for displaying the messages referred to in this point may be the same as the ones used for OBD, correct operation of NOx control measures, or other maintenance purposes.

(c) The visual element of the dual-fuel operating mode indicator shall not be the same as the one used for the purposes of OBD (that is, the MI – Malfunction Indicator), for the purpose of ensuring the correct operation of NOx control measures, or for other engine maintenance purposes.

(d) Safety alerts always have display priority over the operating mode indication.

For a dual-fuel engine retrofit system capable of operating on vehicles equipped with an ADS, it shall send a signal to the ADS, indicating the mode under which the engine operates (dual-fuel mode or diesel mode)."

Paragraph 4.4.1.1., amend to read:

"4.4.1.1.	The dual-fuel system shall warn the driver by means of the dual-fuel mode indicator or an audio signal (or both) when the engine operation is forced to diesel mode as required by paragraph 4.3.

Whilst an ADS feature is active, the dual-fuel system shall send a signal to the ADS when the engine operation is forced to diesel mode as required by paragraph 4.3."

Paragraph 4.4.1.2., amend to read:

"4.4.1.2.	The dual-fuel mode indicator shall be active for at least one minute on the selected mode as soon as the engine operating mode is changed from diesel to dual-fuel mode or vice-versa. This indication is also required for at least one minute at key-on, or at the request of the engine retrofit system manufacturer at engine cranking. The indication shall also be given upon the driver's request."

Paragraph 7.1., amend to read:

"7.1.	The dual-fuel engine retrofit system shall implement an OBD system that complies with the requirements for type 2B engines specified in paragraph 7. of Annex 11 to Regulation No. 49 and with the following restrictions:

(a) During diesel operation the diesel OBD system shall remain the only on board diagnostic system of the vehicle. The MI shall activate in case of a detected malfunction;

(b) During dual-fuel operation the diesel OBD system shall continue to monitor the original emission related components in use. The MI shall activate in case of a detected malfunction;

(c) During dual-fuel operation the dual-fuel ECU shall monitor the dual-fuel emission related components as well as their electrical connections. If the dual-fuel ECU detects a malfunction, the switch to diesel mode shall be performed as soon as possible. Operation in dual-fuel mode shall not be possible until the cause of the malfunction is removed. A clear visual or acoustic signal shall inform the driver and/or the ADS about the situation."

II.	Justification

- The document introduces changes required to incorporate vehicles with Automated Driving Systems into the existing Regulation.

- Reference to the Consolidated Resolution on the Construction of Vehicles (R.E.3.) is updated to include ADS equipped vehicles in the scope. Vehicle categories X and Y are sub-categories which are introduced in R.E.3 by the Task Force on Automated Vehicle Categorisation (TF-AVC) under GRVA and are not capable to be driven manually above 6 km/h.

- Driver messages and warnings are changed to make the requirements suitable for vehicles with and without a human driver.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.