|  | United Nations | ECE/TRANS/WP.29/GRPE/2025/17 |  |
| --- | --- | --- | --- |
|  | Informal Document GRPE-93-43e Submitted by the A-LCA IWG 93rd GRPE, 15-17 October 2025 Agenda item 14 Updates to GRPE/2025/17 are shown with clean version Updates to GRPE/2025/15e are shown using tracked changes Economic and Social Council |  | Distr.: General 5 August 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety-third session

Geneva, 14-17 October 2025

Item 14 of the provisional agenda

Automotive Life Cycle Assessment (A-LCA)

Proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA)

Submitted by the Informal Working Group on Automotive Life Cycle Assessment *

The text reproduced below was prepared by the Informal Working Group on Automotive - Life Cycle Assessment (A-LCA). It is a proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA). It is submitted to the Working Party on Pollution and Energy consideration at its 93rd session.

10.	Verification process

Verification shall be conducted to confirm that the results of the A-LCA have been obtained in accordance with this Resolution while ensuring transparency and consistency. Therefore, for A-LCA studies of level 1, 2, 3 and 4 a critical review shall be conducted according to ISO 14040:2006 paragraph 7 („Critical Review“) with additional focus on compliance with this Resolution. The verification statement shall be available on the report of A-LCA studies  intended for publication .

Verification thus refers to

(a)	methodology applied,

(b)	input data used,

(c)	interpretation of results,

(d)	interpretation of robustness of the study

(e)	reporting of these topics in the background report and

(f)	check of the numerical results of the calculation.

For Level 1 and 2, LCA practitioners should decide whether to perform an internal or external critical review in accordance with ISO 14040:2006 paragraph 7 whereas for Level 3 and 4 an external third party review is mandatory. For Level 3 and 4, the verification shall be done from the perspective of the vehicle and therefore will be conducted at OEM level.  It shall also include the definition of the LCA upstream group as well as the choice of the representative vehicle for which the OEM calculates the LCA. Where the OEM’s LCA includes verified primary information / primary data from suppliers, no re-verification of these data is necessary. The verification of the OEM's LCA calculation system shall include data collection, internal sources, assumptions, calculation processes and tools applied. In case an LCA calculation system is verified and that verification is still valid, no reverification of the system is required for an LCA study based on the verified LCA calculation system. OEMs shall make the system verification statements available to the contracting parties’ according to their requests. The same holds true for suppliers who are responsible for providing data in accordance with this Mutual Resolution.

In the case that no verification body has been nominated by the Contracting Party to the UNECE secretariat, the current practice referring to ISO14071:2024-10 and ISO14064-3:2019 should be continued.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.