|  | United Nations | ECE/TRANS/WP.29/GRPE/2025/17 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 5 August 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety-third session

Geneva, 14-17 October 2025

Item 14 of the provisional agenda

Automotive Life Cycle Assessment (A-LCA)

Proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA)

Submitted by the Informal Working Group on Automotive Life Cycle Assessment *

The text reproduced below was prepared by the Informal Working Group on Automotive - Life Cycle Assessment (A-LCA). It is a proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA). It is submitted to the Working Party on Pollution and Energy consideration at its 93rd session.

Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA)

Contents

Page *

I.	Preamble

II.	Statement of technical rationale and justification

1.	Introduction

2.	Methodology background

3.	Existing regulations and standards

4.	Technical rationale and justification

III.	Text of the Mutual Resolution

1.	Purpose and principles

2.	Scope and application

3.	Definitions

4.	Abbreviations

5.	[Future review]

6.	[Language]

7.	General methodology

8.	Methodology per life cycle stages

9.	Reporting

10.	Verification process

Annex

Service life (in km and years) values for European countries

* Page number will be added at a later stage prior to WP.29 submission

- 3.33.	"Hotspot" refers to an input, a process or a life cycle stage that has the highest contribution to the impact category of the product system under consideration or a group of these inputs or processes or life cycle stages.

- Note: definitions for hotspots exist in other references such as the 2017 UNEP report, the EU Environmental Footprint method (PEF, 2021)

- ]
“Hotspot” means a process or stage within a defined boundary that exhibits GHG emissions significantly higher than the average. However, even if a process or stage is not considered a hotspot in the GHG emissions calculation, it shall be included in the calculation if it is recognized as necessary to satisfy the requirement definition of the cut-off rule in 7.13. Hotspots are defined in other references such as the 2017 UNEP report.

  - 7.3.	[Hotspots

The 2017 UNEP hotspot Definition

“Having attributed impacts to each life cycle stage and normalised / weighted the impacts to allow them to be prioritised, hotspots can be defined. Two approaches may be used. These are illustrated in the figure below.

Figure 3
Options for identifying hotspots

| Hotspot | Warmspot | Cold Spot |
| --- | --- | --- |
| A life cycle stage whose contribution to the impact category is greater than even distribution of that impact across the life cycle stages. | A life cycle stage whose contribution is approximately equivalent to an even distribution of the impact across the life cycle stages. | A life cycle stage whose contribution to any impact category is less than even distribution of that impact across the life cycle stages |

| Hotspot | Cold Spot |
| --- | --- |
| All life cycle stages collectively contributing more than 50% to any impact category. | All life cycle stages collectively contributing less than 50% to any impact category. |

In the first approach a hotspot shall always be a percentage greater than if the impacts were evenly distributed across life cycle stages. So, if there are 5 life cycle stages, a hotspot should not be defined lower than 20% of the impact category, and if there are 7 stages, it should not be lower than 14%.

Where the hotspot has been identified based on qualitative information, it will not be possible to identify a hotspot with quantitative precision. To ensure that hotspots are covered, the analysis should therefore be confident that the majority of impacts (i.e., over 50%) (the second approach) are covered. Depending upon the number of impact categories selected, the number of hotspots may vary.

The first approach should be implementable. That is, if a stage contributes more than the average share of all stages for automotive LCA, then the stageis considered to be a hotspot.

Applications of the hotspot definition in the LCA [guideline]

Material/vehicle manufacturing. The current language for separate estimation of battery emissions from the rest of vehicle manufacturing emissions appears to be based on this rationale: The current guideline version allows linear extrapolation of per-vehicle results with vehicle weight. The guideline realises that battery capacity (and weight) may be independent from vehicle weight. In this regard, the guideline should make battery as an unscalable example with vehicle weight for separation. If this is the case, some general consideration, instead of specific battery consideration, for separation based on un-scalability should be worded.

EoL. As determined on June 19, EoL may use static energy system results if EoL is determined not to be emission hotspot.]

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

Barthel, M., Fava, J., James, K., Hardwick, A., & Khan, S. (2017). Hotspots Analysis: An overarching methodological framework and guidance for product and sector level application. United Nations Environment Programme: Paris, France. https://www.lifecycleinitiative.org/library/hotspots-analysis-an-overarching-methodological-framework-and-guidance-for-product-and-sector-level-application/