Battery Replacement Comments on ALCA Draft Working Document

06 11 2025 GRPE-2026 Draft Working Document – for GRPE submission_V_clean.docx

November 10, 2025

Submitted by USEPA

General Comments:

- USEPA disagrees with a process that does not have a high threshold for the inclusion of traction battery replacements.

  - The inclusion of a single traction battery replacement has the potential completely skew an LCA to a predetermined outcome.

- The starting assumption should be that the traction battery is not replaced unless the LCA practitioner can provide a data-based justification for doing so, such as field data that demonstrates replacement and/or manufacturer statements that the battery will not last the life of the vehicle.

- The ALCA process should also consider other significant repairs such as engine and transmission replacements, both of which occur on a frequent basis in the field.

Table 18
Prioritisation for traction battery/ fuel cell system replacement in order of accuracy and specificity (highest to lowest)

| Level | Traction Battery / fuel cell system Replacement |
| --- | --- |
| Level 1 | Simple assumption is that the traction battery will not be replaced during the life of the vehicle. If the traction battery is assumed to require replacement, the decision should be based on field data and/or manufacturer statements regarding battery durability and replacement. |
| Level 2 |  |
| Level 3 | Same as Level 1 and 2 but including standardised methodology based on battery cycle life and lifetime operational activity (below) fuel cell system durability. An explanation/justification for the result should be provided. |
| Level 4 | Same as Level 1 and 2: Optional, depending on availability OEM-specific / supplier-specific methodological/modelling approach to define the need for a battery replacement (or not) over the operational life of the vehicle. An explanation/justification for the result should be provided. |

8.3.6.2.5.	Standardised Methodology for calculating the need for traction battery replacement (s)

For vehicle traction batteries, the following approach for accounting for the frequency of energy storage replacement for Option 2, which is based on a combination of parameters including the anticipated battery cycle life (i.e. number full charge/discharge cycles).  This methodology also provides a dynamic link to the vehicle battery capacity and the lifetime activity/service life (as defined in the study Goal & Scope), which can be used in product LCA, prospective LCA or other more generic studies.

The methodology for determining the number of traction battery replacements is as follows (i.e. where a value of N > 1 means at least one replacement is likely to be needed):

N= E Average×A [Lifetime]C Battery usable×CL [Battery]				(36)

Where;

N 	means the total number of traction batteries needed over the vehicle lifetime

C [Battery usable] 	means the usable (i.e. ‘net’) traction battery capacity in kWh

CL [Battery] 	means the average battery cycle life – number of full charge/discharge cycles (within the usable capacity)

A [Lifetime] 	means the vehicle lifetime activity (in km). Note: As a sensitivity it is recommended to also explore the potential number of replacements needed based on the warrantied number of km for the battery (where this is present).

E [Average] 	means the vehicle average electrical energy consumption, in kWh per km

In the absence of OEM-specific data on the battery cycle life (parameter ‘CL’ above), then it is proposed to use a value of 2000 charge/discharge cycles, which is representative of a typical value for current technology.  Should the battery come with an expected calendar lifetime lower than the defined vehicle lifetime in the Goal & Scope of the LCA study, then a replacement will also be required.]

As also previously implemented in (Ricardo et al., 2020) based on consultation with stakeholders.