Changes made in bold and strikethrough

Paragraph 5.3.5.1., amend to read:

5.3.5.1. 	The system shall aim to detect the applicable system boundaries when DCAS or a feature of DCAS is in ‘on’ mode. If the system identifies that the system or feature boundary is exceeded, it the system and all its features shall transition into ‘stand-by’ mode, with the exception of the driver unavailability response, and immediately notify the driver in accordance to the strategies described by the manufacturer as outlined in paragraph 5.3.5.2. and according to the HMI requirements defined in paragraph 5.5.4.1.

If the driver is detected to be disengaged whilst the system is transitioning into ‘stand-by’ mode, the system shall continue providing lateral and longitudinal assistance to the extent possible as outlined in the safety concept of the vehicle manufacturer and the driver state monitoring system shall continue to monitor driver disengagement until the driver is motorically and visually reengaged. The system shall initiate a driver unavailability response at the latest 10 seconds after leaving the system boundary, if the driver remains motorically disengaged. The system shall terminate assistance to the driver provided by the affected feature or the system in a controllable way. The assistance termination strategy shall be described by the vehicle manufacturer and assessed according to Annex 3.

Paragraph 5.3.7.3.1. and Paragraph 5.3.7.3.3., amend to read:

5.3.7.3.1. 	The system's driver unavailability response shall comply with the technical requirements and transitional provisions of the 04 or later series of amendments to UN Regulation No. 79 with respect to the Risk Mitigation Function (RMF). In the event that the driver has been determined to be unavailable following a driver disengagement warning escalation sequence as defined in paragraph 5.5.4.2.6., the system shall appropriately activate the Risk Mitigation Function in order to come to a safe stop in accordance with the RMF requirements.

5.3.7.3.3. 	Where the system is equipped with a driver-confirmed or system-initiated lane change feature, the RMF system shall be capable of performing lane changes during the driver unavailability response, in compliance with the technical requirements for systems RMF with the purpose capable of bringing the vehicle to a safe stop outside its own lane of travel of the 04 or later series of amendments to UN Regulation No. 79, during an intervention on a highway to bring the vehicle towards a target stop area in a slower or emergency lane.

Insert new paragraph 5.3.7.3.4., to read:

5.3.7.3.4. 	The requirements of 5.3.2 shall continue to apply whilst the RMF is active.

_________________________________________________________________________________

Insert new paragraph 5.5.4.2.6.5.7., to read:

5.5.4.2.6.5.7 	Additional monitoring requirements for when systems are capable of 			performing system-initiated manoeuvres whilst withholding HORs

5.5.4.2.6.5.7.1. 	The driver state monitoring system shall aim to determine whether or not the 		driver is in an appropriate position to operate the vehicle controls. The strategy 		employed for classification and detection of such positions shall be documented 		by the manufacturer and assessed by the Type Approval Authority.

5.5.4.2.6.5.7.2. 	If the driver is deemed, as per the strategy, not to be in an appropriate position 		to operate the vehicle controls for [10] seconds, an HOR shall be given.

5.5.4.2.6.5.7.3.	Following such an HOR, the system shall only resume operating whilst 			withholding HORs once the driver has been deemed, as per the strategy, to be in 		an appropriate position to operate the vehicle controls and not to be motorically 		disengaged for at least [30s].

_________________________________________________________________________________

Paragraph 5.5.4.2.7., amend to read:

5.5.4.2.7. 	Additional Strategies for Disengagement Detection and Re-Engagement Support

5.5.4.2.7.1. 	When the system is not capable of performing SIM, the driver state monitoring system shall have be equipped with strategies to assess whether the driver is disengaged in the event that no driver input has been determined over prolonged periods (e.g. through a negative determination of driver drowsiness, or assessment of changes in gaze direction) and implement appropriate countermeasures.

5.5.4.2.7.2.	When the system is capable of performing SIM, the driver state monitoring system shall have additional strategies to assess behaviours which indicate that the driver is cognitively engaged with the driving task by assessing the following:

(a)	determination of drowsiness and fatigue, and being equipped with strategies to implement appropriate countermeasures

(b)	that the pattern of their gaze movement and control inputs are appropriate to the current vehicle behaviour, surrounding traffic,  road/junction geometry and upcoming manoeuvres.

If the behaviours of the driver do not indicate sufficient cognitive engagement with the driving task, then the driver shall be deemed to be disengaged, and appropriate countermeasures shall be applied to re-engage the driver (e.g. role reminder, cooperative driving, speed reduction or others as described in the safety concept).

The strategies implemented to assess behaviours which indicate that the driver is cognitively engaged with the driving task shall be documented by the manufacturer and assessed by the Type Approval Authority.

_________________________________________________________________________________

Paragraph 10., amend to read:

10.	Requirements for Software Identification and Updating

10.1.	For the purpose of ensuring the software of the System …

…

10.6	To the extent that is technically feasible to achieve, a software update for in-service vehicles shall be provided to rectify any unreasonable safety risk with the system identified through the safety managements system. This shall be provided within a timeframe that allows for development, testing, and, where necessary, certification of the rectification.

10.6.1.	This shall also be the case where amendments are made to this Regulation to address safety risks that have been identified with existing provisions.

Justification:

- Currently it isn’t clear what happens with a DCAS vehicle when it exits the system boundaries, and the driver is disengaged. In the case of the driver being incapacitated the system should provide a higher level of assistance than what is covered in 5.3.7.1.4.. The scope of ADAS technology has advanced since R79 and DCAS can provide more than just lane-keeping capabilities, so should be able to provide lateral and longitudinal assistance in these cases. There should also be a point where the driver unavailability response is performed if the driver continues to provide no input, which the new wording now covers. The rationale for the 10 seconds is because this is the timing in between issuing a DCA and performing a driver unavailability response during the disengagement cascade.

It’s not quite clear if when an RMF is being performed, if the DCAS has handed over control to the RMF feature or if the DCAS is performing the RMF. Additionally, the RMF in Reg 79 doesn’t covering collision detecting capabilities whilst driving in-lane, only when changing lane. The two suggestions here aim to clarify that either it is a separate RMF that is controlling the vehicle, or that the system is performing the RMF, it must comply to have detection capabilities whilst driving in lane as well as changing lane (5.3.2.).

- There is the expectation that, at any point in time and with very short notice, the driver may need to intervene and make inputs into controls of a DCAS vehicle. The driver, therefore, must be sat in appropriate position (e.g., sat upright, facing forwards, feet and hands near the controls, hands unoccupied, etc) in order to do this quickly and safely. It is recognised that there is a propensity with hands-free system for some drivers to engage in actions not related to or detrimental to performing the driving task, which will be further exacerbated when the system is withholding HORs. Therefore, there is a need to have a form of monitoring to ensure that the driver is in an appropriate position to take control in an instance, especially in periods of long driving where the risk of inattentiveness and complacency can increase. Therefore, these new provisions aim to solve this issue by making the driver monitoring system assess that the driver is in a position to operate the controls when the system is capable of performing SIM whilst HORs are being withheld.

If the driver is deemed not to be in an appropriate position, then a HOR will be given. Additional provisions are included so that once the driver is holding the wheel (i.e. motorically engaged) then hands-free driving cannot resume immediately. Thereby avoiding suppression of the warnings from physical disengagement simply by touching the wheel.

- Hands on the wheel and eyes looking forward do not necessarily represent that the driver is cognitively engaged with the driving task. They could be blankly staring ahead which would constitute engagement according to 5.5.4.2.5.2. but not situationally aware of what’s going on. This can be exacerbated when it comes to long periods of driving where the driver does not provide any input to the driving task. A driver needs to be ‘in the loop’ and demonstrate behaviours which indicate that they are cognitively engaged, so that they can intervene and make inputs into controls of a DCAS vehicle at any point in time and with very short notice. To be cognitively engaged the driver needs to be regularly providing input into the controls of the DCAS vehicle and/or directing their gaze to the different driving task relevant areas when appropriate for the driving environment. These new provisions aim to strengthen the provisions arounds driver monitoring to ensure the driver remains cognitively engaged with the driving task and is situationally aware of their surroundings. Manufacturers can choose which behaviours to assess when systems are not capable of performing SIM, but when it comes to systems that can perform SIM they must assess both behaviours together.

- There is still uncertainty as to how drivers will respond to these systems and certain requirements have been drafted at a high level. The safety management system is designed to deal with this by ensuring that the manufacturer has taken a safety conscious approach to the design and development of their systems to mitigate the potential for safety risks. This also extends to the operation, where it is expected that information is gathered on how safely the systems are functioning. At any point where unreasonable risk is identified then it should be acted upon, even with the vehicles in-service. This provision reiterates that expectation, in that where an issue is identified and can feasibly be rectified by a software update then that should be produced and provided. This prevents unsafe systems from continuing to operate on the road despite there not being

The requirements of this Regulation are also expected to evolve in response to understanding of how these systems operate and are used, including any incidents that results. Any changes that are safety related should be accounted for in the safety management system and acted upon in similar fashion. Updates should be provided rather than in-service vehicles be left to operate in a manner that regulators no longer consider appropriate.