Proposal for supplement to the 03 and 04 series of amendments to UN Regulation No. 79 (Steering equipment)

I.	Proposal

Paragraph 4.4.3., amend to read:

“4.4.3.	a letter “X” preceding the approval number in the case when either:

(a)	ACSF of Category A, B1 or C have been exempted from the technical requirements of this UN Regulation according to paragraph 5.6.1, 5.6.2. and/or 5.6.4.

(b)	The steering system exhibits functionality defined as ACSF of Category B2, D or E.”

Paragraph 5.6.1., amend to read:

“5.6.1.	Special Provisions for ACSF of Category A

Any ACSF of Category A shall fulfil the following requirements.

Vehicles equipped with an ACSF of Category A shall fulfil the following requirements unless the vehicle is equipped with a DCAS which:

(a)	Incorporates this function, and

(b)	Allows the activation of this function only during operation and as part of DCAS, and

(c)	Complies with the technical requirements of the original version or later series of amendments of UN Regulation No. 171.8”

II.	Justification

- This proposal aims to apply the requirements of the ACSF categories B1, B2, C, D, E clarifying the scope between this Regulation and the UN Regulation No. 171 also for the ACSF category A.

- This amendment becomes necessary as the UN Regulation No. 171 fundamentally provides the possibility to type-approve a DCAS system with parking capabilities without complying with specific requirements in this regard. Therefore, this amendment shall clarify that the UN Regulation No. 171 shall always be applied for parking capabilities exceeding the specific requirement defined in this Regulation (e.g. for parking applications with a higher speed range than 10 km/h).

- Until the UN Regulation No. 171 contains specific provisions for Remote Controlled Parking (RCP) functionalities compared to this Regulation (ref. 5.5.3.2.3.) such system capabilities remain within the scope of the UN Regulation No. 79and cannot be type-approved under UN Regulation No. 171.