| Nuber of Item in [] | Content in [] | Proposal for Master document modification |
| --- | --- | --- |
| 2.15.2.1 | “Passive mode” means a DCAS operational condition, when the system or DCAS feature is in ‘stand-by’ mode and considers itself to be within its system boundaries with no preconditions preventing switching to ‘active’ mode. |  |
| 5.3.5.1 | The system shall aim to detect the applicable system boundaries when DCAS or a feature of DCAS is in ‘on’ mode. If the system identifies that the system or feature boundary is exceeded, it the system or the applicable feature shall transition into ‘stand-by’ mode, [with the exception of the driver unavailability response,] and immediately notify the driver in accordance to the strategies described by the manufacturer as outlined in paragraph 5.3.5.2. and according to the HMI requirements defined in paragraph 5.5.4.1. [If the driver is detected to be disengaged whilst the system is transitioning into ‘stand-by’ mode, the system shall continue providing lateral and longitudinal assistance to the extent possible as outlined in the safety concept of the vehicle manufacturer and the driver state monitoring system shall continue to monitor driver disengagement until the driver is motorically and visually reengaged. The system shall initiate a driver unavailability response at the latest 10 seconds after leaving the system boundary, if the driver remains motorically disengaged. ] The system shall terminate assistance to the driver provided by the affected feature or the system in a controllable way. The assistance termination strategy shall be described by the vehicle manufacturer and assessed according to Annex 3. |  |
| 5.3.6.1.2. 5.3.6.2.1. | 5.3.6.1.2. : [ In addition to 5.3.6.1.1. while performing SIM with withholding HORs or while performing SIM in non-highway, the manufacturer shall consider this potential reaction time required for the driver to respond to a DCA, to hold the steering control and to visually engage with the relevant driving related areas. This shall never be assumed to be less than [3] seconds, unless the manufacturer is able to demonstrate that controllability is ensured through specific strategies.] 5.3.6.2.1. :[ In addition to 5.3.6.2. while performing SIM whilst withholding HORs or performing SIM in non-highway the system shall be designed to ensure controllability and consider drivers’ visual engagement with task-relevant areas in accordance with the system’s capabilities and within the defined system boundaries. The system shall take into account that the driver may not had the possibility to perceive or may not be aware of the surrounding traffic and therefore needs additional time to safely control the vehicle.] | Japan understands that the driver should be informed with sufficient time ahead of the initiation of a System-Initiated Manoeuvre. This is covered by paragraph 5.5.4.1.9.2.. On the other hand, while performing SIM with withholding HORs or while performing SIM in non-highway, potential reaction time required for the driver to respond to a DCA, to hold the steering control could be taken into account same time as the one while withholding HORs which is required in paragraph 5.3.6.1.1.. The reason is, it is considered that the driver is visually engaged after initiation of a System-Initiated Manoeuvre. We would like to know the justification of 3s and why this reaction time should be different from the case where in high-way while withholding HORs but not performing SIM. |
| 5.3.7.2.1.1. | [ Unless exempted according to paragraph 5.3.7.2.1.1.1.] a manoeuvre shall only be initiated if the driver is not detected to be disengaged, and - has commanded the system to perform the manoeuvre for a driver-initiated manoeuvre; or - has acknowledged the system’s intention as needed for a driver-confirmed manoeuvre; or - is given sufficient notice to react for a system-initiated manoeuvre. Motoric disengagement may not be considered when HORs are being withheld by the system. [ The SIM shall not be initiated while the assessment of driver disengagement is still pending - that is, before the DMS has confirmed the driver is either disengaged or not disengaged. ] |  |
| 5.3.7.2.1.1.1. | [ A manoeuvre is permitted to be initiated by the system although the driver is detected to be disengaged while not being provided with a disengagement warning yet under any of the following circumstances (a) operation of the vehicle cannot be continued in the current lane (e.g. due to lane closure or the lane being blocked) (b) the manoeuvre is considered to minimize the risk of a collision (c) not initiating the manoeuvre would make the system reach a system boundary and require termination of assistance. In addition the system shall apply strategies to re-engage the driver as soon as the disengagement is detected. ] | Japan would like to know the reason why item (c) is necessary in addition to item (a) and (b). We have concern that item (c) makes the situations too wide in which a manoeuvre is permitted to be initiated by the system although the driver is detected to be disengaged. |
| 5.3.7.2.1.13. | [ The manufacturer shall describe in the safety concept the system behaviour in case the driver is detected to be disengaged during a manoeuvre (e.g., initiation of a risk mitigation function, full execution of the manoeuvre, stop the vehicle).] |  |
| 5.3.7.2.4.2. | A manoeuvre shall not be initiated if system has presented an EOR to the driver in the 7 seconds leading up to the initiation of the manoeuvre,[ unless an exemption according to paragraph 5.3.7.2.1.1.1. applies. ] |  |
| 5.3.7.2.4.10. | [ If the system is designed to perform system-initiated manoeuvres - on any other road than a highway, or - while the system is withholding HORs on a highway according to par. 5.5.4.2.6.5., in addition to applying all relevant requirements of this Regulation the safety of the system shall additionally be assessed according to par. X Appendix 4 to Annex 3. ] |  |
| 【others】 5.3.7.2.5.3.1. | For highway operation, the system shall aim to respond to work zones, lane reductions, lane closures, toll stations and end of highways (e.g., by notifying the driver, issuing a DCA, or continuing operation if capable). | Japan would like to confirm the interpretation of this paragraph. Does the requirement "the system shall aim to respond to work zones, lane reductions, lane closures, toll stations and end of highways" also include "the system shall aim not to initiate a manoeuvre toward work zones, lane reductions, lane closures, toll stations and end of highways"? |
| 5.3.7.2.6.1. | [ The system shall be designed to have multi-target anticipatory behaviour in interaction with other road user(s) and impassable objects aiming to ensure stable, low-amplitude dynamics and/or to minimise risk as appropriate (e.g., when critical situations could become imminent). Option 1: This shall be demonstrated by avoidance of a collision in the following scenarios, accounting for the robustness criteria outlined in Annex 3 Appendix 4: (a) A cut-out of the lead vehicle with blocked… ] Option 2: [ The strategies implemented to achieve this shall be demonstrated on the basis of the scenarios outlined in Annex 4, section Y. For each type of scenario the vehicle manufacturer shall explain the strategies implemented to ensure safety. The manufacturer shall provide evidence (e.g. simulation results, real-world test results, track test results) of the system’s behaviour in the described types of scenarios ] | ・The wording “multi-target anticipatory behaviour” seems not clear. The definition of the word should be added in chapter 2.. ・What kind of dynamics are referred to by the term "low-amplitude dynamics"? ・Regarding the wording of “impassable objects,” clarification is needed. We would like to know what‘passable’ is and what is “impassable”? ・Japan considers that both Option 1 and Option 2 are important. It is considered better to apply both Option 1 and Option 2 rather than selecting one option. However, it is considered necessary to clarify the degree of robustness required. |
| 5.3.7.2.6.2. | [ To minimize the need for driver intervention, the system shall aim to consider its understanding of the situation as well as its understanding of the driver being appropriately engaged when making the decision to initiate a manoeuvre. How this is assessed and how this might vary for different manoeuvres in different driving situations shall be demonstrated to the Technical Service. ] | Japan would like to confirm the interpretation of this requirement. Is the plausibility of understanding of the situation by the system is also included in the assessment (for example, the situation in which the system might not correctly detect the object)? Japan would like to understand the intent behind adding this new provision. |
| 5.3.7.2.6.3. | [ The system’s boundary conditions shall include the maximum operating speed in the test scenarios in Annex 4 and the system should react according to 5.3.7.2.5.3.2. in a controllable way when SIM is active or can be activated in non-HW roads. ] |  |
| 5.3.7.3.1. | The [system’s driver unavailability response] shall comply with the technical requirements and transitional provisions of the 04 or later series of amendments to UN Regulation No. 79 with respect to the Risk Mitigation Function (RMF). In the event that the driver has been determined to be unavailable following a driver disengagement warning escalation sequence as defined in paragraph 5.5.4.2.6., the system shall come to a safe stop[ in accordance with the RMF requirements.] |  |
| 5.3.7.3.3. | Where the system is equipped with a driver-confirmed or system-initiated lane change feature, the [ system shall be capable of performing lane changes during the driver unavailability response, in compliance with the technical requirements for RMF capable] of bringing the vehicle to a safe stop outside its own lane of travel of the 04 or later series of amendments to UN Regulation No. 79, during an intervention on a highway to bring the vehicle towards a target stop area in a slower or emergency lane. |  |
| 5.3.7.3.4. | [ The requirements of 5.3.2. shall continue to apply whilst the driver unavailability response is active. ] | Japan would like to know the reason why this paragraph is added. We understand that the system which comply with the requirement of the RMF in UN R79 is considered to fulfill this requirement. |
| 5.5.4.1.8.1. | The system shall visually inform the driver about a proposed manoeuvre. If informing about a series of manoeuvres, then it shall be a combination that is comprehensible to the driver and of a connected series. The manufacturer shall explain to the Type Approval Authority the timing at which this information is provided to ensure appropriate driver response [ and the modalities adopted to ensure drivers notice the information without undue delay. ] |  |
| 5.5.4.1.9.2. | The system shall aim to provide information ahead of the initiation of a relevant intended manoeuvre with sufficient notice to allow the driver to comprehend the manoeuvre and the traffic situation, taking into account the complexity of the manoeuvre and amount of other road users present. [ This shall never be assumed to be less than [ 3 ] seconds, unless the manufacturer is able to demonstrate that controllability is ensured through specific strategies.] If there is a risk of imminent collision or it would conflict with the information about an ongoing manoeuvre, the time may be reduced and system shall visually inform the driver as far in advance as possible. [ The manufacturer shall explain to the Type Approval Authority the timing at which this information is provided to ensure appropriate driver response and the modalities adopted to ensure drivers notice the information without undue delay.] In addition, the initiation of a lane change procedure shall be announced by another modality unless the system has assessed that the driver has observed the visual information. |  |
| 5.5.4.1.9.4 | [ Option 1: The system shall continuously visualize the result of its environmental perception (focusing on the most relevant elements for the current situation) as well as the planned trajectory which indicates the intended manoeuvre. This visualisation shall not cause additional distraction to the driver. Option 2 The system shall offer the driver the possibility to visualize at least the most relevant information for the current situation as well as the intended manoeuvre. This visualisation shall not cause additional distraction to the driver .] | Japan considers that Option 2 is more reasonable. We have a concern that providing continuously visualize of its environmental perception in Option1 causes additional distraction to the driver. Is it correct to understand that if the first sentence of OP2 is satisfied, the second sentence is automatically satisfied? |
| 5.5.4.1.10.1 | [ The provisions 5.5.4.1.9. shall equally apply. The system shall aim to provide information ahead of the initiation of a relevant intended manoeuvre with sufficient notice to allow the driver to comprehend the manoeuvre and the traffic situation, taking into account the complexity of the manoeuvre and amount of other road users present. This shall never be assumed to be less than 3 seconds, unless the manufacturer is able to demonstrate that controllability is ensured through specific strategies. If there is a risk of imminent collision or it would conflict with the information about an ongoing manoeuvre, the time may be reduced and system shall visually inform the driver as far in advance as possible. The manufacturer shall explain to the Type Approval Authority the timing at which this information is provided to ensure appropriate driver response and the modalities adopted to ensure drivers notice the information without undue delay. ] | These requirements are duplicated with 5.5.4.1.9.2.. In this paragraph, the requirement “The provisions 5.5.4.1.9. shall equally apply.” seems sufficient. |
| XXXX | [ Role reminders (RR) The system may issue reminders to foster the development of a correct mental model of the system. Reminders have no penalty implications as it is described in 5.5.4.2.8. (prolonged driver disengagement) A RR shall be a continual visual information in combination with at least one other modality which are clear and easily perceptible, unless it can be ensured that the driver has observed the visual information. The visual information shall clearly inform drivers about their responsibilities while DCAS is “active”. A RR shall, as a minimum, be provided at the activation of the DCAS function. ] | The wording “Role reminders (RR)” should be defined in chapter 2. While issuing RR is described as “may”, the detailed requirements of RR are stated as “shall”, which seems to be contradictory. |
| 5.5.4.2.5.2. | 5.5.4.2.5.2. : The driver shall be deemed to be visually disengaged when the driver’s eye gaze and/or head posture, as relevant, is directed away from any currently driving task relevant area. An outline of the driving task relevant areas, and when they are relevant, shall be specified by the manufacturer in the documentation provided to the Type Approval Authority. For the purpose of the assessment of visual disengagement, the dashboard and instrument panel shall [ only for a maximum duration of [3] s be ] be considered as a driving task relevant area. [For the case of system-initiated manoeuvres with withholding HORs and system-initiated manoeuvres in non-highway environment, for the purpose of the assessment of visual disengagement, the dashboard and instrument panel may be considered as driving task relevant areas. The manufacturer shall aim to reduce the time required for the drivers to glance towards dashboard and instrumental panel and shall not be longer than [3] s.] |  |
| 5.5.4.2.6.1.5. | [ At speeds above 10km/h, during phases of operation in which system-initiated manoeuvres would occur in non-highway environments, an HOR shall be given latest when the driver is deemed motorically disengaged for more than [2]s, unless the driver is keeping their hands in a position close to the steering wheel, ready to intervene if needed. ] | Why is HOR required after more than 2 seconds when the driver is in a state other than "hands in a position close to the steering wheel"? It is considered that the phrase “keeping their hands in a position close to the steering wheel, ready to intervene” is not very clear. (For example, is keeping one's hands on the knee permitted? Do your palms have to be facing the wheel? What if you have one hand in your lap?) Additionally, we would like to confirm in the case where “the driver is keeping their hands in a position close to the steering wheel,” whether clauses 5.5.4.2.6.1.1 and 5.5.4.2.6.1.2 are appled or not? |
| 5.5.4.2.6.5. | The system may withhold HORs when the vehicle is located on a “Highway” and is operated at a speed up to 130 km/h. As outlined in paragraph 5.3.5.2., the manufacturer shall describe in detail, as part of the documentation required for section 9, the boundary conditions under which HORs can be withheld. Withholding HORs on a road which, by design, is equipped with a physical separation that divides the traffic moving in opposite directions but where pedestrians and cyclists are not prohibited shall only be permitted on types of roads with a general speed limit of at least [60] km/h in absence of crossings at grade and if the system is designed to ensure safe operation in the presence of pedestrians and cyclists. This shall be demonstrated by avoidance of a collision with a longitudinally travelling cyclist ahead in lane as outlined in Annex 4, paragraph 4.2.5.2.16]. Whilst in this mode of operation, the following subparagraphs shall apply: |  |
| 5.5.4.2.6.5.7. | If the system is capable of performing system initiated maneouvres in phases where HORs are withheld and the driver is deemed not to be in an appropriate position to operate the vehicle controls as outlined in 5.5.4.2.X. for [10] seconds, an HOR shall be given. The continuation of the driver not being in an appropriate position shall be escalated according to 5.5.4.2.6.1.2. Following an HOR for this reason, the system shall only resume operating whilst withholding HORs once the driver has been deemed to be in an appropriate position to operate the vehicle controls and not to be motorically disengaged for at least [30s]. | The requirement that the driver be in an "appropriate position" seems to be important not only in the case of withholding HOR in SIM, but also in other cases. Japan would like to ask about the intention and background behind specifying requirements related to "appropriate position" in this section. Japan's understanding of the intent of this provision is that if "the driver" is not in an "appropriate position," a warning must be given to have the driver return to an appropriate position. If the above understanding is correct, it may not necessarily need to use "HOR" to give a warning; using "EOR" would also seem to achieve the same purpose because if the driver is not in an “appropriate position” DMS cannot catch the driver’s face thus it cannot keep DCAS active and is expect to issue an EOR. But if "HOR" is necessary, we would like to know the intention behind this. |
| 5.5.4.2.6.5.7 and sub | 7.1. - The driver state monitoring system shall aim to determine whether or not the driver is in an appropriate position to operate the vehicle controls. The strategy employed for classification and detection of such positions shall be documented by the manufacturer and assessed by the Type Approval Authority. 7.2. - If the driver is deemed, as per the strategy, not to be in an appropriate position to operate the vehicle controls for [10] seconds, an HOR shall be given. 7.3. - Following such an HOR, the system shall only resume operating whilst withholding HORs once the driver has been deemed, as per the strategy, to be in an appropriate position to operate the vehicle controls and not to be motorically disengaged for at least [30s]. | It is considered that 30 s is too long, as the driver may not understand why the system does not resume operation whilst withholding HORs after returning to an appropriate position to operate the vehicle controls. |
| 5.5.4.2.7.1 | Option 1 [5.5.4.2.7.1. Special requirement for system-initiated manoeuvres and withholding of HORs and system-initiated manoeuvres in non-highway environment The driver state monitoring system shall be equipped with strategies to ensure the driver is engaged with the driving task. The manufacturer shall explain to the Type Approval Authority the strategies in place to ensure appropriate driver engagement.] Option 2 [5.5.4.2.7.1. When the system is not capable of performing system-initiated manoeuvres, the driver state monitoring system shall have be equipped with strategies to assess whether the driver is disengaged in the event that no driver input has been determined over prolonged periods (e.g. through a negative determination of driver drowsiness, or assessment of changes in gaze direction) and implement appropriate countermeasures. | (This texts were updated in ADAS 43-02. Following comment is based on ADAS 43-02.) Regarding the “and” in Op 1 of 5.5.4.2.7.1, I want to understand the parallel relationship in the sentence. (For “system-initiated maneuvers and withholding of HORs,” I want to know whether it is “system-initiated maneuvers and withholding of HORs” or “system-initiated maneuvers” and “withholding of HORs.”) |
| 5.5.4.2.7.2. | 5.5.4.2.7.2. When the system is capable of performing SIM, the driver state monitoring system shall have additional strategies to assess behaviours which indicate that the driver is cognitively engaged with the driving task by assessing the following: (a) determination of drowsiness and fatigue, and being equipped with strategies to implement appropriate countermeasures (b) that the pattern of their gaze movement and control inputs are appropriate to the current vehicle behaviour, surrounding traffic, road/junction geometry and upcoming manoeuvres. If the behaviours of the driver do not indicate sufficient cognitive engagement with the driving task, then the driver shall be deemed to be disengaged, and appropriate countermeasures shall be applied to re-engage the driver (e.g. role reminder, cooperative driving, speed reduction or others as described in the safety concept). The strategies implemented to assess behaviours which indicate that the driver is cognitively engaged with the driving task shall be documented by the manufacturer and assessed by the Type Approval Authority.] | Japan would like to understand the meaning of “cooperative driving”. |
| 5.5.4.2.8.2. | The driver is deemed to have insufficient engagement when this leads to: (a) One unavailability response initiation; (b) At most 2 DCAs due to prolonged insufficient engagement; or (c) At most 3 engagement request escalations. [ (d) At most 1 DCAs due to prolonged insufficient engagement during SIM (e) At most 1 [5s] delayed engagement request after DCA ] |  |
| 6.4 and sub | [ 6.4.1. The provisions of par. 6.3.2. to 6.3.8. shall equally apply when going straight across an intersection for systems capable of conducting a system-initiated left or right turn at that intersection. 6.4.2. In non-highway environments and in the absence of a leading vehicle the system shall only make the vehicle drive away from standstill, if at least one of the following conditions is fulfilled at the moment of drive off: - The vehicle has not been stationary for more than 5s. - The driver is not detected to be visually disengaged as per par. 5.5.4.2.5. - The system has confirmed there is no risk of collision detected along the drive-off path. - The driver has confirmed the drive off. ] |  |
| 7.2.3.1. | The manufacturer shall report at least once a year to the Type Approval Authority on the information deemed to be proper evidence of the intended operation collected through the monitoring program and safety of the system in the field until the production is definitively discontinued according to paragraph 14. The manufacturer shall report at least the information listed in the table below, which can be shared in confidence with other Type Approval Authorities on request but are not intended to be shared publicly. The manufacturer shall be notified in this case. Additional information is subject to agreement between the Type Approval Authority and the manufacturer. In the event that the system was subject to significant changes relevant to the reported information during the reporting period, the report shall differentiate the changes of the system. See Table 1 - Information for Periodic Reporting Points 1, 2, 9 and 10 |  |
| 10.6. | [ To the extent that is technically feasible to achieve, a software update for in-service vehicles shall be provided to rectify any unreasonable safety risk with the system identified through the safety managements system. This shall be provided within a timeframe that allows for development, testing, and, where necessary, certification of the rectification. ] | Japan would like to ask about the necessity and intent behind adding this provision. |
| 10.6.1. | [ This shall also be the case where amendments are made to this Regulation to address safety risks that have been identified with existing provisions.] | Japan would like to ask about the necessity and intent behind adding this provision. Is this requirement stated with the intent of applying it retroactively? |
| 16.2 and sub | [ Transitional Provisions applicable to the 02 series of amendments: 16.2.1. As from the official date of entry into force of the 02 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approvals under this Regulation as amended by the 02 series of amendments. 16.2.2. As from 1 September [2029], Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the preceding series of amendments to this Regulation, first issued after 1 September 2029. 16.2.3. Until 1 September 2032, Contracting Parties applying this Regulation shall accept type approvals issued to the preceding series of amendments of this Regulation, first issued before 1 September 2029. 16.2.4. As from 1 September 2032, Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the preceding series of amendments of this Regulation. 16.2.5. Notwithstanding paragraph 16.2.2. and 16.2.4., Contracting Parties applying this Regulation shall continue to accept UN type approvals issued according to a preceding series of amendments to this Regulation, for vehicles which are not possessing the capability to execute system-initiated manoeuvres in non-highway environments or during phases of operation when HOR are withheld.] |  |
| Appendix 2 1.1 & 1.1.1 | [Definitions For the purpose of this appendix, 1.1.1. “Evidence” means material pertinent to demonstrating the validity of a claim such as physical test results, simulation results, analyses with supporting data, etc.] |  |
| Appendix 2 5. and sub | [ Additional aspects to be assessed for systems capable of performing system-initiated manoeuvres during phases of withholding HOR or in non-highway environment 5.1. Evidence of Sufficient System Performance 5.1.1. The manufacturer shall provide evidence of the system’s ability to recognize/interpret its surroundings with sufficient confidence. 5.1.2. This shall include a variety of driving scenarios, environmental conditions and a variety of different types of relevant traffic participants. 5.2. Evidence of controllability 5.2.1. The manufacturer shall provide evidence of the driver’s ability to control system operation. 5.2.2. This shall include situations where: - the driver is cancelling a manoeuvre before it is started; - the driver is aborting an ongoing manoeuvre. 5.4. System validation 5.4.1. The manufacturer shall provide evidence of its system validation, including the overall validation strategy and results of its execution. 5.4.2. This shall include at least relevant real world driving test results.] |  |
| Appendix 4 4 | [Bicycle target longitudinally travelling ahead of the VUT (Annex 4, par. 4.2.5.2.16.)] |  |
| Annex 4 4.2.5.2.12.1.1. | The bicycle target shall travel in a straight line perpendicular to the VUT’s direction of travel at a constant speed of 15 km/h +0/-1 km/h, starting not before the functional part of the test has started. During the acceleration phase of the bicycle target prior to the functional part of the test the bicycle target shall be obstructed. The bicycle target’s positioning shall be coordinated with the VUT in such a way that the impact point of the bicycle target on the front of the VUT is on the longitudinal centreline [[offset of not more than 0.2 m//of the VUT with a tolerance]]of not more than 0.2 m, if the VUT would remain at the prescribed test speed throughout the functional part of the test and does not brake. |  |
| Annex 4 4.2.5.2.16. and sub | [4.2.5.2.16. Longitudinally moving bicycle target ahead in lane 4.2.5.2.16.1. Base Test: The test shall confirm the declared response capability of the system for a longitudinally moving target and any lateral movement navigating around the target, if applicable. 4.2.5.2.16.1.1. The bicycle target shall be positioned within the driving path of the VUT facing away from the subject vehicle. 4.2.5.2.16.1.2. The VUT shall approach the impact point with the cyclist target in a straight line for at least two seconds prior to the functional part of the test. 4.2.5.2.16.2. Extended testing: The test shall demonstrate that the system is not unreasonably changing the control strategy for a stationary bicycle. 4.2.5.2.16.2.1. The test shall be executed at least with: (a) A bicycle target positioned with different offsets up to the target being outside of the driving path of the VUT; (b) A different speed of the VUT;] |  |
| Annex 4 4.2.5.2.17. and sub | 4.2.5.2.17. Cut-out of lead vehicle for multi-target anticipation 4.2.5.2.17.1. Base Test: The test shall confirm the declared response capability of the system for a cut-out of the lead M1 category vehicle when other vehicles are fully or partially blocking the lane 4.2.5.2.17.1.1. The vehicle cutting out shall perform a full lane change (e.g., 3.5 m lateral displacement) into the adjacent lane to avoid the stationary vehicle target, with the measurement behind the stationary vehicle target indicating that start of the lane change, and the measurement in front of the stationary vehicle target indicating the end of the lane change. 4.2.5.2.17.1.2. The indicated TTC is defined as the TTC of the lead vehicle to the target when the lead vehicle will start the lane change. Indicators are not to be used by the lead vehicle during the manoeuvre. 4.2.5.2.17.1.3. The cutting out vehicle shall not deviate from its defined path by more than ±0.2 m. 4.2.5.2.17.1.4. Parallel to the vehicle in the adjacent lane another vehicle drives at the same speed as the VUT and during the cut-out of the lead vehicle the blocking vehicle performs the following trajectories: (a) continue with the same constant speed (b) braking with the deceleration as the VUT blocking the way of the lane change until the VUT stops or passes the static target 4.2.5.2.17.2. Extended Testing: The test shall demonstrate that the system is not unreasonably changing the control strategy for a cut-out of the lead vehicle. 4.2.5.2.17.2.1. The test shall be executed at least with: (a) A stationary vehicle target of a different type or category, including cyclist and pedestrian (if the system is designed to drive in non-highway roads); (b) The cut-out occurring at less than 3 s TTC of the lead vehicle; (c) Different speeds of the VUT and lead vehicle; (d) Different lateral acceleration of the lead vehicle. (e) Impassable objects |  |
| Annex 4 4.2.5.2.18. and sub | Pedestrian target crossing into the path of the VUT for multi-target anticipation 4.2.5.2.18.1. Base Test: The test shall confirm the declared response capability of the system for a crossing pedestrian target, while other vehicle obstructs the lane of travel and the perception of the pedestrian target. 4.2.5.2.18.1.1 A static target shall be placed in front of the VUT with 50% overlap towards the edge of the road. The VUT shall travel at least 30 kmh/h toward the static target with system-initiated manoeuvring activated. The VUT shall avoid collision with the target if performing a drive around manoeuvre. 4.2.5.2.18.1,2 The pedestrian target shall travel in a straight line perpendicular to the VUT’s direction of travel at a constant speed of 5 km/h +0/-0.4 km/h from maximum 10 cm of the front of the static target. The pedestrian target’s positioning shall be coordinated with the VUT in such a way that the impact point of the pedestrian target on the front of the VUT is on the longitudinal centreline of the VUT if the VUT would remain at the speed when reaching the rear of the static target and does not brake. 4.2.5.2.18.2. Extended testing: The test shall demonstrate that the system is not unreasonably changing the control strategy for a crossing pedestrian target. 4.2.5.2.18.2.1. The test shall be executed at least: (a) A static and pedestrian target of a different type and/or size; (b) A pedestrian target moving at a different, but constant speed; (c) A different angle of the pedestrian target path to the VUT path. |  |
| Annex 4 4.2.5.2.19. and sub | 4.2.5.2.19. VUT crosses the straight path of the vehicle target in an intersection for multi-target anticipation 4.2.5.2.19.1. Base Test: The test shall confirm the declared response capability of the system to recognize and offer right of way for a crossing vehicle target driving straight in an intersection. 4.2.5.2.19.1.1. The VUT shall approach the impact point with another vehicle (passenger car or motorcycle) target in an initial straight line in an intersection from either the near side or far side direction to collide the side of the target vehicle at 25 per cent along the length of the target with the centre front of the VUT. 4.2.5.2.19.1.2. The target shall approach at a speed of up to 60 km/h, depending on the declared system boundaries. The VUT is expected to give right of way. 4.2.5.2.19.1.2. A static target shall be placed in the opposite direction of travel at the same side of the intersection which obstructs the view of the coming moving target but does not interfere with the path of the VUT. 4.2.5.2.19.2. Extended testing: The test shall demonstrate that the system is not unreasonably changing the control strategy for a crossing vehicle target driving straight in an intersection. 4.2.5.2.19.2.1. The test shall be executed at least with: (a) Different target vehicles types or categories; (b) Different overlaps; (c) Different lane positions of the VUT and target vehicles. |  |
| Annex 4 4.2.5.2.13. and sub | 4.2.5.2.13. VUT turns across a path of an oncoming vehicle and cyclist for multi-target anticipation 4.2.5.2.13.1. Base Test: The test shall confirm the declared response capability of the system for an oncoming vehicle target while the VUT is turning in an intersection with multiple targets. 4.2.5.2.13.1.1. The VUT shall approach the impact point with another vehicle (passenger car or motorcycle) target in an initial straight line followed by a turn in an intersection to cross front edges of a target vehicle with a lateral position that gives a 50 per cent overlap of the width of the VUT. |  |
| Annex 4 4.2.5.2.14.1.2. | 4.2.5.2.14.1.2. The first target shall approach at a speed of up to 60 km/h. 4.2.5.2.14.1.2. The second cyclist target shall approach 20km/h+0/-0.4 km/h from maximum 1 m of the road edge. The cyclist target’s positioning shall be coordinated with the target in such a way that the cyclist target reaches the centreline of the road after 2s the rear of the moving target passes the same virtual centreline. 4.2.5.2.13.2. Extended testing: The test shall demonstrate that the system is not unreasonably changing the control strategy for an oncoming vehicle target while the VUT is turning in an intersection. 4.2.5.2.13.2.1. The test shall be executed at least with: (a) Different target vehicle types or categories; (b) Different overlaps; (c) Different lane position of the vehicles and target |  |