Resolution XX

Uniform provisions concerning the fast charging performance of light-duty electrified chargeable vehicles using direct current – Fast Charge Power Curve (FCPC)

Draft text proposal prepared by the Drafting Coordinator. 
Table of contents is based on documents F&SC-04-03e and F&SC-05-02e
Yellow highlighted sections are additional to F&SC-04-03e, the main contents of sections are listed in brackets

Version 1.1 (comments Elena (JRC) and Annika Tidblad (Volvo cars) included)

Version 1.2 (discussions of drafting team meeting #4 and F&SC 9th session included)

Version 1.3 (discussions of drafting team meeting #6 and JRC proposal on EVSE preparation included

- Scope

- Definitions

- Abbreviations

- General requirements

  - Definition of vehicle family	(family criteria)

  - Selection of test vehicle 	(worst-case principle)

- Specifications and tests

5.1  EVCE specifications and requirements	(minimum specifications for charger and supporting equipment)

5.2  Measurement system requirements	(measurement tolerances and sample frequencies, test cell)

5.3  Test vehicle 	(vehicle run-in, eligibility criteria for vehicle and battery age)

5.4  Test preparation requirements	(?)

- Test procedure

6.1 General requirements

6.2 Vehicle preparation	(SoC determination, vehicle relocations)

6.3  Soaking	(soak room requirements)

6.4  Vehicle preconditioning 	(battery preheating, driving before the test)

6.5  Fast-charge testing	(start and end SoC, other criteria to start the test, vehicle conditions and clearance)

- Data processing and KPIs			(applicable corrections of test data, KPI calculation, rounding)

- Conformity of Production (CoP)  		(optional)

- In-Service Conformity 				(optional)

Annexes

- Information concerning the conduct of tests

- Communication and test report

- Tests at low and high ambient temperatures (optional)

SCOPE

This Resolution applies to Pure Electric Vehicles (PEVs) and Off-vehicle Charging Hybrid Electric Vehicles (OVC-HEVs) of vehicle categories M1, M2, N1 and N2 with a reference mass not exceeding 2 840 kg with regard to the determination of their DC fast-charging performance.

- DEFINITIONS 
"Battery" means a rechargeable electrical energy storage system (REESS) installed in an electrified vehicle and used mainly for traction purposes.

"Battery preconditioning system" means a system to condition the battery system prior to the fast-charging event to optimise the charging performance.

"EV charge equipment" (EVCE) means the facility to provide electric energy for charging the vehicle’s REESS.

"Fast charging" means charging the battery of an electric vehicle by an off-board charging system at a DC power reaching at least 50 kW.

"Hybrid electric vehicle" (HEV) means a hybrid vehicle where one of the propulsion energy converters is an electric machine.

"Off-vehicle charging hybrid electric vehicle" (OVC-HEV) means a hybrid electric vehicle that can be charged from an external source.

"Pure electric vehicle" (PEV) means a vehicle equipped with a powertrain containing exclusively electric machines as propulsion energy converters and exclusively rechargeable electric energy storage systems as propulsion energy storage systems.

"State of Charge" (SOC) means the indicated value of the residual capacity in a battery available to be discharged expressed as a percentage and as indicated to the vehicle operator.

"State of certified energy" (SOCE) means the durability performance of the battery at a specific point in the lifetime of the vehicle, determined as a measured or estimated usable battery energy divided by the certified usable battery energy, and expressed as a percentage.

"Usable Battery energy (UBE)" means the energy supplied by the battery from the beginning of the WLTP test procedure used for certification until the applicable break-off criterion of the test procedure is reached.

ABBREVIATIONS

- DC		Direct Current

- EVSE		Electric vehicle charging equipment

- KPI		Key performance indicator

- REESS	Rechargeable electric energy storage system

SOC 		State of Charge

SOCE		State of Certified Energy

- UBE		Usable battery energy

- WLTP		Worldwide harmonised Light vehicles Test Procedure

GENERAL REQUIREMENTS

Family criteria

- Only vehicles that are substantially similar with respect to the following characteristics may be part of the same family:

- Battery type and model dimensions, construction, type of cell (including format and chemistry), nominal voltage, nominal power, cooling system, capacity, etc.);

- Battery management system with regard to DC fast charging, including SOC monitor and the algorithm for estimating the on-board SOC;

- Battery passive and active thermal management system (including battery preconditioning system for fast charging);

- Charging system designhaving a non-negligible influence on fast charging;

- Declared maximum charging power;

- Operation strategy of all systems regarding fast charging;

- On-board battery data availability (voltage, current, SOC, battery [preconditioning flag], etc.);

- Any other characteristics having a non-negligible influence on fast charging.

At the request of the manufacturer, with the approval of the responsible authority and with appropriate technical justification, the manufacturer may deviate from the above criteria for families.

Selection of the test vehicle

- The test vehicle shall be representative for the vehicle model [for type-approval] in terms of its battery, charging system and other relevant components.

- In the case of a vehicle family, a test vehicle shall be chosen from the vehicle family with the expected worst-case fast-charging performance. The manufacturer and the responsible authority shall agree which vehicle test model this shall be, based upon appropriate technical justification.

SPECIFICATIONS AND TESTS

EVCE specifications and requirements

- The manufacturer shall specify the recommended DC charging voltage and maximum DC charging current for the test vehicle.

- The EVCE shall be capable of delivering the required DC voltage to the vehicle as specified by the vehicle manufacturer. The EVCE shall be capable of delivering a continuous DC charging current to the vehicle that is the same or higher than the maximum charging current specified by the vehicle manufacturer. These requirements shall be maintained uninterruptedly throughout the fast-charge test phase of the test procedure.

- [The EVCE shall be capable to communicate the delivered voltage and current taken from the grid with the same accuracy, resolution and sample frequency as specified for the vehicle in paragraph 5.2.]

Measurement system requirements

- The sensors and measurement systems used to determine the fast-charging performance of the vehicle during the fast-charge test procedure as described in Section 6. shall fulfil the following requirements with regard to their accuracy,  resolution and sample frequency:

- Time (s) shall be measured with an accuracy of ±1 second and a resolution of ≤1 second;

- SOC monitor (% of UBE) shall be indicated by the vehicle with a sample frequency of 1 Hz and with a resolution of ≤ 1%;

- Battery voltage (V) shall be measured with an accuracy of ±1% at a sampling frequency of [1 Hz] and a resolution of 1 V;

- Battery current (A) shall be measured with an accuracy of ±1% at a sampling frequency of [1 Hz] and a resolution of 1 A;

- Temperature (°C) shall be measured with an accuracy of ±1 °C at a minimum sampling frequency of 0,033 Hz;

- [Battery preconditioning system indicator (enabled/disabled) at a minimum sampling frequency of 1 Hz.]

- In the case that battery voltage and/or current are provided from the vehicle’s on-board data, the accuracy shall be demonstrated by the manufacturer to the responsible authority in accordance with paragraph 2.2 respectively 3.3 of Appendix 3 to Annex B8 of UN Regulation 154. This demonstration may be omitted if the vehicle has been type-approved according to UN Regulation 154 and this accuracy was already demonstrated for that.

- [The accuracy of the SOC monitor for the vehicle family shall be demonstrated by establishing the relationship between the remaining usable battery energy (kWh) and the corresponding SOC value (%). This shall be done by executing the charge-depleting Type 1 test procedure for OVC-HEVs respectively the Shortened Test Procedure (STP) or the Consecutive Cycles Procedure (CCP) for PEVs as described in Annex B8 of UN Regulation 154 while monitoring the indicated SOC and the remaining share of the UBE simultaneously. At the request of the manufacturer and upon approval by the responsible authority this demonstration may be omitted if the vehicle has been type-approved according to UN Regulation 154 and an approved test report is presented showing the relationship of the indicated SOC and  the remaining share of the UBE during the charge-depleting Type 1 test for OVC-HEVs respectively the Shortened Test Procedure (STP) or the consecutive cycles procedure (CCP) for PEVs.

- The SOC monitor accuracy shall be accepted if the following requirements are met:

- An indicated SOC value of 10% corresponds with a remaining share of the UBE between [9 and 11%];

- The SOC interval of 10 to 80% corresponds to [69%[ of the UBE or higher.]

Test vehicle

- The test vehicle and its battery shall be presented in good technical condition. The test vehicle shall have been run-in for a minimum of 300 kilometres or the declared all electric range (AER) for OVC-HEVs respectively the pure electric range (PER) for PEVs, whichever is the higher. The declared AER for OVC-HEVs is determined in paragraph 4.4.1.1. to Annex B8 of UN Regulation 154; the declared pure electric range (PER) is determined in paragraph 4.4.2. to Annex B8 of UN Regulation 154.

- For the battery of the test vehicle, the durability parameter State of Certified Energy (SoCE), as determined by UN R154 Annex C1, shall at be [98%] or higher.

- The test vehicle shall have an SOC monitor installed which shows the remaining energy in the battery as a percentage of the fully charged battery energy. The vehicle manufacturer shall ensure that the algorithm of the SOC monitor is designed to deliver the most reliable SOC value achievable, taking into account the inherent limitations of the battery system’s measurable parameters.

- If a battery preconditioning system is installed, the vehicle manufacturer shall specify how this system can be manually engaged and disengaged.

- The test vehicle shall be able to communicate the following on-board vehicle parameters (e.g. on the OBD port):

- SOC monitor (%), this shall present the same value as indicated to the vehicle user.

- Battery current (A);

- Battery voltage (V);

- Battery preconditioning flag (enabled/disabled), if applicable, indicating whether the battery preconditioning system for fast charging is engaged.

- If the vehicle is not capable to communicate the battery current and/or voltage it may be instrumented according to the requirements of paragraph 2.1 respectively 3.1 of Appendix 3 to Annex B8 of UN Regulation 154.

- If the vehicle is not capable to communicate a flag indicating the battery preconditioning, it shall be tested with the battery conditioning system disengaged.

Test preparation requirements

- (?)

FAST-CHARGE TEST PROCEDURE

General requirements

- The fast-charge test procedure consists of the following phases that shall be consecutively executed in this order:

- Vehicle preparation

- Soaking

- Preconditioning

- Fast-charge testing

- The start SOC condition shall be an indicated SOC value [equal to or lower than 9%].

- The end SOC condition is an SOC value [equal to or greater than 80%].

- In between the phases of the test procedure the vehicle may be transferred to another area. Vehicle transfer shall be undertaken as quickly as possible without undue delay. The maximum transfer time shall be [30 minutes].

- During transfer, the vehicle shall:

- Not be exposed to temperatures [above 30 °C];

- Not be driven under its own power;

- Not have the battery preconditioning system engaged;

- Not be externally charged or discharged nor have any other energy transfer to the battery.

Vehicle preparation

- The test vehicle may be prepared according to the manufacturer recommendation. This preparation may include charging and discharging of the battery but shall be limited to one full charge and one full discharge. The manufacturer shall specify a target SOC range to be achieved by the end of the preparation phase, ensuring that the start SOC condition required for the test procedure is met following the soak period and vehicle preconditioning. The area where the vehicle is prepared shall not have a temperature above [30 °C].

- [JRC proposal]

Soaking

- Following the vehicle preparation phase, the vehicle shall be soaked for a minimum of 12 hours in a soak area which meets the requirements of paragraph 2.2.2.2. of Annex B6 to UN Regulation 154. The battery preconditioning system shall be disengaged throughout the soaking phase.

- At the option of the manufacturer the soaking phase may be omitted if the battery has a pre-warming system engaged during the preconditioning phase and the vehicle is not stored in a room above [23 °C] in the [12 hours] prior to the preconditioning phase. If this option is selected, it shall be noted in the test report.

- [JRC proposal]

Vehicle preconditioning

- Following the soaking phase, the vehicle may be preconditioned to bring the battery towards the start SOC condition for the fast-charge testing phase.

- The time for preconditioning shall be limited to a maximum of [30 minutes]. The area for preconditioning shall fulfil the same requirements as for the soaking area.

- [During the preconditioning phase the battery preconditioning system may be engaged, if installed.]

- [During the preconditioning phase the vehicle may be driven under its own power on a chassis dynamometer to allow the operation of the battery preconditioning system and/or to bring the battery SOC towards the starting condition for the test]

Fast-charge testing

- Following the vehicle preconditioning phase, the vehicle the fast-charge test shall be executed.

- The area where the fast-charge test takes place shall meet the requirements of paragraph 2.2.2.2. of Annex B6 to UN Regulation 154. At the request of the manufacturer and upon approval by the responsible authority the tolerance of the temperature may be increased to ± 5 °C.

- During the entire test the vehicle shall fulfil the following requirements:

- Be in a parked position with the doors closed and without any occupants;

- Be placed in a position with a minimum clearance around the vehicle’s outer edges in a horizontal plane of 1 meter;

- Be solely connected to the EVCE, without any other energy transfer to the battery;

- Have its powertrain disengaged and its driving mode selector in the position normally applied for parking;

- [Have its battery preconditioning system disengaged (if installed)]

- Have any auxiliary systems that can be switched off by the vehicle user disengaged;

- Have any auxiliary system normally activated during charging by control of the vehicle engaged.

- [When the vehicle is charged at an outside location, it shall additionally fulfil the following requirements:

- The vehicle is protected from direct sunlight;

- The average windspeed is less than or equal to 10 km/h over a 5 second moving average window.]

- The fast-charge test shall commence if the vehicle fulfils the SOC start condition and the above-mentioned requirements are met, but no later than [15] minutes after either the end of the preconditioning phase or, if the preconditioning phase was conducted elsewhere, the vehicle transfer.

- During the fast-charge testing phase, all parameters specified in paragraph 5.2 (measurement system requirements) shall be recorded at the indicated accuracy, resolution and measurement sample frequency from the first moment that the current to the battery reaches [1 A].

- The fast-charge test shall end as soon as the end SOC condition has been reached.

Data processing and KPIs

- After completion of the fast-charge test, the following KPIs shall be determined from the recorded measurement data:

- The time in seconds from the first moment that the SOC monitor indicates [10%] to the first moment that the SOC monitor indicates [80%];

- Over the time interval determined in (a), the highest maximum charging power averaged over a 30 second interval;

- Over the time interval determined in (a), the average charging power;

- Over the time interval determined in (a), the average charging efficiency;

- Over a time interval of 600 seconds, starting from the first moment that the SOC monitor indicates [10%], the [recharged electric range or recharged electric energy in kWh/10 min];

- Over a time interval of 600 seconds, starting from the first moment that the SOC monitor indicates [10%], the average charging speed as recharged electric range per hour;

- Over a time interval of 600 seconds, starting from the first moment that the SOC monitor indicates [10%], the average charging efficiency;

- Over a time interval of 600 seconds, starting from the first moment that the SOC monitor indicates [10%], the recharged SOC.

- Intermediate steps in the calculations shall not be rounded unless intermediate rounding is required. Final rounding and units of KPIs are as follows:

- Time-based KPIs are expressed in minutes and rounded to one decimal point;

- Power-based KPIs are expressed in kW and rounded to the nearest whole number;

- Efficiency-based KPIs are expressed as a percentage and rounded to one decimal point;

- Electric range-based KPIs are expressed in km and rounded to the nearest whole number;

- SOC-based KPIs are expressed as a percentage and rounded to the nearest whole number.

- When the digit immediately to the right of the last place to be retained is less than 5, that last digit retained shall remain unchanged.

- Example:

- If a result is 12,34 minutes but only one decimal point has to be retained, the final result shall be 12,3 minutes.

- When the digit immediately to the right of the last place to be retained is greater than or equal to 5, that last

- digit retained shall be increased by 1.

- Example:

- If a result is 12,35 minutes but only one decimal point has to be retained, the final result shall be 12,4 minutes

- At the option of the manufacturer, KPIs may be declared at a different value than which was calculated by this paragraph, but only towards a worse fast-charging performance.

- Example:

- If the time interval measured over the SOC interval from 10-80% is 631 seconds and therefore rounded to 10,5 minutes, the associated KPI may be declared at 10,5 minutes or higher.

CONFORMITY OF PRODUCTION (optional)

- [PM]

IN-SERVICE CONFORMITY (optional)

- [PM]

- ANNEXES

- Information concerning the conduct of tests

[PM]

Communication and test report

[PM]

Tests at low and high ambient temperatures (optional)

[PM]