Proposal (DRAFT)

Colour code:

Purple: agreed during meeting

Green: outcome of preparation group meetings

Orange: OICA feedback to preparation group proposal

Comments from Japan:

General：

We hope to reach an agreement on a document that incorporates realistic modifications based on "WD GRBP/2025/29," while respecting the decisions made regarding the Subgroup and the agreements reached at the 82nd GRBP.

02 Transitional provision: Supporting the WD proposal (24 months)

Based on the results of the monitoring and CP test campaigns, there is no need to impose strict restrictions on the extension approval for the exiting type approved vehicles.

03 Limit curve: Supporting the WD proposal

If any modifications are made, they should be within the acceptable range for the industries.

We believe that adopting the Quick & Interim solution will be a major step toward dramatically eliminating the non-regulated area.

The following amendments are intended to introduce EVs and HEVs into Annex 7 as agreed during the 82nd session of GRBP in Geneva:

Paragraph 2.24., amend to read:

"2.24.	Table of Symbols

…

| nanchor,κ | 1/min | Annex 7 | 3.1. | reported vehicle engine speed for gear ratio i from Annex 3 |
| --- | --- | --- | --- | --- |
| vanchor,κ | km/h | Annex 7 | 3.1. | reported vehicle test speed for gear ratio i at BB’ from Annex 3; value to be reported and used for calculations to the first decimal place |
| aanchor,κ | m/s² | Annex 7 | 3.1. | reported vehicle acceleration for gear ratio i from Annex 3; value to be reported and used for calculations to the first decimal place |
| LASEP κ,j | dB(A) | Annex 7 | 3.3. | sound pressure level calculated for a gear ratio i and for a speed (engine or vehicle) to reflect the estimated linear sound level increase regarding the engine speed (ICEs) or the vehicle speed (EVs) |
| ΔLASEP, j | dB | Annex 7 | 3.4. | estimated delta sound pressure level for ASEP to reflect the relation of the measured performance versus performance at the anchor point; value to be used for calculations to the first decimal place |
| Lκj | dB(A) | Annex 7 | 3.4.3.5. | sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place |

…"

Add new paragraph 2.29., to read

| PREPARATION GROUP | OICA |
| --- | --- |
| "2.29. "Active Sound Source" means a physical part installed on the vehicle - which is powered, and - one of whose purposes is to emit sound to the exterior of the vehicle." | Not needed |

Add new paragraph 2.30., to read

| PREPARATION GROUP | OICA |
| --- | --- |
| "2.30. "Enhanced Exterior Sound (EES)" means any sound - generated by one or more Active Sound Sources, and - which is not subject to another UN Regulation." | "2.29. "Enhanced Exterior Sound (EES)" means sound - generated by active systems installed on a vehicle enhancing the vehicle’s sound emission and - that is not subject to requirements of other UN Regulations. ” |

Add new paragraph 2.31., to read

"2.31.	"Vehicle Master Control Switch" means the device by which the vehicle’s on-board electronics system is brought, from being switched off, as in the case where a vehicle is parked without the driver being present, to normal operation mode (i.e. the vehicle status is "ready to drive")."

Paragraph 6.2., amend 6.2. to read:

| PREPARATION GROUP | OICA |
| --- | --- |
| "6.2. Specification regarding sound levels If the vehicle has different modes which affect sound emissions, the vehicle shall comply with the requirements of this Regulation in all modes.” | Not supported Is already implemented in Annex 7 Paragraph 2.1.: 2.1. Measurement instruments and condition of measurements Unless otherwise specified, the measurement instruments, the conditions of the measurements and the condition of the vehicle are equivalent to those specified in Annex 3, paragraphs 1. and 2. If the vehicle has different modes which affect sound emission, all modes shall comply with the requirements in this annex. In the case where the manufacturer has performed tests to prove to the approval authority compliance with the above requirements, the modes used during those tests shall be reported in a test report. |

Paragraph 6.2., amend to read:

“6.2.	Specifications regarding sound levels

If the vehicle has different modes which affect sound emissions, the vehicle shall comply with the requirements of this Regulation in all modes.”

Paragraph 6.2.3., amend to read:

"6.2.3.	Additional sSound eEmission pProvisions (ASEP)

The Requirements of Annex 7Additional Sound Emission Provisions (ASEP) apply only to vehicles of categories M1 and N1 equipped with an internal combustion engine.

Vehicles are deemed to fulfil the requirements of Annex 7, if the vehicle manufacturer provides technical documents to the type approval authority showing, that the difference between maximum and minimum engine speed of the vehicles at BB' for any test condition inside the ASEP control range (as defined in paragraph 2.3. of Annex 7 to of this Regulation (including Annex 3 conditions) does not exceed 0.15 x S. This article is intended especially for non-lockable transmissions with variable gear ratios (CVT).

Vehicles are exempted from ASEP if one of the following conditions is fulfilled:

(a)	For vehicles of category N1, if the engine capacity does not exceed 660 ccm and the power-to-mass ratio PMR calculated by using the technically permissible maximum laden mass does not exceed 35.

(b)	For vehicles of category N1, if the payload is at least 850 kg and the power-to-mass ratio calculated by using the technically permissible maximum laden mass does not exceed 40.

(c) 	For vehicles of category N1 or M1 derived from N1, if the technically permissible maximum laden mass is greater than 2.5 tons and the R-point height is greater than 850 mm from the ground and the power- to-mass ratio calculated by using the technically permissible maximum laden mass does not exceed 40.

The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type-approval test set out in Annex 3 and Annex 7 was carried out, shall not deviate from the test result in a significant manner.

Any electric sound enhancement system for the purpose of the exterior sound emission emission of EES shall be activated during the type-approval test."

Add new paragraph 6.2.4 and sub paragraphs, to read:

"6.2.4	Specifications for the sound emissions of EVs and HEVs regarding the emission of EES, when electrically propelled only.

6.2.4.1.	Activation, deactivation and control of the emission of EES

When the vehicle is switched on by the vehicle master control switch, the emission of  EES shall be deactivated.

Any activation of the emission of an EES shall require an intentional action of the driver.

The emission of EES shall be deactivatable at any time without affecting the performance and other functions of the vehicle. .

When a vehicle is manually driven, the control of the emissions of EES shall be easily accessible to the driver from the driving seat.

When a vehicle is automated driven, the emission of EES shall be deactivated.”

6.2.4.2.	Sound limits not to be exceeded:

In the speed range from 0 km/h included to 20 km/h included: The sound emitted by the vehicle shall comply with the requirements of UN-Regulation No. 138 paragraph 6 in all driving conditions.

In the speed range from 20 km/h included to 80 km/h included:

The sound emitted by the vehicle shall comply with the requirements of ANNEX 3 and ANNEX 7 of this Regulation.

In the speed range strictly greater than 80 km/h: The sound emitted by the vehicle shall comply with the specific recommendations of GRB 68-03 for EVs and HEVs. “

Paragraph 6.2.3.3. (former), renumber:

| PREPARATION GROUP | OICA |
| --- | --- |
| "6.2.3.4. In applying for type approval, the manufacturer…" | "6.2.3.3.4. In applying for type approval, the manufacturer…" |

Add three new paragraphs 11.18., 11.19. and 11.20., to read:

"11.18. 	Supplement [11] does not apply to existing UN type approvals and their extensions, granted prior to the date of entry into force of Supplement [11].

11.19.	As from the official date of entry into force of Supplement [11] to the 03 series of amendments to this Regulation, no Contracting Party applying this UN Regulation shall refuse to grant or refuse to accept UN type approvals according to Supplement [11] to the 03 series of amendments to this UN Regulation.

| PREPARATION GROUP | OICA |
| --- | --- |
| 11.20. Until [12-18-24] months after the date of entry into force of the Supplement [11] to the 03 series of amendments to this UN Regulation, Contracting Parties applying this UN Regulation can continue to grant extensions to existing UN type approvals to the 03 series of amendments to this UN Regulation, granted prior to the date of entry into force of Supplement [11], without taking into account the provisions of Paragraph 6.2.3.3. of Supplement [11] " | 11.[20]. Until 24 months after the date of entry into force of the Supplement [11] to the 03 series of amendments to this UN Regulation, Contracting Parties applying this UN Regulation can continue to grant extensions to existing UN type approvals to the 03 series of amendments to this UN Regulation, granted prior to the date of entry into force of Supplement [11], without taking into account the provisions of Paragraph 6.2.3.3.1., 6.2.3.3.2. and 6.2.3.3.5. of Supplement [11]. " |

Annex 1 – Appendix 1, add a new paragraph 1.1.10. to read:

"1.1.10	System(s) generating EES, Make and Type: 	"

Annex 1 – Appendix 2, add new paragraph 7.2.  to read:

"7.2.	Details of any system(s) generating EES (e.g., number and position of actuators and active sound sources) including a description of all possibilities to activate and deactivate the EES 	"

Annex 3, paragraph 2.2.3.3., amend to read:

| PREPARATION GROUP | OICA |
| --- | --- |
| "2.2.3.3. Active Sound Systems generating EES Any system generating EES, subject to paragraph 6.2.3. and Annex 7 of this Regulation, either for noise control, or sound enhancement, shall operate as foreseen by the vehicle manufacturer and not be interfered with during the measurements." | "2.2.3.3. Active Sound Systems generating EES Any system generating EES, either for noise control, or sound enhancement, shall operate as foreseen by the vehicle manufacturer and not be interfered with during the measurements." |

Annex 3, paragraph 3.1.2.1.4.3., amend to read:

| PREPARATION GROUP | OICA |
| --- | --- |
|  | "3.1.2.1.4.3. Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV) The gear selector position for forward driving shall be used. The acceleration value awot test shall be calculated as defined in paragraph 3.1.2.1.2.1. The achieved acceleration awot test shall be greater or equal to aurban. If possible, the manufacturer shall take measures to avoid an acceleration value awot test greater than 2.0 m/s². Table 1 in Appendix to Annex 3 provides examples for valid measures to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. The achieved acceleration awot_test , but not more than 2.0 m/s², is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead awot ref.[For Annex 7 calculations, the true achieved acceleration shall be used.] |

Annex 7, add a new paragraph 1.1., to read:

| PREPARATION GROUP | OICA |
| --- | --- |
| "1.1. Tests performed in pure electric mode shall be analysed with Analysis method 1 described in Paragraph 3 of this annex (Slope assessment method)." | "1.1. Vehicles when pure-electrically propelled [with propulsion technologies other than combustion engines] shall comply with Analysis method 1 (Slope assessment method) only. [This shall apply as well to hybrid electric vehicles when tested without an operating combustion engine. Manufacturers of vehicles, according to this paragraph, shall provide a statement of compliance for ASEP, confirming that the vehicle complies with the specifications described in paragraph 3.6. of this Annex, when tested with any operation condition within the control range in paragraph 2.3.]" |

Annex 7, paragraph 2.3., amend to read:

"2.3.	Control rRange

The ASEP requirements apply to every gear ratio κ that leads to test results within the control range as defined below.

| PREPARATION GROUP | OICA |
| --- | --- |
| The control range is specified as: For tests in pure electric mode; For all other tests. Vehicle speed vAA_ASEP; vAA ≥ 20 km/h; vAA ≥ 20 km/h. Vehicle acceleration aWOT_ASEP; aWOT ≤ 5.0 m/s2; aWOT ≤ 5.0 m/s2. Engine speed nBB_ASEP; not applicable; nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest. Vehicle speed vBB_ASEP for vehicles tested in Annex 3 with. - locked gear; vBB ≤ 80 km/h; If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h.. - non locked gears; vBB ≤ 80 km/h; For vehicles tested in non-locked transmission conditions the maximum vehicle speed is 80 km/h.. Gear selection; only if applicable, e.g. for hybrid electric vehicles: gears κ ≤ gear i as determined in Annex 3; gears κ ≤ gear i as determined in Annex 3. Transmission condition for vehicles tested in Annex 3 with. - locked gear; Geari, geari-1,…; Geari, geari-1,…. - non locked gears; Non locked gears; Non locked gears. Vehicle speed VAA_ASEP: vAA ≥ 20 km/h Vehicle acceleration aWOT_ASEP: aWOT ≤ 5.0 m/s2 Engine speed nBB_ASEP: nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest For tests in pure electric mode, the manufacturer shall take measures to achieve an acceleration aWOT_ASEP within the acceleration control range. Table 1 in Appendix 1 to Annex 3 provides examples of valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by a manufacturer for the above-mentioned purposes shall be documented in the test report. Vehicle speed VBB_ASEP: If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h. For vehicles tested in non-locked transmission conditions, the maximum vehicle speed is 80 km/h. Gears κ ≤ gear i as determined in Annex 3 Transmission conditions: Annex 3 gear selection; Annex 7 gear selection. Locked; Geari, geari-1,…. Non-locked; Non-locked. | The control range is specified as: For vehicles according to Annex 7, paragraph 1.1.; For all other vehicles. Vehicle speed vAA_ASEP; vAA ≥ 20 km/h; vAA ≥ 20 km/h. Vehicle acceleration aWOT_ASEP; aWOT ≤ 5.0 m/s2; aWOT ≤ 5.0 m/s2. Engine speed nBB_ASEP; not applicable; nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest. Vehicle speed vBB_ASEP for vehicles tested in Annex 3 with. - locked gear; vBB ≤ 80 km/h; If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h.. - non locked gears; vBB ≤ 80 km/h; For vehicles tested in non-locked transmission conditions the maximum vehicle speed is 80 km/h.. Gear selection; only if applicable, e.g. for hybrid electric vehicles: gears κ ≤ gear i as determined in Annex 3; gears κ ≤ gear i as determined in Annex 3. Transmission condition for vehicles tested in Annex 3 with. - locked gear; Geari, geari-1,…; Geari, geari-1,…. - non locked gears; Non locked gears; Non locked gears. Vehicle speed VAA_ASEP: vAA ≥ 20 km/h Vehicle acceleration aWOT_ASEP: aWOT ≤ 5.0 m/s2 Engine speed nBB_ASEP: nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤0.9 * S, whichever is the lowest For vehicles subject to paragraph 1.1. of this Annex, the manufacturer shall take measures to achieve an acceleration aWOT_ASEP within the acceleration control range. Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. Vehicle speed VBB_ASEP: If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h. For vehicles tested in non-locked transmission conditions, the maximum vehicle speed is 80 km/h. Gears κ ≤ gear i as determined in Annex 3 Transmission conditions: Annex 3 gear selection; Annex 7 gear selection. Locked; Geari, geari-1,…. Non-locked; Non-locked. " |

Annex 7, paragraph 3.1., amend to read:

"3.1.	Determination of the anchor point

The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows:

In the case the test has been carried out with two gear ratios:

Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i;

nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3;

In the case the test has been carried out in a single gear:

Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test;

nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3;

| PREPARATION GROUP | OICA |
| --- | --- |
| For test in pure electric mode: Lanchor is the higher sound pressure level of Lwot of left and right sides of gear ratio selected for the test; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3, but limited to 60 km/h for further calculation; aanchor is the measured acceleration and is not limited to 2.0 m/s² as required in paragraph 3.1.2.1.4.3. of Annex 3 for the determination of the kp-factor, and is the average of awot,test reported from Annex 3, as defined in paragraph 3.1.2.1.2. of Annex 3.” | For vehicles subject to paragraph 1.1. of this Annex: Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test; vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3, but limited to 60 km/h for further calculation. aanchor is the measured acceleration and is not limited to 2.0 m/s² as required in paragraph 3.1.2.1.4.3. of Annex 3 for the determination of the kp-factor, and is the average of awot,test reported from Annex 3, as defined in paragraph 3.1.2.1.2. of Annex 3.” |

Annex 7, paragraph 3.2.2., amend to read:

"3.2.2.	Slope of the regression line for each gear ratio κ

The slopeκ of a particular gear for the further calculation is the derived result of the calculation in paragraph 3.2.1. rounded to the first decimal place, but not higher than 5 dB(A)/1,000 min-1.

In case of non-locked conditions, if Slopeκ  < 0, the selected transmission setup is not valid. In that case the Lurban-Assessment as specified in paragraph 4. shall be applied.

| PREPARATION GROUP | OICA |
| --- | --- |
| For tests in pure electric mode, Slopeκ shall be set to a constant value of 0,25 dB/(km/h) for further calculation." | For vehicles subject to paragraph 1.1. of this annex, Slopeκ shall be set to a constant value of 0,25 dB/(km/h) for further calculation." |

Annex 7, paragraph 3.3., amend to read:

"3.3.	Calculation of the linear sound level increase expected for each measurement

The sound level LASEP,κj for measurement point j and gear ratio κ shall be calculated using the engine speeds measured for each measurement point, using the slope specified in paragraph 3.2. above to the specific anchor point for each gear ratio.

For nBB_κ,j ≤ nanchor:  LASEP κ,j =Lanchor+Slopeκ-Y×(nBB_κ,j- nanchor)1,000

For nBB_κ,j > nanchor: LASEP κ,j=Lanchor+Slopeκ+Y×(nBB_κ,j- nanchor)1,000

| PREPARATION GROUP | OICA |
| --- | --- |
| Where Y= 1 dB(A) /1,000 min-1 For tests in pure electric mode: The sound level LASEP,κj for measurement point j and gear ratio κ shall be calculated using the vehicle speeds measured for each measurement point, using the constant slope specified in paragraph 3.2.2. above to the specific anchor point [for each gear ratio]. For vBB_κ,j ≤ vanchor: LASEP κ,j=Lanchor+Slopeκ-Y×(vBB_κ,j- vanchor) For vBB_κ,j > vanchor: LASEP κ,j=Lanchor+Slopeκ×vBB_κ,j- vanchor Where Y= 0,05 dB/(km/h)" | Where Y= 1 For vehicles subject to paragraph 1.1. of this Annex: The sound level LASEP,κj for measurement point j and gear ratio κ shall be calculated using the vehicle speeds measured for each measurement point, using the constant slope specified in paragraph 3.2.2. above to the specific anchor point for each gear ratio. For vBB_κ,j ≤ vanchor: LASEP κ,j=Lanchor+Slopeκ-Y×(vBB_κ,j- vanchor) For vBB_κ,j > vanchor: LASEP κ,j=Lanchor+Slopeκ×vBB_κ,j- vanchor Where Y= 0,05" |

Annex 7, add new paragraph 3.4., to read:

| PREPARATION GROUP | OICA |
| --- | --- |
| "3.4. Calculation of the sound level increase regarding performance For tests in pure electric mode, the sound level ΔLASEPj shall be calculated: For vBB, test, j × awot, test, j≤ vanchor× aanchor: ΔLASEPj =0 dB For vBB test,j × awot, test, κ,j> vanchor× aanchor : ΔLASEPj = 2-4-6-82-4-6-8dB ×log10 vBB test,j ×awot,test,jvanchor× aanchor For all other tests the sound level ΔLASEPj is set to 0 dB." | "3.4. Calculation of the sound level increase regarding performance For vehicles subject to paragraph 1.1. of this annex, the sound level ΔLASEPj shall be calculated: For vBB, test, j × awot, test, j≤ vanchor× aanchor: ΔLASEPj =0 dB For vBB test,j × awot, test, κ,j> vanchor× aanchor : ΔLASEPj =8dB ×log10 vBB test,j ×awot,test,jvanchor× aanchor For all other vehicles the sound level ΔLASEPj is set to 0 dB." |

Annex 7, paragraph 3.4. (former), renumber:

"3.45.	Additional Samples"

Annex 7, paragraph 3.5. (former), renumber and amend to read:

"3.56.	Specifications

Every individual sound measurement shall be evaluated.

The sound level of every specified measurement point shall not exceed the limits given below:

Lκj - ΔLASEP,j ≤ LASEP,κ.j + x

With:

x = 3 dB(A) + limit value - Lurban	for vehicle  tested with non-locked transmission  conditions

x = 2 dB(A) + (limit value³ - Lurban),	 of Annex 3 for all other vehicles

and only for vehicles

- tested with non-locked transmission conditions given by multiple gear ratios or

- [having multiple electric propulsion sources or]

| PREPARATION GROUP | OICA |
| --- | --- |
| - [(EV only) having an Lcrs_rep greater than Lwot_rep of Annex 3]: x = 3 dB + (limit value - Lurban) | - (EV only) having an Lcrs_rep greater than Lwot_rep of Annex 3: x = 3 dB + (limit value - Lurban) of Annex 3 |

If at any point the measured sound level at a point exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification.

| PREPARATION GROUP | OICA |
| --- | --- |
|  | Annex 9, paragraph 1., amend to read: "1. General The Real Driving Additional Sound Emission Provisions (RD-ASEP) apply only to vehicles of categories M1 and N1 equipped with: - an internal combustion engine (ICE) for propulsion of the vehicle, or - any other propulsion technology fitted with an exterior sound enhancement a system generating EES." Annex 9, paragraph 2.2., delete. Annex 9, paragraphs 2.3.,2.4. and 2.5., renumber and amend to read: "2.32. "Deceleration" is defined as means the vehicle deceleration stipulated by the release of the acceleration control unit only, without any driver applied braking (service brake, retarder, parking brake, etc…)." "2.43. "Performance" is defined as means the product of acceleration and vehicle speed as quantity of the achieved vehicle performance." "2.54. "Power trains Powertrain" is defined as means a propulsion system as combination of the energy storage system, the energy supply system and the powertrain according to the UN Mutual Resolution No. 2 (for example PEV, HEV, FCHEV)." |

Justifications

- ….

For example, audible warning signals (according to UN Regulation No. 28), alarms (according to UN Regulation No. 116), safety sound (according to UN Regulation No. 138) and reverse warning signals (according to UN Regulation No. 165) are not EES.

as already defined in UN Regulation No. 141, No. 171151 and others.

See recommendations provided by informal document GRB-68-03 in its latest version as guidance for technical interpretation. The document can be found in https://unece.org/documents-reference-only-0.

Example: changing a mode or operating a switch.

The Analysis method 2 (Lurban assessment) and the Reference Sound Assessment are not applicable for these vehicles.

As applicable for the approved  type of vehicle

As applicable for the approved  type of vehicle

As applicable for the approved  type of vehicle