| ORG | Paragraph # | Type of comment1 | Original text (Consolidated version prepared by OICA) | Proposed change | Comments / Justification |
| --- | --- | --- | --- | --- | --- |
|  | 2.21. | ge | "Pressure Relief Device (PRD)" means a device that, when activated under specified performance conditions, is used to release hydrogen from a pressurized system and thereby prevent failure of the system. It can include first and second pressure relief devices as a reclosing or non-reclosing type (e.g., safety relief valve and burst disc) as well as boil off valve. | "Pressure Relief Device (PRD)" means a device that, when activated under specified performance conditions, is used to release hydrogen from a pressurized system and thereby prevent failure of the system. It canshould include first and second pressure relief devices as a reclosing or non-reclosing type (e.g., safety relief valve and burst disc) as well as boil off valve. | The LHSS according to this regulation is designed with the aim of using both the first PRD and second PRD. Therefore, “should” should be used instead of “can.” |
|  | 5. | ed | The boundaries of the LHSS are defined by the interfaces which can isolate the stored liquefied (and/or gaseous) hydrogen from the remainder of the fuel system and the environment. All components located within this boundary are subject to the requirements defined in this paragraph. - Figure 1 shows a typical LHSS. The closure devices shall include, at a minimum, the following functions, which may be combined: (a) Shut-off function; (b) Boil-off control; (b) Pressure relief function. | The boundaries of the LHSS are defined by the interfaces which can isolate the stored liquefied (and/or gaseous) hydrogen from the remainder of the fuel system and the environment. All components located within this boundary are subject to the requirements defined in this paragraph. - Figure 1 shows a typical LHSS. The closure devices shall include, at a minimum, the following functions, which may be combined: (a) Shut-off function; (b) Boil-off control; (bc) Pressure relief function. | “(b) Pressure relief function.” should be “(c) Pressure relief function.” |
|  | 5.4. | ge | 5.4. Requirements for pressure relief device(PRD and shut-off device The PRDs and shut-off devices shall be tested and type-approved in accordance with Part II of this Regulation and produced in conformity with the approved type. Retesting of the LHSS is not required if alternative closure devices are provided having comparable function, fittings, materials, strength and dimensions, and satisfy the condition above. | 5.4. Requirements for boil-off valve, pressure relief device(PRD and shut-off device The boil-off valve, PRDs and shut-off devices shall be tested and type-approved in accordance with Part II of this Regulation and produced in conformity with the approved type. Retesting of the LHSS is not required if alternative closure devices are provided having comparable function, fittings, materials, strength and dimensions, and satisfy the condition above. [In addition, requirements for the boil-off valve should be specified in Part II (Specifications of the specific components) and in Annex 4.] | The boil-off valve is not included in 5.4. Therefore, if the type of the boil-off valve is changed, the exemption from retesting the entire LHSS based on 5.4 cannot be applied. The boil-off valve should also be specified in Part II and Annex 4 as a specific component and handled in the same way as the PRD and the shut-off device. |
|  | 5.1.1. | ge | 5.1.1. Proof pressure The inner container and the pipe work situated between the inner container and the outer vacuum jacket is pressurized to a pressure ptest ≥ 130 per cent (MAWP + 0.1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | 5.1.1. Proof pressure The inner container and the pipe work situated between the inner container and the outer vacuum jacket is pressurized to a pressure ptest ≥ 130 per cent (MAWP + 0.1 MPa) in accordance with the test procedure in Annex 3, paragraph 1.1. without visible deformation, degradation of container pressure, or detectable leakage. | (1) The number of proof pressure test samples should be clarified. (2) In the term “degradation of container pressure”, the word “container” is not consistent with the actual test article, which is surely “the inner container and the pipe work situated between the inner container and the vacuum jacket.” If it simply means a pressure decrease, “container” is not needed. |
|  | 5.2.3. | ge | 5.2.3. Vacuum loss The vacuum loss test is performed on an LHSS equipped with all components as described in paragraph 5. (Figure 1). The test is performed on a system filled with liquid hydrogen in accordance with the test procedure in Annex 3, paragraph 2.3. and shall demonstrate that both first and second pressure relief devices limit the pressure to the values specified in Annex 3, paragraph 2.3. if vacuum pressure is lost. | 5.2.3. Vacuum loss The vacuum loss test is performed on an LHSS equipped with all components as described in paragraph 5. (Figure 1). The test is performed on a system filled with liquid hydrogen in accordance with the test procedure in Annex 3, paragraph 2.3. and shall demonstrate that both first and second pressure relief devices limit the pressure to the values specified in Annex 3, paragraph 2.3. if vacuum pressure is lost. | Since the definition of LHSS in 2.15. already includes all components, the term is unnecessary. |
|  | Annex 3 1.1. | ge | 1.1. Proof pressure test The inner container and the pipe work situated between the inner container and the outer vacuum jacket shall withstand an inner pressure test at room temperature according to the following requirements. 1.1.1. The test pressure ptest is defined by the manufacturer and shall fulfil the following requirements: ptest ≥ 130 per cent (MAWP + 0.1 MPa) (a) For metallic containers, either ptest is equal to or greater than the maximum pressure of the inner container during [vacuum loss test] (as determined in Annex 3, paragraph 2.3.) or the manufacturer proves by calculation that at the maximum pressure of the inner container during fault management no yield occurs; [(b) For non-metallic containers, ptest is equal to or greater than the maximum pressure of the inner container during [vacuum loss test] (as determined in Annex 3, paragraph 2.3.).] 1.1.2. The test is conducted according to the following procedure: (a) The test is conducted on the inner storage container and the interconnecting pipes pipe work between inner container and vacuum jacket before the outer vacuum jacket is mounted; (b) The test is either conducted hydraulically with water or a glycol/water mixture, or alternatively with gas. The container is pressurized to test pressure ptest at an even rate and kept at that pressure for at least 10 minutes; (c) The test is done at ambient temperature. In the case of using gas to pressurize the container, the pressurization is done in a way that the container temperature stays at or around ambient temperature. The test is passed successfully if, during the first 10 minutes after applying the proof pressure, no visible permanent deformation, no visible degradation in the container pressure and no visible leakage are detectable. | 1.1. Proof pressure test The inner container and the pipe work situated between the inner container and the outer vacuum jacket shall withstand an inner pressure test at room temperature according to the following requirements. 1.1.1. The test pressure ptest is defined by the manufacturer and shall fulfil the following requirements: ptest ≥ 130 per cent (MAWP + 0.1 MPa) (a) For metallic containers, either ptest is either equal to or greater than the maximum pressure of the inner container during [vacuum loss test] (as determined in Annex 3, paragraph 2.3.) or the manufacturer proves by calculation that at the maximum pressure of the inner container during fault management no yield occurs; [(b) For non-metallic containers, ptest is equal to or greater than the maximum pressure of the inner container during [vacuum loss test] (as determined in Annex 3, paragraph 2.3.).] 1.1.2. The test is conducted according to the following procedure: (a) The test is conducted on the inner storage container and the interconnecting pipes pipe work between inner container and vacuum jacket before the outer vacuum jackettest article is mounted; (b) The test is either conducted hydraulically with water or a glycol/water mixture, or alternatively with gas. The containertest article is pressurized to test pressure ptest at an even rate and kept at that pressure for at least 10 minutes; (c) The test is done at ambient temperature. In the case of using gas to pressurize the containertest article, the pressurization is done in a way that the container temperature stays at or around ambient temperature. The test is passed successfully if, during the first 10 minutes after applying the proof pressure, no visible permanent deformation, no visible degradation in the container pressure and no visible leakage are detectable. | (1) The term “For metallic containers,” is unnecessary due to the deletion of (b). (2) In 1.1.2. (a)–(c), the term for the test target is not consistent. Since 1.1. defines the test article as “the inner container and the pipe work situated between the inner container and the vacuum jacket,” the same wording should be used throughout, or it should be replaced with “the test article.” (3) In 1.1.2., the term “no visible degradation in the container pressure” should not include the word “container”, as it is not needed for the same reason as comment (2) to 5.1.1. |
|  | Annex 3 1.3. | ge | 1.3. Baseline pressure cycle life The Ccontainer is pressure cycled with a number of cycles at least three times the number of possible full pressure cycles (from the lowest to highest operating pressure) for an expected on-road performance. The number of pressure cycles is defined by the manufacturer under consideration of operating pressure range, size of the storage and, respectively, maximum number of refuellings and maximum number of pressure cycles under extreme usage and storage conditions. Pressure cycling is conducted between atmospheric pressure and MAWP at liquid nitrogen temperatures, e.g. by filling the container with liquid nitrogen to certain level and alternately pressurizing and depressurizing it with (pre-cooled) gaseous nitrogen or helium. | 1.3. Baseline pressure cycle life The inner Ccontainer is pressure cycled with a number of cycles at least three times the number of possible full pressure cycles (from the lowest to highest operating pressure) for an expected on-road performance. The number of pressure cycles is defined by the manufacturer under consideration of operating pressure range, size of the storage and, respectively, maximum number of refuellings and maximum number of pressure cycles under extreme usage and storage conditions. Pressure cycling is conducted between atmospheric pressure and MAWP at liquid nitrogen temperatures, e.g. by filling the container with liquid nitrogen to certain level and alternately pressurizing and depressurizing it with (pre-cooled) gaseous nitrogen or helium. | In 2.6. Definitions., the term “container” includes the inner container, the vacuum jacket, and the supporting structure between them. Therefore, the test article in this requirement should be the inner container. |
|  | Annex 3 2.2. | ed ge | 2.2. Boil-off discharge rate 2.2.1 The boil-off discharge rate test shall be performed directly after the boil-off test in paragraph 2.1 under the same conditions. In case a boil-off converter is used it shall be preconditioned until [the nominal operating conditions as per the manufacturer’s specifications or steady-state conditions] are reached before conducting the measurement. The inner container pressure is kept at boil-off pressure during the entire test. The hydrogen boil-off discharge rate shall be measured by an appropriate method (e.g., suitable hydrogen detector and mass flow meter) at the outlet of the boil-off valve, the boil-off vent line or the boil-off converter (if used) as applicable. The test shall at least last for 1 hour after stabilisation. 2.2.2 The boil-off discharge rate shall be recorded during the entire test. | 2.2. Boil-off discharge rate 2.2.1. The boil-off discharge rate test shall be performed directly after the boil-off test in paragraph 2.1. under the same conditions. In case a boil-off converter is used it shall be preconditioned until [the nominal operating conditions as per the manufacturer’s specifications or steady-state conditions] are reached before conducting the measurement. The inner container pressure is kept at boil-off pressure during the entire test. The hydrogen boil-off discharge rate shall be measured by an appropriate method (e.g., suitable hydrogen detector and mass flow meter) at the outlet of the boil-off valve, the boil-off vent line or the boil-off converter (if used) as applicable. The test shall at least last for 1 hour after stabilisation. 2.2.2. The boil-off discharge rate shall be recorded during the entire test. The test is passed successfully if the passing criteria described in paragraph 5.2.2. of this Regulation is fulfilled. | (1) “2.2.1”, “2.2.2”, and “paragraph 2.1” should be “2.2.1.”, “2.2.2.”, and “paragraph 2.1.”, respectively. |
|  |  |  |  |  | (2) In 2.2.2., the acceptance criteria for this test should be specified. |
|  | Annex 3 3.3. | ge | The permissible fault range for steel containers is as follows: (a) If a safety valve is used as second pressure relief device, the pressure inside the container does not exceed 136 per cent of the MAWP of the inner container; (b) If a burst disc with a relief external to the vacuum space is used as a second pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP of the inner container; (c) If a burst disc with a relief internal to the vacuum space is used as second pressure relief device, the pressure inside the container is limited to 150 per cent of the MAWP + 0.1 MPa of the inner container. [For other metallic materials, an equivalent level of safety shall be demonstrated.]] | The permissible fault range for steel containers is as follows: (a) If a safety valve is used as second pressure relief device, the pressure inside the inner container does not exceed 136 per cent of the MAWP of the inner container; (b) If a burst disc with a relief external to the vacuum space is used as a second pressure relief device, the pressure inside the inner container is limited to 150 per cent of the MAWP of the inner container; (c) If a burst disc with a relief internal to the vacuum space is used as second pressure relief device, the pressure inside the inner container is limited to 150 per cent of the MAWP + 0.1 MPa of the inner container. [For other metallic materials, an equivalent level of safety shall be demonstrated.]] | (1) In 2.6. Definitions., “container” includes the inner container, the vacuum jacket, and the supporting structure between them. It should be clearly stated that the pressure refers to only the inner container. (2) The installation of “a burst disc with a relief internal to the vacuum space” should be deleted, because it may cause the vacuum jacket to rupture, for the following reasons: - There is no requirement ensuring the strength of the vacuum jacket against internal pressure. -The installation of a PRD for the vacuum jacket is not mandated. (Already pointed out at the 1st TFH2 meeting on 2 July.) |