DSSAD GUIDANCE DOCUMENT

Committee proposals are in blue. Secretariat notes are in red. UK proposals in purple. EC proposals in green. EME proposals in orange. Japanese proposals in pink. Canadian proposals in teal.

1.	Introduction:

DSSAD refers to a capability of a vehicle to monitor the performance of its Automated Driving System (ADS). This document provides recommendations to enable evaluations of ADS performance.

This document has been prepared to support WP.29 deliberations under the 1958, 1997, and 1998 Agreements.

2.	Terms and Definitions:

This section defines terms used in this document. Use of these terms and their definitions is recommended in the development of legal requirements related to ADS and ADS vehicles.

Source: WP.29 GRVA Guidelines and recommendations for Automated Driving System safety requirements, assessments, and test methods to inform regulatory development.

2.1	Automated Driving System (ADS) means the vehicle hardware and software that 	are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis.

2.2	(ADS) Function means an ADS hardware and software capability designed to perform a specific portion of the DDT.

2.3	ADS feature means an ADS designed specifically for use within an Operational Design Domain (ODD).

2.4	ADS vehicle means a vehicle equipped with an ADS.

2.5	Driver means a human user who performs in real time part or all of the DDT and/or DDT fallback for a particular vehicle.

2.6	Dynamic Driving Task (DDT) means the real-time operational and tactical functions required to operate the vehicle.

2.6.1.	When the ADS is in operation, the DDT is always performed in its entirety by the ADS which means the whole of the tactical and operational functions necessary to operate the vehicle (i.e., the ADS performs “the entire DDT” as stated in the definition of an “Automated Driving System” under paragraph 2.1.). These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control.

2.6.1.1.	Sensing and perception include:

- Monitoring the driving environment via object and event detection, recognition, and classification.

- Perceiving other vehicles and road users, the roadway and its fixtures, objects in the vehicle’s driving environment and relevant environmental conditions.

- Sensing the ODD boundaries, if any, of the ADS feature.

- Positional awareness.

2.6.1.2 Planning and decision include:

- Predicting actions of other road users.

- Response preparation.

- Manoeuvre planning.

2.6.1.3 	Control includes: ·

- Object and event response execution

- Lateral vehicle motion control

- Longitudinal vehicle motion control.

- Enhancing conspicuity via lighting and signaling.

2.7	ADS fallback response means a system-initiated deactivation of the ADS or an ADS-controlled procedure to place the vehicle in a minimal risk condition.

2.8	Fallback user means a user designated to perform the DDT pursuant to an ADS fallback response.

2.9	Mitigated Risk Condition (MRC) means a stable and stopped state of the vehicle that reduces the risk of a crash.

2.10	Operational Design Domain (ODD) means the operating conditions under which an ADS feature is specifically designed to function.

2.11	System-initiated deactivation of the ADS means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle   fallback user.

2.12	User initiated deactivation of the ADS means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user.

2.13 	(ADS) User means a human user of an ADS vehicle.

2.14	Failure means the termination of an intended behaviour of an element (system, component, software) or an item (system or combination of systems that implement a function of a vehicle).

2.15	(DSSAD) Triggering Event means a time stamped data element which triggers the recording and storing of time series data elements.

2.16	Emergency manoeuvre is a manoeuvre performed by the system in case of an event in which the vehicle is at imminent collision risk and has the purpose of avoiding or mitigating a collision.

2.17	Imminent collision risk describes a situation or an event which leads to a collision of the vehicle with another road user or an obstacle which cannot be avoided by a braking demand with lower than 5 m/s2.

2.18	ODD exit means:

(a) the presence of one or more ODD conditions outside the limits defined for use of the ADS feature, and/or

(b) the absence of one or more conditions required to fulfil the ODD conditions of the ADS feature.

2.XX     Detected objects shall mean objects detected by the perception system of the vehicle and classified by the ADS as relevant for the purpose of performing a dynamic driving task. Objects with a negative relative velocity shall be deemed relevant.

OR

2.XX	Detected objects means transient objects (such as road users or parked vehicles) and permanent objects (such as bridge pillars, trees, lamp posts or [lane markings]) detected by the perception system of the vehicle and classified by the ADS as relevant for the purpose of performing the DDT.

2.XX   Time-to-Collision (TTC) trigger means longitudinal and lateral calculation of the Time-to-Collision (TTC), which is the relative Euclidean distance divided by the relative Euclidean velocity between the vehicle and the detected object, calculated based on the fused sensor data.

2.XX 	Time-to-Collision (TTC) trigger means longitudinal and lateral calculation of the Time-to-Collision (TTC), which is the relative Euclidean distance divided by the relative Euclidean velocity between the vehicle and the safety relevant object, calculated based on the fused sensor data.

2.XX	Visual Images means video or series of photos that are produced in the visual spectrum that provides the ground truth or confirmation of the environment surrounding the vehicle during an event.

2.XX	Electronic Communication Interface means a standardized connection point that can be accessed directly via the ADS or via the manufacturer’s database to acquire DSSAD data.

2.XX	Digital map means a layered database providing information to the ADS on the [road infrastructure] on which ADS relies partly or entirely to perform the DDT.

3. 	Data Storage and Security

3.1 	The data storage system should be sufficient to record and store the DSSAD time stamped data elements and time series data elements listed in Paragraph 6.

3.1.1	However, as regards times series data recorded following the trigger referred to in point 6.3.2. (c), the data storage system shall be sufficient to store the data corresponding to 5 occurrences.  The recorded data shall be overwritten by the current occurrence data, on a first-in first-out basis.

3.2	The DSSAD shall be protected against both unauthorized access and manipulation.

3.3	In the case of the data intended to be stored off-board the vehicle cannot be transmitted, it should remain stored on the vehicle.

4.	Data Format:

4.1	Each data element listed in Paragraph 6 shall be available in accordance with Paragraph 5. The output shall be provided in an open standard format (e.g. JSON, CSV, XML), with the exception of ‘sensor data’, and the data shall be in a readable form, aside from ‘sensor data’ [and ‘visual images’].

4.2	Information required to interpret the output to correlate it with respect to the data elements required in Paragraph 6 shall be provided by the manufacturer to an authorised entity on request and subject to applicable national law(s).

4.3	Time stamp data format

4.3.1.	Time stamp data shall be recorded in a clearly identifiable way with the following data:

4.3.1.1.            The time stamped data element, as listed in paragraph 6.2.1.

4.3.1.2.	The additional information noted in 6.2 for each time stamped data element as appropriate.

4.3.1.3.	Date (Resolution: yyyy/mm/dd);

4.3.1.4.	Timestamp

4.3.1.4.1	Resolution: hh/mm/ss timezone e.g. 12:59:59 UTC;

4.3.1.4.2	Accuracy: +/- 1.0 s.

4.3.1.5.	Location: global longitude + latitude; shall be recorded in decimal degrees and to at least five (5) decimal places, but shall be unrounded.

4.3.2.	A single timestamp may be allowed for multiple elements recorded simultaneously within the time resolution of the specific data elements. If more than one element is recorded with the same timestamp, the information from the individual elements shall indicate the chronological order.

5.	Data Accessibility

5.1.	The stored data defined in paragraph 6 shall be readily accessible to [the approval authority and] authorised entities for DSSAD data collected by the vehicles in their territory of deployment. The authorised entities and the conditions of access shall be defined by the applicable laws of the territory of deployment.

5.2. 	The DSSAD data (whether stored on or off-board the vehicle) shall be readily available and retrievable through an electronic communication interface that complies with a publicly available interface standard. It is recommended to use an internationally recognized standard.

5.3. 	Instructions for retrieving the DSSAD data via the electronic communication interface shall be maintained by the manufacturer.3

5.4. 	The stored DSSAD data shall be retrievable even when the main onboard vehicle power supply is not available.

5.5. 	The DSSAD data shall be retrievable even after an impact to the vehicle of a severity level set by relevant regulations.

5.6. 	If the DSSAD data is intended to be stored on-board the vehicle, then the following applies.

5.6.1. 	The data elements concerning the activation and deactivation of the feature in paragraph 6.2.1. shall be available via the vehicle’s information display/user interface where controls related to the manual performance of the DDT are provided.

5.6.2. 	Upon request of an authorized entity, the manufacturer shall make available to them the manufacturer-specific tools, software, webservice interfaces, and/or support to retrieve the DSSAD data.

5.7. 	If the DSSAD data is intended to be stored off-board the vehicle, then the following applies.

5.7.1. 	An authorised entity shall not have to install any manufacturer specific systems or software to retrieve the DSSAD data.

6.	Data Elements:

6.1	The DSSAD shall record and store the data elements listed under Paragraph 6.2 and 6.3. This requirement shall be without prejudice to applicable laws governing access to data, availability, privacy and data protection.

6.2	Data elements of time-stamp data

6.2.1.	The following table details the data elements of time-stamp data to be recorded, along with any additional information and recording condition. These data elements are recorded only whilst the feature is active.

| Event | Additional Information |
| --- | --- |
| Activation of the feature | ADS feature is activated by the: - system, or - user |
| Recording of the following data elements only applies while the ADS feature is active. |  |
| Deactivation of the feature | ADS feature is deactivated by the - system, or - user |
| ODD exit |  |
| Start of ADS fallback to user, if applicable | Deactivation of the ADS feature initiated due to: - Foreseen condition, - Unforeseen condition, - Failure, - Input to the driving controls, or - ODD exit. |
| Start of ADS fallback to an MRC | MRC resulting from: - ODD exit, - ADS failure, - collision detected, - Detection that fallback user is not available when they have no longer met the conditions of paragraph [6.2.2.1.6/4.2.2.1.6] in [draft ADS] regulation (if applicable), or - failure of the fallback user to take control following a system-initiated deactivation of the ADS. |
| User input to the driving controls, if applicable, | Application of: - brake control, - acceleration control, - steering control, or - direction indicator. |
| Application of the passenger stop request as designated in paragraph [4.2.3.1./6.2.3.1.] in [draft ADS] regulation |  |
| Prevention of user takeover, if applicable | Prevention of user takeover (if applicable) due to: - Unintentional user input, - The current situation being unsuitable, - The current situation being unsafe, or - The user not being suitably engaged. |
| Detection that fallback user is not available when they have no longer met the conditions of paragraph 6.2.2.1.6/4.2.2.1.6 in [draft ADS] regulation, if applicable |  |
| Start of Emergency Manoeuvre |  |
| End of Emergency Manoeuvre |  |
| Event Data Recorder (EDR) trigger input |  |
| Detected collision |  |
| MRC achieved | Indication of end states per paragraph [5.3.1.14/7.3.1.14] in [draft ADS] Regulation. |
| Detected failure situation | The failure could include the following: - ADS - Sensor - Other vehicle systems (mechanical, electrical, etc.) |
| Remote intervention in a tactical function, if applicable |  |

6.3	Time series data elements

6.3.1.  The data elements shall be recorded in compliance with paragraph 6.3.1 if the following thresholds are reached or conditions occur:

a)  Detected collision

- EDR trigger input (excluding last stop trigger)

- [ODD warning]

- TTC trigger threshold of [0.2s]

6.3.1.1    If there is no system or sensor designed to provide the data element to be recorded and stored under section 6.3, alternative data may be utilised if the data provides equivalent information to the specified data element.

[6.3.1.2   If the data element in the format (e.g. range, resolution, accuracy, and sample rate) indicated in paragraph 6.3.X is beyond what is achievable by the vehicle’s sensors or instruments, then the native output of the sensor or instrument shall be recorded.]

OR

6.3.2.	The data elements shall be recorded in compliance with paragraph 6.3.1 if the following thresholds are reached or conditions occur:

a) Detected collision

b) EDR trigger input (excluding last stop trigger)

c) [ODD warning]

d) TTC trigger threshold of 0.2s (where the speed of the vehicle is 10kph or more and only for systems capable of explicit detection of objects as referred to in paragraph 2XX).

OR

6.3.2	The data elements as listed in paragraph 6.3 shall be recorded when a triggering event occurs as defined in paragraph 6.3.

6.3.3. 	All detected objects relevant to the triggering event shall be recorded.

6.3.3.1. The vehicle manufacturer shall make available to the relevant authority, together with the detected objects, the full input received by the ADS from digital maps (from all layers of map data), relevant for the geolocation of the vehicle at the time where those objects were detected. This information shall be provided in a format referred to in paragraph 4.1.

6.3.4. 	In case sensor data referred to in the table of data elements are recorded, the manufacturer shall demonstrate that the sensor data, together with any [required/necessary] additional information and/or tools made available to the relevant competent authority, shall provide information about the ADS performance in a readable format and of equivalent utility as to the information derived from the recording of the detected objects [and detected road infrastructure objects].

6.3.X	The following table details the data elements of time-series data to be recorded during a triggering event.

| Data element | Condition for requirement | Recording interval/time (relative to time stamp) |
| --- | --- | --- |
| Visual images | Mandatory |  |
| Detected object distance, longitudinal | Mandatory if available |  |
| Detected object distance, lateral | Mandatory if available |  |
| Detected object relative velocity, longitudinal | Mandatory if available |  |
| Detected object relative velocity, lateral | Mandatory if available |  |
| Detected object classification | Mandatory if available |  |
| [Detected lane] | Mandatory if available |  |
| Sensor data | Mandatory if ‘Detected object elements’ are not available |  |
| [Geolocation] | Mandatory |  |
| Geolocation, vertical |  |  |
| Geolocation, horizontal |  |  |
| [Vehicle orientation heading/direction of travel] |  |  |
| ADS-requested gear |  |  |
| ADS-requested accel demand | Mandatory |  |
| ADS-requested service braking demand | Mandatory |  |
| ADS-requested parking brake demand | Mandatory |  |
| ADS-requested steering demand | Mandatory |  |
| [ADS-requested/ Vehicle-indicated direction indicator demand] |  |  |
| ADS-requested/Vehicle indicated hazard warning signal |  |  |
| ADS-requested/Vehicle indicated exterior lighting |  |  |
| ADS-requested/Vehicle indicated audible warning signal |  |  |
| Vehicle-indicated direction indicator status |  |  |
| Vehicle-indicated hazard warning signal status |  |  |
| Vehicle-indicated exterior lighting status |  |  |
| Vehicle-indicated audible warning system status |  |  |
| Vehicle acceleration, longitudinal | Mandatory |  |
| Vehicle acceleration, lateral | Mandatory |  |
| Vehicle acceleration, vertical |  |  |
| [Yaw rate] | Mandatory |  |
| ADS-determined vehicle speed | Mandatory |  |
| Vehicle indicated gear position |  |  |

This definition is based on SAE J3016 and ISO/PAS 22736 (Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles). These standards define levels of driving automation based on the functionality of the driving automation system feature as determined by an allocation of roles in DDT and DDT fallback performance between that feature and the (human) user (if any). The term “Automated Driving System” is used specifically to describe a Level 3, 4, or 5 driving automation system.

Readable means that the data is of numerical values and natural language which can be understood to represent a specific data point with a value associated with it (e.g. <<speed>>35<<speed/>>, not hexadecimal or binary).

Contracting parties may further define technical specifications for data accessibility and/or availability of instructions under national law.

Leading metrics such as ‘proximity conflict’ shall be considered further as a time stamp data in future iterations of the guidance document, to provide indications of unsafe driving behaviour of the ADS.

Excluding any last stop trigger

This data element is generally represented by a camera image; however, this image may be a construct of other sensor data if camera images are unavailable.

e.g. camera, radar, LiDAR, used by the ADS for decision making. This shall be documented in the information package provided to the Authorised Entity. This shall include a “Visual Representation” submitted to the Authorised Entity at the time of providing the DSSAD Data, and shall comply with the requirements of 4.1 and 5.4.