Proposal

2.		Definitions

…

2.xx.	"Time-to-Collision (TTC) trigger” means longitudinal and lateral calculation of the Time-to-Collision (TTC), which is the relative Euclidean distance divided by the relative Euclidean velocity between the vehicle and the safety relevant object, calculated based on the fused sensor data. TTC is calculated in cases the current trajectories of the ADS and the object along their current routes are crossing.

(…)

3.  Data Storage and Security

(…)

3.1.1.     However, as regards times series data recorded following the trigger referred to in point 6.3.2. (c), the data storage system shall be sufficient to store the data corresponding to 5 occurrences.  The recorded data shall be overwritten by the current occurrence data, on a first-in first-out basis.

(…)

6. Data Elements

6.3.2.  The data elements shall be recorded [in compliance with paragraph 6.3.1] if the following thresholds are reached or conditions occur:

a)  Detected collision

b) EDR trigger input (excluding last stop trigger)

(c) A trigger chosen by the manufacturers resulting in recording of data in all situations featuring an similar level of collision risk to the situation where TTC trigger threshold of 0.2s is reached. In such cases, the similar triggering performance shall be documented in the ADS safety case.

.

Justification

The compromise proposal does not require the manufacturers to use TTC trigger. It introduces the TTC trigger only as a benchmark to define the situations of collisions and near misses in which manufacturers’ triggers should allow for recording DSSAD data.