7.14.	Energy modelling

7.14.1.	Energy modelling at production stage [and for maintenance?]

7.14.1.1.	Fuel modelling at production stage

The definition of emission factors for fuels follows the methodology outlined in the EU Renewable Energy Directive 2018/2001 and other systems such as the U.S. EPA Renewable Fuel Standard and shall include:

(a)	Upstream processes referring to the extraction or cultivation of raw materials.

(b)	Processes related to the fuel’s processing.

(c)	Transport and distribution processes at each life cycle stage.

(d)	Emissions from the fuel in-use.

(Infrastructure and capital goods for the production of fuel should be included by default only within the system boundary for the vehicle use stage. Refer to Section 7.6 Infrastructure and Capital Goods.)

Additionally, concerning fuels originated from bio-materials, the following conditions shall be considered;

(a)	Additional emissions related to changes from soil carbon stock caused by land use change.(e.g. wetlands conversion, some specific forest areas, etc.). See Section 7.8 on Land Use.

(b)	Emission savings from soil carbon accumulation via improved agricultural management (e.g. shifting to reduced or zero-tillage, using cover crops, etc.), CO2 capture and geological sequestration, and CO2 capture and replacement. Refer to section 7.10. Inclusion of specific GHG emissions and removals for reporting of such emissions.

In the case of one or more other products (co-products, e.g. sugar beet pulps, Distillers Dried Grains with Solubles), allocation shall be determined based on Section 7.15.2 Multi Output allocation. in proportion to the energy content of the products, determined by lower heating value in the case of co-products other than electricity and heat, or other allocation methods using the allocation method hierarchy in ISO 14040:2006; or developed for national programs, for example the U.S. Renewable Fuels Standards (RFS). The co-product method used in a study should be transparently presented in the study report, and sensitivity analysis should be conducted with alternative allocation methods for impacts of allocation method choices.

7.14.1.2	Electricity modelling at production stage

Three main approaches exist for modelling electricity consumption by a product system: the location-based approach, the 100% market-based approach and a mixed-method approach.

(a)	The location-based approach is a method of allocating electricity impacts among end-users based on geographical representativeness (economic area, country, bidding zone , etc.); this approach is currently the most commonly used by LCA practitioners, with main LCA databases offering detailed models at a relatively fine geographic scale. The same average profile per kWh is applied to all electricity consumers within a single grid. The GHG Protocol defines it as follows: “A location-based method reflects the average emissions intensity of grids on which energy consumption occurs (using mostly grid-average emission factor data).” [In this approach subnational consumption grid mix (i.e. for the USA California grid mix) or a national consumption grid mix (i.e., country-specific), or a supra-national consumption grid mix (i.e., EU grid mix, Africa grid mix, North America) can be used based on Section 7.11.1.2.     Geographical representativity]

(b)	The 100% market-based approach allocates electricity impacts among end-users based on a principle of individual contractual instruments, which can be claimed by different consumers. This approach seeks a more precise accounting of the environmental impacts of energy choices made by companies and consumers. The GHG Protocol defines it as follows: “A market-based method reflects emissions from electricity that companies have purposefully chosen (or their lack of choice). It derives emission factors from contractual instruments, which include any type of contract between two parties for the sale and purchase of energy bundled with attributes about the energy generation, or for unbundled attribute claims.” [In this approach, every electricity consumption process is modelled using either:

- Processes that reflect the electricity mix purchased via specific contractual instruments related to the considered process and including losses during transmission and distribution of the purchased electricity

- Or, if no contract exists for the given process, a residual consumption mix related to it, which can be derived either at a national level, a sub-national level or supranational level. If no residual mix is available, the fossil generation mix for the respective grid shall be used.

The contractual instruments that are used shall comply with the specific requirements as stated in the following sections 7. 7.14.1.2.1.

Further, this approach should only be used if the entity has enough data (i.e. secondary databases and datasets using residual consumption mixes for every process in the upstream value chain of the product).]

(c)	[In circumstances where industries lack sufficient or appropriate data—whether concerning processes required for residual mixes or the processes utilizing them—or when they do not have the time necessary to develop such data, and likewise in situations pertaining to Level 3 and Level 4, they may employ the following mixed-method approach. The mixed method approach uses available location-based electricity mixes consumption processes from the LCA databases as generic default while using specific processes that reflect the electricity mix purchased via specific contractual instruments from suppliers and / or the electricity mix produced within the factories and including losses during transmission and distribution of the purchased electricity. [This approach should be carefully applied as risk of double counting of inventories and impacts can happen.]

[Specific recommendations regarding the best approach to use per level and per life cycle stages is provided in chapter 8.]

For some geographical region of interest already prescribing official guidelines for such modelling, practitioners shall rely on such specific regulations, e.g. EU RED, most recent U.S. EPA Power Sector Reference Case. Furthermore, temporal and geographical consistency should be ensured.

Development of electricity emission factors shall be based on ISO 14067:2018 and shall include upstream processes and production of electricity generation infrastructure for the electricity supply system (e.g. mining of fuels, transport of fuels to power plants, growing and processing of biomass fuel feedstock, and production of equipment generating renewable energy, such as photovoltaic installations).

7.14.1.2.1.	Electricity from the grid via a contractual instrument

Electricity data from electricity certificates for a specific generator site or supplier documented by energy certificates may be used in the calculation of emission factors for electricity supplied from the grid if it can be ensured that all the following conditions are met. The contractual arrangement should:

(a)	convey the information associated with the unit of electricity delivered;

(b)	assure with a unique claim, to avoid double counting of GHG emissions and GHG removals within the boundary of the subject;

(c)	be tracked and redeemed, retired or cancelled by, or on behalf of, the reporting entity.

(d)	be produced [as close as possible to the period to which the contractual instrument is applied and comprises a corresponding timespan;] OR [For the same year that electricity was produced within the country, or within the market boundaries where consumption occurs and to which the grid is interconnected.]

(e)	be produced within the country, or within the market boundaries where consumption occurs if the grid is interconnected.

(f)	The environmental value of electricity certificates must not be biased;

(g)	There is no double counting with grid electricity (the LCI data for grid electricity uses the residual mix, excluding the value of the electricity certificate. If no residual mix is available, the fossil generation mix for the respective grid shall be used);

Information about the electricity certificate should be clearly stated in the LCA report:

        - Name and location of the generating facility;

        - Method of generation;

        - Amount of electricity generated and amount of electricity certificates issued;

        - If available, tracking number assigned by the electricity certificate system.

Detailed conditions shall be based on ISO 14067 -1:2023.

In ISO 14067:2018, there are examples of certificates that can be used by contractual instruments, such as European Guarantee of Origin (GOs) and US Renewable Energy Certificates (RECs) which meet the above conditions. Electricity certificates that do not guarantee compliance with the above conditions shall not be used. Contractual instruments may include utility tariffs, power purchase agreements (PPAs) or energy attribute certificates (EACs)  themselves.

If processes within the subject are located in Small Island Developing States (SIDS)[3)], the carbon footprint may additionally be quantified using contractual instruments for such processes, irrespective of grid interconnectivity.

Notes on handling power certificates

Note 1  Contractual instruments are any type of contract between two parties for the sale and purchase of energy bundled with attributes about the energy generation, or for unbundled attribute claims. This can include energy attribute certificates (EACs), renewable energy certificates (RECs), guarantees of origin (GOs), power purchase agreements (PPAs), green energy certificates or supplier-specific emission rates.

Note 2  The market-based approach is a method to quantify the indirect emissions from energy of a reporting organisation based on GHG emissions emitted by the generators from which the reporting organisation contractually purchases electricity bundled with contractual instruments, or contractual instruments on their own.

Note 3  The location-based approach is a method to quantify indirect emissions from energy based on physical delivery of energy, using average energy generation emission factors for defined geographic locations at a national or sub-national level.

Note 4  Using biofuels or other bio-based materials based on contractual instruments can result in GHG emissions reductions.

7.14.1.2.2.	Electricity from a directly connected supplier

Site-specific electricity data may be used in the calculation of emission factors for electricity supplied directly from electricity generators via dedicated transmission lines to facilities manufacturing the respective products and consumed in the manufacture of such products, if no contractual instruments have been sold to a third party.

7.14.1.2.3. Electricity from On-Site Production

On-site electricity production can only be accounted for under the following conditions:

(a)	The production asset is owned by the same entity as the factory, or the asset has a direct connection to the factory and is not connected to the grid.

(b)	Only the fraction of electricity that is used on-site and not sourced from the grid shall be accounted for.

(c)	No credits can be claimed for electricity produced in excess and sent to the grid.

(d)	No contractual instruments have been sold to a third party

Any deviation, along with detailed explanations of the calculation methods or models used to revise emission factors, must be reported in the LCA report

7.14.1.2.4. Electricity from the grid without contractual instrument

The emission factors of average consumption electricity grid mix shall be calculated based on:

(a)	The consumption per country of the different electricity sources (coal, natural gas, nuclear, hydropower, wind, solar PV) [Imports should be modelled as the average production mix of the country from which the electricity is imported. The consumption mix and the exported mix are the same.]

(b) The emission factors per country of each electricity source (coal, natural gaz, nuclear, hydropower, wind, solar PV), provided by a same database (EcoInvent, Sphera ..). These emissions factors should include

- Upstream processes referring to the extraction of raw materials,

- Processes and direct emission during electricity production

- Transportation, distribution, and losses

(Infrastructure and capital goods for the production of electricity should be included by default only within the system boundary for the vehicle use stage. Refer to Section 7.6 Infrastructure and Capital Goods.)

7.14.2.	Energy modelling at use stage and EoL stage

Practitioners shall also account for any potential changes in the fuel or electricity production pathways during the lifetime of the vehicle. To model energy at use stage and EoL stage, a consistent approach between fuel and electricity modelling is required, and only location-based approach shall be used.

The energy mix change considered for the use stage and the EoL stage shall be based on the latest available dynamic scenario, in order of preference:

1. Official, government-published dynamic scenario on energy mix or emission factors (where these are consistent with the defined scope) based on currently implemented policy for a country or geographical region of interest

2. If the above government-published energy mix or emissions factors are not available, use the Stated Policies Scenario (STEPS) from the most recent IEA WEO report, with the inclusion of the Sustainable Development Scenario (SDS)  as a sensitivity (or equivalent scenario taking into account international commitments under the UNFCCC)

3. For regions without official government-published energy consumption or emission factors, or for which IEA WEO data is unavailable, dispatch modelling or the use of peer-reviewed published data based upon dispatch modeling may be used if it reflects implement policy.  Any such analysis should comprise a sensitivity consistent with the goals of IEA WEO SDS.

4. Official, government-published static scenario on energy mix or emission factors (where these are consistent with the defined scope) based on currently implemented policy for a country or geographical region of interest. In this case reference year shall be reported.

For levels 3 and 4, preferences 2 and 3 might not be suitable, as the purpose of mandatory declaration and OEM comparison necessitates a single reference. Hence preference 2 and 3 should be used only for level 1 and 2.

The selected scenario should be communicated and justified. The detailed calculations of the fuel or grid mix composition and average representative fuel or grid mix composition over the full service life of the vehicle should be communicated.

(Note) Dispatch modelling:  An electric power sector dispatch model is a mathematical representation of how sources of electricity generation are chosen and scheduled (i.e., “dispatched”) to meet a given electricity demand. It essentially models the decision-making process of electricity system operators and is capable of making projections of future generation and future emissions given cost constraints, environmental policy and emissions constraints, and constraints related to resource adequacy. Dispatch models also include representations of individual electricity generation units (EGU) and regional interconnection of EGUs as part of regional, national, and international electricity grids.

For any prospective LCA, secondary data will be needed. Secondary LCI data should be sourced from data sets provided directly by government or relevant authorities. If such data is not available, region-specific secondary data from IEA should be used.

The scenario and trajectories shall be explained in the final report. Details must be referred to in the “Reporting” section.

8.1.6.	Energy Modelling at material production stage

For general rules refer to paragraph 7.14.1.Energy modelling at production stage and for maintenance

For level 1 and 2, the three approaches: location-based, 100% market-based and mixed method approach can be used. The approach should be clearly stated in the LCA report.

For level 3 and 4 the mixed approach shall be used for the material production stage.

8.2.4.	Energy modelling at production stage

For the general rules reference is made to paragraph 7.14.1. Energy modelling at production stage and for maintenance

For level 1 and 2, the three approaches: location-based, 100% market-based and mixed method approach can be used. The approach should be clearly stated in the LCA report.

For level 3 and 4 the mixed approach shall be used for the Parts production and vehicle assembly stage.

8.3.5.	Energy modelling at use stage

For general rules regarding the use phase (excluding maintenance), refer to paragraph 7.14.2.	Energy modelling at use stage and EoL stage.

For the maintenance phase, refer to paragraph 7.14.1.	Energy modelling at production stage and maintenance stage.

8.3.5.1.	Application of the energy modelling schemes for dynamic scenario

If a dynamic scenario is used, the energy mix composition for each year of vehicle operation shall be estimated (i.e., the shares Si,n of energy supplied by each technology i in the year n), by applying linear interpolation between the respective energy supply shares reported for the nearest pre-defined time horizons in the scenario selected at point 1 above. The average representative energy mix composition over the full service life of the vehicle shall be calculated as follows:

(a)	By default, As the arithmetic average of the individual energy supply shares at point 2 above. Doing so entails the implicit simplifying assumption that the vehicle’s use is distributed homogenously over its full service life (i.e., L/N km are driven each of the N years of operation, where L = total lifetime activity).

(b)	Alternatively, if there is reason to expect that the vehicle’s use intensity will change over time, and if year-specific activities may be estimated with sufficient confidence, then a more refined (and accurate) modelling approach may be adopted, employing a weighted average (as opposed to a simple arithmetic average) of the individual shares Si,n of energy supplied by each technology i in the year n, i.e.: n=1NWnSi,n , where Wn = An/L (An = vehicle activity in year n, L = total lifetime activity).

A bespoke energy mix model shall finally be built using the energy mix composition calculated at point 3 above and leveraging the most up to date database processes available for the individual energy generation technologies. The resulting energy mix thus modelled shall be used to estimate the Emission Factor of the electricity input to the use phase of the vehicle.

8.3.5.2.	Application of the energy modelling schemes for static scenario

If a static scenario is used : [TBD : I suggest here a paragraph to explain how to calculate emissions using a static scenario : eg. Emissions factor the energy based on the year of the SOP x Service life

8.4.7.	Energy modelling at EoL stage

For general rules refer to paragraph 7.14.2.	Energy modelling at use stage and EoL stage.

8.4.7.1.	Future Changes in Energy Mix

Practitioners shall also account for any expected changes in the fuel or electricity production pathways during the lifetime of the vehicle. (detailed methodology shall be considered)

Specifically, the following recommendations are made on how to model energy inputs to the EoL phase:

8.4.7.2.	EoL phase electricity

1. The same scenario for the expected future evolution of the energy mix in the geographical region of interest shall be adopted, as previously selected for the dynamic modelling of the use phase (cf. hierarchical approach in Section 7.14.2.	Energy modelling at use stage and EoL stage) electricity input in paragraph [x.x.x]..

2. The grid mix composition for the specific year of vehicle decommissioning (i.e., year of vehicle registration + expected lifetime) shall be estimated (i.e., the shares Si,N of energy supplied by each technology i in the year N), by applying linear interpolation between the respective energy supply shares reported for the two nearest pre-defined time horizons in the scenario selected at point 1 above

3.	A bespoke grid mix model shall be built using the grid mix composition calculated at point 2 above and leveraging the most up to date database processes available for the individual energy generation technologies. The resulting energy mix thus modelled shall be used to estimate the Emission Factor of the energy input to the EoL phase of the vehicle.

For dynamically evolving fuel mixes (e.g., “green”/”grey” hydrogen; bio/fossil diesel blends; etc.), a similar approach to the one described above for electricity should be employed, whereby the respective fuel mix models for the use phase and EoL phase are arrived at by considering the existing future scenarios. "

In case of the process in the EoL phase is not a hotspot, the static energy modelling may be applied.

This is a summary table for my own utilisation, it will not be included in the draft – except if SG6 thinks it is useful.

|  |  | Material production stage |  | Part & assembly stage |  | Use stage (excluding maintenance) |  | Maintenance only |  |  | EoL stage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level 1 |  | Location-based Or 100% Market-based Or Mixed method |  | Location-based Or 100% Market-based Or Mixed method |  | Location based + Hierarchy : 1, 2, 3, 4 |  | Location-based Or 100% Market-based Or Mixed method + static? |  |  | = Use stage but if a process is not a hotspot is may be static |
| Level 2 |  | Location-based Or 100% Market-based Or Mixed method |  | Location-based Or 100% Market-based Or Mixed method |  | Location based + Hierarchy : 1, 2, 3, 4 |  | Location-based Or 100% Market-based Or Mixed method + static? |  |  | = Use stage but if a process is not a hotspot is may be static |
| Level 3 |  | Mixed method |  | Mixed method |  | Location based + Hierarchy : 1, 4 |  | Mixed method + static? |  |  | = Use stage but if a process is not a hotspot is may be static |
| Level 4 |  | Mixed method |  | Mixed method |  | Location based + Hierarchy : 1, 4 |  | Mixed method + static? |  |  | = Use stage but if a process is not a hotspot is may be static |