| OICA (Volvo Cars) | 1D | 7. Heading | te/ed | “Test modes for the purpose of measurement of interior air emissions entering into the cabin with outside air pollutants” is confusing. This has nothing to do with interior air emissions. | Replace with: “Test modes for the purpose of measurement of air pollutants in the vehicle cabin entering from outside” | Ok |
| --- | --- | --- | --- | --- | --- | --- |
| OICA (Volvo Cars) | IV | 8.4.1 | te | Above the table it says “Detection limit of measurement, not less than”. This is not correctly stated. Correct should be equal or lower than, | Replace with: “Detection limit of measurement, equal or lower than” | Ok |
| OICA (Volvo Cars) | IV | 9.1.1 | te | Regarding the text “Take out cabin air filter and replace by new artificially aged one”. This is wrong, a new filter should be used for the test, | Replace with: “Take out the cabin filter and replace with a new cabin filter” | Ok |
| OICA (Volvo Cars) | IV | 9.3.2 | te | Regarding the minimum pollution level. Does every sampling point need to be over this value or is it the average value? As an example, one sampling point of 13ug/m3 PM 2.5 would disqualify the whole measurement? |  | This statement means single measurement before the test, please see paragraph 3.3 |
| OICA (Volvo Cars) | IV | 9.6.2-9.6.10 | te | PM is mentioned instead of PM 2.5. | Replace with PM 2.5. | Ok |
| CLEPA | C/14 |  | gn | CEN CWA 17934:2022 is neither a standard nor a regulation in its current form. See also comments on C/15/e. It had beed decided to delete the CEN reference in the group. | Delete reference to CEN | Ok |
| CLEPA | C/15/e |  | gn | CEN WS103 and the published CWA 17934:2022 had been decided to be deleted in an earlier session | Delete CEN WS103 and CWA 17934:2022 reference as decided earlier. | Ok |
| CLEPA | 7.1 and 7.2 |  | Ge | Test vehicles should only be new ….but in 7.2 vehicle should have 3.000-15000km, which is not “new” | Delete “new” in 7.1 | Ok |
| CLEPA | 8.2.1 |  | Ed | Typo: …sampling point should be a head-height…. | ….…sampling point should be at head-height…. | Ok |
| CLEPA | 9.1.1 |  | Te | …replace by new artificially aged one. This is wrong, as we had decided to used new, original OEM approved filter! | …replace by new, OEM-approved filter…. | Ok |
| CLEPA | 9.3.8 and 9.3.9 |  | Te,ed | …characterized by vehicle speeds limit… During the last test drives we had come to the conclusion that vehicle speed is not an ideal parameter. Better speed limits of the track. | It should read: …characterized by speed limits… | Ok |
| CLEPA | 9.5.1 |  | Te | Auto mode or fresh air mode Fresh Air Mode might contradict auto mode. Probably it was meant to to use recirculation air mode. But with AUTO mode everything is controlled automatically and fresh air or recirculation air mode do not play a role. | Maybe it make sense to delete that in 9.5.1. and define it more closely in: 9.5.2. Add: HVAC Auto mode switched on 9.5.3. Add: fresh air mode on (no Recirculation Air mode) | Ok |
| ARAI/ India | Clause 9/ 9.1.1 | Paragraph | te | Clause 7.5 mentions “The test vehicle should be equipped with new OEM-approved cabin air filter” However, in clause 9.1.1 artificially aged filter is mentioned. “artificially aged” filter is proposed to be removed | Take out cabin air filter and replace by new artificially aged one. | Ok |
| ESTACA / France | III / 3 | 3.2 | ge | There is no PM measurement for the Emission entering to the vehicle cabin with exhaust gases ? |  | Part III of M.R.3. takes into account only: carbon monoxide, nitrogen monoxide and nitrogen dioxide |
| ESTACA / France | IV / 2 | 2 | ge | “Out of scope are limits of inside and outside air pollutants. Measurements results according to this mutual resolution will vary depending on outside conditions and are therefore not suitable to compare different vehicles.” As showed by Estaca it is important to have the possibility to do measurements in static mode that are to be conducted in controlled conditions in a laboratory. This will facilitate the repetability of simulated outdoor conditions and facilitate the measurement for VIAQ assessment in many conditions (for example if the concentrations are below the minimum values) | Out of scope are limits of inside and outside air pollutants. Measurements results according to this mutual resolution will vary depending on outside conditions and are therefore not suitable to compare different vehicles. However, Static measurements in controlled conditions in laboratory can apply the same protocol. | Developed test procedure applicable only for on-road tests, not for laboratory tests. |
| ESTACA / France | IV / 9 | 9.5.3 | te | 50% for temperature could be very hot and inducing more secondary pollutants, I think also for manually controlled temperature we should impose 22° that is the average temperature that is used by the drivers/passengers | For HVAC systems with manual control: temperature 22˚C: | In the test procedure is not provided the measurement of the temperature inside tested vehicle, so it is not possible to maintain temperature level for manually controlled HVAC systems. |
| ESTACA / France | IV / 9 | 9.6.5 | te | Some instruments are based on principle that takes time before stabilizing such as the Chemiluminescence (NOx) and we remarked that the measurements data are not stabilized before this duration | The measurements should not start before 30min after switching on the instruments | In paragraph 9.6.2 added: “Ensure stabilization of instrument readings in accordance with their operating instructions”. |
| ESTACA / France | IV / 10 | 10.1 | te | The maximal could be only one exceptional value, from our experience adding information of centile 95% couls be representative of the highest concentrations that are encountered many times | Calculate the test substances average, minimal, 95th centile and maximal concertation during the test for PM2.5 and NO2 inside and outside the vehicle and for CO2 inside the vehicle for the whole test. | The 95th centile concentration is an optional parameter, which could be added to test protocol as mentioned in Annex IX “Additional parameters may be added here to characterize test conditions”. |
| ESTACA / France | Annex VII |  | te | These external points for outdoor measurements make it difficult to hae short tubes and not too much bending; but also the difficulty on passing the tube depending on vehicle model. On the other hand the aerodynamics make it not suitable the measurements of the particles in that area | We propose to put the external probe on the rear window at least at 10cm from the windows. | The group decided to place external probe as much as possible close to HVAC intake |
| Palas GmbH, Volker Ziegler | 9.3.8 |  |  | Speed maybe reduced to 50 km/h | Urban part is characterized by vehicle speeds limit lower than or equal to 50 km/h with outside sampling isoaxial to the driving direction | The limit 60 km/h is set for harmonizing with other UN Regulations. Additionally in paragraph 9.3.10 mentioned: “Local speed limits remain in force during a test, notwithstanding other legal consequences”. |
| Palas GmbH, Volker Ziegler | 9.3.9 |  |  | Important is to measure isokinetic, otherwise the results are falsified! | Expressway part is characterized by speeds limit above 50 km/h and up to 100 km/h, with outside sampling with the appropriate Isokinetic inlet | The group decided to place outside probe as much as possible close to HVAC intake |