|  | United Nations | ECE/TRANS/WP.29/GRPE/2026/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General xx December 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety fourth session

Geneva, 17–19 March 2026

Item 3(a) of the provisional agenda

Light vehicles:
UN Regulations Nos. 68 (Measurement of the maximum speed,
including electric vehicles), 83 (Emissions of M1 and N1 vehicles),
101 (CO2 emissions/fuel consumption), 
103 (Replacement pollution control devices) and 
154 (Worldwide harmonized Light vehicles Test Procedures (WLTP))

Proposal for a new Supplement to UN Regulation No. 83 (Emissions of M1 and N1 vehicles)

Submitted by the experts of the European Commission, The United Kingdom of Great Britain and the International Organization of Motor Vehicle Manufacturers.

The text reproduced below was prepared by the representatives of the European Commission, The United Kingdom of Great Britain and the experts from the International Organization of Motor Vehicle Manufacturers (OICA). This document aims to propose a new Supplement to the 09 series of amendments to UN Regulation No. 83 in order to remove the provisions relating to manipulation devices and manipulation strategies. These are now proposed for inclusion in a new Supplement to the 04 series of amendments to UN Regulation No. 154.

The modifications to the current text of the Regulation are marked in bold for new or strikethrough for deleted characters.

INITIAL DISCUSSION DRAFT

I.	Proposal

Paragraph 1., amend to read:

"1. 	Scope

This Regulation establishes technical requirements for the type approval of 	motor vehicles with regard to crankcase emissions (Type 3 test) and exhaust emissions at low ambient temperature (Type 6 test) for emissions of gaseous compounds.

In addition, this Regulation lays down rules for in-service conformity for tailpipe and evaporative emissions, and battery durability, electric range ratio of pure electric vehicles at low temperatures, brake emissions and on-board monitoring; as well as rules for anti-tampering, security and cybersecurity; and manipulation devices and manipulation strategies.

1.1.		This Regulation shall apply to vehicles of categories M1, and N1 .

At the request of the manufacturer, for vehicles of category N2 between 3.5 and 5 tonnes maximum mass originating from a type of vehicle of category N1, the approval authority may grant an emission type-approval if the vehicle meets the requirements for a type of vehicle of category N1.."

Paragraph 2.4.1., amend to read:

"2.4.1. 	"Manipulation device" means any element of design that results in a vehicle not complying with the requirements of this Regulation when driven but not under a regulatory test, despite it resulting in the vehicle appearing to be compliant when tested, or that manipulates data related to sensors, fuel or electric energy consumption, electric range or battery durability;"

Paragraph 2.4.2., amend to read:

"2.4.2. 	"Manipulation strategy" means a strategy that results in a vehicle not complying with the requirements of this Regulation when driven but not under regulatory test, despite it resulting in the vehicle appearing to be compliant when tested, or that manipulates data related to sensors, fuel or electric energy consumption, electric range or battery durability;"

Paragraph 3.1.2. (a), amend to read:

"(a)	All relevant documentation to technically justify the absence of manipulation devices and manipulation strategies. Reserved"

Paragraph 3.3.2., amend to read:

"3.3.2.		For the purpose of paragraph 3.1.2.(a)., the manufacturer shall comply with Annex 7 to this Regulation on tests, methods and procedures to establish the absence of manipulation devices and manipulation strategies

3.3.2.1.	These tests, methods and procedures include the roles and responsibilities assigned to vehicle manufacturers, type-approval authorities, and other actors that shall ensure the absence of manipulation devices and manipulation strategies and are specified in Annex 7.. Reserved"

Paragraph 5.1.1., amend to read:

"5.1.1.	Manufacturers shall demonstrate that all new vehicles are type approved in accordance with this Regulation. These obligations include meeting the emission limits set out in this Regulation.

Manufacturers shall ensure that type approval procedures for verifying conformity of production; in-service conformity; anti-tampering, security and cybersecurity; and manipulation devices and manipulation strategies are met.

In addition, the technical measures taken by the manufacturer must be such as to ensure that the tailpipe emissions are effectively limited, pursuant to this Regulation, throughout the lifetime of the vehicles under the test conditions specified in this Regulation, UN Regulation No. 154 and UN Regulation No. 168. For this Regulation the lifetime consists of the main lifetime extended by the additional lifetime. For vehicles in the additional lifetime, a durability multiplier of 1.2 for adjusting the emission limits for gaseous pollutants shall be used. Therefore, in-service conformity measures shall be checked for a period of up to ten years or 200,000 km, whichever comes first.

In-service conformity shall be checked, in particular, for tailpipe emissions as tested against emission limits set out in UN Regulation No. 154."

Annex 7 and its appendices, delete and replace with:

"Annex 7

Reserved"

Annex 11

Paragraph 1.3., amend the fourth sub-paragraph to read:

"1.3	…

Two decisions are possible for a statistical procedure for a given combination of low temperature vehicle family, as defined in paragraph xx of UN Regulation No. 154:

‘Sample pass’ decision ..."

Annex 13, paragraph 4., amend to read:

"4.	Roles and responsibilities of market surveillance authorities

4.1.	Market surveillance authorities may conduct periodic screening tests to detect manipulation devices and manipulation strategies related to data integrity.

4.2.	If a manipulation device or strategy related to data integrity is identified, market surveillance authorities shall proceed in accordance with Annex 4.

."

II.	Justification

- xxx

- xxx.