|  | United Nations | ECE/TRANS/WP.29/GRPE/2025/17 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General For 5st December 2025 IWG Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety-third session

Geneva, 14-17 October 2025

Item 14 of the provisional agenda

Automotive Life Cycle Assessment (A-LCA)

Proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA)

Submitted by the Informal Working Group on Automotive Life Cycle Assessment *

The text reproduced below was prepared by the Informal Working Group on Automotive - Life Cycle Assessment (A-LCA). It is a proposal for a new Mutual Resolution No. 5 (M.R.5) concerning Automotive Life Cycle Assessment (A-LCA). It is submitted to the Working Party on Pollution and Energy consideration at its 93rd session.

[Table 1
General structure of the Level Concept

|  | Items | Level 1* | Level 2* | Level 3** | Level 4** |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Purpose |  | Strategy |  |  |  | Reporting |  |  |
| Life cycle stage | Possible audiences | Policy makers, researchers | Company strategy/product developers, customers | Government programs, customers |  |  |
| Material production stage (See 6.1) | Activity data (Material weight [kg]) | Primary and secondary data | Primary data |  |  |  |
|  | Carbon intensity of material production [kg-CO2eq/kg] | Secondary data |  | Primary and secondary data | Mainly Primary data |  |
| Parts production and vehicle assembly stage (See 6.2) | Vehicle model | Average for a vehicle fleet | Average for vehicles of a specific segment or a vehicle chosen as representative for a specific segment | Average for specific vehicle models with different equipment variants | Specific vehicle mode |  |
|  | Supply chain model | Generic |  | Partly specific for hotspot part supply chain | Mainly specific for whole supply chain |  |
|  | Waste | Implicit in surcharge factor |  | Implicit in surcharge factor except explicit for hotspot part | Fully explicit |  |
|  | Gross vs. net material input (yield) | Implicit in surcharge factor |  | Implicit in surcharge factor except explicit for hotspot part | Fully explicit |  |
|  | Transport of parts | Implicit in surcharge factor |  | Implicit in surcharge factor except explicit for hotspot part | Fully explicit |  |
|  | Transport of vehicles | Generic |  | Explicitly reported from OEM gate to customer |  |  |
|  | Primary/secondary material use | Implicit in material production carbon intensity |  | Implicit in material production carbon intensity except explicit for hotspot part | Fully explicit |  |
|  | Temporal validity | Most recent data |  | Most recent data except annual update for hotspot part | Annual update |  |
|  | Geographical representativeness | Determined based on the research question |  | As specific as possible while plant level for hotspot part | Plant level |  |
| Use stage (See 6.3) | Service life | Officially available data by CPs; if not available, the values in some peer-reviewed reports or methodology or adopt values from this Resolution |  |  |  |  |
|  | Use stage consumption | Regional typical consumption values or other local representative realistic data |  | Official certification values; if not available, OEM-specific data |  |  |
|  | [Discrepancy factor | OEM/CPs/ Suppliers-specific data ; if not available official monitoring info, inventories like EMEP/EEA, etc. or assume 1.0 |  | Default values provided by CPs | Officially available data by CPs; if not available, verifiable OEM-specific average data matched to the region of operation, for informational purposes. |  |
|  | Future changes in energy mix | [A scenario for the expected future evolution of the fuel or electricity grid mix in the geographical region of interest shall be selected: 1. Official, government published energy mix or emissions factors (where these are consistent with the defined scope) based on currently implemented policy for a country or geographical region of interest, 2. If 1. above is not available, the Stated Policies Scenario (STEPS) from the most recent International Energy Agency’s World Energy Outlook (IEA WEO) report, for the geographical region of interest, shall be used. In addition, the IEA WEO Sustainable Development Scenario (SDS) shall also be used as a sensitivity case (or equivalent scenario taking into account international commitments under the UNFCCC). 3. Only if none of the previous options above are available for the geographical region of interest, a projected future electricity grid mix (or emissions factors for this) from either dispatch modelling or peer-reviewed published literature-based dispatch modelling may be used. Due to the inherent uncertainties involved, any such analysis shall include a sensitivity case consistent with the goals of the IEA WEO SDS (or equivalent scenario taking into account international commitments under the UNFCCC). For fuel mix a peer-reviewed published literature modelling may be used, provided that such scenarios are strictly aligned with the regulatory frameworks in force in the relevant country or region of use (for example, the RED III Directive in Europe the EU or the Renewable Fuels Standard in the US). In the equivalent approach for the average fuels mix, only those scenarios from peer-reviewed data reflecting implemented policies shall be considered that are based on announced production capacities] |  |  |  |  |
|  | Maintenance | Optional: maintenance frequency factors based on practitioner’s technical assessment |  | List of maintenance parts/consumable and associated frequency provided by OEM |  |  |
| End-of-life stage (See 6.4) | Activity data of EoL processes (e.g., weight of vehicle, parts, materials, etc.) | Global secondary data | Primary data based on BoM and MDS |  |  |  |
|  | Carbon intensity data of EoL processes (e.g., dismantling and shredding/sorting, ASR thermal recovery, materials recycling, etc.) | Global, regional, or country specific, primary or secondary data can be chosen, depending on the study |  | Global or Region or Country primary data or secondary data shall be specified and provided by CPs |  |  |
|  | Recovered parts disposal and recycling process (e.g., tyre, lead battery, driving battery, etc.) | Global, regional, or country process can be chosen, depending on the study |  | Global or Region or Country process shall be specified and provided by CPs |  |  |
|  | Material recycling modelling |  | RCM or CFF |  |  |  |  |  |  |

(*) There are multiple approaches possible for Level 1 and Level 2. The descriptions for each item in the rows of Level 1 and Level 2 are the minimum requirements for calculation. In other words, the practitioners can adopt any methodologies for each item from the designated or higher levels. For example, for material weight, Level 1 practitioner can use either secondary data or primary data, and Level 2 practitioner shall use primary data.

(**) Level 3 and Level 4 shall be carried on the single fixed approach throughout the life cycle stages, and the practitioners shall follow the descriptions for each item in the designated row.

6.3.	Use stage

6.3.1.	System boundaries

When defining the system boundaries, it is pivotal to highlight that the scope is to provide a comprehensive methodology for calculating realistic GHG emissions and energy consumption over vehicle use stage at various levels while detail and considering the availability of different information and datasets. As outlined in the figure below, the use stage encompasses the operation of the vehicle itself and direct impacts from this, as well as impacts from the production and distribution of fuel/electricity, and for impacts relating to vehicle maintenance and replacement parts.

Impacts directly from the energy used in operation of the vehicle include in the CO2 equivalent calculation the tank to wheel (TtW) contribution are hereby addressed, hence:

(a)	Electric energy: from vehicle charging port to the wheels, if already included in the certification protocol i.e. includes vehicle internal charging losses);

(b)	Fuel: CO2e emissions related to fuel use from tank to wheel.

Conversely, impacts resulting from the production and distribution of fuel and electricity for the operation of the vehicle, i.e. well to tank (WtT), shall follow the methodology outlined in paragraph 5.5.1.

These latter WtT GHG emissions shall be covered in the conversion factor of each energy type according to paragraph 5.5.1 .

Use stage boundaries are depicted in the figure below:

Figure 22
Use stage Life cycle flow schematic

As a matter of fact, the use stage covers mainly two aspects of the whole life cycle, such as the “In-use energy consumption and GHG emissions” (covering WtT and TtW impacts), and the “Maintenance and consumables” impacts. It is relevant to address also leakages such as emissions of methane and other hydrocarbons, together with emissions of climate active species from the vehicles (e.g. methane leakage from CNG cars) and the formation of non-CO2 GHG gasses (e.g. N2O) during the fuel combustion. More in detail, the figure below shows the system boundaries related to vehicle operation.

Figure 
Vehicle operation system boundaries

The handover point from the “Parts Production and Vehicle Assembly” stage is defined at the point of delivery to the final customer, once the finished vehicle has been handed over. On the disposal and recycling side, GHG emissions after the vehicle service lifetime fall within the EoL (End-of-Life) domain. Therefore, it is acknowledged that the transportation of the vehicle to EoL treatment facilities is included within the EoL domain.

Considering the proposed system boundaries, the CO2 equivalent emissions for the use stage comprise emissions from in-use consumption, emissions due to maintenance parts and consumables, and the contribution of leakages and fluorocarbon emissions. The following equation highlights these components:

C Use Stage=C Energy+C Leakages+C Maintenance	(30)

Where;

C Use Stage 		means carbon emissions for the whole use stage [kg CO2eq];

C Energy	means carbon emissions due to the electricity consumption at vehicle level and/or fuel combustion, including both CO2 and non-CO2 exhaust emissions [kgCO2eq];

C Maintenance 	means carbon emissions due to maintenance and consumables [kg CO2eq];

C Leakages	means carbon emissions due to leakages, including unburnt GHG species emissions [kg CO2eq];

In the following paragraphs, the methodology for assessing each term of the equation is detailed.

6.3.1.1	          Quantification of vehicle emissions exported out of region of sales

Ideally, the GHG emissions related to operation of vehicles exported from the country where they are sold/used shall be evaluated by using energy consumption performance (and fuel mix) and vehicle lifetime (and km activity) of the country where they are exported, used and eventually disposed/recycled. However, to avoid unnecessary complexity, if data in the region to which they are exported cannot be tracked down or it is difficult to grasp the needed values of the country where they are exported, used and eventually disposed/recycled, a simplified approach can be adopted. Hence, the full lifetime period (and km activity) shall be considered but the energy consumption performance (and fuel mix) and vehicle life (activity) of the primary region of sale/use are adopted for the calculations. For those countries and regions that export used vehicles, the actual average vehicle lifetime is higher than the average period during which the vehicles are used domestically.

Therefore, for all vehicles, the lifetime used to calculate life cycle emissions shall align with the average age of vehicles at the time of recycling, considering only those registered and recycled within the same country.

6.3.2.	Service life/Vehicle lifetime activity

The vehicle lifetime (or service life) is defined as the full period of time during which a vehicle is operated and thereby contributes to GHG emissions. This lifetime is expressed in terms of kilometres (km) driven and is chosen to represent typical usage patterns. Alongside the distance travelled, the years of operation for a vehicle is defined as the assumed total number of years a vehicle remains in active use between point of sale and point of recycling/scrapping. Both are relevant for the purpose of the methodology.

Vehicle lifetime mileage is required to translate vehicle production emissions (in tonnes of CO2eq) into the functional unit defined in Section 5.2 Vehicle lifetime in years is needed to calculate the lifetime mileage, but also to determine the period of time over which the changes in the emission intensity of the fuel and electricity will be accounted for. Lifetime duration is also needed to calculate the number of replacements/refills for certain maintenance and consumable items.

6.3.2.1.	Service life: methodology and guideline

Precisely assessing these values for each region may present further challenges, underscoring the need for flexibility and regional adaptation in their definition and application.]

The actual service life and lifetime distance reference flow of a vehicle can vary significantly depending on the region and its specific applications. This variability makes it challenging to accurately estimate these values universally. Therefore, this Resolution proposes to define service life for each region or country of usage.

De-registration in a country (i.e., as indicated by “survival curves”) can be significantly earlier than reaching end-of-life. In some countries, a significant share of vehicles is exported before reaching the end of service life. Hence, the average age when being de-registered is lower than the average age when being recycled. In general, these guidelines recommend to consider statistical evidence on the average vehicle age at the point of recycling. For the reasons discussed above, statistics on the average age at the point of de-registration in a country is not recommended, as these values exclude the potential export and continued operation of a proportion of the vehicle in other countries. Therefore, for all vehicles, the lifetime used to calculate life cycle emissions should align with the average age of vehicles at the time of recycling, considering only those registered and recycled within the same country.

Some frameworks adopt different lifetime values by vehicle segment. There are both advantages and disadvantages to offering differentiation by segments. (1) Differentiation offers a more accurate representation of the typical usage expected, which also partly highlights the greater potential utility that larger vehicles can provide. (2) However, near the boundaries between segments, it may unintentionally influence consumer decisions (for instance, a specific user is unlikely to choose a vehicle solely because it belongs to a different segment – it's typically bought for a particular use). Until further in-depth analysis is conducted, this Resolution does not recommend differentiating service life by segment.

Although statistics indicate that service life varies considerably between powertrains, this may be influenced by user behaviour, as well as current economic/taxation policies, which are subject to change. Since one of the objectives of LCA is to compare different powertrains, based on the same functionality, i.e., the same use case, this Resolution does not recommend assigning different service lives to different powertrains.

Generally, the annual mileage of vehicles decreases as they age. Therefore, it is important to assign greater weight to the higher mileage recorded in the initial years, as the emissions intensity of electricity mixes is generally higher during this period. For regions that have officially adopted a method for modelling the distribution of annual vehicle activity over time, data from government authorities should be prioritised.

Although the methodologies do not prescribe a specific method to model the annual mileage distribution as function of the vehicle age, it is permissible to assume a constant value only in cases where the public authority does not require to use official data regarding age-dependant vehicle annual mileage. In that case, the constant average annual mileage should be based on gathered data on total vehicle activity (in kilometres) and vehicle lifetime., (i.e., L/N km are driven each of the N years of lifetime operation, where L = total lifetime activity in km). The method should be clearly documented and transparently reported. Section 6.3.3.3.2.2 deals with the calculation for the average representative energy mix composition over the full-service life of the vehicle, for both homogeneous and age-dependant annual mileage approaches.

Some vehicles, such as special purpose or sports vehicles, have very low usage, leading to a significantly reduced service life. Practitioners could in principle indicate a shorter service life, where justified and supported by primary data, for communication purposes (in addition to default value). On the other side, the guidelines currently do not advise declaring a longer service life than the regional default value.

6.3.2.2.	Service life values

Service life (in both km and years) will be defined by each contracting parties according to official available data on vehicle service life until point of recycling.

In the absence of the above values, the following values as defined in official databases or peer-reviewed reports (see the Annex to this Resolution for reference) can be used as an indication of typical service life as fall-back option for passenger cars only.

Table 8
[Average age of end-of-life vehicles reported in a selection of countries]

| Region / Country | Year of publication | Duration (Years) | Source |
| --- | --- | --- | --- |
| EU27 | 2024 | 20 | Multiple sources as in the Annex to this Resolution |
| United Kingdom | 2022 | 18 | Centre for Economic Performance (Nguyen-Tien et al., 2025) |
| Japan | 2024 | 17 | Ministry of Economy, Trade and Industry, 2025 - Status of the Enforcement of the Automobile Recycling Law |
| USA | 2025 | varies | MOtor Vehicle Emission Simulator (MOVES) |
| Brazil | 2020 | 22 | Ministry of Science, Technology and Innovations of Brazil, 2020 |

Table 9
Average annual mileage of end-of-life vehicles reported in a selection of countries

| Region / Country | Year of publication | Service Life (km) | Annual Mileage (km) | Source |
| --- | --- | --- | --- | --- |
| EU27 | 2024 | 240,000 km | 12,000 km | Kraftfahrt-Bundesamt (2025); Agence de la Transition Écologique, Enerdata, & Fraunhofer ISI (2025) ACEA 2025 https://www.acea.auto/fact/fact-sheet-cars/* |
| United Kingdom | 2022 | 210,000 km | 11,424 km (7,100 miles per year for all powertrains) | Vehicle mileage and occupancy - GOV.UK (2025) |
| Japan | - | - | - | No statistics available |
| USA | 2025 | Varies | Varies | MOtor Vehicle Emission Simulator (MOVES)34)34 |
| Brazil | 2012 | 288,000 km | 13,000 km (Derived from service life over the first 22 years of usage) | Ministry of Environment (2013) |

6.3.3.	Use stage consumption

Usage of light-duty vehicles depends on regional customer behaviour. To provide a standard approach that ensures repeatability, comparability and verifiability by authorities, each region has defined homologation driving cycles and test conditions to be followed. In some regions, these certification values are repeatedly found to underestimate the average fuel consumption in real-life operation. In this regard, Table 11 provides an overview of the primary certification protocols used globally. This table serves as an illustrative example of the methodologies employed in key regions to determine fuel and energy consumption for light-duty vehicles, depending on the powertrain type. The selection and verification of the appropriate homologation procedure remain the responsibility of each contracting party, to be determined on a case-by-case basis. As a result, using regional certification values is considered to be a good starting point to later reach an accurate representation of in-use energy consumption.

The scientific community widely acknowledges that there remains a substantial disparity between certification values and actual energy or fuel consumption in real-world conditions across all types of powertrains in some regions. The gap may vary per powertrain, region, driving behaviour and other variables that have to be taken into account. However, in some regions such as Europe and Japan, real-world fuel consumption can be monitored through on-board systems. This data is compiled annually and shows the gap between certified values and actual usage for each powertrain. To address this, the Resolution proposes using a "discrepancy factor" if data are available for the region in question. If the factor is not available for a specific region, then the factor should be calculated based on best available data and expert qualified assumptions with a minimum value of ‘1’. For vehicles that have more than one mode of operation (e.g. OVC-HEVs working in Charge-Depleting and Charge-Sustaining mode), distinct discrepancy factors should be applied to accurately characterise each operational mode.

As a matter of fact, for Level 1 and Level 2 analysis, projections or estimations can be made based on available information such as certification values. These should be considered as minimum requirements and do not prevent the use of more realistic or detailed data to better characterise vehicle behaviour, where such data is available. For Level 3 and Level 4, where stricter requirements are in place, please refer to methodologies in table 11.

In a ‘Level 4’ calculation context (see paragraph 2.2.) the substitution of publicly available official data, such as the EU OBFCM data, with OEM-specific average data is not permitted. In instances where publicly available official data is unavailable for a particular manufacturer or model, related to a specific powertrain type or region under consideration, OEM-specific average data, if allowed and approved by relevant government authorities  may be employed to determine the "Discrepancy Factor." This factor should be based on an analysis of real-world data from vehicles with similar powertrains (e.g., internal combustion engine vehicles (ICEVs) or zero-emission/electric powertrains such as plug-in electric vehicles (OVC-HEVs), fuel cell hybrid vehicles (FCHVs), etc.) and aligned with the specified region of operation (i.e., sale region). OEM-specific average data, if allowed and approved by relevant government authorities may also be utilized to provide supplementary information. It is imperative for to disclose detailed information about the fleet sample used to derive the data, including, but not limited to, the sample size and period of data collection. These factors should be defined and updated regularly by the relevant authorities.

Vehicle performance and efficiency may change over time due to wear, component deterioration and other factors, thus effecting fuel consumption and CO2 emission. If "Deterioration factors” for the specific modules are available for a particular region, they should be used to account for this issue. Deterioration factors based upon measurement and used for vehicle emissions certification and/or type approval shall be considered primary data. If a deterioration factor is not available for a region, then a deterioration factor could be calculated based on best available data and expert qualified assumptions with a minimum value of ‘1’. Since the deterioration effect covers vehicle aging, it is important to note that this factor may also be reflected in on-board fuel consumption monitoring data used to quantify the “Discrepancy factor”, and hence double counting shall be avoided.

As far as the mere Fuel and/or Energy consumption is considered, the in-use consumption, ECin-use shall be calculated as follows for vehicles with powertrains with only one mode of operation:

ECin-use=ECcertification×fdiscrepancy×fdeterioration		(31)

Where;

ECcertification 	means the energy consumption or fuel consumption [MJ/km or Wh/km];

fdiscrepancy 	means the discrepancy factor (if not available for a region, then 1 should be used)

fdeterioration 	means the deterioration factor (if not available for a region, then 1 should be used)

[For vehicles with powertrains with two modes of operation the in-use energy consumption shall be defined separately for each mode of operation, so that overall in-use energy consumption may be defined. An additional formula shall be used to account also for changes to the share of operation in Mode 1 and Mode 2 that may result from accounting for the discrepancy and deterioration factors for each mode of operation.

In the specific case of OVC-HEVs, also called plug-in hybrid vehicles (PHEVs), including range-extended electric vehicles (REEVs), the two main modes of operation are the vehicle battery charge-depleting (CD) mode and the charge-sustaining (CS) mode. In some OVC-HEV architectures (e.g., REEVs), this may be 100% operation on electricity. In other OVC-HEV architectures, depending also on factors like power demand and temperature, the CD mode also includes a contribution of the combustion engine. Therefore, the energy consumption in the CD mode need to account for both electricity and fuel consumption. In the CS mode, the on-board battery is not being depleted, and the vehicle is essentially operating as a non-plug-in hybrid vehicle mode. In this case there is only fuel consumption and no net consumption of electricity. The share of operation in CD mode is typically expressed by the utility factor (UF), which is estimated as a function of the vehicle’s range in CD mode.

Based on the mode of operation, both the in-use fuel and electricity consumption should be calculated as follows:

FECin-use=FECin-use [CD mode]×UF in-use CD mode+FECin-use [CS mode]×(1-UF in-use CD mode)

EECin-use=EECin-use [CD mode]×UF in-use CD mode

Where;

FECin-use	means the weighted in-use fuel consumption of driving in CD and CS mode combined [L/100 km].

EECin-use	means the weighted in-use electricity consumption of driving in CD and CS mode combined [MJ/km].

FECin-use[CD mode]	means the in-use fuel consumption of driving in CD mode [L/100 km].

EECin-use[CD mode]	means the in-use electricity consumption of driving in CD mode [MJ/km].

FECin-use[CS mode]	means the in-use fuel consumption of driving in CS mode [L/100 km].

UF in-use [CD mode]	means the in-use utility factor representing the share of operation in CD mode.

And,

| Mode 1 (e.g., CD mode) | FECin-use CD mode=FECcertification [CD mode]×fdiscrepancy, fuel [CD mode]×fdeterioration, fuel [CD mode] EECin-use CD mode=EECcertification [CD mode]×fdiscrepancy, electricity [CD mode]×fdeterioration, electricity [CD mode] |
| --- | --- |
| Mode 2 (e.g., CS mode) | FECin-use CS mode=FECcertification CS mode×fdiscrepancy, fuel CS mode×fdeterioration, fuel CS mode |

Where;

FECCertification [CD mode]	means the certification  fuel consumption of driving in CD mode [L/100 km].

fdiscrepancy,  fuel [i mode]	means the real-world discrepancy factor for fuel consumption in each (i) mode. One for CD and one for CS mode shall be derived [-].

fdeterioration, fuel [i mode]	means the real-world deterioration factor for fuel consumption in each (i) mode. One for CD and one for CS mode shall be derived [-].

EECcertification [CD mode]	means the certification electric-only energy consumption in CD mode [Mj/km];

fdiscrepancy, electricity [CD mode]	means the real-world discrepancy factor for electricity consumption in CD mode [-].

fdeterioration, electricity [CD mode]	means the real-world deterioration factor for electricity consumption in CD mode [-].

FECCertification [CS mode]	means the certification fuel consumption of driving in CS mode [L/100 km].

In many certification procedures, the UF of OVC-HEVs initially has been estimated under the assumption that the vehicle’s battery is fully charged once per driving day and the vehicle drives in the CD mode until the battery is depleted and continues driving in the CS mode for the remainder of the day. In real-world operation, however, it is observed that OVC-HEVs on average are charged less frequent than once per driving day, resulting in a lower share of driving in CD mode and thereby higher fuel consumption than the values considered in certification values.

In reaction to the large-scale evidence on the difference in observed in-use and certified fuel consumption values for OVC-HEVs, the governments in some major markets including China, the EU and the US have updated the official UF assumptions and will gradually introduce these for the certification of new vehicles. For vehicles in markets with such updated certification UF assumptions, such as the European Union, the United States and China, the in-use distribution between of CD and CS mode operation shall be based on the in-use CD mode range and the most recent certification UF curve throughout the vehicle’s lifetime. If new UF values are anticipated and officially released for future years, these values should be adopted and replace the previous UF figures in the lifetime assessment.

For vehicles in markets where the official certification UF has not yet been updated based on the large-scale evidence of the average real-world operation of OVC-HEVs, the same UF as in markets with such updated certification UF shall be used. This is justified by the finding that the updated certification UF in China, the EU, and US reflect a similar OVC-HEV usage across these regions. Also for other markets, the updated UF curves from these regions are thus likely to be more representative than earlier UF curves that are not based on large-scale evidence.

The in-use UF shall consider such updated certification UF and be based on the in-use instead of the certification electricity consumption in Mode 1 (e.g., CD mode). As the range of OVC-HEVs in Mode 1 (e.g., CD mode) is determined by the electricity depleted from the battery, only the electricity consumption in that mode needs to be reflected. The in-use range in Mode 1 shall be determined according to the following equation:

Rangein-use [CD mode]=Rangecertification [CD mode]×EECcertification [CD mode]EECin-use [CDmode] 	(32)

Where;

Rangecertification [CD Mode] means the certified range in CD Mode, e.g. Mode 1, as defined in the regional certification procedure. In some procedures, this may be defined as the full or equivalent all-electric range. In other regions, such as the EU WLTP, the CD mode range may include also some use of the combustion engine in mixed operation, which is included in the certified charge-depleting cycle range (RCDC).

Note that in WLTP, the equivalent all electric range (EAER) is the proportion of the RCDC where the combustion engine is not running. Although the EAER typically is included in public disclosures, it is the RCDC that is used for the weighting of the CD and CS drive modes in WLTP.

Alternatively, in situations where Contracting Parties require the exclusive use of real-world data, such as OBFCM for the EU, to determine the UF, this guideline shall be adhered to. Practitioners shall utilize official certification values and, if these are unavailable for specific calculations, OEM-specific data. The entire process must be thoroughly documented, as stipulated by the Contracting Parties.]

Table 10
Level Concept for Use stage Energy Consumption values*

| Level | Representativeness | Energy Consumption Certification |
| --- | --- | --- |
| Level 1 | LCA family representative vehicle (at the practitioner’s discretion) | Regional typical consumption values or other local representative realistic data** |
| Level 2 |  |  |
| Level 3 | Specific vehicle configuration | Official certification values or OEM-specific data in case the former are not available, if allowed and approved by relevant government authorities. |
| Level 4 |  |  |

Note *: in defining regional discrepancy and deterioration factors for different modes of operation based on real-world energy consumption monitoring, care should be taken to avoid double counting of the effects of the Utility Factor.

Note**: for Level 1 and Level 2, these guidelines are considered as minimum requirements and do not prevent the use of more realistic or detailed data to better characterise vehicle behaviour.

Table 11
Energy consumption and GHG certification protocols for main regions

| Region | Powertrain | Protocol |
| --- | --- | --- |
| EU27 | ICE, HEV, OVC-HEV, FCHV, PEV | WLTP (WLTC 4 phases) |
| United Kingdom | ICE, HEV, OVC-HEV, [FCHV], PEV | WLTP (WLTC 4 phases) |
| Japan | ICE, HEV, OVC-HEV, FCHV, PEV | WLTP (WLTC 3 phases) |
| China | ICE, HEV, OVC-HEV | WLTP (WLTC 4 phases) |
|  | FCHV, PEV (Pass. Car) | CLTC-P |
|  | FCHV, PEV (Vans/LCV) | CLTC-C |
| Korea | ICE, HEV, OVC-HEV, FCHV, PEV | Combined (FTP-75 and HWFET weighted average) |
| US | ICE, HEV, OVC-HEV, PEV, FCHV | FTP and HFET combined-weighted-average |
| Canada | ICE, HEV, OVC-HEV, PEV, FCHV | Combined (FTP-75+HWFET) |

Accurate characterization of methane (CH4) these emissions is critical for comprehensive LCA, ensuring alignment with climate policy targets and improving the accuracy of GHG mitigation strategies.

In regions where methane is monitored as a pollutant and the tailpipe and evaporative methane emissions are determined during the homologation procedure, the emissions measured in the official tests shall be factored into the CO2 eq calculation.

In regions where methane is not monitored (directly or indirectly) as a pollutant and the tailpipe methane emissions are not measured in the homologation procedure, methane emissions shall be included with default values provided by Contracting Parties. If these are not provided, Level 1 and 2 analyses shall use values derived from peer-reviewed literature, provided that they refer to new and fully functional vehicles, while Level 3 and 4 can either use such literature data and sources or ignore methane emissions to ensure that result variability does not impact official reporting or comparisons between vehicles.

As an example, based on studies on Euro 6 cars in Europe (Prussi et al., 2020); Valverde & Giechaskiel, 2020; Vojtíšek-Lom et al., 2018; Hagos & Ahlgren, 2018), the methane emissions from gasoline, diesel, and CNG cars can be considered with 5 mg CH4/km, 9 mg CH4/km, and 60 mg CH4/km. This corresponds to about an additional 0.15 g CO2eq/km, 0.12 g CO2eq/km, and 1.81 g CO2eq/km for the 100-year GWP.

6.3.3.1.	Calculation of ‘Discrepancy Factor’

A significant gap remains between certification values and real-world energy or fuel consumption across all powertrains in some regions. To address this, this methodology suggests applying a "discrepancy factor" where official regional data exists. If unavailable, the factor should be estimated using the best available data and expert assumptions, with a minimum value of 1. For vehicles with multiple modes, such as OVC-HEVs, discrepancy factors should reflect each mode of operation separately, as outlined in the previous section.

Table 12
Level Concept for discrepancy factor calculation

| Level | Discrepancy Factor |
| --- | --- |
| Level 1 | If OEM/ CPs / supplier-specific data is not available, assume official monitoring info, inventories like EMEP/EEA Guidebook, COPERT, MOVES, APEI, etc… |
| Level 2 |  |
| Level 3 | Default values provided by CPs based on methodology development/impact assessment/internal studies on real-world gap (OBFCM or equivalent standardised) |
| Level 4 | Default values provided by CPs shall be used. . As a regional option, verifiable OEM-specific average data from all vehicles of the same powertrain type operating in the real-world (e.g. PEVs, FCHVs, etc.), matched to the region of operation, may be used for informational purposes. |

6.3.3.2.	Calculation of ‘Deterioration Factor’

The current version of this Resolution addresses only the degradation of batteries and fuel cells, as these processes increase fuel consumption as a result of declining efficiency.

Given the loss in (charge/discharge) efficiency of batteries over the lifetime of the vehicle is reportedly relatively low, and no approaches have been identified to quantify this objectively, it is not proposed to include this aspect for PEVs.  However, the situation for OVC-HEVs and fuel cells is different, where efficiency degradation is expected to be significant. Therefore, the following overall methodological approach is proposed for determining fuel cell and OVC-HEV battery efficiency degradation over lifetime of the vehicle.

6.3.3.2.1.	Approach 1 : Where emissions deterioration factors are already incorporated into vehicle emissions certification or type approval procedures, those factors shall be adopted in the calculation of vehicle in-use consumption as per Equation 31. This ensures alignment with regulatory requirements and harmonization across jurisdictions.

6.3.3.2.2.	Approach 2 : In cases where a deterioration factor is not mandated nor available for a specific region, practitioners may adopt the following methodology defined in paragraph 6.3.3.2.2.1. for FCHVs and methodology defined in paragraph 6.3.3.2.2.2. for OVC-HEVs. Given the current state of technical knowledge and data availability, practitioners may apply a minimum “Deterioration factor” of 1.0 where no region-specific data exists. Future revisions to this methodology will incorporate advancements in measurement methodologies and data collection.

6.3.3.2.2.1.	[Fuel Cell Hybrid Electric Vehicles ]

[ For fuel cells, efficiency losses occur over the operational life of the vehicle. It is proposed to calculate the average loss of efficiency (used to calculate an amended lifetime average energy consumption in MJ/km) based on the fuel cell durability assumptions and operational lifetime km, as outlined below. Fuel cell durability is defined as the number operational hours to reach 10% degradation of the original fuel cell rated power (in kW). The following general methodological approach is therefore proposed to determine the average loss in efficiency over the service life of a vehicle using fuel cell based powertrain (i.e. an FCHV or FC-REEV powertrain).

Potential for further development of knowledge in this area is expected. To adjust to this perspective, the recommended practices allow OEM or suppliers to propose an alternative owned methodology to define operational fuel cell efficiency loss, as long as it is validated by an independent third party expert on fuel cells.

For operation on hydrogen for FCHV – amended to account for fuel cell degradation, the efficiency loss should be to a maximum of 5% degradation over the life of the vehicle, i.e. in the case where FC [lifetime energy] >FC [max energy], where a fuel cell replacement will be required in any case. The following formula is proposed to calculate the average efficiency reduction in the intermediate cases up to this point:

EnCon AvLife=  EnCon [Start]1- 10% * FCEV[lifetime energy]FC[max energy]2		(33)

Where;

EnCon [AvLife] 	means the average input hydrogen energy consumption in MJ/km over the entire lifetime of the vehicle.

EnCon [Start] 	means the input hydrogen energy consumption in MJ/km at the start of the vehicle life (i.e. before any FC degradation), as defined in vehicle certification (i.e. before any real-world adjustments being applied).

FCHV [lifetime energy] 	means the lifetime vehicle operational electrical energy requirement (i.e. fuel cell output, kWh) based on the input hydrogen energy consumption (in kWh/km), the lifetime activity (in km) and the average fuel cell efficiency (%).

FC [max energy] 	means the maximum energy delivered by the fuel cell (in kWh) over the defined service life (in hours) at the average fuel cell running power (in kW). As per the formula given below.

NB: Fuel cell durability/service life is defined as based on the number of operational hours to 90% of original peak power rating, hence an efficiency loss of 10% over the life of the fuel cell, equal to an average reduction in overall efficiency of 10% divided by 2.

The (i) maximum lifetime energy that can be delivered by the fuel cell before reaching 10% degradation (FC [max energy]), and (ii) fuel cell electric vehicle lifetime energy requirements (FCHV [lifetime energy]) is proposed be calculated as follows:

(i) Fuel cell lifetime maximum electrical energy output (FC [max energy]):

FC[max energy](kWh)=Fuel cell durability (hrs)* Fuel cell average running power (kW)													(34)

Where;

Fuel cell average running power (kW) 	means the maximum rated fuel cell power (kW) * average operation % of rated fuel cell power. See paragraph underneath to define assumptions and values.

(ii) Fuel cell electric vehicle lifetime electrical energy requirement (i.e. energy output from fuel cell)

(= FCHV [lifetime energy]):

FClifetime energykWh=EnConStartMJkm*Fuel cell average efficiency*EnConConversionkWhMJ* Lifetime activity (km)

(34)

Where;

EnCon [Start] 	means the input hydrogen energy consumption in MJ/km at the start of the vehicle life (i.e. before any FC degradation), as defined in vehicle certification (i.e. before any real-world adjustments being applied).

EnConConversion 	means the conversion factor for converting MJ to kWh = 3,6 MJ/kWh

Prioritisation for fuel cell durability assumptions:

The following recommended prioritisation is proposed for the underlying assumptions of fuel cell life and average operational efficiency, with the choice of which option is most appropriate or feasible left to the practitioner (i.e. depending on the availability of data and objective of the study).

The different options are listed in ascending order of accuracy and preference. The first option is proposed as a default approach, where sufficient information is not available for the other options.

Table 13
Prioritisation for fuel cell durability assumptions

| Priority | Fuel Cell Deterioration Factor |
| --- | --- |
| 1 | If OEM/ supplier-specific data is not available, assume an operational life of 6000/24000 hours (for LDVs/HDVs)(a), an efficiency of 55%/52% (at the start of the fuel cell life for LDVs/HDVs)(b), with efficiency loss of 10% over the life of the fuel cell, and running at an average of 25%(c)/25%(d) (for LDVs/HDVs) of the peak power rating |
| 2 | Optional, depending on availability OEM-specific / supplier-specific data on fuel cell life (to 10% loss in power) and average operational power level (as % of the peak power of the fuel cell, according to regulatory testing cycles) |
| 3 | Optional, depending on availability OEM-specific / supplier-specific methodological approach to define operational fuel cell efficiency loss, if validated by an independent third party expert on fuel cells. |

Notes: (a) based on 2025 targets from FCH2JU KPIs FCH 2 JU - MAWP Key Performance Indicators (KPIs) - European Commission (europa.eu); (b) based on Ricardo review of typical fuel cell efficiency for LDV and HDV applications; (c) based on Fuel Cell Electric Vehicle Durability and Fuel Cell Performance (nrel.gov), (d) average approximation based on Ricardo analysis of VECTO simulation results for different HDVs and cycles.

]

6.3.3.2.2.2.	[Off-Vehicle Charging Hybrid Electric Vehicles (OVC-HEV)]

[For OVC-HEVs (i.e. including plug-in hybrid electric vehicles – OVC-HEVs - and range-extended electric vehicles – REEVs), the proposed methodology should account for the effect of the change in the battery energy (i.e. State of Certified Energy as defined in UN GTR 22) over the lifetime of the vehicle and its impact on electric range and via the Utility Factor (UF) on the share of operation in charge-sustaining and charge-depleting modes, which ultimately affects the combined fuel consumption and CO2 emissions]

[The average lifetime in-use electric range for vehicles operating in dual-mode shall be calculated according to the following formula, accounting for the average loss in energy (i.e. as defined by the battery SOCE State of Certified Energy):

Rangein-use [electric]=Rangecertification [electric] ×Battery CapacityEnergyin-use averageBattery CapacityEnergyStart

(35)

Where;

Rangecertification [electric] 	means the certified electric range (i.e. in charge-depleting mode of operation) in km or miles.

Rangein-use [electric] 	means the in-use electric range (i.e. in charge-depleting mode of operation) in km or miles, i.e. including accounting for the discrepancy and deterioration factors.

Battery CapacityEnergyStart 	means the net usable battery energy (in kWh) available at the start of the vehicle lifetime, defined as UBEcertified in UN GTR 22.

Battery CapacityEnergyin-use average 	means the average net usable battery energy (in kWh) available over the lifetime of the vehicle, which may be defined as the average of the energy available at the start of the vehicle lifetime, and the energy left at the end-of-life of the vehicle (i.e. according to the State of Certified Energy – SOCE), i.e. according to the following equation:

Battery CapacityEnergyvehicle EoL = SoHSOCEvehicle EoL×Battery CapacityEnergyStart	(36)

And Where;

Battery CapacityEnergyvehicle EoL 	means the average net usable battery energy (in kWh) available at the end of the vehicle lifetime.

SoHSOCEvehicle EoL 	means the average battery state of certified energy (SOCE) in % of the original energy at the end-of-life of the vehicle (or battery, whichever comes first).

The average available battery energy at end-of-life of the vehicle (or SoHSOCEvehicle EoL) may be defined according to alternative methodologies, as outlined below.  Where OEM-specific / supplier-specific calculated value for end-of-life available battery energy or SoHSOCEvehicle EoL is not available, the following formula may be used to estimate the SoHSOCEvehicle EoL, which assumes as an approximation a linear relationship for battery degradation based on the number of full charge/discharge cycles:

SoHSOCEvehicle EoL=1-(20% × Batterylifetime cyclesBatterycycle life )			(37)

Where;

Batterycycle life 		means the operational cycle lifetime (i.e. number of full charge/discharge cycles) of the battery to reach 80% SOCE. (Note: depending on the definition of, for example minimum battery durability requirements, in different regions for different vehicle types, the equation may be adjusted to account for the defined minimum SOCE for end-of-life use in a vehicle).

Batterylifetime cycles 	means the average number of full battery charge/discharge cycles over the use stage vehicle lifetime, which may be estimated according to the following formula:

Batterylifetime cycles =ECin-use CD × Service Life×UFcertificationBattery CapacityEnergyStart × (1+Charging Losses) 	(38)

Where;

Service Life 	means the number of km driven over the use stage lifetime.

Charging Losses 	means the charging losses included within the certified or in-use vehicle electricity consumption in charge-depleting mode of operation.

Other parameters are defined as earlier.

Prioritisation for average battery SOCE assumptions:

The following recommended prioritisation is proposed for the underlying assumptions of average battery SOCE, with the choice of which option is most appropriate or feasible left to the practitioner (i.e. depending on the availability of data and objective of the study).

The different options are listed in ascending order of accuracy and preference. The first option is proposed as a default approach, where sufficient information is not available for the other options.

Table 14
Prioritisation for average battery SOCE reduction assumptions

| Priority | Traction Battery Deterioration Factor |
| --- | --- |
| 1 | If OEM / supplier-specific data is not available, assume an operational cycle life of 2000 charge/discharge cycles hours to calculated the average SOCE according to the defined formula. |
| 2 | Optional, depending on availability OEM-specific / supplier-specific data on battery cycle life (i.e. to 80% loss in energy capacity*), according to regulatory durability testing requirements. |

Notes: * Depending on the definition of battery end-of-life in a vehicle in different regions for example via minimum battery durability requirements for different vehicle types, the SoH/SOCE cycle life definition may be adjusted. [For jurisdictions with standards defining a minimum SOCE at specific points of vehicle life,  the SOHvehicle EoL shall be consistent with the SOCE standards in that particular jurisdiction.  For example, the U.S. has standards requiring minimum SOCE of 80% @ 5 years or 62,000 miles and 70% @ 8 years or 100,000 miles. For that jurisdiction, vehicle scrappage at or below 5 years or 62,000 miles would require a minimum SOCE of 80%; and  vehicle scrappage at more than 5 years or 62,000 miles and less than 100,000 miles would require a minimum SOCE of 70%.]

6.3.3.3.	Application of the energy modelling schemes

For general rules, refer to paragraph 5.5.1.

6.3.3.3.1.	Application of the energy modelling schemes for dynamic scenario

If a dynamic scenario is used, the energy mix composition for each year of vehicle operation shall be estimated (i.e., the shares Si,n of energy supplied by each technology i in the year n), by applying linear interpolation between the respective energy supply shares reported for the nearest pre-defined time horizons in the scenario selected. The average representative grid mix composition over the full service life of the vehicle shall be calculated as follows:

(a)	As the arithmetic average of the individual electricity supply shares at point 2 above. Doing so entails the implicit simplifying assumption that the vehicle’s use is distributed homogenously over its full service life (i.e., L/N km are driven each of the N years of operation, where L = total lifetime activity).

(b)	Alternatively, if there is reason to expect that the vehicle’s use intensity will change over time, and if year-specific activities may be estimated with sufficient confidence, then a more refined (and accurate) modelling approach may be adopted, employing a weighted average (as opposed to a simple arithmetic average) of the individual shares Si,n of electricity supplied by each technology i in the year n, i.e.: n=1NWnSi,n , where Wn = An/L (An = vehicle activity in year n, L = total lifetime activity).

A bespoke grid mix model shall finally be built using the grid mix composition calculated above and leveraging the most up to date database processes available for the individual electricity generation technologies. The resulting grid mix thus modelled shall be used to estimate the Emission Factor of the electricity input to the use stage of the vehicle.

6.3.4.	           Quantification of Leakages

This section deals with GHG emissions arising from the fuel system, such as evaporation, hydrogen and methane leakages.

6.3.4.1	Evaporative emissions

Evaporative emissions occur in vehicles due to the nature of the fuel system and the volatility of petrol fuel. As part of the homologation process, vehicles are subjected to emissions testing to ensure compliance with regulations, including limits on evaporative emissions.

Limits of evaporative emission and duration of measurement (24h to 48h) varies between different regions of the world (worst case limit is 2g / 24h).

Given the very low impact on the entire use stage GHG emissions, this methodology  neglect the emissions arising from fuel evaporation.

6.3.4.2	Hydrogen leakage

Hydrogen emissions primarily occur during production and distribution, mainly due to fugitive leakage. To a lesser extent, emissions can also arise directly from hydrogen-fuelled vehicles, although standardised test methods to quantify these emissions are currently lacking. Hydrogen can also escape from combustion engines and storage systems, particularly in the case of liquefied hydrogen.

Recent research suggests that hydrogen leakage rates across the supply chain may be comparable to methane losses in the natural gas supply chain. Estimates indicate that green hydrogen supply chains could experience net leakage rates between 2.6% and 6.9% (Cooper, Dubey, Bakkaloglu, & Hawkes, 2022).

Hydrogen emissions are not commonly included in life cycle inventory (LCI) datasets, and a characterisation factor for hydrogen is currently absent from established impact assessment methods (e.g., the Environmental Footprint (EF) method), due to its omission from the explicit list of greenhouse gases in the latest IPCC Assessment Report. As such, it is recommended that the inclusion of hydrogen as a greenhouse gas in default LCA calculations be deferred until formal consensus is reached on its global warming potential (GWP) and/or its integration into the EF method.

In the interim, for Level 1 and Level 2, practitioners are required to account for hydrogen emissions as a mandatory flow indicator until a standardised GWP value is formally adopted. From both the supply chain and vehicle use, where available, practitioners should use official governmental estimates or supplier-specific data on typical fugitive hydrogen emissions. These should differentiate between various hydrogen production pathways, local versus imported hydrogen, and distinct hydrogen vehicle types. For Levels 3 and 4 LCA, practitioners may apply the methodology for Levels 1 and 2 or omit hydrogen leakage until future guideline revisions mandate inclusion, due to current data limitations and evolving measurement standards.

6.3.4.3	Methane leakage

Methane fugitive emissions from vehicle storage systems can significantly impact use-stage greenhouse gas inventories for certain vehicle using methane as a fuel. In the absence of harmonized measurement standards, practitioners should prioritize official governmental estimates of CH4 leakage when available. Supplementary data from OEMs or suppliers may be used to corroborate these estimates. For Level 1 and Level 2 assessments where official governmental data remains unavailable, validated peer-reviewed literature may constitute an acceptable alternative, provided they are appropriately documented.

6.3.4.4.	Quantification of fluorocarbons emissions

Refrigerant fluids have a direct environmental impact, primarily due to the use of hydrofluorocarbons (HFCs), a class of synthetic gases that replaced hydrochlorofluorocarbons (HCFCs). However, there is no standardized leakage measurement methods available; and refrigerant leakages are considered to be very low. For this reason, this recommended practices neglect the emissions arising from fluorocarbons.

On the other side, refrigerant emissions shall be included in the life cycle inventory as elementary flows pertaining to the maintenance and consumables area, as explained in section 6.3.5.

6.3.5.	Maintenance

Vehicle maintenance encompasses the periodic replacement of components and the replenishment or replacement of operational fluids necessary to ensure reliable and safe vehicle performance throughout its service life. Maintenance schedules may be defined based on elapsed time, distance travelled, predictive algorithms, periodic inspections, or more generally on wear, degradation, or depletion. Maintenance in the use stage of a vehicle's life cycle can have a non-negligible impact on its carbon footprint. Recent life cycle assessment studies estimate that the greenhouse gas emissions produced by vehicle maintenance during the use stage can have a significant impact on total vehicle carbon footprint, depending on the powertrain type and vehicle segment.

6.3.5.1.	Maintenance scope

Two types of maintenance are included in this methodology:

(a)	Consumables: These are materials, fluids, and components that are regularly used, replaced, or replenished during the operation and maintenance of a vehicle. These items typically have a limited lifespan and require periodic replacement to ensure the vehicle's proper functioning, safety, and performance. This includes items such as engine oil, transmission fluid, brake fluid, windshield washer fluid, filter, brake pads and discs, etc…  Carbon footprint related to material, production and EoL should be considered.

(b)	Maintenance parts: This refers to the replacement of parts due to normal wear and tear, including components that are not designed to last the entire lifespan of a vehicle, such as the brakes, clutch, tyres, starter battery etc...  The range of parts that need to be replaced during maintenance varies between different cars, depending on factors such as the driver's behaviour, road conditions and other variables (weather etc.). Hence, it is difficult for OEMs to recommend a frequency of exchange of these parts to the customer. The frequency of maintenance of such parts can only be estimated with acceptable probability through statistical analysis and a generic frequency of these parts shall be used for the purpose of carbon footprint estimation. For these parts recycling aspects must be considered and are same as that of original production parts.

Other repair or maintenance activities not included in the methodology are:

(a)	Vehicle accidents is not considered as a part of maintenance activities within the scope of this Resolution but may be included via future amendment if sufficient data is available.

(b)	Cleaning operations: Effect of car wash may not be a significant factor in the calculation as it is a very small fraction of the overall carbon footprint of a car. Hence, the Resolution does not address cleaning operations.

6.3.5.2.	Maintenance carbon emission estimation

Since the carbon footprint related to maintenance occurs after the vehicle is placed on the market, it can only be calculated based on the known frequency specified by the vehicle manufacturer.

To account for maintenance and consumables emissions, there are two possibilities (considered in a hierarchical order):

(a)	List of maintenance parts/consumable and associated frequency provided by the manufacturer.

(b)	List of maintenance parts/consumable not available

For Level 3 and Level 4,paragraph 6.3.5.2.1. shall be applied.

For Level 1 and Level 2, the approach for maintenance and consumables emissions estimation may instead alternatively follow paragraph 6.3.5.2.2. where a suitable methodology may be defined and documented by the practitioner (e.g. based on assumptions on the parts replaced and the number/frequency, or alternative methods).

6.3.5.2.1.	Maintenance Data Availability

If a list of maintenance parts/consumable and associated frequency is provided by the manufacturer or parts supplier, then the following estimation shall be used.

As this will reflect the effort from the manufacturer to reduce carbon footprint of maintenance, the manufacturer or part supplier should provide all the assumptions (list, frequency, carbon emission) used for carbon footprint estimation.

Cmaintenance= i=1nCEFi*fi.maintainence				(40)

Where;

i	means consumable/ maintenance parts

fi.maintainencemeans the maintenance frequency of the consumable/maintenance parts defined

CEFi  	means the carbon emission factor of the consumable/maintenance parts as used for calculation of upstream and end of life (waste or recycle) emissions [kgCO2e].

6.3.5.2.2.	Determination of list of consumables and parts

The following table provides a list of consumables and parts for guidance and should not be considered as exhaustive. The manufacturer should provide the list adapted to the powertrain and vehicle segment for which the carbon footprint is calculated. However, for Level 3 and Level 4, last column of the table indicates items responsible for the most significant potential impacts that shall be included if a replacement is needed (to be justified) in the considered lifetime of the vehicle.

Table 15
List of consumables and maintenance parts

|  |  | Petrol | Diesel | CNG | NOVC-HEV | OVC-HEV | Pure EV | FCHV | OVC-FCHV | H2-ICE | Level 3/ Level 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consumables | Engine coolant |  |  |  |  |  | N/A | N/A | N/A |  |  |
|  | Engine lubricant |  |  |  |  |  | N/A | N/A | N/A |  |  |
|  | Screen wash |  |  |  |  |  |  |  |  |  |  |
|  | Brake fluids |  |  |  |  |  |  |  |  |  |  |
|  | Engine air/oil filter |  |  |  |  |  | N/A | N/A | N/A |  |  |
|  | AdBlue/Urea/Reagent | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | YES |
| Maintenance parts | Passenger air filter |  |  |  |  |  |  |  |  |  |  |
|  | HVAC Refrigerants |  |  |  |  |  |  |  |  |  |  |
|  | Spark plug |  | - |  |  |  | N/A | N/A | N/A |  |  |
|  | Windshield wiper blades |  |  |  |  |  |  |  |  |  |  |
|  | Tyres |  |  |  |  |  |  |  |  |  | YES |
|  | Brake linings |  |  |  |  |  |  |  |  |  | YES |
|  | Brake discs |  |  |  |  |  |  |  |  |  |  |
|  | SLI battery (i.e. 12V) |  |  |  |  |  |  |  |  |  | YES |
|  | Aftertreatment |  |  |  |  |  | N/A | N/A | N/A |  |  |

The impacts resulting from different maintenance items can vary greatly, with some items being less significant and others usually accounting for a significant proportion of overall impacts. It is therefore proposed that the following items should be mandatorily included (where relevant for a given vehicle powertrain type):

(a)	Consumables: AdBlue/Urea, Refrigerants for HVAC (heating, ventilation and air conditioning) systems

(b)	Maintenance and wear parts: Tyres, SLI battery, brake pads, brake discs, additional auxiliary batteries (if relevant for commercial vehicles – e.g. for temperature controlled payload areas).

Refrigerant emissions shall be included if the refrigerant used has a GWP₁₀₀ value of 150 or higher. Practitioners may also choose to report refrigerant emissions with a GWP₁₀₀ below this threshold as non-exhaust emissions.

Additionally, this Resolution requires the assessment of key components for potential replacement, such as the internal combustion engine in conventional and/or hybrid vehicles, traction or storage battery in electric vehicles and the fuel cell stack in fuel cell electric vehicles in the case that the practitioners provide the justifications to be included.

For certain items, including mandatory items, replacements might not be needed in the vehicle’s typical operational lifetime based on the supplier assessment. In these cases, exclusions made on this basis should be justified.

6.3.5.2.3.	Determination of Frequency

For most maintenance items frequency can be simply calculated by the following two approach:

Frequency by distance

Frequencydistance= Service life [km]Maintainence interval [km]				(941)

Frequency by duration

Frequencyduration= Service life [years]Maintainence interval [years]				(42)

In case the manufacturer/supplier recommends frequency by both distance and duration, both calculations should be performed, and most conservative (i.e. the highest) frequency shall be considered for the carbon footprint estimation.

6.3.5.2.4.	Maintenance Data not available

[RESERVED]

6.3.6.	Methodology for potential replacement(s) of the major powertrain component such as internal combustion engine, traction battery and fuel cell system]

.

This Resolution mandates, as a default assumption, that the major powertrain components such as internal combustion engine, traction battery and fuel cell will not be replaced during the vehicle’s lifetime since they are generally designed to last for the entire lifetime of the vehicle without the need for replacement. However, in instances where the practitioner provides a detailed explanation for why it is included, their significant impact on life cycle assessments necessitates their inclusion in the analysis. This inclusion should be grounded in empirical data from field studies and/or official statements from manufacturers concerning replacement practices. This applies to situations such as niche applications, small battery capacities, low-durability chemistries, low mechanical components or other conditions identified by the practitioner. Such replacement should not be included in a manner that conflicts with minimum performance standards within jurisdictions having such requirements as a condition of granting vehicle certificate of conformity.

This Resolution provides the prioritised methodology approach for the replacement of traction battery/fuel cell system in Table 16. Other major powertrain components may refer to this approach.

Table 16?
Prioritised Methodology for traction battery/ fuel cell system replacement

| Priority |  | Traction Battery / fuel cell system Replacement |
| --- | --- | --- |
| 1 | Based on the justification provided by the practitioner including, but not limited to, a detailed explanation, their significant impact, empirical data from field studies, manufacturers official statements. |  |
| 2 | General methodology defined in paragraph 6.3.6.1. and 6.3.6.2. for traction battery and fuel cell system |  |
| 3 | OEM-specific / supplier-specific methodological/modelling approach to define the need for traction battery/fuel cell system replacement over the operational life of the vehicle. An explanation/justification for the result should be provided |  |

]

6.3.6.1.	General Methodology for calculating the number of traction battery replacement (s)

For vehicle traction batteries, the following approach for accounting for the frequency of energy storage replacement, which is based on a combination of parameters including the anticipated battery cycle life (i.e. number full charge/discharge cycles).  This methodology also provides a dynamic link to the vehicle battery capacity and the lifetime activity/service life (as defined in the study Goal & Scope), which can be used in product LCA, prospective LCA or other more generic studies.

The methodology for determining the number of traction battery replacements is as follows:

Number of traction battery replacement=roundup to integar  E Average×A LifetimeC Battery usable×CL Battery-1				(43)

Where;

N 	means the total number of traction batteries needed over the vehicle lifetime

C [Battery usable] 	means the usable (i.e. ‘net’) traction battery energy in kWh

CL [Battery] 	means the average battery cycle life – number of full charge/discharge cycles (within the usable capacity)

A [Lifetime] 	means the vehicle lifetime activity (in km). Note: As a sensitivity it is recommended to also explore the potential number of replacements needed based on the warrantied number of km for the battery (where this is present).

E [Average] 	means the vehicle average electrical energy consumption, in kWh per km

In the absence of OEM-specific data on the battery cycle life (parameter ‘CL’ above), then it is proposed to use a value of 2000 charge/discharge cycles, which is representative of a typical value for current technology.  Should the battery come with an expected calendar lifetime lower than the defined vehicle lifetime in the Goal & Scope of the LCA study, then a replacement will also be foreseen.]

6.3.6.2.	[General Methodology for calculating the number of fuel cell system replacement(s)]

[For fuel cell systems, the following approach is proposed to determine the need for one or more fuel cell replacements over the service lifetime of a vehicle, consistent with the similar methodology proposed for fuel cell efficiency degradation (see earlier paragraph 6.3.3.2.2.1)).

Fuel cell durability(/life) is defined as the number operational hours to reach 10% degradation of the original fuel cell rated power (in kW).

Fuel cell lifetime max energy output (= FC [max energy]):

Lifetime max energy output (kWh) = Fuel cell durability (hrs)*Fuel cell average running power (kW)				(44)

Where;

Fuel cell average running power (kW) 	means the maximum rated fuel cell power (kW) * average operation % of rated fuel cell power

Fuel cell vehicle lifetime energy requirement (energy output from fuel cell) (= FC [lifetime energy]):

Lifetime vehicle energy requirement fuel cell outputkWh=MJkm*Lifetime km*kWhMJ*fuel cell average efficiency (%)

(45)

Where;

MJ/km 	means the hydrogen energy input to the vehicle, i.e. hydrogen energy consumption.

The number of fuel cells needed (and therefore the number replacements) is defined by the ratio of the vehicle’s lifetime energy requirements and the maximum fuel cell lifetime energy delivered:

Number of fuel cell replacements =roundup to integer FC[lifetime energy] FCmax energy-1

(46)

]

6.3.7.	Data Collection and data type

Data shall be collected for the items outlined below. Where applicable, activity data shall be collected as primary data. In case of the difficulty, secondary data may be applied to activity data. GHG emission intensity data shall be collected as secondary data. Secondary data source shall be reported. The following data shall be collected:

Table 16
Processes and Data to be included in Use stage

| Processes | Activity data | GHG emissions Intensity Data |
| --- | --- | --- |
|  | (Primary Data basis) | (Secondary Data basis) |
| Driving | Certified Fuel consumption [l/100km] and/or electricity consumption [kWh/100km] |  |
|  |  | Discrepancy factor RW data |
|  |  | Deterioration factor |
|  | Separate Certified fuel/electricity consumption for the charge-depleting (CD) and charge-sustaining (CS) modes for OVC-HEVs |  |
|  | Utility Factor definition for dual-mode powertrains (e.g. OVC-HEVs) |  |
|  | Certified charge-depleting (CD) range [km] for dual-mode powertrains (e.g. OVC-HEVs) |  |
|  |  | Average lifetime Battery SoH loss for dual-mode powertrains (e.g. OVC-HEVs) |
|  |  | Average OVC-HEV, FCHV, PEV charging/discharging efficiency if not included in certification values |
|  | Vehicle lifetime (years), and activity (km) | Vehicle recycling statistics/ Nationally authorised statistics/OEM's average vehicle life |
|  |  | GHG emission factor for burning fuel |
|  |  | Vehicle occupancy rates for potential scenario analysis |
| Leakages | Hydrogen, Methane |  |
| Emitting Fluorocarbons | Out of scope | Out of scope |
| Maintenance and Consumables Production | List of maintenance parts and consumables |  |
|  | Frequency of replacement/service intervals |  |
|  |  | GHG emission factor for producing maintenance parts, in line with Section (6.2 Parts production and vehicle assembly stage) |
| Maintenance and Consumables Transportation | List of maintenance parts and consumables |  |
|  | Frequency of replacement/service intervals |  |
|  | Transport Weight [kg] Transport Distance [km] | GHG emission factor for transporting maintenance parts |
| Maintenance and Consumables End-of-Life | To be evaluated in line with Section (6.4. EoL) | To be evaluated in line with Section (6.4. EoL) |
| Vehicle activity out of region of sales | Out of scope | Out of scope |
| Second life of components | Out of scope | Out of scope |

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

Data for the UK from the Centre for Economic Performance (Nguyen-Tien et al. (2025) does not reflect a full vehicle lifetime, but instead estimates the average period that vehicles are used on UK roads.

Annual vehicle distance travelled and vehicle scrappage are functions through an analytical end date (e.g.,2050 for AEO 2025) and vary by vehicle type and by location (U.S. state)

U.S. EPA (2024). Population and Activity of Onroad Vehicles in MOVES5. EPA-420-R-24-019. https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=P101CUN7.pdf.

ACEA 2025 fact sheet reports the average European travels around 12,000 km per year.

While the term "deterioration factor" is commonly used for emissions certification, note that emissions deterioration may be expressed as a multiplicative term, an additive term, or a complex polynomial

https://eur-lex.europa.eu/eli/reg/2023/443/oj/eng

https://www.epa.gov/regulations-emissions-vehicles-and-engines/final-rule-multi-pollutant-emissions-standards-model

http://zxd.catarc.org.cn/zxd/portal/detail/zqyj/622

COMMISSION REGULATION (EU) 2017/1151

The Passenger Car (Fuel Consumption and CO2 Emissions Information) Regulations 2001

JPN to be checked

China to be checked

KR Notification on Test Methods for Energy Consumption Efficiency and GHG Emissions of Automobiles

U.S. Title 40 CFR Part 600. Fuel Economy and Greenhouse Gas Exhaust Emissions of Motor Vehicles. https://www.ecfr.gov/current/title-40/chapter-I/subchapter-Q/part-600.

Passenger Automobile and Light Truck Greenhouse Gas Emission Regulations

As also previously implemented in (Ricardo et al., 2020) based on consultation with stakeholders.

FCH 2 JU - MAWP Key Performance Indicators (KPIs) - European Commission (europa.eu)