# Submitted by the VIAQ Chair and Secretary

Working Paper VIAQ-07-10

(7th VIAQ meeting; January 11th, 2017)

# Report of the 7th Meeting of the

Informal Working Group on Vehicle Interior Air Quality (VIAQ)

Location:	UN, Geneva, Switzerland

Date:		9:30 to 12:30, January 11th, 2017

Chair:		Mr. Jongsoon Lim (Korea)

Co Chair:	Prof. Yunshan Ge (China)

Secretary:	Dr. Andreas Wehrmeier (OICA)

1. 	Welcome and opening remarks

The chair welcomed the participants of the 7th Meeting of the Informal Working Group on Vehicle Interior Air Quality. The agenda for the meeting (VIAQ-07-02) was reviewed and accepted by all participants.

2. 	Review the minutes of the 6th meeting

2.1 	Minutes of the 6th VIAQ meeting

The secretary summarized the meeting minutes of the 6th VIAQ meeting (VIAQ-06-13). The meeting minutes were accepted by the participants without corrections. Therefore all working items in status “provisional conclusion” can be set a status “closed”. Open issues see agenda item 3.

2.2 	Progress report

The chair gave a progress report. He reported that all working items except one have been closed and a draft document is created through the drafter group, being ready to submit to GRPE. (VIAQ-07-03).

3. 	Working Items (VIAQ-07-04)

The one remaining open working item on the vehicle family concept was discussed. The following proposal for a vehicle family concept was accepted by the group:

“The selection of vehicles should be based on a family concept to minimize testing cost. Therefore it is recommended to measure vehicles only in one colour. The colour should be dark in exterior and interior (preferably black) to have the highest solar loading in parking mode testing. Vehicles should be grouped according to similar interior material concepts. Same upholstery (leather or textile), same surface materials, often described by same interior lines are considered as similar interior concepts.”

Reviewing the draft document 4 change items are proposed by the drafter group and OICA:

- Give a specific set temperature for HVAC setting in driving mode and change from “23-25°C” to “23°C”. The temperature in the driving mode will then be going down from the resulting temperature of the parking mode (approx. 40°C) to the set temperature of 23°C.

- The chamber background concentration of measured substances is proposed to be <20µg/m³. With regard to low limit values of the Chinese legislation draft, it is requested to lower the allowed chamber background concentration for each measured substance to <5µg/m³.

- The door opening time to install the sampling equipment inside the cabin is set to 60min. The assembling procedure maybe be done faster. In order to save time for whole vehicle analysis it should be allowed to start soak time directly after installing the equipment. Therefore it is proposed to change the door opening time to 30-60min.

- OICA proposed a data reporting format (Annex 4 of draft document). The format should be checked on amended, if necessary, by all parties.

- To do: The members are requested to review the 4 proposals until the 8th meeting.

4. Draft document

4.1 	Review of the draft document for Recommendation

The secretary presented the updated draft document, which includes the decisions of the working item list and was updated in the drafter group meeting on 04.01.2017 (VIAQ-07-06 Version V72). It was amended in Word change mode with the decisions of the group and comments by Korea in the 7th meeting (VIAQ-07-06 Version V73). A clean version without change documentation (GRPE 74-0x) will be submitted to the GRPE secretary by the chair as an official proposal for a new mutual resolution.

- To do: The members are requested to carefully review the new proposal for a mutual resolution until the 8th meeting.

5. Roadmap

The Roadmap is shown by the chair in his progress report (VIAQ-07-03). The next meeting should be scheduled in April. The draft is to be finished in June 17 session and the mutual resolution on VIAQ should be finished in the November 17 session.

6. Proposed agenda/topics for the next VIAQ IWG meeting

6.1 The second stage of VIAQ mandate

Russia made a proposal for the second stage of the VIAQ mandate. Russia proposed to include a measurement method for exhaust gas entrance into the cabin air to include hazardous substances like CO and NOx. It was stated by OICA that usually CO and NOx entry in vehicle cabin is low. However the toxicity of these substances is high. Therefore OICA, Russia and Korea offered support in a knowledge exchange.

- To do: Russia to draft a Terms of Reference document (ToR) for the 8th meeting.

7. Any other business

7.1 Information exchange concerning VIAQ standards

Prof. Ge on behalf of China presented the current status of the China legislation on VIAQ (VIAQ-07-09). A new draft is proposed for mid-2017. The measurement method will be similar to ambient mode of the draft UN mutual resolution. Prof Ge said that differences in preconditioning temperature will be adapted. Ambient mode test temperature will 25°C +/-1 and therefore remain slightly different from UN mutual resolution.

7.2 Introduction of the research concerning exhaust gas from the vehicle entering cabin Mr. Lim on behalf of Korea presented their research on exhaust gas entering the vehicle cabin air. They found CO entry to the cabin at elevated speed (160 km/h) and when opening the window (VIAQ-07-08). It has been discussed if measured values can cause harm to drivers or passengers. It was concluded that because of undefined limit values for short time exposure of CO under the special situation in a driving car no judgement is possible today.

8. Closing remarks and next meeting schedule

The chair thanked all presenters for their contributions and the participants for their comments.

The 8th meeting is proposed for:

- April 2017, Russia

- To do: Russia to check if meeting in Russia is possible