IWG-DPPS-5bis/skype-28Nov2019

UNECE GRSP Informal Working Group 
on Deployable Pedestrian Protection Systems 
(IWG-DPPS-5bis)

Intermediate Meeting Draft Minutes, 28th Nov 2019, Skype

- Review and approval of the agenda (DPPS-05bis-01, Chair/secretary): 
The agenda was approved.

- ACEA Study Upper Leg Mass Reduction: Dr. Pauer

- HIT calculation - comments from JASIC & IDIADA

- First Comments on the Draft Amendment: structure decision.

And following presentations were discussed:

Fichier PDF IWG_DPPS-6- xx -feedback_IDIADA_linear_regression.pdf (Spain) IDIADA -HIT calculation feedback

Fichier PDF IWG-DPPS_Difference_of_HIT_by_Calculation_Method_rev1_JP.pdf (JASIC) HIT calculation feedback

Présentation Microsoft Powerpoint IWG-DPPS_ULF-Mass-Reduction-Issues_Altran-Concept.pptx (Altran) ULF study 2013

Fichier PDF Upper-Leg_Energy-Equivalence-Study_Concept_2013-04-10.pdf (Altran) ULF study 2013-conclusion for detection

Presents:

+  Apologies: MM. Park, Klöckner.

- ACEA Study Upper Leg Mass Reduction: Dr. G. Pauer

Following the presentation, O. Zander and Tanaka-san ask about a possible necessary validation also for the (normal or lighter weight) ULF for contact, not internal criteria.

GP confirms that it would need a specific validation (also the FEM, not only the physical tool). OZ reminds that 1 option was the ULF light weight, which will need to differentiate validation for:

- Biomechanical criteria

- Detection signal

And to compare with in use Flex-Pli current validated signal.

GP: How to check this ? OZ: Compare HBM & leg impactor signals

GP states that Confor foam properties are very “velocity dependent”: difficult. Secondly, special conditions depending of the vehicle shape, impact velocity…should be tested (Different impact speeds & different heights):

OZ: if ULF is chosen, then an alternative for the confor foam would be needed.

GP: confirms that it is always a challenge because of the non-linear properties.

BB reminds the comparison table from IWG-DPPS-5-06:

- Another point for contact detection: could we use a non-instrumented flex-Pli ? Or the standard Flex-PLI.

- OZ asks Tanaka-san for some clarifications: when looking at slides 8-10 : HBM simulations : Flex-pli simulations were at identical Impact heights ? Against small sedans (only 1 model ? ). Tanaka-san: yes, correct: Will present next meeting.

- OZ: normal shooting (slide 5); what about the PDI2 : propelled or impacted by the car? Yanaoka-san confirms that the PDI2 was not propelled, but set on the ground, and car was propelled.

OZ: General point: how to go forward for Pre-decision of the detection leg impactor:

- PDI2 is out of discussion

- TRL still in discussion ? OZ would have it out of discussion, but needs to have IG opinion.

  - GP: Flex-PLI was better (signal answers)

  - BB: add it in the matrix for capitalization purposes (full picture on all impactors & clarify decision)

- Flex-PLI ! Seems ok

- ULF : would need more validation and modifications, which are out of time scope

- Matrix + - for decision: Ben + OZ for getting the advantages/cons

ID: will circulate it to IWG to get comments and make a final form

=> for decision in March DPPS meeting.

- HIT calculation - comments from JASIC & IDIADA

IDIADA

JASIC

Decision:

Create a Pro-cons matrix decision for March Meeting: - Including all special cases (6yo ot 95th head who don't touch the bonnet) - All shapes of vehicles

Comments following the presentations:

-method 1:  what if 50th  ; 95th  on the windscreen : so no point; but need some point in-between on the bonnet

-sometimes the 6yo does not hit the bonnet; 1st HIT is in-between 6yo & 5th ; => regression method 2 is as in Euro NCAP. OZ: right, but also method 2 depends on 6yo & 95th , when not on bonnet, then method 1 ?

- Corina: at least 2 points left are needed; use only the impact points which are on the bonnet; (interpolation of bonnet stature hit points)

- Yanaoka-san: how to identify HIT & WAD for outside of head area ? ; if only the points on the bonnet; => extrapolation of the line?; => method 1 ? ; easy to eliminate not necessary points, w/o influencing the regression line. (w 2 points or 4 points => difference)

- Corina: only connect the points in M1; while in M2 you draw a regression line until where the stature hits the bonnet.

-Ben: what is the likely effect on car 2 or 4: no last point there.

-OZ: discuss in next meeting for all shapes of cars

-Ben: direct connection between statures ? tbc other scientific data

-Corina: create other statures by scaling (?) for Generic Veh model…; linear trend

-Yanaoka-san: what about real cars ? Scientific question: why regression ? evidence needed

=> OZ : make a HIT matrix decision proposal for March DPPS meeting

=> OICA to compare all fleet cases wrt different shown calculation HIT methods

- Draft structure : agreed on the Japan proposal (based on TF-DPPS-4-06)

Skype meeting: with all CP, Tech Services: OZ, BH, RDW, MW, OICA (BB, GM, Y-san, EB), Tanaka-san,

- Detection group (sensing): EB, TG, DUG, Aurore Loisy

- Simulation group: FR, Corina

- Verification leg impactor: matrix: group

- HIT calculation group: (regression, or…):

- Marking group: GM, FR.