|  | United Nations | ECE/TRANS/WP.29/GRSP/2022/xx |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 9 September 2022 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-second session

Geneva, 5-9 December 2022

Item 13 of the provisional agenda

Regulation No. 145 (ISOFIX anchorage systems, ISOFIX top tether anchorages and i-Size seating positions)

Proposal for supplement 3 to the 00 series of amendments to UN Regulation No. 145 (ISOFIX anchorage systems, ISOFIX top tether anchorages and i-Size seating positions)

Submitted by the expert from the Netherlands *

The text reproduced below was prepared by the expert from the Netherlands, aiming to introduce a definition, requirements and a test procedure for lower tether anchorages.

The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

- Proposal

Paragraph 1, amend to read:

“1.		Scope

This Regulation applies to:

(a)	Vehicles of category M1 with regard to their ISOFIX anchorage systems,  and their ISOFIX top tether anchorages and lower tether anchorages intended for child restraint systems. Other categories of vehicles fitted with ISOFIX anchorages and/or lower tether anchorages have also to comply with the provisions of this Regulation;

(b)	Vehicles of any category with regard to their i-Size seating positions, if any are defined by the vehicle manufacturer.”

Insert new paragraphs 2.25. to 2.29., to read:

"2.25.	"lower tether anchorage (LTA)”: anchorage on the vehicle seat track or on or close to the vehicle floor to which a lower tether can be attached

2.26.	“lower tether”: type of anti-rotational device intended to restrict the rearward rotation of a rearward-facing ECRS

2.27.	“lower tether strap”: a webbing strap (or equivalent) which extends from the  back of an Enhanced Specific Vehicle Child Restraint System to the lower tether anchorage in the vehicle and which is equipped with an adjustment device, a tensioning-relieving device, and a lower tether connector.

2.28.	"lower tether connector" means a device intended to be attached to a lower tether anchorage.

2.29.	"lower tether hook" means a connector typically used to attach a lower tether strap to a lower tether anchorage and which is the same and has the same dimensions as the ISOFIX top tether hook as defined in figure 3 of Annex 4 of this Regulation.”

Paragraph 3, amend to read:

“3.		Application for approval

3.1.	The application for approval of a vehicle type with regard, the ISOFIX anchorages systems, the ISOFIX top tether anchorages, the lower tether anchorages and i-Size seating positions, if any, shall be submitted by the vehicle manufacturer or by his duly accredited representative.

3.2.	It shall be accompanied by the under mentioned documents in triplicate and by the following particulars:

3.2.1.	Drawings of the general vehicle structure on an appropriate scale, showing the positions of the ISOFIX anchorage systems, of ISOFIX top tether anchorages and/or lower tether anchorages if any and in case of i-Size seating positions or applications of a support leg in combination with non-ISOFIX/i-Size seating positions, the vehicle floor contact surface and detailed drawings of the ISOFIX anchorages systems if any, of the ISOFIX top tether anchorages and/or lower tether anchorages if any, and of the points to which they are attached and in case of i-Size seating positions or applications of a support leg in combination with non-ISOFIX/i-Size seating positions, the vehicle floor contact surface;

3.2.2.	A specification of the materials used which may affect the strength of the ISOFIX anchorages systems, and ISOFIX top tether anchorages and/or lower tether anchorages if any and in case of i-Size seating positions or applications of a support leg in combination with non-ISOFIX/i-Size seating positions, the vehicle floor contact surface;

3.2.3.	A technical description of the ISOFIX anchorages systems, and ISOFIX top tether anchorages  and/or lower tether anchorages if any;

3.2.4.	In the case of the ISOFIX anchorages systems and of ISOFIX top tether anchorages and/or lower tether anchorages if any affixed to the seat structure:

3.2.4.1.	Detailed description of the vehicle type with regard to the design of the seats, of the seat anchorages and of their adjustment and locking systems;

3.2.4.2.	Drawings, on an appropriate scale and in sufficient detail, of the seats, of their anchorage to the vehicle, and of their adjustment and locking systems.

3.3.	At the option of the manufacturer, a vehicle representative of the vehicle type to be approved by the technical service conducting approval tests, or the parts of the vehicle considered essential for the ISOFIX anchorages systems, for ISOFIX top tether anchorages and/or lower tether anchorages if any, and in case of i-Size seating positions or applications of a support leg in combination with non-ISOFIX/i-Size seating positions, the vehicle floor contact surface test, shall be submitted to the technical service.”

Insert new paragraph 5.2.4.4., to read:

“5.2.4.4. 	A vehicle manufacturer may declare a non-ISOFIX/i-Size seating position to be suitable for the installation of an ECRS using a support leg.

5.2.4.4.1.	Geometrical requirements for non-ISOFIX/i-Size seating positions suitable for use of ECRS with support leg.

In addition to the requirements defined in 5.2.2. and 5.2.3. it shall be verified that the upper surface of the vehicle floor (incl. trim, carpet, foam, etc.) intersects with both of the limiting surfaces in the x- and y-directions of the support leg foot assessment volume, as shown in figures 1 and 2 of Annex 5 to this Regulation, with the Child Restraint Fixture installed as described in Annex 6.

The support leg foot assessment volume is characterized as follows (see also Annex 5, Figures 1 and 2 of this Regulation):

(a)	In width, by the two planes parallel to and 100 mm apart from the median longitudinal plane of the child restraint fixture installed in the respective seating position; and

(b)	In length, by the two planes perpendicular to the plane given by the child restraint fixture bottom surface and perpendicular to the median longitudinal plane of the child restraint fixture, 585 mm and 695 mm apart from the plane passing through the centerlines of the ISOFIX lower anchorages and being perpendicular to the CRF bottom surface; and

(c)	In height, by two planes which are parallel to and 270 mm and 525 mm below the child restraint bottom surface.

The pitch angle used for the geometrical assessment above shall be measured as in paragraph 5.2.2.4.

Compliance with this requirement may be proven by a physical test or computer simulation or representative drawings.

5.2.4.4.2.	Vehicle floor strength requirements for non-ISOFIX/i-Size seating positions.

The entire vehicle floor contact surface (see Annex 5, Figures 1 and 2) shall be of sufficient strength to withstand the loads imposed when tested in accordance with paragraph 6.2.4.6. This test may be waived in agreement with the Technical Service and Type Approval Authority in case the vehicle floor contact surface strength is considered to be uncritical and does not constitute the boundary with luggage compartment areas and/or the vehicle’s REESS.”

Insert new paragraph 5.4. to read:

“5.4	Lower Tether Anchorages (LTA)

5.4.1.	Applicability

5.4.1.1.	A vehicle type may optionally be equipped with lower tether anchorages meeting the requirements of this Regulation.

5.4.2.	Number and Positioning

5.4.2.1.	Each intended ECRS position may be provided with a single or 2 LTAs, located within the zones shown in figures 5 to 7 of Annex 5 and according to the following specifications.

5.4.2.1.1.	For ISOFIX seating positions, a reference point for measurements is obtained by installing the ISO/R2 fixture in accordance with UN Regulation No. 16, in the vehicle seat. If adjustable, the seat shall be adjusted to its rearmost position.

5.4.2.1.2.	For non-ISOFIX seating positions, the reference point for measurements shall be obtained by installing the ISO/R2 fixture in the vehicle seat, using the envelope positioning procedure in Annex 6. If adjustable, the seat shall be adjusted to its rearmost position.

5.4.2.1.3.	If two lower tether anchorages are chosen, they shall be positioned with a minimum distance of 280 mm for the respective ECRS position.

5.4.2.1.4.	The two anchorages do not need to be symmetrically positioned with respect to the centreline through the fixture. However,

5.4.2.1.4.1.	anchorages shall not be positioned on the same side of the fixture centreline, and

5.4.2.1.4.2.	the offset between the centreline of the installed anchorages and of the fixture should be no more than 200 mm measured perpendicularly from the centrelines.

5.4.2.1.5	If a single lower tether anchorage is chosen for the seating position, it shall be symmetrically positioned with respect to the centreline through the fixture within an offset of ± 50 mm.

5.4.2.2.	Lower tether anchorages may be placed under the vehicle floor, for example in a storage compartment under a floor cover, if such a positioning can be regarded as acceptable from a usability perspective, in agreement with the Technical Service and Type Approval Authority.

5.4.2.3.	The zone intended for a support leg as defined in paragraph 5.2.4.2. and 5.2.4.4. (the support leg foot assessment volume) may not be used for lower tether anchorages.

5.4.2.4.	Rerouting of the lower tether strap path is allowed, although it should be ensured that it does not affect the function of the lower tethers or sensitive parts of the vehicle interior.

5.4.3.	Design

5.4.3.1.	The lower tether anchorage shall have an opening (square, circular, or semi-circular) and clearance space to allow attachment with a standard lower tether connector, in accordance with Annex 4 figure 3.

5.4.3.2.	For anchorages designed to be used for two adjacent CRS positions, the opening and clearance space shall allow for simultaneous attachment with two standard lower tether connectors.

5.4.3.3.	LTA shall be designed to avoid inadvertent injuries to passengers, for example designed to minimise sharp edges and protrusions. The minimum radius of curvature shall be 3.2 mm.”

Paragraph 6.2.4.3., amend to read:

“6.2.4.3.	Tests of ISOFIX anchorages system only:

6.2.4.3.1.	Forward direction force test:

Horizontal longitudinal excursion (after pre-load) of point X of SFAD during application of the 8 kN ± 0.25 kN force shall be limited to 125 mm and permanent deformation including partial rupture or breakage of any ISOFIX low anchorage or surrounding area shall not constitute failure if the required force is sustained for the specified time and the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.

6.2.4.3.2.	Oblique direction force test:

Excursion in the direction of the force (after pre-load) of point X of SFAD during application of the 5 kN  0.25 kN force shall be limited to 125 mm and permanent deformation including partial rupture or breakage of any ISOFIX low anchorage or surrounding area shall not constitute failure if the required force is sustained for the specified time and the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.

6.2.4.4.	Test of ISOFIX anchorages systems and ISOFIX top tether anchorage:

A tension pre-load of 50 N ± 5 N must be applied between the SFAD and the top-tether anchorage. Horizontal excursion (after pre-load) of point X during application of the 8 kN ± 0.25 kN force shall be limited to 125 mm and permanent deformation including partial rupture or breakage of any ISOFIX low anchorage and top tether anchorage, or surrounding area shall not constitute failure if the required force is sustained for the specified time and the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.

6.2.4.5.	Test for i-Size seating positions:

In addition to the tests specified in paragraphs 6.2.4.3. and 6.2.4.4., a test with a modified static force application device, which consists of a SFAD and includes a support leg test probe as defined in Figure 3 of Annex 5, shall be performed. The support leg test device shall be adjusted in length and width to assess the vehicle floor contact surface, as defined in paragraph 5.2.4.2. (see also Figures 1 and 2 of Annex 5 to this Regulation). The height of the support leg test device shall be adjusted in a way that the foot of the support leg test device is in contact with the upper surface of the vehicle floor. In case of incremental height adjustment, the first notch where the foot rests stable on the floor shall be chosen; in case of a non-incremental/continuous adjustment of the support leg test device height, the pitch angle of the SFAD shall be increased by 1.5 +/- 0.5 degrees due to the height adjustment of the support leg test device.

The horizontal excursion (after pre-load) of point X of the SFAD during application of the 8 kN ± 0.25 kN force shall be limited to 125 mm and permanent deformation including partial rupture or breakage of any ISOFIX low anchorage and the vehicle floor contact surface, or surrounding area shall not constitute failure if the required force is sustained for the specified time and the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.”

Insert new paragraph 6.2.4.6., to read:

“6.2.4.6.	Test for support leg for non-ISOFIX/i-Size seating positions

6.2.4.6.1.	A static load of [5,000 N ± 100 N] is to be applied in vertical direction to the vehicle floor contact surface using a device with the dimensions of the support leg foot as shown in Annex 5 figure 3. The exact location of the application of the load is determined in agreement with the Technical Service.

6.2.4.6.2.	A pre-load of 50 N ± 5 N must be applied first. Vertical excursion (after pre-load) during application of the [5,000 N ± 100 N] force shall be limited to [30 mm] and permanent deformation including partial rupture or breakage of any ISOFIX low anchorage and top tether anchorage, or surrounding area shall not constitute failure if the required force is sustained for a minimum of 0.2s and the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.”

Insert new paragraph 6.3., to read:

“6.3.	Lower tether anchorage strength requirements

6.3.1.	In case the lower tether anchorages are part of, or attached to, a seat which has been type approved according to paragraph 6.4.4. of UN Regulation No. 14, the test may be performed on a component level (e.g. seat rail)

6.3.2. 	During the strength test, all relevant interior components (e.g. seat in front of CRS position) shall be mounted. If not otherwise recommended by the vehicle manufacturer, adjustable seats should be positioned as follows:

6.3.2.1.	longitudinally, to the closest position of mid-position between the rearmost position and the foremost position;

6.3.2.2.	vertically, to the closest position of the mid position of its height adjustment.

6.3.3.	Apply a force of 2,500 N to each lower tether anchorage, by means of a representative lower tether strap 38 mm ± 3 mm wide that is fitted at one end with suitable hardware for applying the force and at the other end with a bracket for the attachment to the lower tether anchorage. 
For anchorages designed to be used for two adjacent CRS positions, or in case of a single LTA, the force shall be 5,000 N.

6.3.4.	The force:

6.3.4.1.	shall be applied in a direction determined by the FDRP (force direction reference point), with a tolerance of ± 20 mm in all directions given in figure 8 of Annex 5, noting that the FDRP lateral position coincides with the centreline of the ISO/R2 envelope,

6.3.4.2.	shall be attained within 30 s, and shall be maintained for a minimum of 0.2 s.

6.3.5.	When testing in accordance with paragraph 6.3.3. and 6.3.4., excursion is not limited, and permanent deformation of the lower tether anchorage with respect to the vehicle is acceptable provided that the anchorage does not break or separate from the vehicle.”

In Annex 5, insert new figures 5 – 8 to read:

# “Figure 5  
Lower tether anchorage zone for rear seat row(s), side view

Key:

1.	vehicle seat illustration (rear row seating positions).

2.	ISO/R2 envelope installed in related vehicle seating position.

3.	reference point on ISO/R2 envelope for LTA zone measurements.

4.	lower tether anchorage zone in side view, with upper limitation.

Dimensions in mm.

Note: 	Drawing not to scale.

# Figure 6
Lower tether anchorage zone for front passenger seat, side view

Key:

1.	vehicle seat illustration (front seating positions).

2.	ISO/R2 envelope installed in related vehicle seating position.

3.	reference point on ISO/R2 envelope for LTA zone measurements.

4.	lower tether anchorage zone in side view, with upper limitation.

Dimensions in mm.

Note: 	Drawing not to scale.

Figure 7

Lower tether anchorage zone for rear and front passenger seating positions, top view

Key:

- lower tether anchorage zone, top view

- support leg zone (not to be used)

- ISO/R2 envelope installed in related vehicle seating position

- centreline of ISO/R2 and ISOFIX anchorage positions (or the apparent centreline of the seating position in case of non-ISOFIX/i-Size seating positions, according to Annex 6) (contour only shown)

- lower tether anchorage, example positions

NOTE 	The rearmost surface of the zone is positioned 100 mm rear of a vertical plane through the reference point on the ISO/R2 envelope. See Figures 5 and 6.”

Figure 8

FDRP, Force Direction Reference Point, and test force direction

Key:

- vehicle seat illustration

- ISO/R2 envelope installed in related vehicle seating position

- FDRP (force direction reference point)

- example of actual LTA position in vehicle

- test force direction

NOTE	The FDRP lateral position coincides with the centreline of the ISO/R2 envelope”

Insert new Annex 6, to read:

“Annex 6

ISO/R2 positioning procedure for non-ISOFIX/i-Size seat position

In case of a non-ISOFIX/i-Size vehicle seat position the following procedure should be applied.

1) 	Place a cotton cloth on the vehicle seatback and cushion.

2) 	Remove the ISOFIX attachments from the child restraint fixture (CRF) or retract them to a position fully inside the backseat line.

3) 	Place the CRF on the vehicle seat.

4) 	Ensure that the CRF is located with its centreline on the apparent centreline of the seating position, ± 25 mm, with its centreline parallel with the centreline of the vehicle.

5) 	Push rearwards on the centre of the front of the CRF with a force of 100 N ± 10 N, applied parallel to the lower surface, and remove the force.

6) 	Push vertically downwards on the centre of the upper surface of the CRF with a force of 100 N ± 10 N and remove the force.”

II.		Justification

- this proposal aims at introducing: 
- definitions and requirements for lower tether anchorages
- only provide for a situation that consists of brackets in the vehicle in combination with connectors, similar to ISOFIX top-tether connectors in order to get rid of constructions whereby straps around vehicle seats, straps around seat rails etc. are used. See GRSP-71-19 for background.
- the possibility to approve the vehicle floor contact surface for applications of ECRS using a support leg in combination with non-ISOFIX/i-Size seating positions, in relation to the strength of the floor and the space for positioning the foot. For this situation and for deformations of the ISOFIX anchorages in general, it shall be verified that the deformation does not constitute an additional risk to vulnerable vehicle parts in agreement with the Technical Service and Type Approval Authority.

- Requirements on lower tether anchorages are based upon ISO 13216-4:2020

- Radius requirements added for the lower anchorages, independent of the vehicle zone in which they are mounted (i.e. 5.2 or 5.3 area of UN R21) in order to provide minimum protection for passenger feet in case of contact with these brackets.

- Strength test on the lower tether anchorages may be performed on a component level, for the situation that seat belts are integrated with the seat and have been tested according to UN Regulation No. 14 including the additional inertia load of 20x the seat mass (in case of M1/N1).

*	In accordance with the programme of work of the Inland Transport Committee for 2020 as outlined in proposed programme budget for 2020 (A/74/6 (part V sect. 20) para 20.37), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.